| Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 1st Meeting: Stockholm, SE, 16–20 July 2012 | Document: JCT2-A0056 m25933 |
| --- | --- |

| Title: | 3D-CE1.a related: improvement on NTT’s synthesis with fractional disparity |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yin Zhao, Yichen Zhang and Lu Yu #38 Zheda Road Zhejiang University Hangzhou, China | Tel: Email: | +86-571-87953107 yul@zju.edu.cn |
| Source: | Zhejiang University |  |  |

_____________________________

# Abstract

This document presents a method to derive more accurate fractional-pel disparities for synthesized pixels at depth edges, compared with NTT’s method in M24796 (simplification of in-loop view synthesis with fractional-pel disparity). Two neighbouring pixels, A and B, are warped to quarter-pel sample raster of the target view, and the integer pixels, Ci (0≤i≤N), between the two warped pixels are filled as follows:

1) if A and B present a weak depth edge, it is assumed that the sub-pel depth between A and B changes linearly, and Ci are filled by backward warping;

2) if A and B present a strong depth edge and result in a dis-occlusion region in the target view, Ci are filled by A or B, based on the likelihood of Ci belonging to the foreground or the background.

The proposed method also accomplishes the functionality of pixel splatting at depth boundaries, and obtains more accurate pixel values than the current boundary-aware splatting in ATM4.

This method is reported to bring 0.38% BD-rate saving (in terms of coded texture PSNR and coded texture rate) compared with ATM4, and consumes almost the same decoding time as ATM4 on average.

# Introduction

In m24794, NTT proposed integer-pel-based view synthesis with fractional-pixel disparity compensation, as shown in Fig. 1. Pixel A has a disparity dv = dvi + dvf where dvi and dvf are the integer and fractional parts of the disparity, and accordingly A is projected to pA in the synthesized picture. Then, a sub-pel sA with distance dvf to pixel A on the reference picture is warped to the target integer position C, which aims to compensate the disparity more accurately than just assigning A to C in integer-precision rendering.

Fig. 1: NTT’s warping process.

However, NTT’s method assumes the depth values of A and its neighboring pixel B are the same, which works well for most of the regions since depth maps are mostly plain. When there is a depth change between A and B, the assumption fails and the compensated fractional-pel disparity may not be accurate enough.

# The Proposed Method

2.1 Method description

Figure 2(a) shows an example when A and B have slightly different disparities (e.g., depth of B is smaller than A), which usually belongs to a transitional depth plane (e.g., a slope). In this case, NTT’s method uses A2 to fill C, and leaves pixel D as a hole which will be filled by E in hole filling. In other words, E and D are filled with B1, and C is filled with A2.

Differently, we assume the depth of the sub-pels between A and B changes linearly, and thus the depth between pA and pB is also linear. Using backward warping, we can find the warped position P’ of each integer pixel P between pA and pB in the reference picture, and select one sub-pel or integer-pel pixel (i.e., one from A0 to A4 in Fig. 2(b)) which is closest to P’ to fill the integer pixel P. In such a way, we can improve the geometry accuracy of the target pixels in the synthesized view, and the synthesized region is smoother.

In the linear depth model, the above-mentioned backward warping scheme with sub-pixel rounding can be simplified as

where Px is the distance between P and pB, and Py is that between P and pA, as shown in Fig. 2(b). For instance, D in Fig. 2(b) is filled with A3. The pixel mapping function can be realized by a look-up table.

(a) NTT’s method for pixels with small depth change (depth B< depth A)

(b) the proposal method for pixels with small depth change (depth B< depth A)

Fig. 2. Comparison between NTT’s and the proposed method when the plain depth assumption is not satisfied.

When depth difference between pixels A and B is larger than a threshold (corresponding to 3-pixel disparity in the current implementation), the depth discontinuity is considered appearing at an object boundary, and the depth edge is assumed at the middle of A and B. For such a region, there will be multiple integer pixels Ci (i≤n) between pA and pB, as shown in Fig. 3. We use the following method to derive their pixel values:

1) for the pixel C0 which is closet to pA among Ci, if pA is close to pixel C0 (i.e., fractional-pel of pA is no more than 1/4 pixel), then C0 is assumed to belong to foreground and is assigned with pixel A; otherwise, C0 is considered as the background and is assigned with pixel B;

2) the rest of the pixels Ci (i>0) are regarded as background pixels, and are filled by pixel B.

Fig. 3 The proposed method to handle pixel warping at an object boundary.

It is clear that all pixels between pA and pB are filled if the proposed method is applied at depth edges, and thus hole filling is not needed for these regions.

2.2 Compared with boundary-aware splatting

The boundary-aware splatting allows an integer pixel around object boundary being warped to two neighbouring pixels in the target view, which aims to reduce cracks at object boundaries where depth are not plain. For example, B in Fig. 2(b) is warped to both E and D if splatting is used. The proposed method accomplishes the same functionality, and derives more accurate pixel values by using sub-pel (or integer) pixels around the integer pixel, e.g., B1 for E and A3 for D in Fig. 2(b). Therefore, the proposed method also can replace the current boundary-aware splatting, and thus splatting is turned off.

# Experimental results

3.1 Performance of the proposed method

The proposed method contributes to 0.38% bitrate saving (in terms of texture PSNR and texture rate) and the almost same decoding complexity as ATM4, as listed in Table I. Since splatting is replaced by the proposed method, we also provide the performance of ATM4 with splatting off, as shown in Table II. Compared with ATM4 with splatting off, the proposed method brings around 0.1% rate saving.

Table I RD performance of the proposal method vs. ATM4

|  |  | Texture Coding |  | Depth Coding |  | Total (Coded PSNR) |  | Total (Synthesed PSNR) |  | Complexity |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | dBR, % | dPSNR,dB | dBR, % | dPSNR,dB | dBR, % | dPSNR,dB | dBR, % | dPSNR,dB | Decoder Time, % |
| Hall2 | S01 | -0.52 | 0.01 | 0.00 | 0.00 | -0.47 | 0.01 | -0.50 | 0.01 | 0.995 |
| Street | S02 | -0.02 | 0.00 | 0.00 | 0.00 | -0.02 | 0.00 | 0.02 | 0.00 | 0.990 |
| Dancer | S03 | -1.00 | 0.03 | 0.00 | 0.00 | -0.93 | 0.03 | -0.70 | 0.02 | 1.038 |
| GT | S04 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.03 | 0.00 | 0.963 |
| Kendo | S05 | -0.37 | 0.02 | 0.00 | 0.00 | -0.29 | 0.01 | -0.30 | 0.01 | 0.970 |
| Balloons | S06 | -0.41 | 0.02 | 0.00 | 0.00 | -0.35 | 0.02 | -0.32 | 0.01 | 1.064 |
| News | S08 | -0.33 | 0.01 | 0.00 | 0.00 | -0.27 | 0.01 | -0.23 | 0.01 | 0.992 |
|  | Average | -0.38 | 0.01 | 0.00 | 0.00 | -0.33 | 0.01 | -0.29 | 0.01 | 1.002 |

Table II RD performance of ATM4 with splatting off vs. ATM4

|  |  | Texture Coding |  | Depth Coding |  | Total (Coded PSNR) |  | Total (Synthesed PSNR) |  | Complexity |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | dBR, % | dPSNR,dB | dBR, % | dPSNR,dB | dBR, % | dPSNR,dB | dBR, % | dPSNR,dB | Decoder Time, % |
| Hall2 | S01 | -0.42 | 0.01 | 0.00 | 0.00 | -0.38 | 0.01 | -0.33 | 0.01 | 0.958 |
| Street | S02 | -0.06 | 0.00 | 0.00 | 0.00 | -0.06 | 0.00 | -0.05 | 0.00 | 1.011 |
| Dancer | S03 | -0.85 | 0.03 | 0.00 | 0.00 | -0.80 | 0.03 | -0.55 | 0.01 | 1.094 |
| GT | S04 | -0.07 | 0.00 | 0.00 | 0.00 | -0.07 | 0.00 | -0.06 | 0.00 | 0.947 |
| Kendo | S05 | -0.26 | 0.01 | 0.00 | 0.00 | -0.21 | 0.01 | -0.25 | 0.01 | 0.949 |
| Balloons | S06 | -0.19 | 0.01 | 0.00 | 0.00 | -0.16 | 0.01 | -0.15 | 0.01 | 1.008 |
| News | S08 | -0.15 | 0.01 | 0.00 | 0.00 | -0.12 | 0.00 | -0.13 | 0.00 | 0.856 |
|  | Average | -0.29 | 0.01 | 0.00 | 0.00 | -0.26 | 0.01 | -0.22 | 0.01 | 0.97 |

3.2 Further discussion

As shown in Table II, enabling splatting in ATM4 results in 0.29% BD-rate loss. However, with QCOM_ASYMMTRIC_RESOLUTION off, enabling splatting leads to around 0.03% BD-rate gain, as indicated from Table III and IV. The coding loss of splatting under mixed resolution view synthesis may be caused by not-well-harmonized three techniques: fractional-pel warping, splatting and mixed-resolution synthesis. This problem is avoided by using the proposed method.

Table III RD performance of ATM4 with QCOM_VSP off vs. ATM4

|  |  | Texture Coding |  | Depth Coding |  | Total (Coded PSNR) |  | Total (Synthesed PSNR) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | dBR, % | dPSNR,dB | dBR, % | dPSNR,dB | dBR, % | dPSNR,dB | dBR, % | dPSNR,dB |
| Hall2 | S01 | -0.28 | 0.01 | 0.00 | 0.00 | -0.25 | 0.00 | -0.26 | 0.01 |
| Street | S02 | -0.16 | 0.01 | 0.00 | 0.00 | -0.16 | 0.00 | -0.12 | 0.00 |
| Dancer | S03 | -0.61 | 0.02 | 0.00 | 0.00 | -0.57 | 0.02 | -0.44 | 0.01 |
| GT | S04 | 0.38 | -0.01 | 0.00 | 0.00 | 0.36 | -0.01 | 0.36 | -0.01 |
| Kendo | S05 | 0.04 | 0.00 | 0.00 | 0.00 | 0.06 | 0.00 | 0.04 | 0.00 |
| Balloons | S06 | -0.26 | 0.01 | 0.00 | 0.00 | -0.23 | 0.01 | -0.20 | 0.01 |
| News | S08 | -0.09 | 0.00 | 0.00 | 0.00 | -0.06 | 0.00 | -0.04 | 0.00 |
|  | Average | -0.14 | 0.00 | 0.00 | 0.00 | -0.12 | 0.00 | -0.09 | 0.00 |

Table IV RD performance of ATM4 with both QCOM_VSP and splatting off vs. ATM4

|  |  | Texture Coding |  | Depth Coding |  | Total (Coded PSNR) |  | Total (Synthesed PSNR) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | dBR, % | dPSNR,dB | dBR, % | dPSNR,dB | dBR, % | dPSNR,dB | dBR, % | dPSNR,dB |
| Hall2 | S01 | -0.10 | 0.00 | 0.00 | 0.00 | -0.08 | 0.00 | -0.15 | 0.00 |
| Street | S02 | -0.18 | 0.01 | 0.00 | 0.00 | -0.18 | 0.01 | -0.13 | 0.00 |
| Dancer | S03 | -0.60 | 0.02 | 0.00 | 0.00 | -0.56 | 0.02 | -0.43 | 0.01 |
| GT | S04 | 0.40 | -0.01 | 0.00 | 0.00 | 0.37 | -0.01 | 0.37 | -0.01 |
| Kendo | S05 | 0.04 | 0.00 | 0.00 | 0.00 | 0.05 | 0.00 | 0.07 | 0.00 |
| Balloons | S06 | -0.25 | 0.01 | 0.00 | 0.00 | -0.21 | 0.01 | -0.17 | 0.01 |
| News | S08 | -0.07 | 0.00 | 0.00 | 0.00 | -0.05 | 0.00 | -0.04 | 0.00 |
|  | Average | -0.11 | 0.00 | 0.00 | 0.00 | -0.09 | 0.00 | -0.07 | 0.00 |

# Conclusion

This proposal improves the geometry accuracy of warped pixels at regions of depth changes, which is an enhancement on NTT’s fractional-pel disparity compensation scheme and provides a better implementation of boundary-aware splatting with more accurate pixel values.

We recommend adopting the proposed method.

# Patent rights declaration(s)

Zhejiang University may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).