| Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 1st Meeting: Stockholm, SE, 16–20 July 2012 | Document: JCT2-A0057 |
| --- | --- |

| Title: | 3D-CE8.h results on JRDO |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Li Wang, Lu Yu #38 Zheda Road Zhejiang University Hangzhou, China | Tel: Email: | +86-571-87953107 yul@zju.edu.cn |
| Source: | Zhejiang University |  |  |

_____________________________

# Abstract

This document presents the RD performance of JRDO implemented on HTM3.1, evaluated with two view synthesis tools. A harmonization scheme of JRDO and VSO is also proposed. The harmonized scheme achieves better RD performance than JRDO and VSO on both VSRS and VSRS-1D-Fast.

# Introduction

In Geneva meeting, we evaluated RDO using different renderers and suggested that the RD performance of RDO for depth coding should be robust under different view synthesis algorithms. We have integrated JRDO (proposed in m24899) into the reference SW HTM3.1 for further evaluation and performance comparison. Besides, we try to harmonize JRDO with VSO in HTM3.1, which improves RD performance when either VSRS or VSRS-1D-Fast is used as the renderer. The harmonized scheme also reduces the encoding complexity compared with VSO, since part of the VSO is replaced with JRDO.

# The Proposed Method

In proposals [1][2], we have introduced a new RD optimization (JRDO) for depth coding which estimates the synthesized view distortion instead of conventional SSD. In this proposal, we try to harmonize the proposed JRDO and VSO, in which part of mode decision using VSO is replaced with JRDO. More specifically, in CU-level mode decision (i.e., the xCompressCU() function, which determines merge, PU partitioning of Intra and Inter modes), VSO is used; in the rest RD cost calculation below CU level, JRDO is used.

# Experimental results

In conventional RDO, the distortion is the SSD between original and reconstructed pixel values. In the experiments below, we use the conventional SSD-based RDO as an anchor. We compare the BD-rate performance of JRDO, RDO and the harmonized RDO vs. SSD-based RDO, using VSRS 1D fast and VRSR 3.5 as the renderers. The results are summarized in Table 1, Table 2 and Table 3, respectively, in which the BD-rate performance refers to PSNR of synthesized views vs. total bit rate of texture and depth.

Table 1 BD-rate performance of VSO when using VSRS 1D fast and VSRS 3.5 as the renderer

| Sequences | VSRS 1D Fast | VSRS 3.5 |
| --- | --- | --- |
| Balloons | -10.9% | -8.3% |
| Kendo | -18.1% | -14.9% |
| Newspapercc | -14.2% | -4.8% |
| GT Fly | -9.7% | -6.8% |
| PoznanHall2 | -16.1% | -18.5% |
| PoznanStreet | -7.2% | -9.3% |
| UndoDancer | -13.4% | -1.6% |
| Average | -12.8% | -9.2% |

Table 2 BD-rate performance of JRDO when using VSRS 1D fast and VSRS 3.5 as the renderer

| Sequences | VSRS 1D Fast | VSRS 3.5 |
| --- | --- | --- |
| Balloons | -4.7% | -6.0% |
| Kendo | -9.3% | -11.6% |
| Newspapercc | -8.8% | -11.1% |
| GT Fly | -5.4% | -5.2% |
| PoznanHall2 | -7.7% | -10.8% |
| PoznanStreet | -3.4% | -10.3% |
| UndoDancer | -8.6% | -8.5% |
| Average | -6.8% | -9.1% |

Table 3 BD-rate performance of harmonized method when using VSRS 1D fast and VSRS 3.5 as the renderer

| Sequences | VSRS 1D Fast | VSRS 3.5 |
| --- | --- | --- |
| Balloons | -11.0% | -9.3% |
| Kendo | -17.9% | -15.3% |
| Newspapercc | -14.3% | -8.3% |
| GT Fly | -10.0% | -7.0% |
| PoznanHall2 | -16.0% | -17.7% |
| PoznanStreet | -7.5% | -11.2% |
| UndoDancer | -14.0% | -3.3% |
| Average | -12.9% | -10.3% |

The performance of the harmonized RD optimization method slightly improves RD-performance when using 1D Fast (0.1% BD-rate saving) and VSRS 3.5 (1.1% BD-rate saving) as the renderer.

The encoder complexity of the harmonized RDO is supposed to be lower than VSO, since distortion calculation in JRDO is much simpler than that in VSO as reported in [1][2]. We have measured the encoding time of harmonized RDO using VSO based on HTM3.1 as an anchor. And the result is summarized in Table 4 below.

Table 4	The result of encoding time for harmonized RDO

| Sequences | Enc_time |
| --- | --- |
| Balloons | 82.5% |
| Kendo | 80.4% |
| Newspapercc | 78.9% |
| GT Fly | 63.6% |
| PoznanHall2 | 83.3% |
| PoznanStreet | 70.9% |
| UndoDancer | 59.8% |
| Average | 73.7% |

- Conclusion

Harmonizing JRDO with VSO achieves slightly better RD performance with different renderers.

We recommend to further study JRDO and the harmonization method in CE8.h.

# References

- Li Wang, Deliang Fu, Yin Zhao, Lu Yu, "3D-CE8.h results on JRDO and VSO with different view synthesis algorithms ", ISO/IEC JTC1/SC29/WG11, M24899, Geneva, May 2012.

- Li Wang, Deliang Fu, Yichen Zhang, Lu Yu, "3D-HEVC-CE10 results on Joint RDO for depth coding of 3D video by ZJU ", ISO/IEC JTC1/SC29/WG11, M23829, San Jose, February 2012.

# Patent rights declaration(s)

Zhejiang University may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).