| Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 1st Meeting: Stockholm, SE, 16–20 July 2012 | Document: JCT2-A0058 |
| --- | --- |

| Title: | 3D-CE1.a cross check report of dilation based view synthesis prediction with upsampling by Samsung (m25879/JCT2-A0035) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | report |  |  |
| Author(s) or Contact(s): | Yin Zhao and Lu Yu #38 Zheda Road Zhejiang University Hangzhou, China | Tel: Email: | +86-571-87953107 {zhaoyin,yul}@zju.edu.cn |
| Source: | Zhejiang University |  |  |

_____________________________

# Cross-check results

The cross-check results on Samsung’s dilation filter for VSP (JCT2-A0035) match exactly with the coding and synthesis results reported by Samsung. The decoding time (with disabling output of YUV) is on average 162% of that of ATM4, which is slightly higher than 152% which was reported by Samsung. The encoding and synthesis time are not measured in this cross-check.

The code fulfills the technical description in m24821.

Table I RD performance of the Samsung’s method vs. ATM4

|  |  | Texture Coding |  | Depth Coding |  | Total (Coded PSNR) |  | Total (Synthesed PSNR) |  | Complexity |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | dBR, % | dPSNR,dB | dBR, % | dPSNR,dB | dBR, % | dPSNR,dB | dBR, % | dPSNR,dB | Decoder Time, % |
| Hall2 | S01 | 0.00 | 0.00 | 0.00 | 0.00 | 0.02 | 0.00 | -0.05 | 0.00 | 1.624 |
| Street | S02 | -0.48 | 0.01 | 0.00 | 0.00 | -0.44 | 0.01 | -0.44 | 0.01 | 1.694 |
| Dancer | S03 | -0.66 | 0.02 | 0.00 | 0.00 | -0.61 | 0.02 | -0.68 | 0.02 | 1.518 |
| GT | S04 | -0.02 | 0.00 | 0.00 | 0.00 | -0.01 | 0.00 | -0.05 | 0.00 | 1.667 |
| Kendo | S05 | -0.22 | 0.01 | 0.00 | 0.00 | -0.15 | 0.01 | -0.19 | 0.01 | 1.584 |
| Balloons | S06 | -0.46 | 0.02 | 0.00 | 0.00 | -0.39 | 0.02 | -0.39 | 0.02 | 1.507 |
| News | S08 | -0.05 | 0.00 | 0.00 | 0.00 | -0.03 | 0.00 | -0.01 | 0.00 | 1.775 |
|  | Average | -0.27 | 0.01 | 0.00 | 0.00 | -0.23 | 0.01 | -0.26 | 0.01 | 1.62 |