| Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 1st Meeting: Stockholm, SE, 16–20 July 2012 | Document: JCT2-A0061 |
| --- | --- |

| Title: | 3D-CE1.a : Cross check on sub-pel VSP simplification of NTT |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Seok Lee, Jaejoon Lee San 14, Nongseo-dong, Giheung-gu, Yongin-si, Gyeonggi-do | Tel: Email: | 82-31-280-6696 lee.seok@samsung.com |
| Source: | Samsung Electronics Co., Ltd . |  |  |

_____________________________

# Abstract

Cross check results for NTT’s sub-pel VSP simplification are reported. Performance evaluation results reported by proponent were verified. -0.47% BD-rate gain for decoded views and -0.43% BD-rate gain for synthesized views were observed. These are same results provided by proponent.

# Test results

Test was done on linux cluster computing environment. Test results are summarized in Table 1.1.

Table 1.1 Test results for NTT’s sub-pel VSP simplification.

| Sequences | BD-rate with PSNR of decoded views over texture + depth bit rate | BD-rate with PSNR of synthesized views over texture + depth bit rate |
| --- | --- | --- |
| Poznan Hall2 | -0.54 | -0.56 |
| Poznan Street | -0.17 | -0.14 |
| Undo Dancer | -1.04 | -0.84 |
| GT Fly | -0.30 | -0.27 |
| Kendo | -0.36 | -0.38 |
| Balloons | -0.56 | -0.56 |
| Newspaper | -0.30 | -0.29 |
| Average | -0.47 | -0.43 |
| Encoding Time (%) | 103.40% |  |
| Decoding Time (%) | 97.18% |  |

# Summary

Cross check of NTT’s sub-pel VSP simplification was done. Test was performed by running NTT SW which sub-pel VSP simplification is enabled. Test results we obtained are same with the results provided by proponent.

# Patent rights declaration(s)

Samsung Electronics Co., Ltd. does not have any current or pending patent rights relating to the technology described in this contribution.