| Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 1st Meeting: Stockholm, SE, 16–20 July 2012 | Document: JCT2-A0064 |
| --- | --- |

| Title: | 3D-CE3.a : Cross check on depth post processing of Nokia |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Hocheon Wey, Jaejoon Lee San 14, Nongseo-dong, Giheung-gu, Yongin-si, Gyeonggi-do | Tel: Email: | 82-31-280-6612 hc.wey@samsung.com |
| Source: | Samsung Electronics Co., Ltd . |  |  |

_____________________________

# Abstract

Cross check results for Nokia’s depth post processing are reported. Performance evaluation results reported by proponent were verified. The depth post processing achieves -0.96% BD-rate gain in terms of synthesized texture PSNR with deblocking filter turn-off, and -1.13% BD-rate gain with deblocking filter turned on. These are same results provided by proponent.

# Test results

Test was done on linux cluster computing environment. Test results are summarized in Table 1.1 and 1.2.

Table 1.1 Test results for Nokia’s depth post processing (deblocking filter turn-off).

| Sequences | BD-rate with PSNR of decoded views over texture + depth bit rate | BD-rate with PSNR of synthesized views over texture + depth bit rate |
| --- | --- | --- |
| Poznan Hall2 | 0.00 | -0.04 |
| Poznan Street | 0.00 | 0.20 |
| Undo Dancer | 0.00 | -7.05 |
| GT Fly | 0.00 | 0.98 |
| Kendo | 0.00 | -0.71 |
| Balloons | 0.00 | -0.36 |
| Newspaper | 0.00 | -0.26 |
| Average | 0.00 | -0.96 |
| Encoding Time (%) | 102.5% |  |
| Decoding Time (%) | 104.4% |  |

Table 1.2 Test results for Nokia’s depth post processing (deblocking filter turn-on).

| Sequences | BD-rate with PSNR of decoded views over texture + depth bit rate | BD-rate with PSNR of synthesized views over texture + depth bit rate |
| --- | --- | --- |
| Poznan Hall2 | 0.00 | -0.04 |
| Poznan Street | 0.00 | -0.18 |
| Undo Dancer | 0.00 | -7.31 |
| GT Fly | 0.00 | 0.47 |
| Kendo | 0.00 | -0.22 |
| Balloons | 0.00 | -0.37 |
| Newspaper | 0.00 | -0.28 |
| Average | 0.00 | -1.13 |
| Encoding Time (%) | 104.7% |  |
| Decoding Time (%) | 112.6% |  |

# Summary

Cross check of Nokia’s depth post processing was done. Test results we obtained are same with the results provided by proponent.