| Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 1st Meeting: Stockholm, SE, 16–20 July 2012 | Document: JCT2-A0065 |
| --- | --- |

| Title: | 3D-CE3.a : Cross check on depth pre-post processing of Nokia |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Hocheon Wey, Jaejoon Lee San 14, Nongseo-dong, Giheung-gu, Yongin-si, Gyeonggi-do | Tel: Email: | 82-31-280-6612 hc.wey@samsung.com |
| Source: | Samsung Electronics Co., Ltd . |  |  |

_____________________________

# Abstract

Cross check results for Nokia’s depth pre-post processing are reported. Performance evaluation results reported by proponent were verified. The depth pre-post processing achieves -5.79% BD-rate gain in terms of synthesized texture PSNR with deblocking filter turn-off, and -5.85% BD-rate gain with deblocking filter turned on. These are same results provided by proponent.

# Test results

Test was done on linux cluster computing environment. Test results are summarized in Table 1.1 and 1.2.

Table 1.1 Test results for Nokia’s depth pre-post processing (deblocking filter turn-off).

| Sequences | BD-rate with PSNR of decoded views over texture + depth bit rate | BD-rate with PSNR of synthesized views over texture + depth bit rate |
| --- | --- | --- |
| Poznan Hall2 | 0.35 | -1.60 |
| Poznan Street | 0.23 | -1.81 |
| Undo Dancer | 1.31 | -22.69 |
| GT Fly | 0.32 | -9.35 |
| Kendo | 0.59 | -1.23 |
| Balloons | 0.83 | -0.82 |
| Newspaper | 1.01 | -3.04 |
| Average | 0.66 | -5.79 |
| Encoding Time (%) | 102.3% |  |
| Decoding Time (%) | 109.8% |  |

Table 1.2 Test results for Nokia’s depth pre-post processing (deblocking filter turn-on).

| Sequences | BD-rate with PSNR of decoded views over texture + depth bit rate | BD-rate with PSNR of synthesized views over texture + depth bit rate |
| --- | --- | --- |
| Poznan Hall2 | 0.09 | -1.62 |
| Poznan Street | 0.19 | -2.13 |
| Undo Dancer | 1.41 | -22.65 |
| GT Fly | 0.32 | -9.54 |
| Kendo | 0.48 | -1.33 |
| Balloons | 0.79 | -0.84 |
| Newspaper | 1.07 | -2.84 |
| Average | 0.62 | -5.85 |
| Encoding Time (%) | 102.7% |  |
| Decoding Time (%) | 108.1% |  |

# Summary

Cross check of Nokia’s depth pre-post processing was done. Test results we obtained are same with the results provided by proponent.