| Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 1st Meeting: Stockholm, SE, 16–20 July 2012 | Document: JCT2-A0066 |
| --- | --- |

| Title: | 3D-CE3.a : Cross check on depth boundary filter of GIST |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Hocheon Wey, Jaejoon Lee San 14, Nongseo-dong, Giheung-gu, Yongin-si, Gyeonggi-do | Tel: Email: | 82-31-280-6612 hc.wey@samsung.com |
| Source: | Samsung Electronics Co., Ltd . |  |  |

_____________________________

# Abstract

Cross check results for GIST’s depth boundary filter are reported. Performance evaluation results reported by proponent were verified. The depth boundary filtering achieves -3.53% BD-rate gain in terms of synthesized texture PSNR with deblocking filter turn-off, and -3.18% BD-rate gain with deblocking filter turned on.

# Test results

Test was done on linux cluster computing environment. Test results are summarized in Table 1.1 and 1.2.

Table 1.1 Test results for GIST’s depth boundary filter (deblocking filter turn-off).

| Sequences | BD-rate with PSNR of decoded views over texture + depth bit rate | BD-rate with PSNR of synthesized views over texture + depth bit rate |
| --- | --- | --- |
| Poznan Hall2 | 0.00 | -0.59 |
| Poznan Street | 0.00 | -1.16 |
| Undo Dancer | 0.00 | -11.34 |
| GT Fly | 0.00 | -3.84 |
| Kendo | 0.00 | -2.01 |
| Balloons | 0.00 | -1.88 |
| Newspaper | 0.00 | -3.88 |
| Average | 0.00 | -3.53 |
| Encoding Time (%) | 104.4% |  |
| Decoding Time (%) | 359.8% |  |

Table 1.2 Test results for GIST’s depth boundary filter (deblocking filter turn-on).

| Sequences | BD-rate with PSNR of decoded views over texture + depth bit rate | BD-rate with PSNR of synthesized views over texture + depth bit rate |
| --- | --- | --- |
| Poznan Hall2 | 0.00 | -0.27 |
| Poznan Street | 0.00 | -0.80 |
| Undo Dancer | 0.00 | -10.80 |
| GT Fly | 0.00 | -3.42 |
| Kendo | 0.00 | -1.68 |
| Balloons | 0.00 | -1.65 |
| Newspaper | 0.00 | -3.62 |
| Average | 0.00 | -3.18 |
| Encoding Time (%) | 103.1% |  |
| Decoding Time (%) | 392.8% |  |

# Summary

Cross check of GIST’s depth boundary filter was done. Test results for SD sequences we obtained are same with the results provided by proponent.