| Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 1st Meeting: Stockholm, SE, 16–20 July 2012 | Document: JCT2-A0068 |
| --- | --- |

| Title: | CE6.h Simple Region-based Intra Prediction for Depth-map |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jin Heo, Eunyong Son, Sehoon Yea 19, Yangjae-daero 11gil Seocho-gu, Seoul 137-130, Korea | Tel: Email: | 82-2-6912-6477 jin78.heo@lge.com eunyong.son@lge.com sehoon.yea@lge.com |
| Source: | LG Electronics |  |  |

_____________________________

# Abstract

This contribution presents a simple region-based intra prediction method for depth-map. The proposed method separates a block into two partitions using a straight line without any extra signalling bits by replacing the conventional intra prediction. The proposal provides approximately 4% and 2.5% encoding and decoding complexity reduction with 0.1% BD-rate loss compared to 3DV-HTM3.1 under the common test condition (CTC).

# Introduction

The simple region-based intra prediction for depth-map was introduced at the last meeting [1] and it was integrated into the current 3DV-HTM software 3.1 (3DV-HTM3.1). In 3DV-HTM3.1, there are two kinds of intra prediction methods for depth-map: 1) the directional intra prediction in HM6.1 and 2) the depth modeling modes (DMM) [2]. Although the depth-map block is coded more frequently with the former, the latter can enhance the BD-rate of synthesized views especially on sharp edges. However, it takes long time to find the best wedgelet pattern and uses additional syntax elements to signal its prediction mode. Therefore, this contribution proposes the region-based intra prediction which employs the characteristics that a depth-map block can be partitioned into disjoint depth regions and the DMM mode 1 and DMM mode 2 were replaced by the region-based intra prediction. The proposed method has two advantages; prediction mode reuse and less complexity compared to the DMM.

# Algorithm description

The method consists of the following five steps.

Step 1: Select reference pixels

The reference pixels are chosen within the left and above adjacent pixels depending on the prediction direction. For simplicity, we adopt the same reference pixels used for the conventional intra prediction in HM6.1. For example, the vertical mode uses the above pixels.

Step 2: Choose an edge starting point

It decides the boundary edge using a maximal slope point within the reference pixels. It eliminates additional signaling bits to inform the start point.

Step 3: Partition the block

The depth-map block is separated into two constant-valued regions using the edge starting point and the intra prediction mode that implies the direction of edges inside the block. The method exploits the same directional modes defined in the base HM, that is, 35 modes for all blocks in this proposal.

Step 4: Calculate predictors and fill them into the block

The value representing each region is calculated from the reference pixels. The averages of reference pixels for the regions are used as the predictors. Then the block is filled with the calculated predictors.

Step 5: Update the prediction block

After the Step 4, an offset signaling method similar to [2] was used and the offset value was added to the predictors.

# Experimental Results

Tests were based on the common test condition for the 3-view configuration [3]. The results are summarized in Table 1 and Table 2. The proposed method obtains 3.9% and 2.5% encoding and decoding time reduction and 0.1% BD-rate loss for 3-view test condition.

Table 1 BD rate results for 3-view case

|  | video 0 | video 1 | video 2 | video only | synthesized only | coded & synthesized |
| --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | 0.1% |
| Kendo | 0.0% | 0.0% | 0.0% | 0.0% | 0.2% | 0.2% |
| Newspapercc | 0.0% | 0.0% | 0.0% | 0.0% | 0.2% | 0.1% |
| GhostTownFly | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| PoznanHall2 | 0.0% | 0.0% | 0.0% | 0.0% | 0.3% | 0.2% |
| PoznanStreet | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | 0.0% |
| UndoDancer | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% | -0.1% |
| 1024x768 | 0.0% | 0.0% | 0.0% | 0.0% | 0.2% | 0.1% |
| 1920x1088 | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | 0.0% |
| average | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | 0.1% |

Table 2 Complexity results for 3-view case

|  | enc time | dec time | ren time |
| --- | --- | --- | --- |
| Balloons | 95.9% | 96.4% | 99.6% |
| Kendo | 98.3% | 97.3% | 102.0% |
| Newspapercc | 93.1% | 89.9% | 97.7% |
| GhostTownFly | 96.9% | 98.6% | 97.9% |
| PoznanHall2 | 97.9% | 99.7% | 100.2% |
| PoznanStreet | 94.7% | 97.7% | 105.2% |
| UndoDancer | 96.3% | 103.6% | 99.3% |
| 1024x768 | 95.7% | 94.5% | 99.8% |
| 1920x1088 | 96.4% | 99.9% | 100.6% |
| average | 96.1% | 97.5% | 100.2% |

# Conclusion

The simple region-based intra prediction was proposed to express edges inside a depth block well. By integrating the proposed method on 3DV-HTM3.1, it reduces the encoding and decoding complexity by 4.0% and 2.5%, respectively with 0.1 BD-rate loss compared to 3DV-HTM3.1. Therefore, we recommend the adoption of the simple region-based intra prediction method into the current 3DV-HTM software.

# Reference

- Doowon Lee, Jaewon Sung, Jin Heo, and Sehoon Yea, “3D-CE6.h related: Simple Region-based Intra Prediction for Depth-map,” ISO/IEC JTC1/SC29/WG11 MPEG2012/M25011, Geneva, Switzerland, May 2012.

- Heiko Schwarz, Christian Bartnik, Sebastian Bosse, Heribert Brust, Tobias Hinz, Haricharan Lakshman, Detlev Marpe, Philipp Merkle, Karsten Muller, Hunn Rhee, Gerhard Tech, Martin Winken, Thomas Wiegand, “Description of 3D Video Technology Proposed by Fraunhofer HHI,” ISO/IEC JTC1/SC29/WG11 MPEG2011/M22570, Geneva, Switzerland, Nov. 2011.

- Heiko Schwarz and Dmytro Rusanovskyy, “Common Test Conditions for 3DV experimentation,” ISO/IEC JTC1/SC29/WG11 MPEG2011/N12745, Geneva, Switzerland, May 2012.

# Patent rights declaration(s)

LG Electronics Co., Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).