| Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 1st Meeting: Stockholm, SE, 16–20 July 2012 | Document: JCT2-A0069 M26011 |
| --- | --- |

| Title: | 3DV CE7.h results on Global Depth and View Prediction by NICT |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Takanori Senoh, Kenji Yamamoto, Ryutaro Oi, Yasuyuki Ichihashi Nukui-Kitamachi 4-2-1, Koganei, Tokyo 184-8795, Japan | Tel: Email: | +81-42-327-7262 {senoh, k.yamamoto, oi, y-ichihashi}@nict.go.jp |
| Source: | National Institute of Information and Communications Technology(NICT) |  |  |

_____________________________

# Abstract

This contribution reports 3DV-CE7.h results on Global Depth and View Prediction (GVDP) proposed by NICT in N12746 [1]. CE7 was executed on 3view case based on 3DV HTM v3.1 [2]. CE7 is common for both ATM- and HTM-based 3DV. Since results on ATM -based CE7 reported in CE7.a summary [3] and Subjective evaluation [4] showed high coding efficiency and reasonable synthesis quality with the encoding and decoding speed faster than ATM0.3, HTM-based CE7 was tested this time. Experimental results show shorter encoding time of 14.3% and shorter decoding time of 17.4% comparing to the anchor codec, longer but improved 8-view rendering time of 125.7% comparing to VSRS_1D_fast for 6-views, better depth BD rate of -89.5%, although some sequences couldn’t be calculated, and worse but improved coded & synthesized BD rate of 82.8%. Since BD rate doesn’t exactly show the synthesized view quality, we propose a subjective test for a correct evaluation.

# Introduction

Global Depth and View Prediction (GDVP: CE7) is a pre-process and post-process for anchor codec of 3DV-TM (ATM, HTM) as shown in Fig.1. Pre-process consists of Global Depth generation and View Prediction. Main view (center view for 3view case and right view for 2view case), Global Depth, and Residual from View Prediction are encoded/decoded by the anchor codec. Post-process consists of View Synthesis from the decoded output of anchor codec. Global depth is a depth sequence combining three input depth sequences into one sub-sampled sequence. Residual is also one sequence combining two non-anchor residual texture sequences. It should be noted that GDVP is using 3DV TM as a single view codec, AVC or HEVC codecs are easily extended to 3DV codec.

Figure 1: Structure of CE7: Global Depth and View Prediction

# Experiments

## Software

CE7.h experiments are executed with anchor codec of 3DV-HTM3.1 software with the cfg files provided by HHI and CE7 software provided by NICT.

## Test conditions

The common test conditions [5] are applied.

## Experimental environment

The experimental environment is summarized in Table1.

Table 1 Experimental environment

| CPU | Intel Core i7 2600 3.4GHz |
| --- | --- |
| Memory | 12GB (DDR3) |
| OS | Windows 7 Professional SP1 |
| Compiler | Microsoft Visual Studio 2010 Professional |
| Executable | x64 |

## Simulation results

### Summary

Table2 shows a summary of experimental results for 3view case. CE7 yielded -89.5% lower BD rate for depth, although some sequences couldn’t be calculated, -25.4% lower BD rate for video only coding,113.6% higher BD rate for synthesized only quality, and 82.8% higher BD rate for coded & synthesized views. Regarding the complexity, encoding time is 14.3%, decoding time is 17.4%, and synthesis time is 125.7% comparing to the anchor.

Table 2 Simulation results for 3 view test case

### Execution time

Fig.2 shows the ratio (%) of encoding, decoding and view synthesis time comparing to the anchor codec for each sequence. Since CE7 performs inter-view prediction based on homography projection without search and uses anchor codec as a single-view codec, encoding and decoding speed is fast. Although CE7 software is partially optimized, since it requires view synthesis for Residual views, total synthesis time is still longer than the anchor.

14.3%

17.4%

125.7%

Figure 2: Execution time

### Output view quality

Fig.3 shows output view quality for each sequence. PSNRs are the average of all PSNRs of input views and synthesized views, measured by referring to the synthesized views by VSRS_1D_fast with input depth maps. Bitrates are the total bitrate of all views and depth maps. Since reference views include artifacts caused by imperfect depth maps and the software, the PSNRs are not reliable.

Figure 3: Output view PSNR vs. total bit rate

### Synthesized views

Figu.4 shows the last frames of synthesized view1.5 between view1 (left) and view2 (center) input sequences at rate R4 (QP25) and R1 (QP40) for CE7 (left) and Anchor (right). Subjective test is necessary to evaluate the CE7 performance correctly.

(S01) Poznan_Hall2	CE7	Anchor

Q25

Q40

(S02) Poznan_Street	CE7	Anchor

Q25

Q40

(S03) Undo_Dancer	CE7	Anchor

Q25

Q40

(S04) GT_Fly	CE7	Anchor

Q25

Q40

(S05) Kendo	CE7	Anchor

Q25

Q40

(S06) Balloons	CE7	Anchor

Q25

Q40

(S08) Newspaper	CE7	Anchor

Q25

Q40

Figure 4: Synthesized view1.5 by CE7 and anchor HTM3.1

# Patent rights declaration(s)

NICT may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

[1]	ISO/IEC JTC1/SC29/WG11 MPEG2011/N12746: "Description of Core Experiments in 3D Video Coding," (May 2012)

[2]	ISO/IEC JTC1/SC29/WG11 MPEG2011/N12744: "Test Model under consideration for HEVC based 3D Video Coding V3.0," (May 2012)

[3]	ISO/IEC JTC1/SC29/WG11 MPEG2011/M24984: "3D-CE7.a summary report on Global Depth and View Prediction," (May 2012)

[4]	ISO/IEC JTC1/SC29/WG11 MPEG2011/M25242: "3D-CE7.a subjective evaluation results on Global Depth and View Prediction," (May 2012)

[5] ISO/IEC JTC1/SC29/WG11 MPEG2011/N12754: "Common Test Conditions for 3DV Experimentation," (May 2012)