| Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 1st Meeting: Stockholm, SE, 16–20 July 2012 | Document: JCT2-A0070 |
| --- | --- |

| Title: | CE6.h Region Boundary Chain Coding for Depth-map |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jin Heo, Eunyong Son, Sehoon Yea 19, Yangjae-daero 11gil Seocho-gu, Seoul 137-130, Korea | Tel: Email: | 82-2-6912-6477 jin78.heo@lge.com eunyong.son@lge.com sehoon.yea@lge.com |
| Source: | LG Electronics |  |  |

_____________________________

# Abstract

This contribution represents a region boundary chain coding method for the depth-map. The proposed method introduces an edge chain that separates the block into two regions and codes the chain explicitly. The proposal achieves 0.2% and 0.4% BD-rate gains compared to 3DV-HTM3.1 under the common test condition (CTC) and all intra test condition with negligible encoding/decoding time.

# Introduction

The depth-map in 3D video coding has different characteristics compared to the texture video. First, many depth-map blocks include two regions: the foreground and background. Hence the depth values change abruptly near edges between the regions. In addition, the depth values have little variation within a region [1]. The region boundary chain coding for depth-map based on these characteristics was introduced at the last meeting [2] and it was integrated into the current 3DV-HTM software 3.1 (3DV-HTM3.1). The depth modeling mode (DMM) which was integrated in 3DV-HTM consists of four modes including wedgelet and contour partitioning modes. In the wedgelet modes, the block is separated with a straight line. On the other hand, the contour partitioning utilizes inter-component correlation between texture and depth in order to reduce signaling bits to describe an arbitrary shape of the partition. The proposed method partitions the block into two regions by signaling the region boundaries with chain codes [3]. Compared with the contour partitioning modes in the DMM, it has a distintive feature that it can well describe even a curved depth pattern that has weak or little inter-component correlation between texture and depth.

# Algorithm description

The proposed method consists of four steps.

Step 1: Find internal edges

The internal edges inside a depth-map block are calculated in the encoder. The step consists of several fine procedures.

- Calculate differences between vertically and horizontally adjacent pixels.

- Mark as edge candidates if the difference is greater than the threshold.

- Eliminate the edge candidates which have smaller differences than neighboring edge candidates.

- Connect unlinked edges if necessary.

- Prune unconnected edges.

- Check whether the block consists of two regions exactly. Otherwise, the method will not be applied further.

Step 2: Code the edges using chain codes

The edges are encoded into chain codes. Firstly, an edge starts from the block boundaries. Then, the next edge is chosen as an edge connected to the current one. The final edge ends at the boundaries. To construct chain codes, it defines seven traverse types (0, 45, -45, 90, -90, 135, -135 degree) and their edge patterns.

Step 3: Convert the chain codes into bitstream syntax

After the Step 1 and Step 2, the region boundaries are expressed as a set of an edge starting point and the traverse codes. Two syntax elements (edge_start_left_flag and edge_start_position) are used for the starting point. And the traverse codes are signaled with edge_count_minus_1 and edge_code as described in Figure 1. The edge_code is constructed to have a different codeword length according to the frequency of traverse types [3].

... // intra_mode is signaled with intra prediction syntax elements with escape code

if ( intra_mode is equal to Region Boundary Chain Coding ) {

edge_start_left_flag

edge_start_position

edge_count_minus_1

for( i = 0; i <= edge_count_minus_1; i++ )

edge_code[i]

}

Figure 1. Pseudo syntax elements for region boundary chain coding

A new intra prediction mode is added to the conventional prediction syntax for the purpose of signaling the proposed mode. The method is signaled with an escape code as the final intra prediction mode whereas the DMM mode exploits an additional flag for each prediction unit to determine whether it is coded with the conventional prediction or the DMM.

Step 4: Calculate the predictors and fill them into the block

The regional average values are chosen as the predictors for partitioned regions. We used the averaging method because it showed the best PSNR results and is expected to be easy to implement on hardware. Moreover an offset signaling method similar to [1] was added to the current method to enhance the accuracy of the predictors

# Experimental Results

Tests were based on the common test condition (CTC) and all intra test condition for the 3-view configuration [4]. The results are summarized in from Table 1 to Table 4. The proposed method achieved 0.2% and 0.4% BD-rate gains with negligible decoding time.

Table 1 BD rate results for 3-view case under CTC

|  | video 0 | video 1 | video 2 | video only | synthesized only | coded & synthesized |
| --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.0% | 0.0% | 0.0% | 0.0% | -0.2% | -0.1% |
| Kendo | 0.0% | 0.0% | 0.0% | 0.0% | -0.2% | -0.1% |
| Newspapercc | 0.0% | 0.0% | 0.0% | 0.0% | -0.5% | -0.3% |
| GhostTownFly | 0.0% | 0.0% | 0.0% | 0.0% | -0.2% | -0.1% |
| PoznanHall2 | 0.0% | 0.0% | 0.0% | 0.0% | -0.3% | -0.2% |
| PoznanStreet | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| UndoDancer | 0.0% | 0.0% | 0.0% | 0.0% | -0.9% | -0.6% |
| 1024x768 | 0.0% | 0.0% | 0.0% | 0.0% | -0.3% | -0.2% |
| 1920x1088 | 0.0% | 0.0% | 0.0% | 0.0% | -0.3% | -0.2% |
| average | 0.0% | 0.0% | 0.0% | 0.0% | -0.3% | -0.2% |

Table 2 Complexity results for 3-view case under CTC

|  | enc time | dec time | ren time |
| --- | --- | --- | --- |
| Balloons | 103.6% | 98.2% | 99.1% |
| Kendo | 104.4% | 99.7% | 102.4% |
| Newspapercc | 106.4% | 97.0% | 100.7% |
| GhostTownFly | 100.7% | 100.5% | 98.0% |
| PoznanHall2 | 100.6% | 99.0% | 102.2% |
| PoznanStreet | 100.5% | 100.0% | 100.6% |
| UndoDancer | 100.5% | 101.2% | 96.2% |
| 1024x768 | 104.8% | 98.3% | 100.7% |
| 1920x1088 | 100.6% | 100.2% | 99.2% |
| average | 102.4% | 99.4% | 99.9% |

Table 3 BD rate results for 3-view case under all intra test condition

|  | video 0 | video 1 | video 2 | video only | synthesized only | coded & synthesized |
| --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.0% | 0.0% | 0.0% | 0.0% | -0.5% | -0.3% |
| Kendo | 0.0% | 0.0% | 0.0% | 0.0% | -0.5% | -0.3% |
| Newspapercc | 0.0% | 0.0% | 0.0% | 0.0% | -0.8% | -0.5% |
| GhostTownFly | 0.0% | 0.0% | 0.0% | 0.0% | -0.4% | -0.2% |
| PoznanHall2 | 0.0% | 0.0% | 0.0% | 0.0% | -0.2% | -0.1% |
| PoznanStreet | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% | -0.1% |
| UndoDancer | 0.0% | 0.0% | 0.0% | 0.0% | -1.3% | -0.9% |
| 1024x768 | 0.0% | 0.0% | 0.0% | 0.0% | -0.6% | -0.4% |
| 1920x1088 | 0.0% | 0.0% | 0.0% | 0.0% | -0.5% | -0.3% |
| average | 0.0% | 0.0% | 0.0% | 0.0% | -0.5% | -0.4% |

Table 4 Complexity results for 3-view case all intra test condition

|  | enc time | dec time | ren time |
| --- | --- | --- | --- |
| Balloons | 99.2% | 99.4% | 99.9% |
| Kendo | 99.5% | 100.3% | 100.0% |
| Newspapercc | 99.3% | 101.4% | 101.7% |
| GhostTownFly | 101.3% | 98.9% | 99.9% |
| PoznanHall2 | 100.4% | 100.7% | 100.6% |
| PoznanStreet | 99.7% | 101.9% | 119.2% |
| UndoDancer | 99.8% | 98.7% | 98.7% |
| 1024x768 | 99.3% | 100.4% | 100.5% |
| 1920x1088 | 100.3% | 100.0% | 104.3% |
| average | 99.9% | 100.2% | 102.7% |

# Conclusion

The proposed method utilizes a chain coding along region boundaries inside the depth-map block and transmits the chain information explicitly. It achieves a 0.2% and 0.4% BD-rate reduction on 3DV-HTM3.1 under CTC and all intra test condition.

# References

- Heiko Schwarz, Christian Bartnik, Sebastian Bosse, Heribert Brust, Tobias Hinz, Haricharan Lakshman, Detlev Marpe, Philipp Merkle, Karsten Muller, Hunn Rhee, Gerhard Tech, Martin Winken, Thomas Wiegand, “Description of 3D Video Technology Proposed by Fraunhofer HHI (HEVC compatible; configuration A),” ISO/IEC JTC1/SC29/WG11 MPEG2011/M22570, Geneva, Switzerland, Nov. 2011.

- Doowon Lee, Jaewon Sung, Jin Heo, and Sehoon Yea, “3D-CE6.h related: Region Boundary Chain Coding for Depth-map,” ISO/IEC JTC1/SC29/WG11 MPEG2012/M25011, el under consideration for AVC-based 3D video coding (3DV-ATM),” M12349, Nov. 2011.

- Yong Kui Liu and Borut Zalik, “An efficient chain code with Huffman coding,” Pattern Recognition, vol. 38, Issue 4, pp. 553-557, April 2005.

- Heiko Schwarz and Dmytro Rusanovskyy, “Common Test Conditions for 3DV experimentation,” ISO/IEC JTC1/SC29/WG11 MPEG2011/N12745, Geneva, Switzerland, May 2012.

# Patent rights declaration(s)

LG Electronics Co., Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).