| Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 1st Meeting: Stockholm, SE, 16–20 July 2012 | Document: JCT2-A0071 M26013 |
| --- | --- |

| Title: | 3DV CE7 summary on Global Depth and View Prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Takanori Senoh Nukui-Kitamachi 4-2-1, Koganei, Tokyo 184-8795, Japan | Tel: Email: | +81-42-327-7262 senoh@nict.go.jp |
| Source: | National Institute of Information and Communications Technology(NICT) |  |  |

_____________________________

# Abstract

3DV CE7 results are summarized. Global Depth and View Prediction (GVDP) proposed by NICT was tested by NICT and cross-checked by NISRI and MediaTek on 3view test case. Experimental results show shorter encoding time of 14.3% and shorter decoding time of 17.4% comparing to the anchor codec, longer but improved rendering time of 125.7% comparing to VSRS_1D_fast, better depth BD rate of -89.5%, although some sequences couldn’t be calculated, and worse but improved coded & synthesized BD rate of 82.8%.. Since BD rate doesn’t exactly show the synthesized view quality, subjective test is requested for a correct evaluation. It is also proposed MPEG should create 3DV Simple Test Model based on CE7.

# Introduction

3DV CE7: Global Depth and View Prediction:

The goal of this Core Experiment (CE) is to investigate Global Depth and View Prediction (GDVP) based on a concept of Epipolar Plane Image (EPI) of depth and view which is applicable to AVC- or HEVC-based 3DV. Two tools were proposed and one tool was evaluated according to the impact on compression efficiency and computational complexity.

# Participants

Six organizations were participated and three organizations reported the experimental results.

| Participant | Contact | Email | Proponent/ Cross checker |
| --- | --- | --- | --- |
| NICT | Takanori Senoh(coordinator) | senoh@nict.go.jp | P/C |
| NISRI | Masayuki Tanimoto Kazuyoshi Suzuki | tanimototentative3@yahoo.co.jp kazuyoshi-suzuki@cb3.so-net.ne.jp | P/C |
| Poznan U | Krzysztof Wegner | kwegner@multimedia.edu.pl | C |
| Orange Labs | Joelb Jung Elie Mora | joelb.jung@orange.com elie.mora@orange.com | C |
| Qualcomm | Ying Chen | cheny@qualcomm.com | C |
| MediaTek | Yu-Lin Chang Yu-Pao Tsai Shawmin Leo | yulin.chang@mediatek.com yupao.tsai@mediatek.com shawmin.leo@mediatek.com | C |

# Tools under test

One tool for Global Depth and View Prediction was tested under this CE with the cross checkers.

| Tool | Cross-checker | results |
| --- | --- | --- |
| NICT [M26014/JCT2-A0072] | NISRI, MediaTek | Available Not yest |

# Document list

Three documents were uploaded.

| Participants | Doc No. | Title | Type |
| --- | --- | --- | --- |
| NICT | M26011/ JCT2-A0069 | 3DV CE7.h results on Global Depth and View Prediction by NICT | Proposer |
| NISRI | M25945/ JCT2-A0059 | Cross Check of 3DV CE7.h on Global Depth and View Prediction of NICT by NISRI | Cross checker |

# Description of experiments and summary results

## Description of experiments

NICT has implemented and executed CE7.h: Global Depth and View Prediction tool based on 3DV-HTM v3.1 anchor codec as depicted in Fig.1 and Table 1. It has been crosschecked by NISRI and MediaTek as shown in Table 2. All simulation results matched well each other except for small differences caused by the test platforms.

Figure 1: Structure of CE7: Global Depth and View Prediction

## Summary of experiments

Two CE7.h results matched well. Results show Video1,2 coding: 138.4% worse BD rate, Depth coding: -89.5% better BD rate, Synthesized view: 113.6% worse but subjectively not bad BD rate, Execution time: shorter encoding time of 14.3%, shorter decoding time of 17.4%, and longer but improved 8-view synthesis time of 125.7% comparing to VSRS_1D_fast for 6-views.

Table 1 CE7.h results by NICT

Table 2 CE7.h results by NISRI

Table 3 CE7.h results by MediaTek

14.3%

17.4%

125.7%

Figure 2: Encoding, Decoding and synthesis time from NICT results

Figure 3: Coded + synthesized view average PSNR vs. total bit rate from NICT results

## Synthesized views

Although BD rate is not better, Subjective quality of CE7 stereo pairs is as good as the anchor. The following Figures shows the last frames of synthesized view1.5 between view1 (left) and view2 (center) input sequences at rate R4 (QP25) and R1 (QP40) for CE7 (left) and Anchor (right). Subjective evaluation is required for correct evaluation.

(S01) Poznan_Hall2	CE7	Anchor

Q25

Q40

(S02) Poznan_Street	CE7	Anchor

Q25

Q40

(S03) Undo_Dancer	CE7	Anchor

Q25

Q40

(S04) GT_Fly	CE7	Anchor

Q25

Q40

(S05) Kendo	CE7	Anchor

Q25

Q40

(S06) Balloons	CE7	Anchor

Q25

Q40

(S08) Newspaper	CE7	Anchor

Q25

Q40

Figure 2: Synthesized view1.5 by CE7 and anchor

# Conclusion

CE7.h experimental results showed shorter encoding time, shorter decoding time, longer but comparable rendering time, better depth BD rate, and worse but subjectively not-bad synthesized view BD rate for 8-views. Subjective test is requested for a correct evaluation. It is also proposed to create a 3DV Simple Test Model base on CE7.