| Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 1st Meeting: Stockholm, SE, 16–20 July 2012 | Document: JCT2-A0073 M26016 |
| --- | --- |

| Title: | Proposal of Depth-aware 3DV Test Condition and Evaluation |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Takanori Senoh, Kenji Yamamoto, Ryutaro Oi, Yasuyuki Ichihashi Nukui-Kitamachi 4-2-1, Koganei, Tokyo 184-8795, Japan Masayuki Tanimoto, Kazuyoshi Suzuki | Tel: Email: | +81-42-327-7262 senoh@nict.go.jp k.yamamoto@nict.go.jp oi@nict.go.jp y-ichihashi@nict.go.jp +81-52-223-6639 tanimototentative3@yahoo.co.jp kazuyoshi-suzuki@cb3.so-net.ne.jp |
| Source: | National Institute of Information and Communications Technology (NICT) , Nagoya Industry Science Research Institute (NISRI) |  |  |

_____________________________

# Abstract

This contribution requests a depth-aware 3DV test condition and evaluation method which provide depth maps of higher coding quality, especially for HEVC-based model. The currently decoded depth map quality even for the highest bit rate looks un-acceptable for emerging Free Viewpoint TV [1][2 ]or new 3D displays, which are directly displaying the depth of 3D objects, such as volume-scanning display [3][4], plasma 3D display[5], electronic holography [6], etc. For the longevity of 3DV standard, its codec should be specified considering wide application range, which will come soon. For that purpose, Common Test Condition should use the same QP values for both texture and depth map. BD rate for depth maps should be evaluated in the summary of HTM evaluation chart.

# Introduction

Originally, 3DV standardization started aiming applications for Free Viewpoint TV [1][2] and 3D images. Later, the target was focused only on auto-stereoscopic (multi-view) 3D images. Since 3DV standard will be used for a long time, however, we should consider about not only multi-view displays but also FTVs and depth-aware 3D displays.

Free viewpoint TV requires high quality depth maps, since it synthesizes images at different baseline from the base line for reference views. If the depth maps are heavily distorted, synthesized view will be corrupted.

Figure 1: Free viewpoint video (from reference [2])

Auto-stereoscopic 3D images are not only based on multi-view images, but many auto-stereoscopic 3D images are based on the depth maps. Light Field display [3] displays depth of objects in a 3D space as shown in Figure 2. If the depth map is distorted, it distorts 3D images.

Figure 2: Light Field display (from reference [3])

Volume-scanning 3D display [4] displays slices of 3D images as shown in Figure 3. If depth maps are distorted, displayed 3D images will be distorted.

Figure 3: Volume scanning 3D display (from reference [3])

Plasma 3D display [5] also displays the depth of objects in a 3D space as shown in Figure 4. If depth map is distorted, it distorts 3D images.

Figure 4: Aerial 3D display (from reference [5])

Electronic holography [6], which is expected to be an ultimate 3D display, requires depth maps to display 3D image as shown in Figure 5. If depth maps are distorted, they distort 3D images.

Figure 5: Aerial 3D display (from reference [6])

# Problem of Current Common Test Condition

Current Common Test Condition for HEVC-based mode is using very large QP values for depth maps comparing to texture QPs as seen in Table1. Especially for high bit rates, the QP difference between texture and depth is large. This causes a heavy distortion on the depth maps as shown in Figure.6. This depth map may work well for current common test condition, where multi-views are synthesized on the same base line as the reference views. However, it is clear that heavily distorted depth maps cannot be used for the other applications which are displaying the depth of 3D objects. If we keep the current test condition in future, the 3DV standard will limit itself into very narrow application area.

Table 1: QP for HEVC-based test

| QPV0 | 51 | 50 | 49 | 48 | 47 | 46 | 45 | 44 | 43 | 42 | 41 | 40 | 39 | 38 | 37 | 36 | 35 | 34 | 33 | 32 | 31 | 30 | 29 | 28 | 27 | 26 | 25 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| QPD0 | 51 | 50 | 50 | 50 | 50 | 49 | 48 | 47 | 47 | 46 | 45 | 45 | 44 | 44 | 43 | 43 | 42 | 42 | 41 | 41 | 40 | 39 | 38 | 37 | 36 | 35 | 34 |

Figure 6: Decoded depth map at QP=34 (highest bit rate) by HTM3.1 (35dB/231kbps)

# Proposal on Common Test Condition and Evaluation

It is proposed that

- the same QP values should be used in the Common Test Condition for depth maps and texture like AVC-based test condition. Since depth data is originally smaller than texture data, heavy compression of depth maps gains only few bits with sacrificing the depth quality largely. If higher compression is required, sub-sampling of depth maps like AVC-based test condition is a good candidate. Figure 7 shows an example where depth maps are sub-sampled to half horizontally and vertically.

Figure 7: Decoded depth map at QP=35 (2nd low bit rate) by CE7 (51dB/144kbps)

- PSNR and bit rate comparison for depth map should be added in the summary in HTM evaluation sheet. It will save potential un-expected problems of the depth map quality in future.

Table 2: CE evaluation summary including depth map

# Patent rights declaration(s)

NICT and NISRI may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

[1] M. Tanimoto, M. Panahpour Teherani, T. Fujii and T. Yendo, “Free-Viewpoint TV,” IEEE Singal Pocessing, pp.67-76, Jan. 2011.

[2] A. Ishikawa, M. P. Teherani, S. Naito, S. Sakazawa and A. Koike, “Free Viewpoint Generation for Walk-through Experience Using Image –based Rendering,” M M’08, ACM Multimedia Conference 2008, ACM-978-1-60558-303-7/08/10, pp.1007-1008, Oct. 2008.

[3] A. Jones, I. McDowall, H. Yamada, M. Bolas and P. Debevec, “Rendering for an Interactive 360°Light Field Display,” ACM Trans on Graphics, 26(3), pp.40-1 -10, 2007.

http://www.youtube.com/watch?feature=endscreen&v=G10bzatpuFc&NR=1

[4] D. Miyazaki and K. Matsushita, “Volume-scanning three-dimensional display that uses an inclined image plane,” Applied Optics, Vol. 40, Issue 20, pp. 3354-3358 (2001)

http://jstshingi.jp/abst/p/10/1035/osaka402.pdf

[5] H. Watanabe, H. Ishikawa, S. Aoki, S. Suzuki, H. Sato, S. Shimada, T. Kimura, M. Kakehata, Y. Tsukada and H. Kimura, “Vision-based Interaction with Aerial 3D Display,” Proc. of 3DSA2010, pp.193-196, 2010.

http://www.youtube.com/watch?v=EndNwMBEiVU

[6] T. Senoh, T. Mishina, K. Yamamoto, R. Oi, Y. Ichihashi and T. Kurita, “Full-Color Wide Viewing-Zone-Angle Electronic Holography System,” OSA DH 2011, DTuA3, pp.1-3, May, 2011.