| Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 1st Meeting: Stockholm, SE, 16–20 July 2012 | Document: JCT2-A0074 |
| --- | --- |

| Title: | 3DV-ATM HP: Proposed HRD text for ISO/IEC 14496-10:2012/PDAM 2 MVC extensions for inclusion of depth maps |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Sally Hattori (Sony) Teruhiko Suzuki (Sony) Ying Chen (Qualcomm) Anthony Vetro (MERL) Sony City Osaki. 2-10-1 Osaki Shinagawa-ku , Tokyo, 141-8610 JAPAN | Tel: Email: | +81-50-3750-7411 Shinobu.Hattori@jp.sony.com teruhikos@jp.sony.com cheny@qualcomm.com avetro@merl.com |
| Source: | Sony, Qualcomm, Mitsubishi Electric Research Labs |  |  |

_____________________________

# Abstract

This document proposes text on HRD for ISO/IEC 14496-10:2012/PDAM 2 MVC extensions for inclusion of depth maps[1]. The proposal extends current Annex C in AVC[2] for Annex I in MVC extensions for inclusion of depth maps[1] with similar concept of extending process applied for MVC.

# Introduction

The followings are the basic concept of proposed extensions made for Annex I:

- When the decoding process specified in Annex I is applied, the process specified for Annex H in Annex C is invoked for particular texture view and depth view with view order index VOIdx.

- During the invocation of the process for a particular texture view, only the texture view components of the particular view are considered and during the invocation of the process for a particular depth view, only the depth view components of the particular view are considered.

- When the coded video sequence conforms to a profile specified in Annex I and the decoding process specified in Annex I is used, the view components of all the target output views of a picture are output at the same time instant and in increasing order of the view order index VOIdx.

- Inside each view component of the current primary coded picture, depth view component if present, is processed after the texture view component within the same view component.

- The same setting of OutputFlag applies to the texture view component and depth view component wit same view order index VOIdx within the same view component.

Attached text is the proposed changes in Annex C.

# Reference

- Video Group, “Text of ISO/IEC 14496-10:2012/PDAM 2 MVC extensions for inclusion of depth maps,” ISO/IEC JTC1/SC29/WG11 N12731, Geneva, CH, May 2012

- “Advanced Video Coding”, ISO/IEC 14496-10 and ITU-T Rec. H.264

# Patent rights declaration(s)

Sony Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Qualcomm may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Mitsubishi Electric Research Labs may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

- Annex C

Hypothetical reference decoder

(This annex forms an integral part of this Recommendation | International Standard)

This annex specifies the hypothetical reference decoder (HRD) and its use to check bitstream and decoder conformance.

Two types of bitstreams are subject to HRD conformance checking for this Recommendation | International Standard. The first such type of bitstream, called Type I bitstream, is a NAL unit stream containing only the VCL NAL units and filler data NAL units for all access units in the bitstream. The second type of bitstream, called a Type II bitstream, contains, in addition to the VCL NAL units and filler data NAL units for all access units in the bitstream, at least one of the following:

–	additional non-VCL NAL units other than filler data NAL units,

–	all leading_zero_8bits, zero_byte, start_code_prefix_one_3bytes, and trailing_zero_8bits syntax elements that form a byte stream from the NAL unit stream (as specified in Annex Error: Reference source not found).

Figure C-1 shows the types of bitstream conformance points checked by the HRD.

- Figure C-1 – Structure of byte streams and NAL unit streams for HRD conformance checks

The syntax elements of non-VCL NAL units (or their default values for some of the syntax elements), required for the HRD, are specified in the semantic subclauses of clause Error: Reference source not found, Annexes Error: Reference source not found and Error: Reference source not found, and subclauses Error: Reference source not found, Error: Reference source not found, Error: Reference source not found, Error: Reference source not found, Error: Reference source not found, Error: Reference source not found, Error: Reference source not found, Error: Reference source not found, and Error: Reference source not found.

Two types of HRD parameter sets (NAL HRD parameters and VCL HRD parameters) are used. The HRD parameter sets are signalled as follows:

–	When the coded video sequence conforms to one or more of the profiles specified in Annex Error: Reference source not found and the decoding process specified in clauses Error: Reference source not found-Error: Reference source not found is applied, the HRD parameter sets are signalled through video usability information as specified in subclauses Error: Reference source not found and Error: Reference source not found, which is part of the sequence parameter set syntax structure.

–	When the coded video sequence conforms to one or more of the profiles specified in Annex Error: Reference source not found and the decoding process specified in Annex Error: Reference source not found is applied, the HRD parameter sets are signalled through the SVC video usability information extension as specified in subclauses Error: Reference source not found and Error: Reference source not found, which is part of the subset sequence parameter set syntax structure.

NOTE 1 – For coded video sequences that conform to both, one or more of the profiles specified in Annex Error: Reference source not found and one or more of the profiles specified in Annex Error: Reference source not found, the signalling of the applicable HRD parameter sets is depending on whether the decoding process specified in clauses Error: Reference source not found-Error: Reference source not found or the decoding process specified in Annex Error: Reference source not found is applied.

–	When the coded video sequence conforms to one or more of the profiles specified in Annex Error: Reference source not found and the decoding process specified in Annex Error: Reference source not found is applied, the HRD parameter sets are signalled through the MVC video usability information extension as specified in subclauses Error: Reference source not found and Error: Reference source not found, which is part of the subset sequence parameter set syntax structure.

NOTE 2 – For coded video sequences that conform to both, one or more of the profiles specified in Annex Error: Reference source not found and one or more of the profiles specified in Annex Error: Reference source not found, the signalling of the applicable HRD parameter sets is depending on whether the decoding process specified in clauses Error: Reference source not found-Error: Reference source not found or the decoding process specified in Annex Error: Reference source not found is applied.

–	When the coded video sequence conforms to one or more of the profiles specified in Annex I and the decoding process specified in Annex I is applied, the HRD parameter sets are signalled through the MVC video usability information extension as specified in subclauses I.14, which is part of the subset sequence parameter set syntax structure.

NOTE 3 – For coded video sequences that conform to one or more of the profiles specified in Annex Error: Reference source not found, one or more of the profiles specified in Annex H and one or more of the profiles specified in Annex I, the signalling of the applicable HRD parameter sets is depending on whether the decoding process specified in clauses Error: Reference source not found-Error: Reference source not found, the decoding process specified in Annex Error: Reference source not found or the decoding process specified in Annex I is applied.

All sequence parameter sets and picture parameter sets referred to in the VCL NAL units, and corresponding buffering period and picture timing SEI messages shall be conveyed to the HRD, in a timely manner, either in the bitstream (by non-VCL NAL units), or by other means not specified in this Recommendation | International Standard.

In Annexes C, Error: Reference source not found, and Error: Reference source not found and subclauses Error: Reference source not found, Error: Reference source not found, Error: Reference source not found, Error: Reference source not found, Error: Reference source not found, Error: Reference source not found, Error: Reference source not found, Error: Reference source not found, and Error: Reference source not found, the specification for "presence" of non-VCL NAL units is also satisfied when those NAL units (or just some of them) are conveyed to decoders (or to the HRD) by other means not specified by this Recommendation | International Standard. For the purpose of counting bits, only the appropriate bits that are actually present in the bitstream are counted.

NOTE 3 – As an example, synchronization of a non-VCL NAL unit, conveyed by means other than presence in the bitstream, with the NAL units that are present in the bitstream, can be achieved by indicating two points in the bitstream, between which the non-VCL NAL unit would have been present in the bitstream, had the encoder decided to convey it in the bitstream.

When the content of a non-VCL NAL unit is conveyed for the application by some means other than presence within the bitstream, the representation of the content of the non-VCL NAL unit is not required to use the same syntax specified in this annex.

NOTE 4 – When HRD information is contained within the bitstream, it is possible to verify the conformance of a bitstream to the requirements of this subclause based solely on information contained in the bitstream. When the HRD information is not present in the bitstream, as is the case for all "stand-alone" Type I bitstreams, conformance can only be verified when the HRD data is supplied by some other means not specified in this Recommendation | International Standard.

The HRD contains a coded picture buffer (CPB), an instantaneous decoding process, a decoded picture buffer (DPB), and output cropping as shown in Figure C-2.

- Figure C-2 – HRD buffer model

The CPB size (number of bits) is CpbSize[ SchedSelIdx ]. The DPB size (number of frame buffers) is Max( 1, max_dec_frame_buffering ). When the coded video sequence conforms to one or more of the profiles specified in Annex Error: Reference source not found and the decoding process specified in Annex Error: Reference source not found is applied, the DPB size is specified in units of view components. When the coded video sequence conforms to one or more of the profiles specified in Annex I and the decoding process specified in Annex I is applied, the DPB size is specified in units of texture view components and depth view components.

The HRD operates as follows. Data associated with access units that flow into the CPB according to a specified arrival schedule are delivered by the HSS. The data associated with each access unit are removed and decoded instantaneously by the instantaneous decoding process at CPB removal times. Each decoded picture is placed in the DPB at its CPB removal time unless it is output at its CPB removal time and is a non-reference picture. When a picture is placed in the DPB it is removed from the DPB at the later of the DPB output time or the time that it is marked as "unused for reference".

For each picture in the bitstream, the variable OutputFlag for the decoded picture and, when applicable, the reference base picture is set as follows:

–	If the coded video sequence containing the picture conforms to one or more of the profiles specified in Annex Error: Reference source not found and the decoding process specified in clauses Error: Reference source not found-Error: Reference source not found is applied, OutputFlag is set equal to 1.

–	Otherwise, if the coded video sequence containing the picture conforms to one or more of the profiles specified in Annex Error: Reference source not found and the decoding process specified in Annex Error: Reference source not found is applied, the following applies:

–	For a reference base picture, OutputFlag is set equal to 0.

–	For a decoded picture, OutputFlag is set equal to the value of the output_flag syntax element of the target layer representation.

–	Otherwise (the coded video sequence containing the picture conforms to one or more of the profiles specified in Annex Error: Reference source not found and the decoding process specified in Annex Error: Reference source not found is applied), the following applies:

–	For the decoded view components of the target output views, OutputFlag is set equal to 1.

–	For the decoded view components of other views, OutputFlag is set equal to 0.

–	Otherwise (the coded video sequence containing the picture conforms to one or more of the profiles specified in Annex I and the decoding process specified in Annex I is applied), the following applies:

–	For the decoded texture view components and depth view component with same VOIdx of the target output views, OutputFlag is set equal to 1.

–	For the decoded texture view components and depth view component with same VOIdx of other views, OutputFlag is set equal to 0.

The operation of the CPB is specified in subclause C.1. The instantaneous decoder operation is specified in clauses Error: Reference source not found-Error: Reference source not found (for coded video sequences conforming to one or more of the profiles specified in Annex Error: Reference source not found) and in Annex Error: Reference source not found (for coded video sequences conforming to one or more of the profiles specified in Annex Error: Reference source not found) and in Annex Error: Reference source not found (for coded video sequences conforming to one or more of the profiles specified in Annex Error: Reference source not found) and in Annex I (for coded video sequences conforming to one or more of the profiles specified in Annex I). The operation of the DPB is specified in subclause C.2. The output cropping is specified in subclause C.2.2.

NOTE 5 – Coded video sequences that conform to both, one or more of the profiles specified in Annex Error: Reference source not found and one or more of the profiles specified in Annex Error: Reference source not found, can be decoded either by the decoding process specified in clauses Error: Reference source not found-Error: Reference source not found or by the decoding process specified in Annex Error: Reference source not found. The decoding result and the HRD operation may be depending on which of the decoding processes is applied.

NOTE 6 – Coded video sequences that conform to both, one or more of the profiles specified in Annex Error: Reference source not found and one or more of the profiles specified in Annex Error: Reference source not found, can be decoded either by the decoding process specified in clauses Error: Reference source not found-Error: Reference source not found or by the decoding process specified in Annex Error: Reference source not found. The decoding result and the HRD operation may be depending on which of the decoding processes is applied.

NOTE 7 – Coded video sequences that conform to one or more of the profiles specified in Annex Error: Reference source not found, one or more of the profiles specified in Annex H and one or more of the profiles specified in Annex I, can be decoded either by the decoding process specified in clauses Error: Reference source not found-Error: Reference source not found, by the decoding process specified in Annex H or by the decoding process specified in Annex I. The decoding result and the HRD operation may be depending on which of the decoding processes is applied.

HSS and HRD information concerning the number of enumerated delivery schedules and their associated bit rates and buffer sizes is specified in subclauses Error: Reference source not found, Error: Reference source not found, Error: Reference source not found, Error: Reference source not found, Error: Reference source not found, Error: Reference source not found, Error: Reference source not found, Error: Reference source not found and I.14. The HRD is initialised as specified by the buffering period SEI message as specified in subclauses Error: Reference source not found and Error: Reference source not found. The removal timing of access units from the CPB and output timing from the DPB are specified in the picture timing SEI message as specified in subclauses Error: Reference source not found and Error: Reference source not found. All timing information relating to a specific access unit shall arrive prior to the CPB removal time of the access unit.

When the coded video sequence conforms to one or more of the profiles specified in Annex Error: Reference source not found and the decoding process specified in Annex Error: Reference source not found is applied, the following is specified:

- When an access unit contains one or more buffering period SEI messages that are included in scalable nesting SEI messages and are associated with values of DQId in the range of ( ( DQIdMax >> 4) << 4 ) to ( ( ( DQIdMax >> 4 ) << 4 ) + 15 ), inclusive, the last of these buffering period SEI messages in decoding order is the buffering period SEI message that initialises the HRD. Let hrdDQId be the largest value of 16 * sei_dependency_id[ i ] + sei_quality_id[ i ] that is associated with the scalable nesting SEI message containing the buffering period SEI message that initialises the HRD, let hrdDId and hrdQId be equal to hrdDQId >> 4 and hrdDQId & 15, respectively, and let hrdTId be the value of sei_temporal_id that is associated with the scalable nesting SEI message containing the buffering period SEI message that initialises the HRD.

- The picture timing SEI messages that specify the removal timing of access units from the CPB and output timing from the DPB are the picture timing SEI messages that are included in scalable nesting SEI messages associated with values of sei_dependency_id[ i ], sei_quality_id[ i ], and sei_temporal_id equal to hrdDId, hrdQId, and hrdTId, respectively.

- The HRD parameter sets that are used for conformance checking are the HRD parameter sets, included in the SVC video usability information extension of the active SVC sequence parameter set, that are associated with values of vui_ext_dependency_id[ i ], vui_ext_quality_id[ i ], and vui_ext_temporal_id[ i ] equal to hrdDId, hrdQId, and hrdTId, respectively. For the specification in this annex, num_units_in_tick, time_scale, fixed_frame_rate_flag, nal_hrd_parameters_present_flag, vcl_hrd_parameters_present_flag, low_delay_hrd_flag, and pic_struct_present_flag are substituted with the values of vui_ext_num_units_in_tick[ i ], vui_ext_time_scale[ i ], vui_ext_fixed_frame_rate_flag[ i ], vui_ext_nal_hrd_parameters_present_flag[ i ], vui_ext_vcl_hrd_parameters_present_flag[ i ], vui_ext_low_delay_hrd_flag[ i ], and vui_ext_pic_struct_present_flag[ i ], respectively, with i being the value for which vui_ext_dependency_id[ i ], vui_ext_quality_id[ i ], and vui_ext_temporal_id[ i ] are equal to hrdDId, hrdQId, and hrdTId, respectively.

When the coded video sequence conforms to one or more of the profiles specified in Annex Error: Reference source not found and the decoding process specified in Annex Error: Reference source not found is applied, the following is specified:

- When an access unit contains one or more buffering period SEI messages that are included in MVC scalable nesting SEI messages, the buffering period SEI message that is associated with the operation point being decoded is the buffering period SEI message that initialises the HRD. Let hrdVId[ i ] be equal to sei_op_view_id[ i ] for all i in the range of 0 to num_view_components_op_minus1, inclusive, and let hrdTId be the value of sei_op_temporal_id, that are associated with the MVC scalable nesting SEI message containing the buffering period SEI message that initialises the HRD.

- The picture timing SEI messages that specify the removal timing of access units from the CPB and output timing from the DPB are the picture timing SEI messages that are included in MVC scalable nesting SEI messages associated with values of sei_op_view_id[ i ] equal to hrdVId[ i ] for all i in the range of 0 to num_view_components_op_minus1, inclusive, and sei_temporal_id equal to hrdTId.

- The HRD parameters that are used for conformance checking are the HRD parameters, included in the MVC video usability information extension of the active MVC sequence parameter set, that are associated with values of vui_mvc_view_id[ i ][ j ] for all j in the range of 0 to vui_mvc_num_target_output_views_minus1[ i ], inclusive, equal to hrdVId[ j ], and the value of vui_mvc_temporal_id[ i ] equal to hrdTId. For the specification in this annex, num_units_in_tick, time_scale, fixed_frame_rate_flag, nal_hrd_parameters_present_flag, vcl_hrd_parameters_present_flag, low_delay_hrd_flag, and pic_struct_present_flag are substituted with the values of vui_mvc_num_units_in_tick[ i ], vui_mvc_time_scale[ i ], vui_mvc_fixed_frame_rate_flag[ i ], vui_mvc_nal_hrd_parameters_present_flag[ i ], vui_mvc_vcl_hrd_parameters_present_flag[ i ], vui_mvc_low_delay_hrd_flag[ i ], and vui_mvc_pic_struct_present_flag[ i ], respectively, with i being the value for which vui_mvc_view_id[ i ] is equal to hrdVId[ j ] for all j in the range of 0 to vui_mvc_num_traget_output_views_minus1[ i ], inclusive, and vui_mvc_temporal_id[ i ] equal to hrdTId.

When the coded video sequence conforms to one or more of the profiles specified in Annex I and the decoding process specified in Annex I is applied, the following is specified:

- When an access unit contains one or more buffering period SEI messages that are included in 3DVC scalable nesting SEI messages, the buffering period SEI message that is associated with the operation point being decoded is the buffering period SEI message that initialises the HRD. Let hrdVId[ i ] be equal to sei_op_view_id[ i ] for all i in the range of 0 to num_view_components_op_minus1, inclusive, and let hrdTId be the value of sei_op_temporal_id, that are associated with the 3DVC scalable nesting SEI message containing the buffering period SEI message that initialises the HRD.

- The picture timing SEI messages that specify the removal timing of access units from the CPB and output timing from the DPB are the picture timing SEI messages that are included in 3DVC scalable nesting SEI messages associated with values of sei_op_view_id[ i ] equal to hrdVId[ i ] for all i in the range of 0 to num_view_components_op_minus1, inclusive, and sei_temporal_id equal to hrdTId.

- The HRD parameter sets that are used for conformance checking are the HRD parameter sets, included in the MVC video usability information extension of the active 3DV sequence parameter set, that are associated with values of vui_mvc_view_id[ i ][ j ] for all j in the range of 0 to vui_mvc_num_target_output_views_minus1[ i ], inclusive, equal to hrdVId[ j ], and the value of vui_mvc_temporal_id[ i ] equal to hrdTId. For the specification in this annex, num_units_in_tick, time_scale, fixed_frame_rate_flag, nal_hrd_parameters_present_flag, vcl_hrd_parameters_present_flag, low_delay_hrd_flag, and pic_struct_present_flag are substituted with the values of vui_mvc_num_units_in_tick[ i ], vui_mvc_time_scale[ i ], vui_mvc_fixed_frame_rate_flag[ i ], vui_mvc_nal_hrd_parameters_present_flag[ i ], vui_mvc_vcl_hrd_parameters_present_flag[ i ], vui_mvc_low_delay_hrd_flag[ i ], and vui_mvc_pic_struct_present_flag[ i ], respectively, with i being the value for which vui_mvc_view_id[ i ] is equal to hrdVId[ j ] for all j in the range of 0 to vui_mvc_num_traget_output_views_minus1[ i ], inclusive, and vui_mvc_temporal_id[ i ] equal to hrdTId.

The HRD is used to check conformance of bitstreams and decoders as specified in subclauses C.3 and C.4, respectively.

NOTE 7 – While conformance is guaranteed under the assumption that all frame-rates and clocks used to generate the bitstream match exactly the values signalled in the bitstream, in a real system each of these may vary from the signalled or specified value.

All the arithmetic in this annex is done with real values, so that no rounding errors can propagate. For example, the number of bits in a CPB just prior to or after removal of an access unit is not necessarily an integer.

The variable tc is derived as follows and is called a clock tick:

tc = num_units_in_tick  time_scale		(C-1)

The following is specified for expressing the constraints in this annex:

–	Let access unit n be the n-th access unit in decoding order with the first access unit being access unit 0.

–	Let picture n be the primary coded picture or the decoded primary picture of access unit n.

  - Operation of coded picture buffer (CPB)

The specifications in this subclause apply independently to each set of CPB parameters that is present and to both the Type I and Type II conformance points shown in Figure C-1.

    - Timing of bitstream arrival

The HRD may be initialised at any one of the buffering period SEI messages. Prior to initialisation, the CPB is empty.

NOTE – After initialisation, the HRD is not initialised again by subsequent buffering period SEI messages.

- Each access unit is referred to as access unit n, where the number n identifies the particular access unit. The access unit that is associated with the buffering period SEI message that initialises the CPB is referred to as access unit 0. The value of n is incremented by 1 for each subsequent access unit in decoding order.

The time at which the first bit of access unit n begins to enter the CPB is referred to as the initial arrival time tai( n ).

The initial arrival time of access units is derived as follows:

–	If the access unit is access unit 0, tai( 0 ) = 0,

–	Otherwise (the access unit is access unit n with n > 0), the following applies:

–	If cbr_flag[ SchedSelIdx ] is equal to 1, the initial arrival time for access unit n, is equal to the final arrival time (which is derived below) of access unit n − 1, i.e.,

tai( n ) = taf( n − 1 )		(C-2)

–	Otherwise (cbr_flag[ SchedSelIdx ] is equal to 0), the initial arrival time for access unit n is derived by

tai( n ) = Max( taf( n − 1 ), tai,earliest( n ) )		(C-3)

where tai,earliest( n ) is derived as follows:

–	If access unit n is not the first access unit of a subsequent buffering period, tai,earliest( n ) is derived as

tai,earliest( n ) = tr,n( n ) − ( initial_cpb_removal_delay[ SchedSelIdx ] +
                                       initial_cpb_removal_delay_offset[ SchedSelIdx ] )  90000	(C-4)

with tr,n( n ) being the nominal removal time of access unit n from the CPB as specified in subclause C.1.2 and initial_cpb_removal_delay[ SchedSelIdx ] and initial_cpb_removal_delay_offset[ SchedSelIdx ] being specified in the previous buffering period SEI message.

–	Otherwise (access unit n is the first access unit of a subsequent buffering period), tai,earliest( n ) is derived as

tai,earliest( n ) = tr,n( n ) − ( initial_cpb_removal_delay[ SchedSelIdx ]  90000 )	(C-5)

with initial_cpb_removal_delay[ SchedSelIdx ] being specified in the buffering period SEI message associated with access unit n.

The final arrival time for access unit n is derived by

taf( n ) = tai( n ) + b( n )  BitRate[ SchedSelIdx ]		(C-6)

- where b( n ) is the size in bits of access unit n, counting the bits of the VCL NAL units and the filler data NAL units for the Type I conformance point or all bits of the Type II bitstream for the Type II conformance point, where the Type I and Type II conformance points are as shown in Figure C-1.

The values of SchedSelIdx, BitRate[ SchedSelIdx ], and CpbSize[ SchedSelIdx ] are constrained as follows:

–	If the content of the active sequence parameter sets for access unit n and access unit n − 1 differ, the HSS selects a value SchedSelIdx1 of SchedSelIdx from among the values of SchedSelIdx provided in the active sequence parameter set for access unit n that results in a BitRate[ SchedSelIdx1 ] or CpbSize[ SchedSelIdx1 ] for access unit n. The value of BitRate[ SchedSelIdx1 ] or CpbSize[ SchedSelIdx1 ] may differ from the value of BitRate[ SchedSelIdx0 ] or CpbSize[ SchedSelIdx0 ] for the value SchedSelIdx0 of SchedSelIdx that was in use for access unit n − 1.

–	Otherwise, the HSS continues to operate with the previous values of SchedSelIdx, BitRate[ SchedSelIdx ] and CpbSize[ SchedSelIdx ].

When the HSS selects values of BitRate[ SchedSelIdx ] or CpbSize[ SchedSelIdx ] that differ from those of the previous access unit, the following applies:

–	the variable BitRate[ SchedSelIdx ] comes into effect at time tai( n )

–	the variable CpbSize[ SchedSelIdx ] comes into effect as follows:

–	If the new value of CpbSize[ SchedSelIdx ] exceeds the old CPB size, it comes into effect at time tai( n ),

–	Otherwise, the new value of CpbSize[ SchedSelIdx ] comes into effect at the time tr( n ).

    - Timing of coded picture removal

When an access unit n is the access unit with n equal to 0 (the access unit that initialises the HRD), the nominal removal time of the access unit from the CPB is specified by

tr,n( 0 ) = initial_cpb_removal_delay[ SchedSelIdx ]  90000	(C-7)

When an access unit n is the first access unit of a buffering period that does not initialise the HRD, the nominal removal time of the access unit from the CPB is specified by

tr,n( n ) = tr,n( nb ) + tc * cpb_removal_delay( n )		(C-8)

where tr,n( nb ) is the nominal removal time of the first access unit of the previous buffering period and cpb_removal_delay( n ) is the value of cpb_removal_delay specified in the picture timing SEI message associated with access unit n.

The nominal removal time tr,n(n) of an access unit n that is not the first access unit of a buffering period is given by

tr,n( n ) = tr,n( nb ) + tc * cpb_removal_delay( n )		(C-9)

where tr,n( nb ) is the nominal removal time of the first access unit of the current buffering period and cpb_removal_delay( n ) is the value of cpb_removal_delay specified in the picture timing SEI message associated with access unit n.

The removal time of access unit n is specified as follows:

–	If low_delay_hrd_flag is equal to 0 or tr,n( n ) >= taf( n ), the removal time of access unit n is specified by

tr( n ) = tr,n( n )		(C-10)

–	Otherwise (low_delay_hrd_flag is equal to 1 and tr,n( n ) < taf( n )), the removal time of access unit n is specified by

tr( n ) = tr,n( n ) + tc * Ceil( ( taf( n ) − tr,n( n ) ) tc ) 		(C-11)

NOTE – The latter case indicates that the size of access unit n, b( n ), is so large that it prevents removal at the nominal removal time.

When an access unit n is the first access unit of a buffering period, nb is set equal to n at the removal time tr( n ) of the access unit n.

  - Operation of the decoded picture buffer (DPB)

The decoded picture buffer contains frame buffers. When a coded video sequence conforming to one or more of the profiles specified in Annex Error: Reference source not found is decoded by applying the decoding process specified in clauses Error: Reference source not found-Error: Reference source not found, each of the frame buffers may contain a decoded frame, a decoded complementary field pair or a single (non-paired) decoded field that is marked as "used for reference" (reference pictures) or is held for future output (reordered or delayed pictures). When a coded video sequence conforming to one or more of the profiles specified in Annex Error: Reference source not found is decoded by applying the decoding process specified in Annex Error: Reference source not found, each frame buffer may contain a decoded frame, a decoded complementary field pair, a single (non-paired) decoded field, a decoded reference base frame, a decoded reference base complementary field pair or a single (non-paired) decoded reference base field that is marked as "used for reference" (reference pictures) or is held for future output (reordered or delayed pictures). When a coded video sequence conforming to one or more of the profiles specified in Annex Error: Reference source not found is decoded by applying the decoding process specified in Annex Error: Reference source not found, each of the frame buffers may contain a decoded frame view component, a decoded complementary field view component pair, or a single (non-paired) decoded field view component that is marked as "used for reference" (reference pictures) or is held for future output (reordered or delayed pictures) or is held as reference for inter-view prediction (inter-view only reference components). When a coded video sequence conforming to one or more of the profiles specified in Annex I is decoded by applying the decoding process specified in Annex I, each of the frame buffers may contain a decoded texture frame view component, a decoded depth frame view component, a decoded complementary texture field view component pair, or a single (non-paired) decoded texture field view component that is marked as "used for reference" (reference pictures) or is held for future output (reordered or delayed pictures) or is held as reference for inter-view prediction (inter-view only reference components).

Prior to initialisation, the DPB is empty (the DPB fullness is set to zero). The following steps specified in this subclause all happen instantaneously at tr( n ) and in the order listed. When the decoding process specified in Annex Error: Reference source not found or Annex I is applied, the view components of the current primary coded picture are processed by applying the ordered steps to each view component in increasing order of the associated view order index VOIdx. During the invocation of the process for a particular texture view, only the texture view components of the particular view are considered and during the invocation of the process for a particular depth view, only the depth view components of the particular view are considered. Prior to initialisation, the DPB is empty (the DPB fullness is set to zero). Inside each view component of the current primary coded picture, depth view component if present, is processed after the texture view component within the same view component.

- The process of decoding gaps in frame_num and storing "non-existing" frames as specified in subclause C.2.1 is invoked.

- The picture decoding and output process as specified in subclause C.2.2 is invoked.

- The process of removing pictures from the DPB before possible insertion of the current picture as specified in subclause C.2.3 is invoked.

- The process of marking and storing the current decoded picture as specified in subclause C.2.4 is invoked.

NOTE – When the decoding process specified in Annex Error: Reference source not found is applied, the DPB is only operated for decoded pictures and reference base pictures associated with decoded pictures. The DPB is not operated for layer pictures with dependency_id less than DependencyIdMax (and associated reference base pictures). All decoded pictures and associated reference base pictures are decoded pictures and associated reference base pictures for dependency_id equal to DependencyIdMax, which represent the results of the decoding process specified in subclause Error: Reference source not found.

    - Decoding of gaps in frame_num and storage of "non-existing" frames

When the decoding process specified in Annex Error: Reference source not found is applied, the process specified in this subclause is invoked for a particular view with view order index VOIdx, with "picture" being replaced by "view component", "frame" being replaced by "frame view component", and "field" being replaced by "field view component". During the invocation of the process for a particular view, only view components of the particular view are considered and view components of other views are not marked as "unused for reference" or removed from the DPB. The DPB fullness represents the total number of non-empty frame buffers, including frame buffers that contain view components of other views. When the decoding process specified in Annex I is applied, the process specified in this subclause for Annex Error: Reference source not found is invoked for particular texture view or depth view with view order index VOIdx, with each "view component" being replaced by "texture view component" or "depth view component", "frame view component" being replaced by "texture frame view component" or "depth frame view component", and "field view component" being replaced by "texture field view component". During the invocation of the process for a particular texture view, only the texture view components of the particular view are considered and during the invocation of the process for a particular depth view, only the depth view components of the particular view are considered and view components of other views are not marked as "unused for reference" or removed from the DPB. The DPB fullness represents the total number of non-empty frame buffers, including frame buffers that contain view components of other views.

When applicable, gaps in frame_num are detected by the decoding process and the generated frames are marked and inserted into the DPB as specified below.

Gaps in frame_num are detected by the decoding process and the generated frames are marked as specified in subclauses Error: Reference source not found and Error: Reference source not found.

After the marking of each generated frame, each picture m marked by the "sliding window" process as "unused for reference" is removed from the DPB when it is also marked as "non-existing" or its DPB output time is less than or equal to the CPB removal time of the current picture n; i.e., to,dpb( m ) <= tr( n ), or it has OutputFlag equal to 0. When a frame or the last field in a frame buffer is removed from the DPB, the DPB fullness is decremented by one. The "non-existing" generated frame is inserted into the DPB and the DPB fullness is incremented by one.

    - Picture decoding and output

When the decoding process specified in Annex Error: Reference source not found is applied, the process specified in this subclause is invoked for a particular view with view order index VOIdx.

When the decoding process specified in Annex I is applied, the process specified in this subclause in a particular texture view or depth view with view order index VOIdx.

The decoding of the current picture or view component (when applying the decoding process specified in Annex Error: Reference source not found or Annex I) and the derivation of the DPB output time (if applicable) is specified as follows:

–	If the decoding process specified in clause Error: Reference source not found or Annex Error: Reference source not found is applied, the following applies:

–	The current primary coded picture n is decoded.

–	When picture n has OutputFlag equal to 1, its DPB output time to,dpb( n ) is derived by

to,dpb( n ) = tr( n ) + tc * dpb_output_delay( n )		(C-12)

where dpb_output_delay( n ) is the value of dpb_output_delay specified in the picture timing SEI message associated with access unit n.

–	Otherwise (the decoding process specified in Annex Error: Reference source not found or Annex I is applied), the following applies:

–	The view component with view order index VOIdx of the current primary coded picture n is decoded.

–	When VOIdx is equal to VOIdxMin and any of the view components of picture n has OutputFlag equal to 1, the DPB output time to,dpb( n ) for picture n is derived by Equation C-12, where dpb_output_delay( n ) is the value of dpb_output_delay specified in the picture timing SEI message associated with access unit n.

The output of the current picture or view component (when applying the decoding process specified in Annex Error: Reference source not found or Annex I) is specified as follows:

–	If OutputFlag is equal to 1 and to,dpb( n ) = tr( n ), the current picture or view component is output.

NOTE 1 – When the current picture or view component has nal_ref_idc greater than 0 (when using the decoding process specified in Annex Error: Reference source not found, nal_ref_idc is the syntax element of the target layer representation), it will be stored in the DPB.

–	Otherwise, if OutputFlag is equal to 0, the current picture or view component is not output, but it may be stored in the DPB as specified in subclause C.2.4.

–	Otherwise (OutputFlag is equal to 1 and to,dpb( n ) > tr( n ) ), the current picture or view component is output later and will be stored in the DPB (as specified in subclause C.2.4) and is output at time to,dpb( n ) unless indicated not to be output by the decoding or inference of no_output_of_prior_pics_flag equal to 1 at a time that precedes to,dpb( n ).

NOTE 2 – When the coded video sequence conforms to a profile specified in Annex Error: Reference source not found and the decoding process specified in Annex Error: Reference source not found is used, the view components of all the target output views of a picture are output at the same time instant and in increasing order of the view order index VOIdx.

NOTE 3 – When the coded video sequence conforms to a profile specified in Annex I and the decoding process specified in Annex I is used, the view components of all the target output views of a picture are output at the same time instant and in increasing order of the view order index VOIdx. A depth view component, if present, follows the texture view component within the same view component.

When output, the picture or view component shall be cropped, using the cropping rectangle specified in the active sequence parameter set for the picture or view component.

When the decoding process specified in clause Error: Reference source not found or Annex Error: Reference source not found is applied, the current picture n is a picture that is output and is not the last picture of the bitstream that is output, the value of to,dpb( n ) is derived by

to,dpb( n ) = to,dpb( nn ) − to,dpb( n )		(C-13)

where nn indicates the picture that follows after picture n in output order and has OutputFlag equal to 1.

When the decoding process specified in Annex Error: Reference source not found or Annex I is applied, the current picture n is a picture that contains at least one view component that is output and the current picture is not the last picture of the bitstream that contains at least one view component that is output and VOIdx is equal to VOIdxMin, the value of to,dpb( n ) is derived by Equation C-13, where nn indicates the picture that follows after picture n in output order and contains at least one any view component with OutputFlag equal to 1.

The decoded picture or view component is temporarily stored (not in the DPB).

    - Removal of pictures from the DPB before possible insertion of the current picture

When the decoding process specified in Annex Error: Reference source not found is applied, the process specified in this subclause is invoked for a particular view with view order index VOIdx, with "picture" being replaced by "view component", "frame" being replaced by "frame view component", and "field" being replaced by "field view component".

When the decoding process specified in Annex I is applied, the process specified in this subclause for Annex Error: Reference source not found is invoked for particular texture view and depth view with view order index VOIdx, with each "view component" being replaced by "texture view component" or "depth view component", "frame view component" being replaced by "texture frame view component" or "depth frame view component", and "field view component" being replaced by "texture field view component". During the invocation of the process for a particular texture view, only the texture view components of the particular view are considered and during the invocation of the process for a particular depth view, only the depth view components of the particular view are considered.

When the decoding process specified in Annex Error: Reference source not found or Annex I is applied, the following process is specified for removing inter-view only reference components of the current access unit from the DPB. By this process, view components of the current view with view order index VOIdx are not removed from the DPB, but inter-view only reference components of other views may be removed. The removal of inter-view only reference components is specified as follows:

–	If the view order index VOIdx of the current view is equal to VOIdxMax, all inter-view only reference components m for which any of the following conditions are true are removed from the DPB:

–	OutputFlag is equal to 0,

–	The DPB output time to,dpb( m ) of the picture containing the view component m is less than or equal to the CPB removal time tr( n ) of the current picture.

–	Otherwise (the view order index VOIdx of the current view is less than VOIdxMax), all inter-view only reference components m for which both of the following conditions are true are removed from the DPB:

–	OutputFlag is equal to 0 or the DPB output time to,dpb( m ) of the picture containing the view component m is less than or equal to the CPB removal time tr( n ) of the current picture,

–	One of the following conditions is true:

–	the current view component is a view component of an anchor picture and the view_id of the inter-view only reference component m is not equal to any value of anchor_ref_lX[ k ][ j ], with X being equal to 0 or 1, k being any integer value greater than the view order index VOIdx of the current view, and j being any integer value in the range of 0 to Max( 0, num_anchor_refs_lX[ k ] − 1 ), inclusive,

–	the current view component is not a view component of an anchor picture and the view_id of the inter-view only reference component m is not equal to any value of non_anchor_ref_lX[ k ][ j ], with X being equal to 0 or 1, k being any integer value greater than the view order index VOIdx of the current view, and j being any integer value in the range of 0 to Max( 0, num_non_anchor_refs_lX[ k ] − 1 ), inclusive.

When the decoding process specified in Annex Error: Reference source not found is applied, for the following processes specified in this subclause, only view components of the particular view for which this subclause is invoked are considered, and view components of other views are not marked as "unused for reference" or removed from the DPB. When the decoding process specified in Annex I is applied, for the following processes specified for Annex I in this subclause, during the invocation of the process for a particular texture view, only texture view components of the particular texture view are considered and during the invocation of the process for a particular depth view, only depth view components of the particular depth view  are considered, and view components of other views are not marked as "unused for reference" or removed from the DPB. The DPB fullness represents the total number of non-empty frame buffers, including frame buffers that contain texture or depth view components of the current view and other views.

The removal of pictures from the DPB before possible insertion of the current picture proceeds as follows:

–	If the decoded picture is an IDR picture the following applies:

- All reference pictures in the DPB are marked as "unused for reference" as specified in subclause Error: Reference source not found when a coded video sequence conforming to one or more of the profiles specified in Annex Error: Reference source not found is decoded by applying the decoding process specified in clauses Error: Reference source not found-Error: Reference source not found, or as specified in subclause Error: Reference source not found when a coded video sequence conforming to one or more of the profiles specified in Annex Error: Reference source not found is decoded by applying the decoding process specified in Annex Error: Reference source not found, or as specified in subclause Error: Reference source not found when a coded video sequence conforming to one or more of the profiles specified in Annex Error: Reference source not found is decoded by applying the decoding process specified in Annex Error: Reference source not found, or as specified in subclause Error: Reference source not found when a coded video sequence conforming to one or more of the profiles specified in Annex I is decoded by applying the decoding process specified in Annex I.

- When the IDR picture is not the first IDR picture decoded and the value of PicWidthInMbs or FrameHeightInMbs or max_dec_frame_buffering derived from the active sequence parameter set is different from the value of PicWidthInMbs or FrameHeightInMbs or max_dec_frame_buffering derived from the sequence parameter set that was active for the preceding picture, respectively, no_output_of_prior_pics_flag is inferred to be equal to 1 by the HRD, regardless of the actual value of no_output_of_prior_pics_flag.

NOTE – Decoder implementations should try to handle frame or DPB size changes more gracefully than the HRD in regard to changes in PicWidthInMbs or FrameHeightInMbs.

- When no_output_of_prior_pics_flag is equal to 1 or is inferred to be equal to 1, all frame buffers in the DPB are emptied without output of the pictures they contain, and DPB fullness is set to 0.

–	Otherwise (the decoded picture is not an IDR picture), the following applies:

–	If the slice header of the current picture includes memory_management_control_operation equal to 5, all reference pictures in the DPB are marked as "unused for reference".

–	Otherwise (the slice header of the current picture does not include memory_management_control_operation equal to 5), the decoded reference picture marking process specified in subclause Error: Reference source not found is invoked when a coded video sequence conforming to one or more of the profiles specified in Annex Error: Reference source not found is decoded by applying the decoding process specified in clauses Error: Reference source not found-Error: Reference source not found, or the decoded reference picture marking process specified in subclause Error: Reference source not found is invoked when a coded video sequence conforming to one or more of the profiles specified in Annex Error: Reference source not found is decoded by applying the decoding process specified in Annex Error: Reference source not found, or the decoded reference picture marking process specified in subclause Error: Reference source not found is invoked when a coded video sequence conforming to one or more of the profiles specified in Annex Error: Reference source not found is decoded by applying the decoding process specified in Annex Error: Reference source not found, or the decoded reference picture marking process specified in subclause Error: Reference source not found is invoked when a coded video sequence conforming to one or more of the profiles specified in Annex I is decoded by applying the decoding process specified in Annex I.

All pictures m in the DPB, for which all of the following conditions are true, are removed from the DPB:

–	picture m is marked as "unused for reference" or picture m is a non-reference picture. When a picture is a reference frame, it is considered to be marked as "unused for reference" only when both of its fields have been marked as "unused for reference",

–	picture m is marked as "non-existing" or it has OutputFlag equal to 0 or its DPB output time to,dpb( m ) is less than or equal to the CPB removal time tr( n ) of the current picture n.

When a frame or the last field in a frame buffer is removed from the DPB, the DPB fullness is decremented by one.

    - Current decoded picture marking and storage

When the decoding process specified in Annex Error: Reference source not found is applied, the process specified in this subclause is invoked for a particular view with view order index VOIdx, with "picture" being replaced by "view component", "frame" being replaced by "frame view component", and "field" being replaced by "field view component". When the decoding process specified in Annex I is applied, the process specified in this subclause for Annex Error: Reference source not found is invoked for particular texture view and depth view with view order index VOIdx, with each "view component" being replaced by "texture view component" and "depth view component", "frame view component" being replaced by "texture frame view component" and "depth frame view component", and "field view component" being replaced by "texture field view component".In subclause C.2.4.2, the DPB output time to,dpb(n) and the CPB removal time tr(n) of a view component are the DPB output time and the CPB removal time of the picture n containing the view component, depth view component or texture view component.

The marking and storage of the current decoded picture is specified as follows:

–	If the current picture is a reference picture, the marking and storage process for reference pictures as specified in subclause C.2.4.1 is invoked.

–	Otherwise (the current picture is a non-reference picture), the storage process for non-reference pictures as specified in subclause C.2.4.2 is invoked.

      - Marking and storage of a reference picture into the DPB

The current picture is stored in the DPB as follows:

–	If the current decoded picture is a second field (in decoding order) of a complementary reference field pair, and the first field of the pair is still in the DPB, the current decoded picture is stored in the same frame buffer as the first field of the pair.

–	Otherwise, the current decoded picture is stored in an empty frame buffer, and the DPB fullness is incremented by one.

When the coded video sequence conforms to one or more of the profiles specified in Annex Error: Reference source not found and the decoding process specified in Annex Error: Reference source not found is applied and the current picture has store_ref_base_pic_flag equal to 1 (i.e., the current picture is associated with a reference base picture), the associated reference base picture is stored in the DPB as follows:

–	If the reference base picture is a second field (in decoding order) of a complementary reference base field pair, and the first field of the pair is still in the DPB, the reference base picture is stored in the same frame buffer as the first field of the pair.

–	Otherwise, the reference base picture is stored in an empty frame buffer, and the DPB fullness is incremented by one.

      - Storage of a non-reference picture into the DPB

The variable storePicFlag is derived as follows:

–	If any of the following conditions are true, storePicFlag is set equal to 1:

–	the current picture n has OutputFlag equal to 1 and to,dpb(n) > tr(n),

–	the decoding process specified in Annex Error: Reference source not found or Annex I is used and the current view component has a view order index VOIdx less than VOIdxMax and inter_view_flag equal to 1.

–	Otherwise, storePicFlag is set equal to 0.

When storePicFlag is equal to 1, the current picture is stored in the DPB as follows:

–	If the current decoded picture is a second field (in decoding order) of a complementary non-reference field pair, and the first field of the pair is still in the DPB, the current decoded picture is stored in the same frame buffer as the first field of the pair.

–	Otherwise, the current decoded picture is stored in an empty frame buffer, and the DPB fullness is incremented by one.

  - Bitstream conformance

A bitstream of coded data conforming to this Recommendation | International Standard fulfils the following requirements.

The bitstream is constructed according to the syntax, semantics, and constraints specified in this Recommendation | International Standard outside of this annex.

The bitstream is tested by the HRD as specified below:

For Type I bitstreams, the number of tests carried out is equal to cpb_cnt_minus1 + 1 where cpb_cnt_minus1 is either the syntax element of hrd_parameters( ) following the vcl_hrd_parameters_present_flag or is determined by the application by other means not specified in this Recommendation | International Standard. One test is carried out for each bit rate and CPB size combination specified by hrd_parameters( ) following the vcl_hrd_parameters_present_flag. Each of these tests is conducted at the Type I conformance point shown in Figure C-1.

For Type II bitstreams there are two sets of tests. The number of tests of the first set is equal to cpb_cnt_minus1 + 1 where cpb_cnt_minus1 is either the syntax element of hrd_parameters( ) following the vcl_hrd_parameters_present_flag or is determined by the application by other means not specified in this Recommendation | International Standard. One test is carried out for each bit rate and CPB size combination. Each of these tests is conducted at the Type I conformance point shown in Figure C-1. For these tests, only VCL and filler data NAL units are counted for the input bit rate and CPB storage.

The number of tests of the second set, for Type II bitstreams, is equal to cpb_cnt_minus1 + 1 where cpb_cnt_minus1 is either the syntax element of hrd_parameters( ) following the nal_hrd_parameters_present_flag or is determined by the application by other means not specified in this Recommendation | International Standard. One test is carried out for each bit rate and CPB size combination specified by hrd_parameters( ) following the nal_hrd_parameters_present_flag. Each of these tests is conducted at the Type II conformance point shown in Figure C-1. For these tests, all NAL units (of a Type II NAL unit stream) or all bytes (of a byte stream) are counted for the input bit rate and CPB storage.

NOTE 1 – NAL HRD parameters established by a value of SchedSelIdx for the Type II conformance point shown in Figure C-1 are sufficient to also establish VCL HRD conformance for the Type I conformance point shown in Figure C-1 for the same values of initial_cpb_removal_delay[ SchedSelIdx ], BitRate[ SchedSelIdx ], and CpbSize[ SchedSelIdx ] for the VBR case (cbr_flag[ SchedSelIdx ] equal to 0). This is because the data flow into the Type I conformance point is a subset of the data flow into the Type II conformance point and because, for the VBR case, the CPB is allowed to become empty and stay empty until the time a next picture is scheduled to begin to arrive. For example, when a coded video sequence conforming to one or more of the profiles specified in Annex Error: Reference source not found is decoded by applying the decoding process specified in clauses Error: Reference source not found-Error: Reference source not found, when NAL HRD parameters are provided for the Type II conformance point that not only fall within the bounds set for NAL HRD parameters for profile conformance in item Error: Reference source not found of subclause Error: Reference source not found or item Error: Reference source not found of subclause Error: Reference source not found (depending on the profile in use) but also fall within the bounds set for VCL HRD parameters for profile conformance in item Error: Reference source not found of subclause Error: Reference source not found or item Error: Reference source not found of subclause Error: Reference source not found (depending on the profile in use), conformance of the VCL HRD for the Type I conformance point is also assured to fall within the bounds of item Error: Reference source not found of subclause Error: Reference source not found.

For conforming bitstreams, all of the following conditions shall be fulfilled for each of the tests:

- For each access unit n, with n>0, associated with a buffering period SEI message, with tg,90( n ) specified by

tg,90( n ) = 90000 * ( tr,n( n ) − taf( n − 1 ) )		(C-14)

the value of initial_cpb_removal_delay[ SchedSelIdx ] shall be constrained as follows:

–	If cbr_flag[ SchedSelIdx ] is equal to 0,

initial_cpb_removal_delay[ SchedSelIdx ] <= Ceil( tg,90( n ) )	(C-15)

–	Otherwise (cbr_flag[ SchedSelIdx ] is equal to 1),

Floor( tg,90( n ) ) <= 	initial_cpb_removal_delay[ SchedSelIdx ] <= Ceil( tg,90( n ) )	(C-16)

- NOTE 2 – The exact number of bits in the CPB at the removal time of each picture may depend on which buffering period SEI message is selected to initialise the HRD. Encoders must take this into account to ensure that all specified constraints must be obeyed regardless of which buffering period SEI message is selected to initialise the HRD, as the HRD may be initialised at any one of the buffering period SEI messages.

- A CPB overflow is specified as the condition in which the total number of bits in the CPB is larger than the CPB size. The CPB shall never overflow.

- A CPB underflow is specified as the condition in which tr,n( n ) is less than taf( n ). When low_delay_hrd_flag is equal to 0, the CPB shall never underflow.

- The nominal removal times of pictures from the CPB (starting from the second picture in decoding order), shall satisfy the constraints on tr,n( n ) and tr( n ) expressed in subclauses Error: Reference source not found through Error: Reference source not found for the profile and level specified in the bitstream when a coded video sequence conforming to one or more of the profiles specified in Annex Error: Reference source not found is decoded by applying the decoding process specified in clauses Error: Reference source not found-Error: Reference source not found, and they shall satisfy the constraints on tr,n( n ) and tr( n ) expressed in subclauses Error: Reference source not found and Error: Reference source not found for profile and level specified in the bitstream when a coded video sequence conforming to one or more of the profiles specified in Annex Error: Reference source not found is decoded by applying the decoding process specified in Annex Error: Reference source not found, and they shall satisfy the constraints on tr,n( n ) and tr( n ) expressed in subclause Error: Reference source not found for the profile and level specified in the bitstream when a coded video sequence conforming to one or more of the profiles specified in Annex Error: Reference source not found is decoded by applying the decoding process specified in Annex Error: Reference source not found, and they shall satisfy the constraints on tr,n( n ) and tr( n ) expressed in subclause Error: Reference source not found for the profile and level specified in the bitstream when a coded video sequence conforming to one or more of the profiles specified in Annex I is decoded by applying the decoding process specified in Annex I.

- Immediately after any decoded picture is added to the DPB, the fullness of the DPB shall be less than or equal to the DPB size as constrained by Annexes Error: Reference source not found, Error: Reference source not found, and Error: Reference source not found and subclauses Error: Reference source not found, Error: Reference source not found, Error: Reference source not found, Error: Reference source not found, Error: Reference source not found, Error: Reference source not found, and I.14 for the profile and level specified in the bitstream.

- All reference pictures shall be present in the DPB when needed for prediction. Each picture shall be present in the DPB at its DPB output time unless it is not stored in the DPB at all, or is removed from the DPB before its output time by one of the processes specified in subclause C.2.

- The value of to,dpb( n ) as given by Equation C-13, which is the difference between the output time of a picture and that of the first picture following it in output order and having OutputFlag equal to 1, shall satisfy the constraint expressed in subclause Error: Reference source not found for the profile and level specified in the bitstream when a coded video sequence conforming to one or more of the profiles specified in Annex Error: Reference source not found is decoded by applying the decoding process specified in clauses Error: Reference source not found-Error: Reference source not found, and it shall satisfy the constraint expressed in subclause Error: Reference source not found for profile and level specified in the bitstream when a coded video sequence conforming to one or more of the profiles specified in Annex Error: Reference source not found is decoded by applying the decoding process specified in Annex Error: Reference source not found, and it shall satisfy the constraints expressed in subclause Error: Reference source not found for the profile and level specified in the bitstream when a coded video sequence conforming to one or more of the profiles specified in Annex Error: Reference source not found is decoded by applying the decoding process specified in Annex Error: Reference source not found, and it shall satisfy the constraints expressed in subclause Error: Reference source not found for the profile and level specified in the bitstream when a coded video sequence conforming to one or more of the profiles specified in Annex I is decoded by applying the decoding process specified in Annex I.

  - Decoder conformance

A decoder conforming to this Recommendation | International Standard fulfils the following requirements.

A decoder claiming conformance to a specific profile and level shall be able to decode successfully all conforming bitstreams specified for decoder conformance in subclause C.3, provided that all sequence parameter sets and picture parameter sets referred to in the VCL NAL units, and appropriate buffering period and picture timing SEI messages are conveyed to the decoder, in a timely manner, either in the bitstream (by non-VCL NAL units), or by external means not specified by this Recommendation | International Standard.

There are two types of conformance that can be claimed by a decoder: output timing conformance and output order conformance.

To check conformance of a decoder, test bitstreams conforming to the claimed profile and level, as specified in subclause C.3 are delivered by a hypothetical stream scheduler (HSS) both to the HRD and to the decoder under test (DUT). All pictures output by the HRD shall also be output by the DUT and, for each picture output by the HRD, the values of all samples that are output by the DUT for the corresponding picture shall be equal to the values of the samples output by the HRD.

For output timing decoder conformance, the HSS operates as described above, with delivery schedules selected only from the subset of values of SchedSelIdx for which the bit rate and CPB size are restricted as specified in Annex Error: Reference source not found, Annex Error: Reference source not found, Annex Error: Reference source not found and Annex I for the specified profile and level, or with "interpolated" delivery schedules as specified below for which the bit rate and CPB size are restricted as specified in Annex Error: Reference source not found, Annex Error: Reference source not found, Annex Error: Reference source not found, and Annex I. The same delivery schedule is used for both the HRD and DUT.

When the HRD parameters and the buffering period SEI messages are present with cpb_cnt_minus1 greater than 0, the decoder shall be capable of decoding the bitstream as delivered from the HSS operating using an "interpolated" delivery schedule specified as having peak bit rate r, CPB size c( r ), and initial CPB removal delay ( f( r )r ) as follows:

 = ( r − BitRate[ SchedSelIdx − 1 ] )  ( BitRate[ SchedSelIdx ] − BitRate[ SchedSelIdx − 1 ] ),	(C-17)

c( r ) =  * CpbSize[ SchedSelIdx ] + (1 −  * CpbSize[ SchedSelIdx−1 ],	(C-18)

f( r ) = initial_cpb_removal_delay[ SchedSelIdx ] * BitRate[ SchedSelIdx ] + 
		( 1 −  initial_cpb_removal_delay[ SchedSelIdx − 1 ] * BitRate[ SchedSelIdx − 1 ]	(C-19)

for any SchedSelIdx > 0 and r such that BitRate[ SchedSelIdx − 1 ] <= r <= BitRate[ SchedSelIdx ] such that r and c( r ) are within the limits as specified in Annex Error: Reference source not found, Annex Error: Reference source not found, Annex Error: Reference source not found, and Annex I for the maximum bit rate and buffer size for the specified profile and level.

NOTE 1 – initial_cpb_removal_delay[ SchedSelIdx ] can be different from one buffering period to another and have to be re-calculated.

For output timing decoder conformance, an HRD as described above is used and the timing (relative to the delivery time of the first bit) of picture output is the same for both HRD and the DUT up to a fixed delay.

For output order decoder conformance, the HSS delivers the bitstream to the DUT "by demand" from the DUT, meaning that the HSS delivers bits (in decoding order) only when the DUT requires more bits to proceed with its processing.

NOTE 2 – This means that for this test, the coded picture buffer of the DUT could be as small as the size of the largest access unit.

A modified HRD as described below is used, and the HSS delivers the bitstream to the HRD by one of the schedules specified in the bitstream such that the bit rate and CPB size are restricted as specified in Annex Error: Reference source not found, Annex Error: Reference source not found, Annex Error: Reference source not found, and Annex I. The order of pictures output shall be the same for both HRD and the DUT.

For output order decoder conformance, the HRD CPB size is equal to CpbSize[ SchedSelIdx ] for the selected schedule and the DPB size is equal to MaxDpbFrames. Removal time from the CPB for the HRD is equal to final bit arrival time and decoding is immediate. The operation of the DPB of this HRD is specified in subclause C.4.1.

    - Operation of the output order DPB

The decoded picture buffer contains frame buffers. When a coded video sequence conforming to one or more of the profiles specified in Annex Error: Reference source not found is decoded by applying the decoding process specified in clauses Error: Reference source not found-Error: Reference source not found, each of the frame buffers may contain a decoded frame, a decoded complementary field pair or a single (non-paired) decoded field that is marked as "used for reference" or is held for future output (reordered pictures). When a coded video sequence conforming to one or more of the profiles specified in Annex Error: Reference source not found is decoded by applying the decoding process specified in Annex Error: Reference source not found, each frame buffer may contain a decoded frame, a decoded complementary field pair, a single (non-paired) decoded field, a decoded reference base frame, a decoded reference base complementary field pair or a single (non-paired) decoded reference base field that is marked as "used for reference" (reference pictures) or is held for future output (reordered or delayed pictures). When a coded video sequence conforming to one or more of the profiles specified in Annex Error: Reference source not found is decoded by applying the decoding process specified in Annex Error: Reference source not found, each of the frame buffers may contain a decoded frame view component, a decoded complementary field view component pair, or a single (non-paired) decoded field view component that is marked as "used for reference" (reference pictures) or is held for future output (reordered or delayed pictures) or is held for inter-view prediction (inter-view only reference components). When a coded video sequence conforming to one or more of the profiles specified in Annex I is decoded by applying the decoding process specified in Annex I, each of the frame buffers may contain a decoded texture frame view component, a decoded depth frame view component, a decoded complementary texture field view component pair, a single (non-paired) decoded texture field view component that is marked as "used for reference" (reference pictures) or is held for future output (reordered or delayed pictures) or is held for inter-view prediction (inter-view only reference components).

At HRD initialisation, the DPB fullness, measured in non-empty frame buffers, is set equal to 0. The following steps all happen instantaneously when an access unit is removed from the CPB, and in the order listed. When the decoding process specified in Annex Error: Reference source not found or Annex I is applied, the view components of the current primary coded picture are processed by applying the ordered steps to each view component in increasing order of the associated view order index VOIdx. The invocation of the process for a depth view component, if present, follows the invocation of the process for the texture view component within the same view component.

- The process of decoding gaps in frame_num and storing "non-existing" frames as specified in subclause C.4.2 is invoked.

- The picture decoding and output process as specified in subclause C.4.3 is invoked.

- The process of removing pictures from the DPB before possible insertion of the current picture as specified in subclause C.4.4 is invoked.

- The process of marking and storing the current decoded picture as specified in subclause C.4.5 is invoked.

NOTE – When the decoding process specified in Annex Error: Reference source not found is applied, the DPB is only operated for decoded pictures and reference base pictures associated with decoded pictures. The DPB is not operated for layer pictures with dependency_id less than DependencyIdMax (and associated reference base pictures). All decoded pictures and associated reference base pictures are decoded pictures and associated reference base pictures for dependency_id equal to DependencyIdMax, which represent the results of the decoding process specified in subclause Error: Reference source not found.

    - Decoding of gaps in frame_num and storage of "non-existing" pictures

When the decoding process specified in Annex Error: Reference source not found is applied, the process specified in this subclause is invoked for a particular view with view order index VOIdx, with "picture" being replaced by "view component", "frame" being replaced by "frame view component", and "field" being replaced by "field view component". During the invocation of the process for a particular view, only view components of the particular view are considered and view components of other views are not marked as "unused for reference" or removed from the DPB. The DPB fullness represents the total number of non-empty frame buffers, including frame buffers that contain texture or depth view components of the current view and other views.

When the decoding process specified in Annex I is applied, the process specified in this subclause for Annex Error: Reference source not found is invoked for particular texture view and depth view with view order index VOIdx, with each "view component" being replaced by "texture view component" or "depth view component", "frame view component" being replaced by "texture frame view component" or "depth frame view component", and "field view component" being replaced by "texture field view component". During the invocation of the process for a particular texture view, only the texture view components of the particular view are considered and during the invocation of the process for a particular depth view, only the depth view components of the particular view are considered and view components of other views are not marked as "unused for reference" or removed from the DPB.

When applicable, gaps in frame_num are detected by the decoding process and the necessary number of "non-existing" frames are inferred in the order specified by the generation of values of UnusedShortTermFrameNum in Equation Error: Reference source not found and are marked as specified in subclauses Error: Reference source not found and Error: Reference source not found. Frame buffers containing a frame or a complementary field pair or a non-paired field which are marked as "not needed for output" and "unused for reference" are emptied (without output), and the DPB fullness is decremented by the number of frame buffers emptied. Each "non-existing" frame is stored in the DPB as follows:

–	When there is no empty frame buffer (i.e., DPB fullness is equal to DPB size), the "bumping" process specified in subclause C.4.5.3 is invoked repeatedly until there is an empty frame buffer in which to store the "non-existing" frame.

–	The "non-existing" frame is stored in an empty frame buffer and is marked as "not needed for output", and the DPB fullness is incremented by one.

    - Picture decoding

When the decoding process specified in Annex Error: Reference source not found is applied, the process specified in this subclause is invoked for a particular view with view order index VOIdx.

When the decoding process specified in Annex I is applied, the process specified for Annex Error: Reference source not found in this subclause is invoked for particular texture view and depth view with view order index VOIdx.

The decoding of the current picture or view component (when applying the decoding process specified in Annex Error: Reference source not found or Annex I) is specified as follows:

–	If the decoding process specified in clause Error: Reference source not found or Annex Error: Reference source not found is applied, the current primary coded picture n is decoded and is temporarily stored (not in the DPB).

–	Otherwise (the decoding process specified in Annex Error: Reference source not found or Annex I is applied), the view component with view order index VOIdx of the current primary coded picture n is decoded and is temporarily stored (not in the DPB).

    - Removal of pictures from the DPB before possible insertion of the current picture

When the decoding process specified in Annex Error: Reference source not found is applied, the process specified in this subclause is invoked for a particular view with view order index VOIdx, with "picture" being replaced by "view component", "frame" being replaced by "frame view component", and "field" being replaced by "field view component".

When the decoding process specified in Annex I is applied, the process specified in this subclause for Annex Error: Reference source not found is invoked for particular texture view and depth view with view order index VOIdx, with each "view component" being replaced by "texture view component" or "depth view component", "frame view component" being replaced by "texture frame view component" or "depth frame view component", and "field view component" being replaced by "texture field view component". During the invocation of the process for a particular texture view, only the texture view components of the particular view are considered and during the invocation of the process for a particular depth view, only the depth view components of the particular view are considered.

When the decoding process specified in Annex Error: Reference source not found or Annex I is applied, the following process is specified for emptying frame buffers containing inter-view only reference components of the current access unit. By this process, frame buffers that contain view components of the current view with view order index VOIdx are not emptied, but frame buffers that contain inter-view only reference components of other views may be emptied. The process is specified as follows:

–	If the view order index VOIdx of the current view is equal to VOIdxMax, all frame buffers containing a frame or a complementary field pair or a non-paired field which are marked as "not needed for output" and "unused for reference" are emptied (without output), and the DPB fullness is decremented by the number of frame buffers emptied.

NOTE 1 – At this stage of the process, all frame buffers that contain a frame or a complementary field pair or a non-paired field marked as "not needed for output" and "unused for reference" are frame buffers that contain an inter-view only reference component (of the current access unit and a view with view order index less than VOIdx) with OutputFlag equal to 0.

–	Otherwise (the view order index VOIdx of the current view is less than VOIdxMax), frame buffers containing a frame or a complementary field pair or a non-paired field for which both of the following conditions are true are emptied (without output), and the DPB fullness is decremented by the number of frame buffers emptied:

–	the frame or complementary field pair or non-paired field is marked as "not needed for output" and "unused for reference",

NOTE 2 – At this stage of the process, all frame buffers that contain a frame or a complementary field pair or a non-paired field marked as "not needed for output" and "unused for reference" are frame buffers that contain an inter-view only reference component (of the current access unit and a view with view order index less than VOIdx) with OutputFlag equal to 0.

–	one of the following conditions is true:

–	the current view component is a view component of an anchor picture and the view_id of the frame or complementary field pair or non-paired field is not equal to any value of anchor_ref_lX[ k ][ j ], with X being equal to 0 or 1, k being any integer value greater than the view order index VOIdx of the current view, and j being any integer value in the range of 0 to Max( 0, num_anchor_refs_lX[ k ] − 1 ), inclusive,

–	the current view component is not a view component of an anchor picture and the view_id of the frame or complementary field pair or non-paired field is not equal to any value of non_anchor_ref_lX[ k ][ j ], with X being equal to 0 or 1, k being any integer value greater than the view order index VOIdx of the current view, and j being any integer value in the range of 0 to Max( 0, num_non_anchor_refs_lX[ k ] − 1 ), inclusive.

When the decoding process specified in Annex Error: Reference source not found or Annex I is applied, for the following processes specified in this subclause, only view components of the particular view for which this subclause is invoked are considered, and frame buffers containing view components of other views are not emptied. The DPB fullness represents the total number of non-empty frame buffers, including frame buffers that contain view components of other views.

The removal of pictures from the DPB before possible insertion of the current picture proceeds as follows:

–	If the decoded picture is an IDR picture the following applies:

- All reference pictures in the DPB are marked as "unused for reference" as specified in subclause Error: Reference source not found when a coded video sequence conforming to one or more of the profiles specified in Annex Error: Reference source not found is decoded by applying the decoding process specified in clauses Error: Reference source not found-Error: Reference source not found, or as specified in subclause Error: Reference source not found when a coded video sequence conforming to one or more of the profiles specified in Annex Error: Reference source not found is decoded by applying the decoding process specified in Annex Error: Reference source not found, or as specified in subclause Error: Reference source not found when a coded video sequence conforming to one or more of the profiles specified in Annex Error: Reference source not found is decoded by applying the decoding process specified in Annex Error: Reference source not found, or as specified in subclause Error: Reference source not found when a coded video sequence conforming to one or more of the profiles specified in Annex I is decoded by applying the decoding process specified in Annex I.

- When the IDR picture is not the first IDR picture decoded and the value of PicWidthInMbs or FrameHeightInMbs or max_dec_frame_buffering derived from the active sequence parameter set is different from the value of PicWidthInMbs or FrameHeightInMbs or max_dec_frame_buffering derived from the sequence parameter set that was active for the preceding picture, respectively, no_output_of_prior_pics_flag is inferred to be equal to 1 by the HRD, regardless of the actual value of no_output_of_prior_pics_flag.

NOTE 3 – Decoder implementations should try to handle changes in the value of PicWidthInMbs or FrameHeightInMbs or max_dec_frame_buffering more gracefully than the HRD.

- When no_output_of_prior_pics_flag is equal to 1 or is inferred to be equal to 1, all frame buffers in the DPB are emptied without output of the pictures they contain, and DPB fullness is set to 0.

–	Otherwise (the decoded picture is not an IDR picture), the decoded reference picture marking process is invoked as specified in subclause Error: Reference source not found when a coded video sequence conforming to one or more of the profiles specified in Annex Error: Reference source not found is decoded by applying the decoding process specified in clauses Error: Reference source not found-Error: Reference source not found, or as specified in subclause Error: Reference source not found when a coded video sequence conforming to one or more of the profiles specified in Annex Error: Reference source not found is decoded by applying the decoding process specified in Annex Error: Reference source not found, or as specified in subclause Error: Reference source not found when a coded video sequence conforming to one or more of the profiles specified in Annex Error: Reference source not found is decoded by applying the decoding process specified in Annex Error: Reference source not found, or as specified in subclause Error: Reference source not found when a coded video sequence conforming to one or more of the profiles specified in Annex I is decoded by applying the decoding process specified in Annex I. Frame buffers containing a frame or a complementary field pair or a non-paired field which are marked as "not needed for output" and "unused for reference" are emptied (without output), and the DPB fullness is decremented by the number of frame buffers emptied.

When the current picture has a memory_management_control_operation equal to 5 or is an IDR picture for which no_output_of_prior_pics_flag is not equal to 1 and is not inferred to be equal to 1, the following two steps are performed.

1.	Frame buffers containing a frame or a complementary field pair or a non-paired field which are marked as "not needed for output" and "unused for reference" are emptied (without output), and the DPB fullness is decremented by the number of frame buffers emptied.

2.	All non-empty frame buffers in the DPB are emptied by repeatedly invoking the "bumping" process specified in subclause C.4.5.3, and the DPB fullness is set to 0.

    - Current decoded picture marking and storage

When the decoding process specified in Annex Error: Reference source not found is applied, the process specified in this subclause is invoked for a particular view with view order index VOIdx, with "picture" being replaced by "view component", "frame" being replaced by "frame view component", and "field" being replaced by "field view component". During the invocation of the process for a particular view, only view components of the particular view are considered and frame buffers containing view components of other views are not emptied. The DPB fullness represents the total number of non-empty frame buffers, including frame buffers that contain texture view components and depth view components of the current and other views.

When the decoding process specified in Annex I is applied, the process specified in this subclause for Annex Error: Reference source not found is invoked for particular texture view and depth view with view order index VOIdx, with each "view component" being replaced by "texture view component" or "depth view component", "frame view component" being replaced by "texture frame view component" or "depth frame view component", and "field view component" being replaced by "texture field view component". During the invocation of the process for a particular texture view, only the texture view components of the particular view are considered and during the invocation of the process for a particular depth view, only the depth view components of the particular view are considered and frame buffers containing view components of other views are not emptied.

The marking and storage of the current decoded picture is specified as follows:

–	If the current picture is a reference picture, the storage and marking process for decoded reference pictures as specified in subclause C.4.5.1 is invoked.

–	Otherwise (the current picture is a non-reference picture), the storage and marking process for decoded non-reference pictures as specified in subclause C.4.5.2 is invoked.

      - Storage and marking of a reference decoded picture into the DPB

The current picture is stored in the DPB as follows:

–	If the current decoded picture is the second field (in decoding order) of a complementary reference field pair, and the first field of the pair is still in the DPB, the current picture is stored in the same frame buffer as the first field of the pair and the following applies:

–	If the current decoded picture has OutputFlag equal to 1, it is marked as "needed for output".

–	Otherwise (the current decoded picture has OutputFlag equal to 0), it is marked as "not needed for output".

–	Otherwise, the following operations are performed:

- When there is no empty frame buffer (i.e., DPB fullness is equal to DPB size), the "bumping" process specified in subclause C.4.5.3 is invoked repeatedly until there is an empty frame buffer in which to store the current decoded picture.

- The current decoded picture is stored in an empty frame buffer, the DPB fullness is incremented by one, and the following applies:

–	If the current decoded picture has OutputFlag equal to 1, it is marked as "needed for output".

–	Otherwise (the current decoded picture has OutputFlag equal to 0), it is marked as "not needed for output".

When the coded video sequence conforms to one or more of the profiles specified in Annex Error: Reference source not found and the decoding process specified in Annex Error: Reference source not found is applied and the current picture has store_ref_base_pic_flag equal to 1 (i.e., the current picture is associated with a reference base picture), the associated reference base picture is stored in the DPB as follows:

–	If the reference base picture is a second field (in decoding order) of a complementary reference base field pair, and the first field of the pair is still in the DPB, the reference base picture is stored in the same frame buffer as the first field of the pair and marked as "not needed for output".

–	Otherwise, the following operations are performed:

- When there is no empty frame buffer (i.e., DPB fullness is equal to DPB size), the "bumping" process specified in subclause C.4.5.3 is invoked repeatedly until there is an empty frame buffer in which to store the reference base picture.

- The reference base picture is stored in an empty frame buffer and marked as "not needed for output" and the DPB fullness is incremented by one.

      - Storage and marking of a non-reference decoded picture into the DPB

The current picture is associated with a variable StoreInterViewOnlyRefFlag, which is derived as follows:

–	If the decoding process specified in Annex Error: Reference source not found or Annex I is applied, the current view component has a view order index VOIdx less than VOIdxMax and inter_view_flag equal to 1, StoreInterViewOnlyRefFlag is set equal to 1.

–	Otherwise, StoreInterViewOnlyRefFlag is set equal to 0.

The current picture is stored in the DPB or output as follows:

–	If the current decoded picture is the second field (in decoding order) of a complementary non-reference field pair and the first field of the pair is still in the DPB, the current picture is stored in the same frame buffer as the first field of the pair and the following applies:

–	If the current decoded picture has OutputFlag equal to 1, it is marked as "needed for output".

–	Otherwise (the current decoded picture has OutputFlag equal to 0), it is marked as "not needed for output".

–	Otherwise, if the current picture has OutputFlag equal to 0 and StoreInterViewOnlyRefFlag equal to 0, the DPB is not modified and the current picture is not output.

–	Otherwise, if the current picture has StoreInterViewOnlyRefFlag equal to 1, the following operations are performed:

- When there is no empty frame buffer (i.e., DPB fullness is equal to DPB size), the "bumping" process specified in subclause C.4.5.3 is invoked repeatedly until there is an empty frame buffer in which to store the current decoded picture.

- The current decoded picture is stored in an empty frame buffer, the DPB fullness is incremented by one, and the following applies:

–	If the current decoded picture has OutputFlag equal to 1, it is marked as "needed for output".

–	Otherwise (the current decoded picture has OutputFlag equal to 0), it is marked as "not needed for output".

–	Otherwise, the following operations are performed repeatedly until the current decoded picture has been cropped and output or has been stored in the DPB:

–	If there is no empty frame buffer (i.e., DPB fullness is equal to DPB size), the following applies:

–	If the current picture does not have a lower value of PicOrderCnt( ) than all pictures in the DPB that are marked as "needed for output", the "bumping" process described in subclause C.4.5.3 is performed.

–	Otherwise (the current picture has a lower value of PicOrderCnt( ) than all pictures in the DPB that are marked as "needed for output"), the current picture is cropped, using the cropping rectangle specified in the active sequence parameter set for the picture and the cropped picture is output.

–	Otherwise (there is an empty frame buffer, i.e., DPB fullness is less than DPB size), the current decoded picture is stored in an empty frame buffer and is marked as "needed for output", and the DPB fullness is incremented by one.

      - "Bumping" process

When the decoding process specified in Annex Error: Reference source not found is applied, the process specified in this subclause is invoked for a particular view with view order index VOIdx, with "picture" being replaced by "view component", "frame" being replaced by "frame view component", and "field" being replaced by "field view component". During the invocation of the process for a particular view, only view components of the particular view are considered and frame buffers containing view components of other views are not emptied. The DPB fullness represents the total number of non-empty frame buffers, including frame buffers that contain texture view components and depth view components of the current view and other views.

When the decoding process specified in Annex I is applied, the process specified in this subclause for Annex Error: Reference source not found is invoked for particular texture view and depth view with view order index VOIdx, with each "view component" being replaced by "texture view component" or "depth view component", "frame view component" being replaced by "texture frame view component" or "depth frame view component", and "field view component" being replaced by "texture field view component". During the invocation of the process for a particular texture view, only the texture view components of the particular view are considered and during the invocation of the process for a particular depth view, only the depth view components of the particular view are considered and frame buffers containing view components of other views are not emptied. The DPB fullness represents the total number of non-empty frame buffers, including frame buffers that contain view components of other views.

The "bumping" process is invoked in the following cases.

–	There is no empty frame buffer (i.e., DPB fullness is equal to DPB size) and an empty frame buffer is needed for storage of an inferred "non-existing" frame, as specified in subclause C.4.2.

–	The current picture is an IDR picture and no_output_of_prior_pics_flag is not equal to 1 and is not inferred to be equal to 1, as specified in subclause C.4.4.

–	The current picture has memory_management_control_operation equal to 5, as specified in subclause C.4.4.

–	There is no empty frame buffer (i.e., DPB fullness is equal to DPB size) and an empty frame buffer is needed for storage of a decoded (non-IDR) reference picture or a reference base picture, as specified in subclause C.4.5.1.

–	There is no empty frame buffer (i.e., DPB fullness is equal to DPB size) and the current picture is a non-reference picture that is not the second field of a complementary non-reference field pair and the current picture has OutputFlag equal to 1 and there are pictures in the DPB that are marked as "needed for output" that precede the current non-reference picture in output order, as specified in subclause C.4.5.2, so an empty buffer is needed for storage of the current picture.

–	There is no empty frame buffer (i.e., DPB fullness is equal to DPB size) and the current picture is a non-reference picture that is not the second field of a complementary non-reference field pair and the current picture has StoreInterViewOnlyRefFlag equal to 1, as specified in subclause C.4.5.2, so an empty buffer is needed for storage of the current picture.

The "bumping" process consists of the following ordered steps:

- The picture or complementary reference field pair that is considered first for output is selected as follows:

- The frame buffer is selected that contains the picture having the smallest value of PicOrderCnt( ) of all pictures in the DPB marked as "needed for output".

- Depending on the frame buffer, the following applies:

–	If this frame buffer contains a complementary non-reference field pair with both fields marked as "needed for output" and both fields have the same PicOrderCnt( ), the first of these two fields in decoding order is considered first for output.

–	Otherwise, if this frame buffer contains a complementary reference field pair with both fields marked as "needed for output" and both fields have the same PicOrderCnt( ), the entire complementary reference field pair is considered first for output.

NOTE – When the two fields of a complementary reference field pair have the same value of PicOrderCnt( ), this "bumping" process will output these pictures together, although the two fields have different output times from a decoder that satisfies output timing conformance criteria (as specified in subclause C.2.2).

–	Otherwise, the picture in this frame buffer that has the smallest value of PicOrderCnt( ) is considered first for output.

- Depending on whether a single picture or a complementary reference field pair is considered for output, the following applies:

–	If a single picture is considered first for output, this picture is cropped, using the cropping rectangle specified in the active sequence parameter set for the picture, the cropped picture is output, and the picture is marked as "not needed for output".

–	Otherwise (a complementary reference field pair is considered first for output), the two fields of the complementary reference field pair are both cropped, using the cropping rectangle specified in the active sequence parameter set for the pictures, the two fields of the complementary reference field pair are output together, and both fields of the complementary reference field pair are marked as "not needed for output".

- The frame buffer that included the picture or complementary reference field pair that was cropped and output is checked, and when any of the following conditions are true, the frame buffer is emptied and the DPB fullness is decremented by 1:

–	The frame buffer contains a non-reference non-paired field.

–	The frame buffer contains a non-reference frame.

–	The frame buffer contains a complementary non-reference field pair with both fields marked as "not needed for output".

–	The frame buffer contains a non-paired reference field marked as "unused for reference".

–	The frame buffer contains a reference frame with both fields marked as "unused for reference".

–	The frame buffer contains a complementary reference field pair with both fields marked as "unused for reference" and "not needed for output".