| Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 1st Meeting: Stockholm, SE, 16–20 July 2012 | Document: JCT2-A0075 |
| --- | --- |

| Title: | 3DV-ATM HP Technical Input : A study on depth range based weighted prediction tool |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Sally Hattori Teruhiko Suzuki Sony City Osaki. 2-10-1 Osaki Shinagawa-ku , Tokyo, 141-8610 JAPAN | Tel: Email: | +81-50-3750-7411 Shinobu.Hattori@jp.sony.com teruhikos@jp.sony.com |
| Source: | Sony Corporation |  |  |

_____________________________

# Abstract

This document reports a comparison between depth range based weighted prediction (DBWP) and explicit weighted prediction (EWP) in AVC. In this document, we compared coding efficiency of both methods with 3D-ATM software. The experimental result shows that coding efficiency of each method is almost the same.

# Introduction

- This document reports a comparison between depth range based weighted prediction (DBWP) and explicit weighted prediction (EWP) in AVC [1]. DBWP is an implicit weighted prediction specialized for depth maps in 3D-ATM [2]. DBWP estimates weighting coefficients from depth range parameters in both encoder and decoder. When EWP is used in place of DBWP, it is possible to use exactly the same prediction by providing weighting coefficients estimated from the depth range parameters in the encoder.

- In this document, the coding efficiency of both methods are experimented with 3D-ATM software [3]. The experimental result shows that coding efficiency of the two methods are almost the same.

# Comparison of DBWP and EWP

- Both DBWP and EWP use the same equation of weighted prediction as follows:

- vref'=w⋅vref+o=1Zrefnear−1Zreffar1Zcurnear−1Zcurfarvref+255⋅1Zreffar−1Zcurfar1Zcurnear−1Zcurfar

- v’ref is the reference picture after weighted prediction, vref is the reference picture before weighted prediction, and Znear and Zfar are the depth range parameters, which represents the nearest depth value and the farthest depth value, respectively.

- There are two differences between DBWP and EWP.

- The derivation process of weighting coefficients

- The first difference is the derivation process of weighting coefficients. DBWP provides the depth range parameters (Znear, Zfar) in the sequence parameter set and depth range update NAL unit, and estimates weighting coefficients (w,o) from these depth range parameters in both encoder and decoder. Thus, the weighting coefficients need not to be encoded. On the other hand, EWP estimates weighting coefficients (w,o) from the depth range parameters (Znear, Zfar) in the encoder side, and provides the weighting coefficients in the slice header as normative explicit mode in AVC. Thus, the depth range parameters need not to be encoded.

- Additional flag

- The second difference is whether an additional flag is needed or not. DBWP need to provide an additional flag depth_weighted_pred_flag (See Table 3) in slice header in order to distinguish the conventional weighted prediction. On the other hand, EWP does not need to provide an additional flag since DBWP is operated as a part of explicit mode in AVC.

- The differences are summarized in Table.1.

Table 1: The comparison of DBWP and EWP

|  | - DBWP | - EWP |
| --- | --- | --- |
| - Depth range parameters (Znear, Zfar) | - Need to be encoded | - Does not need to be encoded |
| - Weighting coefficients (w,o) | - Does not need to be encoded - (estimated from (Znear, Zfar) in decoder) | - Need to be encoded - (explicitly provided to the decoder) |
| - Additional flag | - Need to be encoded - (in order to distinguish WP in AVC) | - Does not need to be encoded - (operating DBWP as a part of WP in AVC) |

# Experiments

- The contribution compared the coding efficiency of DBWP and EWP with 3D-ATM software in 3 view case of the common test condition. The anchor is used as DBWP. EWP is implemented with weighting coefficients estimated from depth range parameters in the encoder. Note that the implementation does not omit encoding of depth range parameters in sequence parameter set and depth range update NAL unit although these depth range parameters are not needed for EWP.

- Table 2 shows the BD-rate of EWP compared with DBWP. The results only show the coding efficiency for the depth, since the texture coding is not effected by the experiment.

- Table2: The BD-rate of EWP compared with DBWP

|  | Depth Coding |  | Total (Texture + Depth) |  |
| --- | --- | --- | --- | --- |
|  | dBR, % | dPSNR,dB | dBR, % | dPSNR,dB |
| S01 | -0.05 | 0.00 | -0.01 | 0.00 |
| S02 | -0.05 | 0.00 | -0.01 | 0.00 |
| S03 | -0.03 | 0.00 | 0.00 | 0.00 |
| S04 | 0.77 | -0.04 | 0.08 | 0.00 |
| S05 | -0.02 | 0.00 | -0.01 | 0.00 |
| S06 | -0.04 | 0.00 | -0.01 | 0.00 |
| S08 | -0.04 | 0.00 | -0.01 | 0.00 |
| Average | 0.08 | 0.00 | 0.01 | 0.00 |

- The coding efficiency of EWP is slightly better than DBWP in all sequences except for S04. Both DBWP and EWP are conceptually ineffective in these sequences since the depth ranges are unchanged through all of frames. Thus, the difference of coding efficiency is achieved from encoding of an additional flag, depth_weighted_pred_flag, mentioned in Section 2.

- The coding efficiency of EWP is slightly worse than DBWP in S04. The depth ranges change in all frames in S04. EWP uses exactly the same weighted prediction as DBWP, however, the overhead is increased since the implementation did not omit coding of depth ranges in the sequence parameter set and depth range update NAL unit. The actual coding loss may be less than as shown in the result if the parameters were omitted.

# Conclusion

This document reports a comparison between depth range based weighted prediction (DBWP) and explicit weighted prediction (EWP) in AVC. The experimental result shows that coding efficiency of the two methods is almost the same.

# Reference

- “Advanced Video Coding”, ISO/IEC 14496-10 and ITU-T Rec. H.264

- Video Group, “Specification text for 3DV-ATM,” ISO/IEC JTC1/SC29/WG11 N22915, Geneva, CH, January 2012.

- 3DV-ATM Software; http://mpeg3dv.research.nokia.com/svn/mpeg3dv/trunk,  Revision: 52

# Patent rights declaration(s)

Sony Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Annex

- Table 3: Slice header Syntax in Specification text for 3DV-ATM[2]

| slice_header( ) { | C | Descriptor |
| --- | --- | --- |
| first_mb_in_slice | 2 | ue(v) |
| slice_type | 2 | ue(v) |
| pic_parameter_set_id | 2 | ue(v) |
| if( separate_colour_plane_flag = = 1 ) |  |  |
| colour_plane_id | 2 | u(2) |
| frame_num | 2 | u(v) |
| if( !frame_mbs_only_flag ) { |  |  |
| field_pic_flag | 2 | u(1) |
| if( field_pic_flag ) |  |  |
| bottom_field_flag | 2 | u(1) |
| } |  |  |
| if( IdrPicFlag ) |  |  |
| idr_pic_id | 2 | ue(v) |
| if( pic_order_cnt_type = = 0 ) { |  |  |
| pic_order_cnt_lsb | 2 | u(v) |
| if( bottom_field_pic_order_in_frame_present_flag && !field_pic_flag ) |  |  |
| delta_pic_order_cnt_bottom | 2 | se(v) |
| } |  |  |
| if( pic_order_cnt_type = = 1 && !delta_pic_order_always_zero_flag ) { |  |  |
| delta_pic_order_cnt[ 0 ] | 2 | se(v) |
| if( bottom_field_pic_order_in_frame_present_flag && !field_pic_flag ) |  |  |
| delta_pic_order_cnt[ 1 ] | 2 | se(v) |
| } |  |  |
| if( redundant_pic_cnt_present_flag ) |  |  |
| redundant_pic_cnt | 2 | ue(v) |
| if( slice_type = = B ) |  |  |
| direct_spatial_mv_pred_flag | 2 | u(1) |
| if( slice_type = = P ¦ ¦ slice_type = = SP ¦ ¦ slice_type = = B ) { |  |  |
| num_ref_idx_active_override_flag | 2 | u(1) |
| if( num_ref_idx_active_override_flag ) { |  |  |
| num_ref_idx_l0_active_minus1 | 2 | ue(v) |
| if( slice_type = = B ) |  |  |
| num_ref_idx_l1_active_minus1 | 2 | ue(v) |
| } |  |  |
| } |  |  |
| if( nal_unit_type = = 21 ) |  |  |
| ref_pic_list_3dv_modification( ) /* specified in Annex I */ |  |  |
| if( nal_unit_type = = 20 ) |  |  |
| ref_pic_list_mvc_modification( ) /* specified in Annex Error: Reference source not found */ | 2 |  |
| else |  |  |
| ref_pic_list_modification( ) | 2 |  |
| if( ( weighted_pred_flag && ( slice_type = = P ¦ ¦ slice_type = = SP ) ) ¦ ¦ ( weighted_bipred_idc = = 1 && slice_type = = B ) ) |  |  |
| pred_weight_table( ) | 2 |  |
| if( nal_ref_idc != 0 ) |  |  |
| dec_ref_pic_marking( ) | 2 |  |
| if( entropy_coding_mode_flag && slice_type != I && slice_type != SI ) |  |  |
| cabac_init_idc | 2 | ue(v) |
| slice_qp_delta | 2 | se(v) |
| if( slice_type = = SP ¦ ¦ slice_type = = SI ) { |  |  |
| if( slice_type = = SP ) |  |  |
| sp_for_switch_flag | 2 | u(1) |
| slice_qs_delta | 2 | se(v) |
| } |  |  |
| if( deblocking_filter_control_present_flag ) { |  |  |
| disable_deblocking_filter_idc | 2 | ue(v) |
| if( disable_deblocking_filter_idc != 1 ) { |  |  |
| slice_alpha_c0_offset_div2 | 2 | se(v) |
| slice_beta_offset_div2 | 2 | se(v) |
| } |  |  |
| } |  |  |
| if( num_slice_groups_minus1 > 0 && slice_group_map_type >= 3 && slice_group_map_type <= 5) |  |  |
| slice_group_change_cycle | 2 | u(v) |
| if( ( nal_unit_type = = 21 ) && ( slice_type != I && slice_type != SI ) ) |  |  |
| if( DepthFlag = = 0 ) |  |  |
| dmvp_flag | 1 | u(1) |
| else if( ( ( slice_type==P ¦ ¦ slice_type = = SP ) && ( weighted_pred_flag = = 0 ) ) ¦ ¦ ( slice_type = = B && ( weighted_bipred_idc = = 0 ) ) ) |  |  |
| depth_weighted_pred_flag | 1 | u(1) |
| } |  |  |