| Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 1st Meeting: Stockholm, SE, 16–20 July 2012 | Document: JCT2-A0086 |
| --- | --- |

| Title: | 3D-CE1.h related: Illumination Compensation for Inter-View Prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hongbin Liu (hongbin.liu@lge.com) Jiwook Jung (jiwook.jung@lge.com) Jaewon Sung (jw.sung@lge.com) Jie Jia (jie.jia@lge.com) Sehoon Yea (sehoon.yea@lge.com) |  |  |
| Source: | LG Electronics |  |  |

_____________________________

# Abstract

This contribution proposes illumination compensation (IC) method to improve coding efficiency of dependent views. The basic idea is that illumination discrepancy between different views should be corrected in inter-view prediction. Linear IC model is employed to compensate illumination discrepancy between different views. Parameters in IC model are estimated for each PU (prediction unit) using available reconstructed neighbouring pixels and therefore are not need to be encoded. Applying IC or not is decided at CU (coding unit) level, and an IC flag is coded for CU in which inter-view prediction is used. A method is also designed to adaptively determine whether there is illumination discrepancy in a picture, and an IC flag is encoded for the first slice of each picture in dependent views.

It is reported that in 2-view case, proposed method can achieve -0.8% gain on dependent video, -0.2% gain on coded video, -0.4% gain on synthesized view and -0.4% gain on coded and synthesized view. In 3-view case, proposed method can achieve -0.8%, -1.0% gain on two dependent video respectively, -0.4% gain on coded video, -0.6% gain on synthesized view and -0.5% gain on coded and synthesized view.

# Introduction

In multi-view video coding (MVC) or 3D video coding, there are multiple views to be encoded. Because different cameras maybe not perfectly calibrated, they may have different exposure time. In this case, there is illumination discrepancy between different views.

To tackle this problem, Y. Lee etc. [1] propose to add a DC component to prediction block. The DC component is calculated based on illumination discrepancy between original block and reference block. To save bits, original DC component is first predicted from neighboring macroblocks’s DC component and then difference between them is quantized and encoded. In [2], A. Fartukov etc. propose to employ pixel level linear IC model in 3DV-ATM. They train respective linear parameters for each position of the current coding block by using neighboring reconstructed blocks and their corresponding blocks in reference picture.

In this contribution, we propose to employ PU level linear IC model in 3DV-HEVC. One pair of linear parameters is used for the whole PU. When training the linear parameters, only the above row and right column of the current CU (CU that contains current PU) are used. Same training method is used for both encoder and decoder and therefore there is no linear parameters encoded.

# Proposed Method

This contribution proposes to employ linear IC model to compensate illumination discrepancy between different views. Parameters in IC model are estimated for each PU using available reconstructed neighbouring pixels and therefore are not need to be encoded. Applying IC or not is decided at CU level, and an IC flag is coded for CU in which inter-view prediction is used. Details are as follows.

- Linear IC model

Linear IC model used in inter-view prediction is shown in Eq. (1):

(1)

Here, PUc is the current PU, (i, j) is the coordinate of pixels in PUc, (dvx, dvy) is the disparity vector of PUc. p(i, j) is the prediction of PUc, r is the PU’s reference picture from neighboring view. a and b are parameters of the linear IC model.

To estimate parameter a and b for a PU, two set of pixels as shown in Fig.2 are used: 1. available reconstructed neighboring pixels in left column and above row of current CU (the CU that contains current PU), 2. Corresponding neighboring pixels of current CU’s reference block. Reference block of current CU is found by using current PU’s disparity vector.

Fig.2. Neighboring pixels used to estimate parameters in IC model. Reference block of current CU is found by using current PU’s disparity vector.

Let Recneig and Recrefneig denote used neighboring pixel set of current CU and its reference block respectively, and let 2N denote the pixel number in Recneig and Recrefneig. Then, a and b can be calculated as:

(2)                   (3)

- Motion estimation criterion

At encoder, when IC model is used, criterion used in motion estimation is changed from SAD to the well-known MR_SAD.

(4)

where PUc , (i, j), (dvx, dvy) and r are defined same with Eq. (1).  and  are the mean of PUc and its reference block respectively.

- Adaptive IC on slice-level

For adaptive IC, the encoder decides whether the IC should be applied or not to current picture and transmits the decision result to decoder. One bit flag is encoded in the slice header of the first slice to indicate whether IC is applied for the first slice and its subsequent slices in the picture. The decision process is shown in Table1. The colour distributions of the current and inter-view reference pictures are represented by histograms and the similarity of two distributions are measured by SAD of the two histograms.  The threshold is determined by preliminary experiments. When the flag is turned off, the encoder does not try IC applied case for CU and the CU-level flags are not transmitted to decoder. Therefore, inefficient IC process can be prevented at both encoder and decoder.

Table 1. The decision process of the picture level IC flag

1) Make histograms of the current picture and the inter-view reference original picture

2) Calculate SAD between two histograms

3) If the SAD is over a certain threshold, the IC flag is set to 1

4) else, the IC flag is set to 0

- Syntax change

- Slice header syntax

We modify the slice head syntax to include the picture level IC flag, as highlighted in Table 2.

Table 2. Slice header syntax modification

| slice_header( ) { | Descriptor |
| --- | --- |
| first_slice_in_pic_flag | u(1) |
| If( first_slice_in_pic_flag && !base_view && !is_depth) |  |
| illumination_comp_flag | u(1) |
| pic_parameter_set_id | ue(v) |
| if( !first_slice_in_pic_flag ) |  |
| slice_address | u(v) |

- base_view: Boolean variable to indicate whether current view is base view. If current view is base view, base_view = true; else base_view = false.

- is_depth: Boolean variable to indicate whether current picture is a depth picture. If current picture is depth picture, is_depth = true; else is_depth = false.

- illumination_comp_flag: flag to indicate whether IC is enabled for the picture that includes the current slice  or not. If illumination_comp_flag = 1, IC is enabled for the current picture and CU in it can use IC; otherwise, IC is disabled for the current picture, and CU in it cannot use IC, therefore no CU level IC flag is coded.

- CU syntax

Proposed method needs an IC flag for CU in which inter-view prediction is used and therefore we change the CU syntax correspondingly, as highlighted in Table 3.

Table 3. CU syntax modification

| if( skip_flag[ x0 ][ y0 ] ) |  |
| --- | --- |
| { |  |
| prediction_unit( x0, y0 , log2CbSize ) |  |
| If(!base_view && !is_depth && !cu_not_pred_from_interview) |  |
| illumination_comp_flag | ae(v) |
| … |  |
| } |  |
| else { |  |
| if( slice_type != I ) |  |
| pred_mode_flag | ae(v) |
| if( PredMode != MODE_INTRA ¦ ¦ log2CbSize = = Log2MinCbSize ) |  |
| part_mode | ae(v) |
| x1 = x0 + ( ( 1 << log2CbSize ) >> 1 ) |  |
| y1 = y0 + ( ( 1 << log2CbSize ) >> 1 ) |  |
| x2 = x1 − ( ( 1 << log2CbSize ) >> 2 ) |  |
| y2 = y1 − ( ( 1 << log2CbSize ) >> 2 ) |  |
| x3 = x1 + ( ( 1 << log2CbSize ) >> 2 ) |  |
| y3 = y1 + ( ( 1 << log2CbSize ) >> 2 ) |  |
| if( PartMode = = PART_2Nx2N ) |  |
| prediction_unit( x0, y0 , log2CbSize ) |  |
| else if( PartMode = = PART_2NxN ) { |  |
| prediction_unit( x0, y0 , log2CbSize ) |  |
| prediction_unit( x0, y1 , log2CbSize ) |  |
| } else if( PartMode = = PART_Nx2N ) { |  |
| prediction_unit( x0, y0 , log2CbSize ) |  |
| prediction_unit( x1, y0 , log2CbSize ) |  |
| } else if( PartMode = = PART_2NxnU ) { |  |
| prediction_unit( x0, y0 , log2CbSize ) |  |
| prediction_unit( x0, y2 , log2CbSize ) |  |
| } else if( PartMode = = PART_2NxnD ) { |  |
| prediction_unit( x0, y0 , log2CbSize ) |  |
| prediction_unit( x0, y3 , log2CbSize ) |  |
| } else if( PartMode = = PART_nLx2N ) { |  |
| prediction_unit( x0, y0 , log2CbSize ) |  |
| prediction_unit( x2, y0 , log2CbSize ) |  |
| } else if( PartMode = = PART_nRx2N ) { |  |
| prediction_unit( x0, y0 , log2CbSize ) |  |
| prediction_unit( x3, y0 , log2CbSize ) |  |
| } else { /* PART_NxN */ |  |
| prediction_unit( x0, y0 , log2CbSize ) |  |
| prediction_unit( x1, y0 , log2CbSize ) |  |
| prediction_unit( x0, y1 , log2CbSize ) |  |
| prediction_unit( x1, y1 , log2CbSize ) |  |
| } |  |
| If(PredMode != MODE_INTRA && !base_view && !is_depth && !cu_not_pred_from_interview) |  |
| illumination_comp_flag | ae(v) |
| … |  |
| } |  |

- base_view: Boolean variable to indicate whether current view is base view. If current view is base view, base_view = true; else base_view = false.

- is_depth: Boolean variable to indicate whether current picture is a depth picture. If current picture is depth picture, is_depth = true; else is_depth = false.

- cu_not_pred_from_interview: Boolean variable to indicate whether there is no PU predicted from inter-view reference picture in current CU. If all PUs in the current CU are not predicted from inter-view reference picture, cu_not_pred_from_interview = true; otherwise cu_pred_only_from_temporal = false.

- illumination_comp_flag: flag to indicate whether current CU uses IC or not. If illumination_comp_flag = 1, current CU uses IC; otherwise current CU does not use IC.

# Results

Proposed method is integrated into 3DV-HTM 3.1 software and compared with it. Experiments are executed following the configuration and test condition defined in [3].

Computer of the specification: Intel (R) Xeon (R) CPU X5670@2.93GHz, 12cores, Memory 24GB，Centos and 64 bit compiler, is used in the simulation.

Table 4. Summary results of 3 view-case

|  | video 0 | video 1 | video 2 | video only | synthesized only | coded & synthesized | enc time | dec time | ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.0% | -1.4% | -1.0% | -0.6% | -0.4% | -0.4% | 136.7% | 98.9% | 94.6% |
| Kendo | 0.0% | -3.1% | -3.5% | -1.4% | -1.1% | -1.2% | 138.3% | 93.2% | 95.1% |
| Newspapercc | 0.0% | -0.7% | -1.0% | -0.4% | -0.7% | -0.6% | 132.1% | 93.4% | 95.9% |
| GhostTownFly | 0.0% | 0.0% | -0.1% | 0.0% | 0.2% | 0.1% | 103.6% | 100.4% | 84.9% |
| PoznanHall2 | 0.0% | -0.6% | -1.6% | -0.5% | -1.3% | -1.1% | 137.0% | 94.1% | 88.9% |
| PoznanStreet | 0.0% | 0.4% | -0.3% | 0.0% | -0.5% | -0.4% | 133.1% | 97.1% | 87.6% |
| UndoDancer | 0.0% | 0.1% | 0.1% | 0.0% | -0.2% | -0.1% | 122.7% | 100.7% | 82.7% |
| 1024x768 | 0.0% | -1.7% | -1.8% | -0.8% | -0.7% | -0.7% | 135.7% | 95.1% | 95.2% |
| 1920x1088 | 0.0% | 0.0% | -0.5% | -0.1% | -0.5% | -0.4% | 123.4% | 98.0% | 86.0% |
| average | 0.0% | -0.8% | -1.1% | -0.4% | -0.6% | -0.5% | 128.5% | 96.8% | 89.8% |

Table 5. Summary results of 2 view-case

|  | video 0 | video 1 |  | video only | synthesized only | coded & synthesized |
| --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.0% | -1.4% |  | -0.4% | -0.4% | -0.4% |
| Kendo | 0.0% | -3.1% |  | -0.9% | -0.7% | -0.7% |
| Newspapercc | 0.0% | -0.7% |  | -0.2% | -0.4% | -0.3% |
| GhostTownFly | 0.0% | 0.0% |  | 0.0% | 0.2% | 0.1% |
| PoznanHall2 | 0.0% | -0.6% |  | -0.2% | -1.1% | -0.8% |
| PoznanStreet | 0.0% | 0.4% |  | 0.0% | -0.5% | -0.3% |
| UndoDancer | 0.0% | 0.1% |  | 0.0% | -0.2% | -0.1% |
| 1024x768 | 0.0% | -1.7% |  | -0.5% | -0.5% | -0.5% |
| 1920x1088 | 0.0% | 0.0% |  | 0.0% | -0.4% | -0.3% |
| average | 0.0% | -0.8% |  | -0.2% | -0.4% | -0.4% |

# Conclusion

This proposal improves coding efficiency of dependent views and it has negligible influence on decoder complexity. We recommend that proposed method is adopted into 3DV-HEVC.

# Reference

[1] Y. Lee, J. Hur*, etc., “Results of CE2 on Multi-view Video Coding”, ISO/IEC JTC1/SC29/WG11 and ITU-T SG16 Q.6, Doc. JVT-T110, July 2006, Klagenfurt, Austria.

[2] A. Fartukov, I. Kovliga, etc., “3D-AVC results on inter-view coding with adaptive luminance compensation algorithm”, ISO/IEC JTC1/SC29/WG11, Doc. M24816, May 2012, Geneva, Switzerland

[3] H.Schwarz, D.Rusanovskyy, “Common test conditions for 3DV experimentation”, ISO/IEC JTC1/SC29/WG11, Doc. w12745, May 2012, Geneva, Switzerland

# Patent rights declaration(s)

LG Electronics / LG Electronics (china) R&D Center may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).