| Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 1st Meeting: Stockholm, SE, 16–20 July 2012 | Document: JCT2-A0088 |
| --- | --- |

| Title: | 3D-CE6.h Intra Skip mode for depth map I-slices |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Eunyong Son, Jin Heo, Hongbin Liu, Sehoon Yea 19, Yangjae-daero 11gil Seocho-gu, Seoul 137-130, Korea | Tel: Email: | 82-2-6912-6477 eunyong.son@lge.com jin78.heo@lge.com hongbin.liu@lge.com sehoon.yea@lge.com |
| Source: | LG Electronics |  |  |

_____________________________

# Abstract

This contribution presents the intra skip mode for depth map I-slices. Intra skip mode does not transmit residual data and only sends intra prediction modes in depth map coding. The proposed method achieves 0.0% (3-view) and 0.1% (2-view) BD-rate gains under common test condition (CTC) and 0.4% (3-view) and 0.3% (2-view) BD-rate gains under all intra test condition.

# Introduction

In 3D video coding common test condition (CTC) [1], both multiview/stereo video coding with depth data and without depth data have been defined as two general test cases. This contribution proposes a depth map intra skip mode for multiview/stereo video coding with depth map. Since depth map data is simpler than texture data, a skip method of the inter prediction can be applied to the intra prediction with the similar concept. The proposed intra skip mode does not encode any residual signal like a inter skip mode; the intra skip mode only encodes intra prediction mode and then prediction information is employed for the reconstruction process in the decoder side.

# Algorithm description

In the 3DV-HTM software 3.1 (3DV-HTM3.1), the skip flag is transmitted for the current coding unit (CU) in the inter slices (P or B slices) and is not employed in the intra slices (I slices). This contribution uses the conventional skip flag in both inter and intra slices of the depth map. The skip flag used in intra slice is checked whether the current CU is intra skip or not. The coding unit syntax and the prediction unit syntax were modified as follows.

| coding_unit( x0, y0, log2CbSize ) { | Descriptor |
| --- | --- |
| CurrCbAddrTS = MinCbAddrZS[ x0 >> Log2MinCbSize ][ y0 >> Log2MinCbSize ] |  |
| if( slice_type != I ¦¦ is_depth_slice ) |  |
| if( slice_type != I ) |  |
| skip_flag[ x0 ][ y0 ] | ae(v) |
| if( skip_flag[ x0 ][ y0 ] ) |  |
| prediction_unit( x0, y0 , log2CbSize ) |  |
| else { |  |
| ... |  |

Coding unit syntax changes

| prediction_unit( x0, y0, log2CbSize ) { | Descriptor |
| --- | --- |
| if( skip_flag[ x0 ][ y0 ] ) { |  |
| if( is_depth_slice && slice_type == I ) { |  |
| prev_intra_luma_pred_flag[ x0 ][ y0 ] | ae(v) |
| if( prev_intra_luma_pred_flag[ x0 ][ y0 ] ) |  |
| mpm_idx[ x0 ][ y0 ] | ae(v) |
| else |  |
| rem_intra_luma_pred_mode[ x0 ][ y0 ] | ae(v) |
| else if( MaxNumMergeCand > 1 ) |  |
| if( MaxNumMergeCand > 1 ) |  |
| merge_idx[ x0 ][ y0 ] | ae(v) |
| } else if( PredMode = = MODE_INTRA ) { |  |
| if( PartMode = = PART_2Nx2N && pcm_enabled_flag && log2CbSize >= Log2MinIPCMCUSize && log2CbSize <= Log2MaxIPCMCUSize ) |  |
| pcm_flag | ae(v) |
| if( pcm_flag ) { |  |
| num_subsequent_pcm | tu(3) |
| NumPCMBlock = num_subsequent_pcm + 1 |  |
| while( !byte_aligned( ) ) |  |
| pcm_alignment_zero_bit | u(v) |
| pcm_sample( x0, y0, log2CbSize ) |  |
| } else { |  |
| prev_intra_luma_pred_flag[ x0 ][ y0 ] | ae(v) |
| if( prev_intra_luma_pred_flag[ x0 ][ y0 ] ) |  |
| mpm_idx[ x0 ][ y0 ] | ae(v) |
| else |  |
| rem_intra_luma_pred_mode[ x0 ][ y0 ] | ae(v) |
| intra_chroma_pred_mode[ x0 ][ y0 ] | ae(v) |
| SignalledAsChromaDC = ( chroma_pred_from_luma_enabled_flag ? intra_chroma_pred_mode[ x0 ][ y0 ] = = 3 : intra_chroma_pred_mode[ x0 ][ y0 ] = = 2 ) |  |
| } |  |
| } else { /* MODE_INTER */ |  |
| ... |  |

Prediction unit syntax changes

Here, is_depth_slice specifies whether the current slice is a depth map slice. If is_depth_slice is equal to 1, the current slice is included in a depth map slice.

# Experimental Results

The proposed method was integrated into the 3DV-HTM3.1 and simulations were conducted for 3-view configurations. The results are summarized in Table 1 and Table 2. Table 1 shows that the proposed method shows 0.0% and 0.1% BD-rate gains for 3-view and 2-view cases under common test condition. Table 3 and Table 4 show that the proposed method achieves 0.4% and 0.3% BD-rate gains for 3-view and 2-view cases under all intra test condition.

Table 1 BD rate results for 3-view case under common test condition

Table 2 BD rate results for 2-view case under common test condition

Table 3 BD rate results for 3-view case under all intra test condition

Table 4 BD rate results for 2-view case under all intra test condition

# Conclusion

This contribution proposed the intra skip mode for depth map I-slice. By integrating the proposed method on 3DV-HTM3.1, it achieved 0.0% and 0.1% BD-rate gains under common test condition with negligible encoding and decoding time and 0.4% and 0.3% BD-rate gains under all intra test condition.

# Reference

- Heiko Schwarz and Dmytro Rusanovskyy, “Common Test Conditions for 3DV experimentation,” ISO/IEC JTC1/SC29/WG11 MPEG2011/N12745, Geneva, Switzerland, May 2012.

# Patent rights declaration(s)

LG Electronics Co., Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).