| Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 1st Meeting: Stockholm, SE, 16–20 July 2012 | Document: JCT2-A0097 |
| --- | --- |

| Title: | CE5.h: Disparity vector generation results |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Li Zhang Ying Chen Marta Karczewicz 5775 Morehouse Drive San Diego, CA 92121 USA | Tel: Email: | 1-858-651-6660 lizhang@qualcomm.com 1-858-845-6589 cheny@qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

In the current 3DV-HTM software, disparity vectors can be estimated by two methods, and are used for inter-view motion prediction and inter-view residual prediction. One method is to maintain a depth map for each picture of each view and the depth map is generated from disparity motion vectors and predicted by propagating the depth map to a different time instance with temporal motion vectors and propagating the depth map to a different view using view synthesis. The other method, as proposed in m24937, derives a disparity vector from spatial and temporal neighboring blocks, once a disparity motion vector in any of the blocks is identified, the disparity vector is derived. The coding efficiency of the two methods is analyzed in the latest HTM software this proposal. It is reported that deriving the disparity vector from neighbouring blocks will reduce the complexity greatly and the cost in terms of compression efficiency is negligible.

# Introduction

Instead of using estimated depth maps as described in [1], it was proposed in m24937 that the spatial or temporal neighboring blocks are utilized to generate a disparity vector for inter-view motion prediction and inter-view residual prediction [2]. Once a disparity motion vector of the neighboring block is identified, it is used for inter-view motion prediction or inter-view residual prediction. This technique has been adopted and integrated to the reference software and used for CE5.h. The software was committed to the HTM software branch.

For simplicity, the depth map based method is named Depth Map Disparity Vector (DMDV) in this document and the neighboring blocks based method is named Neighboring Blocks Disparity Vector (NBDV). Main drawbacks of DMDV are highlighted as follows:

- A depth map buffer is required for each view of each access unit.

- Predicting from one depth map buffer to a depth map buffer in a different access unit based on temporal motion vector has a similar complexity as motion compensation.

- Predicting from one depth map buffer to a depth map buffer in a different view requires view synthesis where hole-filling is needed.

- To use the maximum disparity within a prediction unit (PU) in the AMVP mode, up to 256 estimated depth values are needed to be compared to find the maximum one for each PU.

# Simulation results

This section provides simulation results of both methods. One set of results are based on the current CE5.h software implemented on HTM3.1rc1 [3] and the corresponding results are reported in section 2.1. Another set of results are based on the latest software HTM3.1 [4] and presented in section 2.2. To have a better understanding of the performance of NBDV and DMDV, the following cases are tested and all the simulations are based on the common test conditions [5] :

- NBDV Case 1: NBDV with improved merge mode, as proposed in JCT2-A0096 [6].

- DMDV Case 1: DMDV with bug fix of independent estimated depth maps for non-base views, as proposed in JCT2-A0095 [7].

- DMDV Case 2: DMDV with bug fix and the improved merge mode, as proposed in [6], [7].

- NBDV Case 2: NBDV in current HTM

NBDV Case 1 is the best performing configuration for the NBDV method from the proponent. DMDV Case 1 is the DMDV with bug fix (to not allow decoding dependency between the two enhancement views).  So firstly, NBDV Case 1 and DMDV Case 1 are compared.

Since the merge improvement (as in JCT2-A0096) can be applicable to DMDV as well. The above DMDV configuration plus the technique in JCT2-A0096, denoted as DMDV Case 2 was tested and compared with NBDV Case 1, assuming JCT2-A0096 is adopted.

Finally NBDV, denoted as NBDV Case 2, can be directly compared with DMDV Case 1 and the results are also informative.

## Experimental results on CE5.h software

The experimental results of the above three comparisons are presented in section 2.1 where all the related coding tools are integrated to CE5.h software.

### Comparison of NBDV Case 1 and DMDV Case 1

This is the comparison of the best performing NBDV configuration and the DMDV method with bug fix (no dependency between two enhancement views). The coding gain of NBDV Case 1 compared to DMDV Case 1 is listed in Table 1: Coding performance comparison between NBDV Case 1 and DMDV Case 1. Minor coding gain can be achieved, i.e., 0.1% bitrate reduction for synthesized views.

Table 1: Coding performance comparison between NBDV Case 1 and DMDV Case 1

|  | Video 0 | Video 1 | Video 2 | Video only | Synthesized only | Coded & synthesized | Enc time | Dec time | Ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.0% | 0.2% | 0.6% | 0.1% | 0.1% | 0.1% | 108.1% | 103.0% | 107.6% |
| Kendo | 0.0% | -0.5% | -0.8% | -0.3% | -0.4% | -0.3% | 103.6% | 98.7% | 101.8% |
| Newspapercc | 0.0% | 0.1% | 0.1% | 0.0% | 0.0% | 0.0% | 109.4% | 104.2% | 106.6% |
| GhostTownFly | 0.0% | 0.3% | 0.5% | 0.1% | 0.0% | 0.0% | 102.2% | 102.1% | 104.1% |
| PoznanHall2 | 0.0% | -1.2% | -0.1% | -0.3% | -0.3% | -0.3% | 99.0% | 93.7% | 98.0% |
| PoznanStreet | 0.0% | 0.4% | 0.6% | 0.2% | 0.1% | 0.1% | 102.3% | 104.1% | 101.9% |
| UndoDancer | 0.0% | 0.1% | -0.8% | 0.0% | 0.0% | 0.0% | 100.1% | 99.2% | 99.5% |
| 1024x768 | 0.0% | 0.0% | 0.0% | -0.1% | -0.1% | -0.1% | 107.0% | 101.9% | 105.3% |
| 1920x1088 | 0.0% | -0.1% | 0.0% | 0.0% | -0.1% | 0.0% | 100.9% | 99.7% | 100.8% |
| average | 0.0% | -0.1% | 0.0% | 0.0% | -0.1% | -0.1% | 103.5% | 100.7% | 102.7% |

### Comparison of NBDV Case 1 and DMDV Case 2

As shown in Table 2, NBDV and DMDV are compared when both of them have the same merge improvement. Minor coding loss is observed, i.e., 0.1% bitrate increase for texture views and almost the same results for synthesized views.

Table 2: Coding performance comparison between NBDV Case 1 and DMDV Case 2

|  | Video 0 | Video 1 | Video 2 | Video only | Synthesized only | Coded & synthesized | Enc time | Dec time | Ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.0% | 0.5% | 0.8% | 0.3% | 0.2% | 0.2% | 114.1% | 107.1% | 110.6% |
| Kendo | 0.0% | -0.1% | -0.5% | -0.1% | -0.2% | -0.2% | 106.0% | 100.5% | 104.6% |
| Newspapercc | 0.0% | 0.5% | 0.6% | 0.2% | -0.3% | -0.1% | 106.6% | 100.2% | 102.8% |
| GhostTownFly | 0.0% | 0.4% | 0.7% | 0.2% | 0.0% | 0.1% | 102.3% | 99.5% | 101.9% |
| PoznanHall2 | 0.0% | -0.7% | 0.9% | 0.1% | 0.0% | 0.1% | 100.2% | 95.4% | 97.6% |
| PoznanStreet | 0.0% | 0.5% | 0.7% | 0.2% | 0.1% | 0.2% | 107.5% | 105.4% | 107.3% |
| UndoDancer | 0.0% | 0.3% | -0.5% | 0.1% | 0.0% | 0.0% | 103.2% | 98.2% | 102.0% |
| 1024x768 | 0.0% | 0.3% | 0.3% | 0.1% | -0.1% | 0.0% | 108.8% | 102.5% | 105.9% |
| 1920x1088 | 0.0% | 0.1% | 0.4% | 0.1% | 0.1% | 0.1% | 103.3% | 99.6% | 102.1% |
| average | 0.0% | 0.2% | 0.4% | 0.1% | 0.0% | 0.0% | 105.6% | 100.8% | 103.8% |

### Comparison of NBDV Case 2 and DMDV Case 1

As shown in Table 3 NBDV, when compared with DMDV (with bug fix: no prediction between left and right views), has minor coding loss, i.e., 0.1% bitrate increase for synthesized views.

Table 3: Coding performance comparison between NBDV Case 2 and DMDV Case 1

|  | Video 0 | Video 1 | Video 2 | Video only | Synthesized only | Coded & synthesized | Enc time | Dec time | Ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.0% | 0.6% | 1.0% | 0.3% | 0.3% | 0.3% | 100.1% | 101.6% | 107.2% |
| Kendo | 0.0% | -0.1% | -0.4% | -0.1% | -0.2% | -0.1% | 98.6% | 97.9% | 101.3% |
| Newspapercc | 0.0% | 0.5% | 0.5% | 0.2% | -0.1% | 0.0% | 106.9% | 104.4% | 110.9% |
| GhostTownFly | 0.0% | 0.1% | 0.8% | 0.2% | 0.1% | 0.1% | 100.5% | 106.2% | 107.7% |
| PoznanHall2 | 0.0% | -0.1% | 0.6% | 0.2% | 0.1% | 0.1% | 98.8% | 101.5% | 100.4% |
| PoznanStreet | 0.0% | 0.7% | 0.7% | 0.3% | 0.1% | 0.1% | 100.7% | 103.5% | 102.5% |
| UndoDancer | 0.0% | 0.5% | -0.3% | 0.1% | 0.1% | 0.1% | 99.5% | 107.3% | 104.6% |
| 1024x768 | 0.0% | 0.3% | 0.4% | 0.1% | 0.0% | 0.1% | 101.8% | 101.3% | 106.4% |
| 1920x1088 | 0.0% | 0.3% | 0.5% | 0.2% | 0.1% | 0.1% | 99.8% | 104.6% | 103.8% |
| average | 0.0% | 0.3% | 0.4% | 0.2% | 0.1% | 0.1% | 100.7% | 103.2% | 104.9% |

## Experimental results on HTM3.1

The experimental results with the same test conditions and comparisons used in section 2.1 are reported in this section where all the related coding tools are integrated to the latest software of 3DV-HTM version 3.1. The source code with NBDV integrated to 3DV-HTM version 3.1 can be downloaded from the SVN branch: https://hevc.hhi.fraunhofer.de/svn/svn_3DVCSoftware/branches/HTM-3.1-Qualcomm.

### Comparison of NBDV Case 1 and DMDV Case 1

Table 4 lists the coding gain of the best performing NBDV configuration compared to DMDV with bug fix (no dependency between two enhancement views). It is easily observed that slightly coding gain can be achieved, i.e., 0.1% bitrate reduction for synthesized views. Meanwhile, the decoding time is also saved by 3% on average.

Table 4: Coding performance comparison between NBDV Case 1 and DMDV Case 1

|  | Video 0 | Video 1 | Video 2 | Video only | Synthesized only | Coded & synthesized | Enc time | Dec time | Ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.0% | 0.1% | 0.5% | 0.1% | 0.1% | 0.1% | 83.3% | 87.4% | 85.9% |
| Kendo | 0.0% | -0.5% | -0.9% | -0.3% | -0.3% | -0.3% | 98.5% | 96.9% | 102.4% |
| Newspapercc | 0.0% | 0.0% | 0.3% | 0.0% | 0.0% | 0.0% | 108.1% | 100.4% | 104.4% |
| GhostTownFly | 0.0% | -0.1% | 0.5% | 0.1% | 0.1% | 0.1% | 92.8% | 97.2% | 95.1% |
| PoznanHall2 | 0.0% | -1.4% | 0.1% | -0.3% | -0.3% | -0.3% | 100.6% | 98.5% | 101.5% |
| PoznanStreet | 0.0% | 0.6% | 0.8% | 0.2% | 0.1% | 0.1% | 95.5% | 100.0% | 97.9% |
| UndoDancer | 0.0% | 0.0% | -0.7% | 0.0% | -0.2% | -0.1% | 97.1% | 96.3% | 99.4% |
| 1024x768 | 0.0% | -0.1% | 0.0% | -0.1% | -0.1% | -0.1% | 96.1% | 94.7% | 97.2% |
| 1920x1088 | 0.0% | -0.2% | 0.2% | 0.0% | -0.08% | -0.07% | 96.5% | 98.0% | 98.4% |
| average | 0.0% | -0.2% | 0.1% | 0.0% | -0.1% | -0.1% | 96.3% | 96.6% | 97.9% |

### Comparison of NBDV Case 1 and DMDV Case 2

The best coding performance of both NBDV and DMDV with bug fix (no dependency between two enhancement views) are compared in this sub-section. Table 5 lists the coding gain of NBDV Case 1 compared to DMDV Case 2. Minor coding loss is observed, i.e., 0.1% bitrate increase for texture views and almost the same results for synthesized views.

Table 5: Coding performance comparison between Case 1 and DMDV Case 2

|  | Video 0 | Video 1 | Video 2 | Video only | Synthesized only | Coded & synthesized | Enc time | Dec time | Ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.0% | 0.4% | 0.7% | 0.3% | 0.3% | 0.3% | 107.7% | 102.8% | 104.8% |
| Kendo | 0.0% | 0.0% | -0.5% | -0.1% | -0.1% | -0.1% | 101.4% | 98.3% | 103.5% |
| Newspapercc | 0.0% | 0.4% | 0.6% | 0.2% | 0.1% | 0.1% | 108.5% | 99.4% | 105.0% |
| GhostTownFly | 0.0% | 0.1% | 0.5% | 0.1% | 0.1% | 0.1% | 103.3% | 104.1% | 107.1% |
| PoznanHall2 | 0.0% | -0.8% | 1.0% | 0.1% | -0.1% | -0.1% | 102.5% | 100.2% | 104.4% |
| PoznanStreet | 0.0% | 0.7% | 1.1% | 0.3% | 0.1% | 0.2% | 103.5% | 106.7% | 107.0% |
| UndoDancer | 0.0% | 0.2% | -0.4% | 0.1% | 0.0% | 0.0% | 105.1% | 102.0% | 108.5% |
| 1024x768 | 0.0% | 0.3% | 0.3% | 0.1% | 0.1% | 0.1% | 105.8% | 100.1% | 104.4% |
| 1920x1088 | 0.0% | 0.1% | 0.6% | 0.2% | 0.01% | 0.05% | 103.6% | 103.2% | 106.7% |
| average | 0.0% | 0.1% | 0.4% | 0.1% | 0.1% | 0.1% | 104.5% | 101.9% | 105.7% |

### Comparison of NBDV Case 2 and DMDV Case 1

Table 6 lists the coding gain of current NBDV compared to CE5.h with bug-fix (no dependency between two enhancement views). It is observed there is minor coding loss, i.e., 0.2% and 0.1% bitrate increase for the texture views and synthesized views, respectively.  Meanwhile, the decoding time can be saved by 8% on average.

Table 6: Coding performance comparison between NBDV Case 2 and DMDV Case 1

|  | Video 0 | Video 1 | Video 2 | Video only | Synthesized only | Coded & synthesized | Enc time | Dec time | Ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.0% | 0.3% | 1.1% | 0.3% | 0.2% | 0.2% | 79.9% | 84.4% | 84.4% |
| Kendo | 0.0% | 0.0% | -0.5% | -0.1% | -0.1% | -0.1% | 94.0% | 95.5% | 99.3% |
| Newspapercc | 0.0% | 0.4% | 0.7% | 0.2% | 0.0% | 0.1% | 94.3% | 98.3% | 100.0% |
| GhostTownFly | 0.0% | 0.1% | 0.4% | 0.1% | 0.1% | 0.1% | 94.5% | 94.6% | 95.6% |
| PoznanHall2 | 0.0% | -0.8% | 1.4% | 0.1% | 0.0% | 0.0% | 95.8% | 94.0% | 100.8% |
| PoznanStreet | 0.0% | 0.8% | 0.7% | 0.3% | 0.1% | 0.2% | 93.6% | 93.5% | 92.9% |
| UndoDancer | 0.0% | 0.5% | -0.2% | 0.2% | 0.0% | 0.1% | 93.4% | 90.8% | 95.7% |
| 1024x768 | 0.0% | 0.2% | 0.4% | 0.2% | 0.0% | 0.1% | 89.2% | 92.5% | 94.3% |
| 1920x1088 | 0.0% | 0.1% | 0.6% | 0.2% | 0.07% | 0.10% | 94.3% | 93.2% | 96.2% |
| average | 0.0% | 0.2% | 0.5% | 0.2% | 0.1% | 0.1% | 92.1% | 92.9% | 95.4% |

# Conclusion

NBDV provides comparable coding efficiency as DMDV, while greatly reduces the memory consumption, the decoder complexity as well as the expected hardware implementation efforts. Both NBDV and DMDV are in the current HTM reference software, it is recommended to further adopt NBDV into the Test Model under Consideration (TMuC) for 3D-HEVC and to discuss the need of DMDV in TMuC.

# References

- "Test Model under Consideration for HEVC based 3D video coding v3.0," ISO/IEC JTC1/SC29/WG11 MPEG2012/N12744, Apr. 2012.

- Li Zhang, Ying Chen, Marta Karczewicz, "3D-CE5.h related: Disparity vector derivation for multiview video and 3DV," ISO/IEC JTC1/SC29/WG11 MPEG2012/m24937, Apr. 2012.

- 3DV-HTM version 3.1rc1: https://hevc.hhi.fraunhofer.de/svn/svn_3DVCSoftware/tags/HTM-3.1rc1/.

- 3DV-HTM version 3.1: https://hevc.hhi.fraunhofer.de/svn/svn_3DVCSoftware/tags/HTM-3.1/.

- "Common test conditions for 3DV experimentation," ISO/IEC JTC1/SC29/WG11 MPEG2012/N12745, Apr. 2012.

- Li Zhang, Ying Chen, Marta Karczewicz, "3D-CE5.h related: Improved merge mode for inter-view predicted motion," JCT2-A0096, July 2012.

- Li Zhang, Ying Chen, Marta Karczewicz, "3D-CE5.h related: Bug fix for independent estimated depth maps of non-base views in CTC," JCT2-A0095, July 2012.

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).