| Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 1st Meeting: Stockholm, SE, 16–20 July 2012 | Document: JCT2-A0100 |
| --- | --- |

| Title: | 3DV-HLS: Signalling of the number of to-be-decoded texture/depth views for MVC based 3DV |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ying Chen Ye-Kui Wang 5775 Morehouse Drive San Diego, CA 92121 USA | Tel: Email: | 1-858-845-6589 cheny@qualcomm.com 1-858-651-8345 yekuiw@qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

In MVC, for each operation point, the number of views to be decoded is signalled in the MVC SPS extension (by the syntax element applicable_op_num_views_minus1[ i ][ j ]). In MVC based 3DV, the semantics of this syntax element becomes ambiguous and insufficient, since the current PDAM allows a depth view to have a different view dependency than that of the texture view. It is proposed that applicable_op_num_views_minus1[ i ][ j ] is used to signal the number of texture views to be decoded while the number of depth views to be decoded for each operation point is further signalled by another syntax element in the subset SPS.

# Introduction

In 3DV, a subset SPS contains both SPS MVC extension and SPS 3DVC extension. The SPS MVC extension is shown below. The syntax elements apply to either the depth views or the 3DV operation points (each includes both texture and depth).

| seq_parameter_set_mvc_extension( ) { | C | Descriptor |
| --- | --- | --- |
| - num_views_minus1 | 0 | ue(v) |
| for( i = 0; i <= num_views_minus1; i++ ) |  |  |
| view_id[ i ] | 0 | ue(v) |
| for( i = 1; i <= num_views_minus1; i++ ) { |  |  |
| num_anchor_refs_l0[ i ] | 0 | ue(v) |
| for( j = 0; j < num_anchor_refs_l0[ i ]; j++ ) |  |  |
| anchor_ref_l0[ i ][ j ] | 0 | ue(v) |
| num_anchor_refs_l1[ i ] | 0 | ue(v) |
| for( j = 0; j < num_anchor_refs_l1[ i ]; j++ ) |  |  |
| anchor_ref_l1[ i ][ j ] | 0 | ue(v) |
| } |  |  |
| for( i = 1; i <= num_views_minus1; i++ ) { |  |  |
| num_non_anchor_refs_l0[ i ] | 0 | ue(v) |
| for( j = 0; j < num_non_anchor_refs_l0[ i ]; j++ ) |  |  |
| non_anchor_ref_l0[ i ][ j ] | 0 | ue(v) |
| num_non_anchor_refs_l1[ i ] | 0 | ue(v) |
| for( j = 0; j < num_non_anchor_refs_l1[ i ]; j++ ) |  |  |
| non_anchor_ref_l1[ i ][ j ] | 0 | ue(v) |
| } |  |  |
| num_level_values_signalled_minus1 | 0 | ue(v) |
| for( i = 0; i <= num_level_values_signalled_minus1; i++ ) { |  |  |
| level_idc[ i ] | 0 | u(8) |
| num_applicable_ops_minus1[ i ] | 0 | ue(v) |
| for( j = 0; j <= num_applicable_ops_minus1[ i ]; j++ ) { |  |  |
| applicable_op_temporal_id[ i ][ j ] | 0 | u(3) |
| applicable_op_num_target_views_minus1[ i ][ j ] | 0 | ue(v) |
| for( k = 0; k <= applicable_op_num_target_views_minus1[ i ][ j ]; k++ ) |  |  |
| applicable_op_target_view_id[ i ][ j ][ k ] | 0 | ue(v) |
| applicable_op_num_views_minus1[ i ][ j ] | 0 | ue(v) |
| } |  |  |
| } |  |  |
| } |  |  |

In the current specification, it is not clear to what exactly the syntax element applicable_op_num_views_minus1[ i ][ j ] applies. It can be used to indicate the number of texture views to be decoded, the number of depth views to be decoded, or the number of views to be decoded when each view includes both depth and texture, because it is allowed that texture and depth of the same view can have different view dependencies.

# Proposal

To resolve the above ambiguity, it is proposed that

- applicable_op_num_views_minus1[ i ][ j ] in an MVC SPS extension specifies the number of texture views to be decoded for the operation point;

- A new syntax element applicable_op_num_depth_views_minus1[ i ][ j ] is added in the 3DVC SPS extension to indicate the number of depth views to be decoded for the operation point.

The syntax and semantics changes are as follows.

### Sequence parameter set 3DVC extension syntax

| seq_parameter_set_3dvc_extension( ) { | C | Descriptor |
| --- | --- | --- |
| for( i = 0; i <= num_level_values_signalled_minus1; i++ ) |  |  |
| for( j = 0; j <= num_applicable_ops_minus1[ i ]; j++ ) |  |  |
| applicable_op_num_depth_views_minus1[ i ][ j ] | 0 | ue(v) |
| … |  |  |
| } |  |  |

### Sequence parameter set 3DVC extension semantics

applicable_op_num_depth_views_minus1[ i ][ j ] plus 1 specifies the number of depth views required for decoding the target output views corresponding to the j-th operation point to which the level indicated by level_idc[ i ] applies. The number of depth views specified by applicable_op_num_depth_views_minus1 includes the target output views and the depth views that the depth views of the target output views depend on. The value of applicable_op_num_depth_views_minus1[ i ][ j ] shall be in the range of 0 to 1023, inclusive.

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).