| Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 1st Meeting: Stockholm, SE, 16–20 July 2012 | Document: JCT2-A0103 |
| --- | --- |

| Title: | 3D-CE1.a: Generalized view synthesis prediction (GVSP) mode |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xin Zhao Li Zhang Ying Chen Marta Karczewicz 5775 Morehouse Drive San Diego, CA 92121 USA | Tel: Email: | 86-10-5776-0696 xinzhao@qualcomm.com 1-858-651-6660 lizhang@qualcomm.com 1-858-845-6589 cheny@qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

It is proposed to change the signaling of View Synthesis Prediction (VSP) so that a VSP picture is not added into a reference picture list and the motion vectors to the other reference pictures are not used to predict the motion to a VSP picture. Instead, zero motion is always assumed for a block predicted from a co-located block of a VSP picture and a flag is used to indicate such prediction. This proposal is technically identical to m24940, but integrated to the latest 3D-ATM. Without complexity increase, the proposed method gives 0.8% gain in a stable CE configuration.

# Introduction

In-loop View Synthesis Prediction (VSP) is supported in 3DV-TM for enhanced texture coding. To enable VSP for encoding the current view, the view synthesized picture of the current view is generated from the previously coded texture and depth view components of the same access unit. Currently the resulted view synthesized picture is included in the initial reference picture list List0 following temporal and inter-view reference frames. If view synthesized picture is chosen as the reference, the reference index and MVD are signaled in the corresponding MB or MB partition level.

In the current WD, a new VSP skip/direct [1] mode is enabled. It adds a flag called mb_skip_type_flag after the MB skip flag. If mode decision determines that the reference picture is view synthesized picture for skip mode, the mb_skip_type_flag is 1, otherwise it is 0. In addition, another flag called mb_direct_type_flag is added to indicate whether the reference picture for direct mode is view synthesized picture or not. To further improve the coding efficiency, a context-based adaptive skip flag positioning method was proposed and adopted in the last meeting [2].

The view synthesized picture is the synthesized picture for the current view. The motion vector to the view synthesized picture is always very close to zero. Therefore, in this document, we propose to introduce a new VSP mode GVSP mode by adding a flag at the MB partition level to indicate whether a MB partition is predicted from a view synthesized picture. As there is virtually no motion between the coded picture and its view synthesized picture, it is proposed to remove the view synthesized picture from the reference picture list. Instead, the view synthesized picture is enlisted in the decoded picture buffer. In this way, bits used in encoding the motion information can be saved.

# Proposal

The proposed GVSP mode is summarized as follows.

For a non-skip MB, a flag is introduced to indicate whether the whole MB is predicted from VSP or not.

For a MB which is not totally predicted from VSP, each of its MB partition (no smaller than 8x8) has a flag to signal whether the MB partition is predicted from VSP or not.

If a MB or MB partition is predicted from VSP, no reference index or motion vector information is transmitted. Residual information such as CBP and coded coefficients are signaled as usual.

Since no motion information is signaled, VSP picture will never be inserted into the reference picture list. Instead it is stored in the decode picture buffer. When a MB partition or MB partition is signaled to be predicted from VSP picture, the collocated MB or MB partition in VSP picture will be used as the prediction.

The value of vsp_flag will affect the encoding of motion vectors of neighboring MBs. When a MB needs to encode the motion vectors, a predicted motion vector is formed based on motion vectors of nearby, previously coded partitions. If it is discovered that the vsp_flag of the neighboring block is set to 1, the block is set to be unavailable, thus has no impact on the motion field encoding.

# Experimental results

In this section, objective coding performance of the proposed method is presented and compared with 3DV-ATM v0.4 anchor EHP profile.

The proposed algorithm is implemented based on 3DV-ATM v0.4 software. As specified in the common testing condition document (CTC) [3], four rate distortion points are generated for each sequence with QP equal to 41, 36, 31 and 26.

## Bitrates vs. decoded texture view PSNRs

In Table 1, the results are reported for coding gain with respect to the anchor where the bitrates represent the bitrates of texture and the PSNR values are the average values of all decoded texture views. The BD bitrate reduction is about 0.46%.

Table 2 lists the coding gain of the proposed algorithm with respect to the anchor where the bitrates represent the total bitrates of bitstreams containing both texture and depth of all three views and the PSNR values are the average PSNR values of the three decoded texture views. The BD bitrate reduction is about 0.42%.

Table 1: Coding gain with respect to anchor for 3-view case 
(total bitrates of texture vs. average PSNR of decoded texture views)

|  |  |  | Coding Gain relative to Anchor (3-view) |  |  |
| --- | --- | --- | --- | --- | --- |
| Resolution | Class | Seq. No. | Seq. Name | BD Bitrate (%) | BD PSNR Gain (dB) |
| 1920x1088 | Class A | S01 | Poznan_Hall2 | 0.01 | 0.00 |
|  |  | S02 | Poznan_Street | -0.31 | 0.01 |
|  |  | S03 | Undo_Dancer | -0.53 | 0.02 |
|  |  | S04 | GT_Fly | -0.82 | 0.03 |
| 1024x768 | Class C | S05 | Kendo | -0.71 | 0.04 |
|  |  | S06 | Balloons | -0.44 | 0.02 |
|  |  | S08 | Newspaper | -0.40 | 0.02 |
|  |  |  | Overall Avg | -0.46 | 0.02 |

Table 2: Coding gain with respect to anchor for 3-view case 
(total bitrates vs. average PSNR of decoded texture views)

|  |  |  | Coding Gain relative to Anchor (3-view) |  |  |
| --- | --- | --- | --- | --- | --- |
| Resolution | Class | Seq. No. | Seq. Name | BD Bitrate (%) | BD PSNR Gain (dB) |
| 1920x1088 | Class A | S01 | Poznan_Hall2 | 0.02 | 0.00 |
|  |  | S02 | Poznan_Street | -0.28 | 0.01 |
|  |  | S03 | Undo_Dancer | -0.49 | 0.02 |
|  |  | S04 | GT_Fly | -0.77 | 0.03 |
| 1024x768 | Class C | S05 | Kendo | -0.64 | 0.03 |
|  |  | S06 | Balloons | -0.41 | 0.02 |
|  |  | S08 | Newspaper | -0.36 | 0.01 |
|  |  |  | Overall Avg | -0.42 | 0.02 |

Table 3: Coding gain with respect to anchor for 3-view case when QC_VSP_Asymmetric_Resolution is off
(dependent texture view bitrates vs. PSNR of dependent texture view)

|  |  |  | Coding Gain relative to Anchor (3-view) |  |  |
| --- | --- | --- | --- | --- | --- |
| Resolution | Class | Seq. No. | Seq. Name | BD Bitrate (%) | BD PSNR Gain (dB) |
| 1920x1088 | Class A | S01 | Poznan_Hall2 | -0.29 | 0.01 |
|  |  | S02 | Poznan_Street | -0.46 | 0.01 |
|  |  | S03 | Undo_Dancer | -0.76 | 0.03 |
|  |  | S04 | GT_Fly | -1.59 | 0.06 |
| 1024x768 | Class C | S05 | Kendo | -1.27 | 0.06 |
|  |  | S06 | Balloons | -0.86 | 0.04 |
|  |  | S08 | Newspaper | -0.75 | 0.03 |
|  |  |  | Overall Avg | -0.85 | 0.03 |

Table 4: Coding gain with respect to anchor for 3-view case when QC_VSP_Asymmetric_Resolution is off
(total views bitrates vs. average PSNR of dependent texture views)

|  |  |  | Coding Gain relative to Anchor (3-view) |  |  |
| --- | --- | --- | --- | --- | --- |
| Resolution | Class | Seq. No. | Seq. Name | BD Bitrate (%) | BD PSNR Gain (dB) |
| 1920x1088 | Class A | S01 | Poznan_Hall2 | -0.24 | 0.01 |
|  |  | S02 | Poznan_Street | -0.40 | 0.01 |
|  |  | S03 | Undo_Dancer | -0.70 | 0.02 |
|  |  | S04 | GT_Fly | -1.49 | 0.05 |
| 1024x768 | Class C | S05 | Kendo | -1.09 | 0.05 |
|  |  | S06 | Balloons | -0.77 | 0.04 |
|  |  | S08 | Newspaper | -0.65 | 0.03 |
|  |  |  | Overall Avg | -0.76 | 0.03 |

In Table 3, for the case of QC_VSP_Asymmetric_Resolution off for both anchor and proposed method, the results are reported for coding gain with respect to the anchor where the bitrates represent the bitrates of texture and the PSNR values are the average values of all decoded texture views. The BD bitrate reduction is about 0.85%.

In Table 4, for the case of QC_VSP_Asymmetric_Resolution off for both anchor and proposed method, the results are reported for coding gain with respect to the anchor where the bitrates represent the total bitrates of bitstreams containing both texture and depth of all three views and the PSNR values are the average PSNR values of the three decoded texture views. The BD bitrate reduction is about 0.76%.

## Bitrates vs. synthesized texture view PSNRs

As specified in the CTC document [2], intermediate views are synthesized between decoded views for each sequence. Table 5 lists the coding gain with respect to the anchor where the bitrates represent the total bitrates of texture and depth and the PSNR values are the average synthesized view PSNRs. The BD rate reduction is about 0.37% for 3-view case.

Table 5: Coding gain with respect to anchor for 3-view case 
(total bitrates vs. average PSNR of synthesized views)

|  |  |  | Coding Gain relative to Anchor (3-view) |  |  |
| --- | --- | --- | --- | --- | --- |
| Resolution | Class | Seq. No. | Seq. Name | BD Bitrate (%) | BD PSNR Gain (dB) |
| 1920x1088 | Class A | S01 | Poznan_Hall2 | 0.09 | 0.00 |
|  |  | S02 | Poznan_Street | -0.27 | 0.01 |
|  |  | S03 | Undo_Dancer | -0.49 | 0.01 |
|  |  | S04 | GT_Fly | -0.88 | 0.03 |
| 1024x768 | Class C | S05 | Kendo | -0.44 | 0.02 |
|  |  | S06 | Balloons | -0.27 | 0.01 |
|  |  | S08 | Newspaper | -0.30 | 0.01 |
|  |  |  | Overall Avg | -0.37 | 0.01 |

Again if we disable QC_VSP_Asymmetric_Resolution in both anchor and proposed method, the coding gain with respect to the anchor where the bitrates represent the total bitrates of texture and depth and the PSNR values are the average synthesized view PSNRs are reported in Table 6. The BD rate reduction is about 0.69% for 3-view case.

Table 6: Coding gain with respect to anchor for 3-view case 
(total bitrates vs. average PSNR of synthesized views)

|  |  |  | Coding Gain relative to Anchor (3-view) |  |  |
| --- | --- | --- | --- | --- | --- |
| Resolution | Class | Seq. No. | Seq. Name | BD Bitrate (%) | BD PSNR Gain (dB) |
| 1920x1088 | Class A | S01 | Poznan_Hall2 | -0.24 | 0.01 |
|  |  | S02 | Poznan_Street | -0.39 | 0.01 |
|  |  | S03 | Undo_Dancer | -0.63 | 0.02 |
|  |  | S04 | GT_Fly | -1.39 | 0.04 |
| 1024x768 | Class C | S05 | Kendo | -0.93 | 0.04 |
|  |  | S06 | Balloons | -0.62 | 0.03 |
|  |  | S08 | Newspaper | -0.64 | 0.02 |
|  |  |  | Overall Avg | -0.69 | 0.02 |

## Complexity analysis

Table 7 summarizes the computational time of the proposed method compared to the anchor. Our encoding times were collected from log files generated by the encoder, which were obtained using the same process as used by 3DV-ATM anchor. Compared to anchor, the proposed encoder has about 11% increased runtime for 3-view case which is mainly due to our added modes in mode decision. The decoding times were collected from log files generated by the decoder, which were obtained using the same process as the anchor. The I/O times (for output YUV file generation) are not included in the reported decoding times for both decoders. Compared to anchor, the proposed decoder is about 95.9% of the anchor runtime for 3-view case.

Table 7: Computational time of the proposed method

|  | Complexity estimate (ratio to anchor) |  |  |
| --- | --- | --- | --- |
|  | Encoder Time, % | Decoder Time, % | Rendering Time, % |
| S01 | 124.7% | 129.2% | 101.8% |
| S02 | 108.1% | 121.8% | 101.4% |
| S03 | 103.6% | 93.4% | 104.7% |
| S04 | 108.9% | 86.4% | 99.9% |
| S05 | 112.5% | 88.5% | 101.1% |
| S06 | 110.9% | 89.2% | 99.7% |
| S08 | 108.2% | 63.0% | 98.8% |
| Average | 111.0% | 95.9% | 101.1% |

# Conclusion

The proposed GVSP method is able to bring additional 0.8% gain in a stable CE configuration without complexity increase. Therefore, it is recommended to adopt this proposal for the next ATM and working draft.

# References

- “Description of 3D video coding technology proposal by ETRI and Kwangwoon University,” ISO/IEC JTC1/SC29/WG11/M23915, San Jose, USA, Feb. 2012.

- Jin Young Lee, Jaejoon Lee, and Du-Sik Park, “3D-CE1.a results on a context-based adaptive skip flag positioning method by Samsung”, ISO/IEC JTC1/SC29/WG11 MPEG2011/m24819, Geneva, Switzerland, April 2012.

- “Common test conditions for 3DV experimentation,” ISO/IEC JTC1/SC29/WG11 MPEG2011/N12745, Geneva, Switzerland, April 2012.

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).