| Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 1st Meeting: Stockholm, SE, 16–20 July 2012 | Document: JCT2-A0105 |
| --- | --- |

| Title: | 3D-CE6.h related: Fast Wedgelet search |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Philipp Merkle, Hunn Rhee, Karsten Mueller, Thomas Wiegand | Email: | philipp.merkle@hhi.fraunhofer.de, hunn.rhee@hhi.fraunhofer.de, karsten.mueller@hhi.fraunhofer.de, thomas.wiegand@hhi.fraunhofer.de |
| Source: | Fraunhofer HHI |  |  |

_____________________________

# Abstract

This contribution proposes a fast search method fort the Wedgelet-based depth coding modes. It introduces a two-step search strategy as an alternative to the full search currently used in 3DV-HTM-3.1 [1]. Under common test condition [2] this method reduces the encoder complexity to 95.7% and for intra-only configuration the encoder/decoder complexity reduces to 46.5%/78.3%. For both configurations the BD-rate increases negligibly by less than 0.1%.

# Introduction

The additional depth intra coding modes in 3DV-HTM, referred to as DMM, are adapted to the specific characteristics of depth maps by applying non-rectangular block partitions for approximating the signal of a depth block containing an edge. Two of the modes use a minimum distortion Wedgelet search for finding the best matching partition for a block. For this purpose a Wedgelet pattern look-up list is provided that contains all possible Wedgelet partition patterns for a certain block size. By searching the full list of patterns the minimum distortion Wedgelet partition is definitely found, but such a full search requires a substantial amount of processing. In this contribution we propose a fast search strategy that significantly reduces the complexity of the minimum distortion Wedgelet search.

# Proposed method

The proposed method is an alternative to the full Wedgelet pattern list search and basically consists of two steps. In a first step a coarse Wedgelet pattern list is searched for the minimum distortion partition and in a second step the resulting Wedgelet partition is refined. This requires modifying the Wedgelet pattern list initialization process as well as the search function.

## Wedgelet pattern list initialization

In 3DV-HTM the Wedgelet pattern lists are generated during encoder and decoder initialization. For a given block size they contain all unique binary Wedgelet patterns. By combining all possible start and end positions for the Wedgelet separation line, all possible partition patterns are generated and added to the list, if an identical pattern is not already part of the list. For the proposed fast Wedgelet search the list initialization is extended as follows: According to the example in Fig.1 a coarse subset of patterns is marked during the list generation process described above, such that the coarse subset contains all Wedgelet patterns generated from every second start and end position.

Figure 1. Coarse subset of Wedgelet separation line start/end positions

In a next step a Wedgelet node list is derived from the pattern list, where each Wedgelet marked as part of the coarse subset corresponds to a Wedgelet node. Besides a reference to the coarse Wedgelet, a node contains up to eight references to refinement Wedgelet patterns. All refinement patterns are not part of the coarse subset, as shown in the example in Fig. 2. By varying the start and end position of the separation line in the range [-1, 1] eight refinement candidates are defined and the reference to the Wedgelet pattern is added to the node, if an identical pattern is not already part of the refinement list.

Figure 2. Wedgelet node and refinements

## Search function

In 3DV-HTM the minimum distortion Wedgelet search consist of testing each pattern contained in the Wedgelet pattern list corresponding to the block size. Such a search strategy is usually referred to as full search. For the proposed fast Wedgelet search the processing is modified as follows: In a first step the minimum distortion Wedgelet pattern is searched for the coarse subset by iterating over the Wedgelet node list and testing the referenced pattern. In a second step the Wedgelet partition found in the first step is refined. For this purpose the minimum distortion Wedgelet pattern is searched from the up to eight refinement patterns referenced in the corresponding node.

Table 1. Number of Wedgelet patterns tested for full and fast search
depending on the block size

|  | full search | fast search | ratio fast/full |
| --- | --- | --- | --- |
| 4×4 | 86 | ≤ 58 + 8 | 76.7% |
| 8×8 | 782 | ≤ 310 + 8 | 40.7% |
| 16×16 | 1394 | ≤ 338 + 8 | 24.8% |
| 32×32 | 1503 | ≤ 368 + 8 | 25.0% |

The main difference between the full search and the proposed search strategy is the number of Wedgelet patterns that has to be tested, as shown in Table 1 for the different block sizes.

# Experimental results

Table 2. Result for 3-view scenario under CTC

|  | video only | synthesized only | coded & synthesized | enc time | dec time | ren time |
| --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.0% | 0.0% | 0.0% | 98.2% | 102.2% | 103.1% |
| Kendo | 0.0% | 0.1% | 0.1% | 93.2% | 98.8% | 98.4% |
| Newspapercc | 0.0% | -0.1% | -0.1% | 93.6% | 98.1% | 99.3% |
| GhostTownFly | 0.0% | 0.0% | 0.0% | 96.6% | 94.8% | 97.8% |
| PoznanHall2 | 0.0% | 0.2% | 0.2% | 97.7% | 98.8% | 99.9% |
| PoznanStreet | 0.0% | 0.0% | 0.0% | 94.7% | 99.5% | 101.1% |
| UndoDancer | 0.0% | 0.0% | 0.0% | 96.3% | 97.9% | 98.9% |
| 1024x768 | 0.0% | 0.0% | 0.0% | 95.0% | 99.7% | 100.2% |
| 1920x1088 | 0.0% | 0.0% | 0.0% | 96.3% | 97.7% | 99.4% |
| average | 0.0% | 0.0% | 0.0% | 95.7% | 98.5% | 99.8% |

Table 3. Result for 3-view scenario under intra-only configuration

|  | video only | synthesized only | coded & synthesized | enc time | dec time | ren time |
| --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.0% | 0.1% | 0.1% | 48.4% | 80.7% | 100.5% |
| Kendo | 0.0% | 0.1% | 0.1% | 49.0% | 83.5% | 98.2% |
| Newspapercc | 0.0% | 0.1% | 0.1% | 43.2% | 69.8% | 101.4% |
| GhostTownFly | 0.0% | 0.1% | 0.0% | 48.2% | 71.8% | 97.4% |
| PoznanHall2 | 0.0% | 0.1% | 0.0% | 50.0% | 90.7% | 102.7% |
| PoznanStreet | 0.0% | 0.0% | 0.0% | 42.5% | 76.1% | 101.2% |
| UndoDancer | 0.0% | 0.1% | 0.1% | 44.8% | 77.4% | 102.8% |
| 1024x768 | 0.0% | 0.1% | 0.1% | 46.8% | 77.8% | 100.0% |
| 1920x1088 | 0.0% | 0.1% | 0.0% | 46.3% | 78.7% | 101.0% |
| average | 0.0% | 0.1% | 0.1% | 46.5% | 78.3% | 100.6% |

According to the CE6.h description [3], the proposed method is additionally evaluated for an intra-only configuration. This is achieved by the following modifications to the CTC encoder configuration files provided with 3DV-HTM-3.1:

#========== coding structure ==========

IntraPeriod               : 1

DecodingRefreshType       : 0

GOPSize                   : 1

# V0        Type POC QPoffset QPfactor temporal_id #ref_pics_act ref_pic #ref_pics predict #IV_ref_pics

Frame1    : B    1   1        1        0           4             1       0         0       0

Frame2    :

Frame3    :

Frame4    :

Frame5    :

Frame6    :

Frame7    :

Frame8    :

# V1        Type POC QPoffset QPfactor temporal_id #ref_pics_act ref_pic #ref_pics predict #IV_ref_pics

FrameI_v1 : I    0   0        1        0           0             1       0         0       0

Frame1_v1 : B    1   1        1        0           4             1       0         0       0

Frame2_v1 :

Frame3_v1 :

Frame4_v1 :

Frame5_v1 :

Frame6_v1 :

Frame7_v1 :

Frame8_v1 :

# V2        Type POC QPoffset QPfactor temporal_id #ref_pics_act ref_pic #ref_pics predict #IV_ref_pics

FrameI_v2 : I    0   0        1        0           0             1       0         0       0

Frame1_v2 : B    1   1        1        0           4             1       0         0       0

Frame2_v2 :

Frame3_v2 :

Frame4_v2 :

Frame5_v2 :

Frame6_v2 :

Frame7_v2 :

Frame8_v2 :

# Conclusion

The results show that the complexity is reduced by 4.3% (encoder) for random access configuration and by 53.5% (encoder) and 21.7% (decoder) for intra-only configuration. The BD-rate increase is negligible, being less than 0.1% for both configurations. In conclusion the fast Wedgelet search method leads to a significant complexity reduction at almost the same RD performance and should therefore be considered for adoption to 3DV-HTM.

# References

- [1]	Heiko Schwarz and Krzysztof Wegner, “Test Model under Consideration for HEVC based 3D video coding v3.0,” ISO/IEC JTC1/SC29/WG11 document W12744, Geneva, Switzerland, Apr. 2012.

- [2]	Heiko Schwarz and Dmytro Rusanovskyy, “Common test conditions for 3DV experimentation,” ISO/IEC JTC1/SC29/WG11 document W12745, Geneva, Switzerland, Apr. 2012.

- [3]	Anthony Vetro and Karsten Müller, “Description of Core experiments in 3D video coding,” ISO/IEC JTC1/SC29/WG11 document W12746, Geneva, Switzerland, Apr. 2012.

Fraunhofer-Gesellschaft zur Förderung der angewandten Forschung e.V. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).