| Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 1st Meeting: Stockholm, SE, 16–20 July 2012 | Document: JCT2-A0107 |
| --- | --- |

| Title: | 3DV-CE1.a: Block-based View Synthesis Prediction for 3DV-ATM |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Wenyi Su University of Science and Technology of China Dmytro Rusanovskyy, Miska M. Hannuksela Nokia Research Center Visiokatu 3, 33720 Tampere, Finland | Tel: Email: | Firstname.Lastname@nokia.com |
| Source: | Nokia Corporation |  |  |

_____________________________

# Abstract

The current implementation of View Synthesis Prediction (VSP) in 3DV-ATM utilizes a forward warping to produce a complete reference image (VSP frame). An asserted drawback of such frame-level implementation is its high complexity at the decoder side and its incompatibility with a block-based processing architecture which is traditionally utilized for decoder.

This contribution proposes a block-based VSP implementation (B-VSP) which is based on a backward warping process. With the B-VSP, texture pixels of a dependent view are predicted not from the synthesized VSP-frame, but directly from the texture pixels of the base view. Displacement vectors required for this process are produced from the depth map data of the dependent view. It is asserted that B-VSP reduces the complexity of view synthesis prediction considerably and avoids the need to allocate a frame buffer for the VSP reference frame.

The presented simulation results show that proposed B-VSP scheme outperforms the current 3DV-ATM design of VSP by 0.4% dBR for coded texture views and by 0.7% of dBR for synthesized views. It is asserted that the estimated decoding time for B-VSP is ~20% lower on average than the decoding time of the 3DV-ATM v4 anchor. When considering the execution time of the VSP modules only, B-VSP reportedly provides 70% complexity reduction in comparison to the anchor VSP module in 3DV-ATM v4.

# Introduction

Current implementation of 3DV-ATM utilizes in-loop View Synthesis based Prediction (VSP). This tool brings a feasible coding gain but remains the most complex part of the 3DV-ATM decoder. A rough complexity estimates shows that current design of in-loop VSP occupies about 30-40% of decoding time.

And what is more important, the current design of in-loop VSP requires a frame level implementation which impose significant computational and memory access requirements at the decoder side. E.g. decoder needs to allocate a memory for reference frame, need to access a texture and depth map and produce VS results for a complete VSP frame, even if 10-15% of it to be utilized at the decoder side with VSP-prediction.

# Block-based VSP (B-VSP) based on backward warping

The concept of proposed B-VSP is shown in Figure 1.

Let us assume that the following coding order is utilized: (T0, D0, D1, T1). Texture component T0 is a base view and T1 is dependent view coded with the VSP. Depth map components D0 and D1 are respective depth map associated with T0 and T1.

In dependent view T1, sample values of currently coded block Cb are predicted from reference area R(Cb) that consists of sample values of the base view T0. The displacement vector (motion vector) between coded and reference samples is found as a disparity between T1 and T0 from a depth map value associated with a currently coded texture sample. In such implementation, B-VSP is comparable with a tradition Motion Compensated Prediction (MCP) where displacement (motion) vectors are replaced with disparity vectors.

Figure 1. Visualization of Block-based VSP (B-VSP) based on backward warping

The process of conversion may be performed for example with following equations:

ZCbj,i=1dCbj,i255⋅1Znear−1Zfar+1Zfar;DCbj,i=f⋅bZCbj,i; 	(1)

where j and i are local spatial coordinates within Cb, d(Cbj,i)  is a depth map value in depth map image of a view #1, Z is its actual depth value, and D  is a disparity to a particular view #0. The parameters f, b, Znear and Zfar are parameters specifying the camera setup; i.e. the used focal length (f), camera separation (b) between view #1 and view #0i  and depth range (Znear,Zfar) representing parameters of depth map conversion.

In a current 3DV-ATM implementation [2] equation (1) is already precomputed for every depth map value (0…255) and stored as a look up-table. Therefore, its complexity in B-VSP is equal to complexity of fetching a table value.

# B-VSP Implementation in 3DV-ATM

Taking into consideration current 3DV-ATM architecture and statistics of motion vectors utilized in VSP B-VSP was implemented as following:

## Spatial Resolution of Disparity Information

3DV-ATM assumes coding of depth map at quarter resolution compare to the texture. To avoid in-loop depth map resampling, B-VSP process depth map of dependent view D1 at the original resolution. Therefore, a single disparity value is calculated as in (1) for a 2x2 block texture samples of T1.

## Motion Vectors Restrictions in B-VSP

Following study on the MV distribution for VSP-frames, vertical displacement in VSP prediction has been restricted to zero value, and accuracy of horizontal displacement has been limited to a quarter-pixel resolution.

mv_y = 0.0;

mv_x = Disparity(Cb(j,i);

Since vertical component of motion vector is a-priori known to be equal to zero and horizontal component is extracted from pre-coded depth map data, no motion information is transmitted to the decoder for every motion partition predicted with VSP. This solution was proposed earlier in [3].

## Bug-fix of DMVP for B-VSP

Current design of 3DV-ATM utilizes two contradicting approaches to select VSP_SKIP and VSP_DIRECT modes.

The first approach is derivation of VSP_SKIP or VSP_DIRECT through a Depth-based Motion Completion (DMC) where synthesized depth map is utilized for mode derivation at the decoder side. No signaling shall be utilized in this case.

The second approach is explicit signaling of VSP_SKIP and VSP_DIRECT modes with a flag introduced by MERL in recent adoption [4]. These coding modes are evaluated at the encoder side through RDO and overlap/disturb the decision making process utilized in DMC. Even if DMC selects VSP_SKIP or VSP_DIRECT mode explicit signaling is still utilized for these coding modes. With respect to these recent changes to 3DV-ATM, motion vector derivation in DMC is redundant for blocks coded with VSP_SKIP and/or VSP_DIRECT modes.

Table 1. Simulation results for disabled DMC for VSP, 3DV-ATM v4 utilized as anchor.

|  | Texture Coding |  | Total (Coded PSNR) |  | Complexity estimate (ratio to anchor) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | dBR, % | dPSNR,dB | dBR, % | dPSNR,dB | Encoder Time, % | Decoder Time, % | Rendering Time, % |
| S01 | 0.98 | -0.03 | 0.89 | -0.03 | 111 | 94 | 100 |
| S02 | -0.06 | 0.00 | -0.06 | 0.00 | 110 | 97 | 100 |
| S03 | -0.06 | 0.00 | -0.07 | 0.00 | 102 | 89 | 100 |
| S04 | -0.19 | 0.01 | -0.18 | 0.01 | 103 | 94 | 100 |
| S05 | -0.27 | 0.01 | -0.26 | 0.01 | 126 | 94 | 100 |
| S06 | 0.08 | 0.00 | 0.05 | 0.00 | 99 | 95 | 100 |
| S08 | -0.07 | 0.00 | -0.07 | 0.00 | 126 | 101 | 100 |
| Average | 0.06 | 0.00 | 0.04 | 0.00 | 111 | 95 | 100 |

The proposed B-VSP does not produce virtual depth map. To resolve this problem we either should produce virtual depth map for purposes of DMC or disable DMC decision making for VSP. We have studied these alternatives and found that disabling of DMC for VSP does not produce any significant penalty on coding gain, but it provides about 5% reduction of the decoding time in comparison to the 3DV-ATM anchor.  Simulation results for 3DV-ATM v4 anchor and disabled DMC for VSP are shown in Table 1. Results of Table 1 have been cross-checked within CE5 as an independent proposal [5].

# Simulation results for B-VSP

Proposed B-VSP scheme was implemented in the 3DV-ATM software and tested with EHP C3 coding configuration under the Common Test Conditions [1]. Simulation results are reported in the excel document attached to this email, and table 2 and Table 3 given below provides the summary.

Simulation results show that proposed B-VSP scheme outperforms the current design of VSP by 0.4% dBR for coded texture views and by 0.7% of dBR for synthesized views.

Estimated decoding time for codec with B-VSP is ~20% lower on average than decoding time of the 3DV-ATM v4 anchor. Estimates for execution time of VSP modules only shows, that B-VSP provides 70% complexity reduction in comparison to the anchor VSP module in 3DV-ATM v4.

Table 2. Coding performance of the B-VSP in comparison against the 3DV-ATM v4 anchor

|  | Texture Coding |  | Depth Coding |  | Total (Coded PSNR) |  | Total (Synthesized PSNR) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | dBR, % | dPSNR, dB | dBR, % | dPSNR, dB | dBR, % | dPSNR, dB | dBR, % | dPSNR,dB |
| S01 | 0.77 | -0.02 | 0.00 | 0.00 | 0.70 | -0.02 | 0.27 | -0.01 |
| S02 | 0.29 | -0.01 | 0.00 | 0.00 | 0.26 | -0.01 | 0.02 | 0.00 |
| S03 | 0.03 | 0.00 | 0.00 | 0.00 | 0.02 | 0.00 | -0.60 | 0.02 |
| S04 | -0.01 | 0.00 | 0.00 | 0.00 | -0.01 | 0.00 | -0.31 | 0.01 |
| S05 | -1.41 | 0.07 | 0.00 | 0.00 | -1.19 | 0.06 | -1.45 | 0.06 |
| S06 | -1.99 | 0.10 | 0.00 | 0.00 | -1.72 | 0.08 | -2.18 | 0.10 |
| S08 | -0.62 | 0.03 | 0.00 | 0.00 | -0.54 | 0.02 | -0.57 | 0.02 |
| Average | -0.42 | 0.02 | 0.00 | 0.00 | -0.35 | 0.02 | -0.69 | 0.03 |

Table 3. Execution time estimates for the B-VSP in comparison against the 3DV-ATM v4 anchor

|  | Complexity estimate (ratio to anchor) |  |  |
| --- | --- | --- | --- |
|  | Encoder Time, % | Decoder Time, % | VSP module, % |
| S01 | 1.09 | 79.84 | 21.85 |
| S02 | 1.03 | 80.60 | 21.60 |
| S03 | 0.92 | 84.26 | 48.07 |
| S04 | 1.05 | 87.04 | 74.24 |
| S05 | 1.03 | 80.09 | 23.06 |
| S06 | 0.93 | 77.52 | 29.49 |
| S08 | 1.02 | 78.25 | 7.31 |
| Average | 1.01 | 81.09 | 32.23 |

# References

[1] 	“Common test conditions for HEVC- and AVC-based 3DV”, MPEG 3DV output document, w12745, May 2012.

[2] “Working Draft 2 of AVC compatible video with depth”, MPEG 3DV output document, w12732, May 2012.

[3] “3DV-ATM CE1 - Low complexity block-based View Synthesis Prediction”, Input MPEG 3DV document, m24915, April 2012.

[4]	“3D-AVC-CE1 results on VSP skip /direct mode by Mitsubishi”, Input MPEG 3DV document m23915, February 2012.

[5] “CE5: Modified DMVP for purposes of View Synthesis Prediction”, Input MPEG 3DV document, JCT2-A0111, July 2012.

# Patent rights declaration(s)

Nokia Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).