| Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 1st Meeting: Stockholm, SE, 16–20 July 2012 | Document: JCT2-A0108 |
| --- | --- |

| Title: | 3DV-CE3: Enabling H.264/AVC deblocking for depth map coding in 3DV-ATM |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Dmytro Rusanovskyy (Nokia), Payman Aflaki (TUT), Miska Hannuksela (Nokia) Nokia Visiokatu 3, 37221 Tampere, Finland | Tel: Email: | Firstname.Lastname@nokia.com |
| Source: | Nokia Corporation |  |  |

_____________________________

# Abstract

This contribution proposes to enable normative H.264/AVC deblocking filter for coding depth map data in the common test conditions of 3DV-ATM. The simulation results presented in this document show that normative H.264/AVC deblocking brings a coding gain for all components of 3DV data. Enabling of deblocking improves coding performance by 6.7% of dBR on average for depth map data. Better quality of depth map in term improves the coding gain for texture data by 0.3% of dBR on average and results of view synthesis by 0.7% of dBR on average.

# Introduction

The recent Common Test Conditions [1] specifies that normative H.264/AVC deblocking filter shall be disabled for depth map data coding.  However, it is asserted with simulation results that this setting provides a significant penalty on the 3DV coding gain.

# Proposal

This contribution proposes to enable normative H.264/AVC deblocking filter for coding depth map data in 3DV-ATM.

# Simulation Results for Proposed Solution

The proposed solution was integrated to the 3DV-ATM CE3 anchor and compared against the CE3 anchor method. Detailed simulation results are attached to this contribution, the summary of results is given in  Table 1.

Simulations results show that proposed solution provide a noticeable and consistent gain over the entire test set. Proposal provides coding gain in 6.7% of dBR on average for depth map data, 0.3% of dBR on average for coded texture and 0.7% of dBR on average for synthesized views.

Table 1. Codding efficiency of proposed method in comparison against the CE3 anchor

|  | Texture Coding |  | Depth Coding |  | Total (Coded PSNR) |  | Total (Synthesed PSNR) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | dBR, % | dPSNR,dB | dBR, % | dPSNR,dB | dBR, % | dPSNR,dB | dBR, % | dPSNR,dB |
| S01 | -0.55 | 0.02 | -5.55 | 0.34 | -0.72 | 0.03 | -0.78 | 0.03 |
| S02 | 0.01 | 0.00 | -7.88 | 0.39 | -0.16 | 0.00 | -0.54 | 0.02 |
| S03 | -0.60 | 0.02 | -6.26 | 0.52 | -0.76 | 0.03 | -0.36 | 0.01 |
| S04 | -0.14 | 0.00 | -6.73 | 0.39 | -0.22 | 0.01 | -0.62 | 0.02 |
| S05 | -0.28 | 0.01 | -6.44 | 0.32 | -0.42 | 0.02 | -0.95 | 0.04 |
| S06 | -0.33 | 0.02 | -7.07 | 0.35 | -0.50 | 0.02 | -0.93 | 0.04 |
| S08 | -0.06 | 0.00 | -7.48 | 0.34 | -0.27 | 0.01 | -0.89 | 0.03 |
| Average | -0.28 | 0.01 | -6.77 | 0.38 | -0.44 | 0.02 | -0.72 | 0.03 |

# Patent rights declaration(s)

Nokia Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).