| Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 1st Meeting: Stockholm, SE, 16–20 July 2012 | Document: JCT2-A0109 |
| --- | --- |

| Title: | 3DV-CE3: Non-linear Depth Map Upsampling for 3DV-ATM Post-Processing |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Payman Aflaki (TUT), Dmytro Rusanovskyy (Nokia), Miska Hannuksela (Nokia) Nokia Visiokatu 3, 37221 Tampere, Finland | Tel: Email: | Firstname.Lastname@nokia.com |
| Source: | Nokia Corporation |  |  |

_____________________________

# Abstract

3DV-ATM utilizes coding of depth map data at reduced spatial resolution. After decoding of depth map it is upsampled to its original resolution prior to rendering of virtual views. In the current implementation, a basic bi-linear up-sampling is utilized at the post-processing stage of 3DV-ATM.

This contribution proposes a non-linear depth map up-sampling method for post-processing purposes. Simulation results show that proposed method improves objective quality of synthesized views by 0.96% of average dBR for 3DV-ATM bitstreams produced with disabled H.264/AVC deblocking for depth. For bitstreams produced with enabled H.264/AVC deblocking, the proposed scheme improves objective quality of synthesized views by 1.13% of dBR on average.

In addition to the objective gain, it is asserted by the proponent that proposed method provides a reduction in visual artifacts of synthesized views comparing with the anchor approach (i.e. bi-linear up-sampling).

# Introduction

Design of 3DV-ATM utilizes coding of depth map data at reduced spatial resolution [1]. After decoding of depth map it is upsampled to its original resolution prior to rendering of virtual views. In the current implementation, a basic bi-linear up-sampling [1] is utilized at the post-processing stage of 3DV-ATM.

Subjective viewing of anchor results performed during last MPEG 3DV meeting revealed strong visual artifacts present in some sequences (e.g. Dancer) that dominated the visual quality and affect the overall subjective scores.

# Proposal

This contribution proposes a non-linear depth map up-sampling method for post-processing purposes. Detailed description of this method can be found in  [2]. In this document we provide a summary of it.

Values of a, b, c, and d (the group of pixels in upsampled image) will be calculated based on values of A, B, C, D, E, F, G, H, and I (the surrounding pixel values in downsampled image), see Figure 1. The algorithm to calculate the values for a, b, c, and d is as following:

if (abs(A-I)<th) & (abs(C-G)<th)

a = E; b = E; c = E; d = E;

else

a = f(A, B, D, E);

b = f(C, B, F, E);

c = f(G, D, H, E);

d = f(I, F, H, E);

end

where: Y = f(X1, X2, X3, X4) is defined as following:

if (abs(X2-X4)> Th) & (abs(X3-X4)> Th)

Y = (X1 + X2 + X3)/3;

else

Y = X4;

End

Th  was derived empirically and set to 16 for all sequences.

Figure 1. Upsampling one pixel in downsampled image to create a group of pixels

# Simulation results

Proposed up-sampling techniques were evaluated with 3DV-ATM v4 with EHP/C3 configuration under the 3DV common test conditions [1] and according to the CE3a specification.  Complete set of simulation results has been cross-checked through a CE3 and provided as Excel attached to this contribution. Summary information from results is shown in Tables below.

Table 1. Coding performance of the NLDR up-sampling (Deblocking OFF) in comparison against the CE3 anchor (3DV-ATM v4 with Deblocking OFF)

|  | Texture Coding |  | Depth Coding |  | Total (Coded PSNR) |  | Total (Synthesed PSNR) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | dBR, % | dPSNR,dB | dBR, % | dPSNR,dB | dBR, % | dPSNR,dB | dBR, % | dPSNR,dB |
| S01 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | -0.04 | 0.00 |
| S02 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.20 | -0.01 |
| S03 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | -7.05 | 0.21 |
| S04 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.98 | -0.03 |
| S05 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | -0.17 | 0.01 |
| S06 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | -0.36 | 0.02 |
| S08 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | -0.26 | 0.01 |
| Average | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | -0.96 | 0.03 |

Table 2. Coding performance of the NLDR up-sampling with Deblocking ON in comparison against the 3DV-ATM v4 anchor (Deblocking ON)

|  | Texture Coding |  | Depth Coding |  | Total (Coded PSNR) |  | Total (Synthesed PSNR) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | dBR, % | dPSNR,dB | dBR, % | dPSNR,dB | dBR, % | dPSNR,dB | dBR, % | dPSNR,dB |
| S01 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | -0.04 | 0.00 |
| S02 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | -0.18 | 0.01 |
| S03 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | -7.31 | 0.22 |
| S04 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.47 | -0.01 |
| S05 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | -0.22 | 0.01 |
| S06 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | -0.37 | 0.02 |
| S08 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | -0.28 | 0.01 |
| Average | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | -1.13 | 0.04 |

In addition to the objective gain, it is observed that proposed method provides a significant reduction in visual artifacts of synthesized views comparing the anchor approach.

# References

[1] 	“Common test conditions for HEVC- and AVC-based 3DV”, MPEG 3DV output document, w12745, May 2012.

[2]	P. Aflaki, D. Rusanovskyy, M. M. Hannuksela, Non-linear Depth Map Resampling for 3DV-ATM Coding, MPEG m23721

# Patent rights declaration(s)

Nokia Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).