| Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 1st Meeting: Stockholm, SE, 16–20 July 2012 | Document: JCT2-A0111 |
| --- | --- |

| Title: | 3DV-CE5.a: Modified DMVP for purposes of View Synthesis Prediction (Bug-fix) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Wenyi Su University of Science and Technology of China Dmytro Rusanovskyy, Miska M. Hannuksela Nokia Research Center Visiokatu 3, 33720 Tampere, Finland | Tel: Email: | Firstname.Lastname@nokia.com |
| Source: | Nokia Corporation |  |  |

_____________________________

# Abstract

It is asserted that current design of 3DV-ATM utilizes two contradicting approaches to select VSP_SKIP and VSP_DIRECT modes. The first approach is derivation of VSP_SKIP or VSP_DIRECT through a Depth-based Motion Competition (DMC). In this scheme a synthesized depth map is utilized for the mode derivation at the decoder side and no signaling is required. The second approach is the explicit signaling of VSP_SKIP and VSP_DIRECT modes with a flag for which the encoder performs mode selection using the Rate-Distortion Optimization. The latter scheme overwrites the decision making of the DMC and uses explicit signalling even if it is not required by the DMC design.

To resolve these conflicting approaches we propose to disable the DMC for VSP. Simulation results show that proposed scheme provides less than 0.1 dBR penalty on the texture and depth coding gain on average, but it provides about 5% decoding time reduction in comparison to the 3DV-ATM anchor scheme.

# Introduction

Current design of 3DV-ATM utilizes two contradicting approaches to select VSP_SKIP and VSP_DIRECT modes.

The first approach is derivation of VSP_SKIP or VSP_DIRECT through a Depth-based Motion Competition (DMC) where synthesized depth map is utilized for mode derivation at the decoder side. No signaling shall be utilized in this case.

The second approach is explicit signaling of VSP_SKIP and VSP_DIRECT modes with a flag introduced by MERL in recent adoption [4]. These coding modes are evaluated at the encoder side through RDO and overlap/disturb the decision making process utilized in DMC. Even if DMC selects VSP_SKIP or VSP_DIRECT mode explicit signaling is still utilized for these coding modes. With respect to these recent changes to 3DV-ATM, motion vector derivation in DMC is redundant for blocks coded with VSP_SKIP and/or VSP_DIRECT modes.

# Modified DMVP for purposes of VSP

To resolve these conflicting issues we propose to disable of DMC for VSP and preserve explicit signaling of VSP_SKIP and VSP_DIRECT as it proposed in [4].  Proposed scheme was submitted for cross-checked within CE5.a.

Simulation results presented in Table 1 show that proposed brings a negligible penalty on the coding gain (0.04-0.06%) of dBR, but it provides about 5% decoding time reduction in comparison to the 3DV-ATM anchor scheme.

Table 1. Simulation results for disabled DMC for VSP, 3DV-ATM v4 utilized as anchor.

|  | Texture Coding |  | Total (Coded PSNR) |  | Complexity estimate (ratio to anchor) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | dBR, % | dPSNR,dB | dBR, % | dPSNR,dB | Encoder Time, % | Decoder Time, % | Rendering Time, % |
| S01 | 0.98 | -0.03 | 0.89 | -0.03 | 111 | 94 | 100 |
| S02 | -0.06 | 0.00 | -0.06 | 0.00 | 110 | 97 | 100 |
| S03 | -0.06 | 0.00 | -0.07 | 0.00 | 102 | 89 | 100 |
| S04 | -0.19 | 0.01 | -0.18 | 0.01 | 103 | 94 | 100 |
| S05 | -0.27 | 0.01 | -0.26 | 0.01 | 126 | 94 | 100 |
| S06 | 0.08 | 0.00 | 0.05 | 0.00 | 99 | 95 | 100 |
| S08 | -0.07 | 0.00 | -0.07 | 0.00 | 126 | 101 | 100 |
| Average | 0.06 | 0.00 | 0.04 | 0.00 | 111 | 95 | 100 |

# References

[1] 	“Common test conditions for HEVC- and AVC-based 3DV”, MPEG 3DV output document, w12745, May 2012.

[2] “Working Draft 2 of AVC compatible video with depth”, MPEG 3DV output document, w12732, May 2012.

[3] “3DV-ATM CE1 - Low complexity block-based View Synthesis Prediction”, Input MPEG 3DV document, m24915, April 2012.

[4] “3D-AVC-CE1 results on VSP skip /direct mode by Mitsubishi”, Input MPEG 3DV document m23915, February 2012.

# Patent rights declaration(s)

Nokia Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).