| Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 1st Meeting: Stockholm, SE, 16–20 July 2012 | Document: JCT2-A0113 |
| --- | --- |

| Title: | On coding configurations for 3DV-HTM development |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Srikanth Gopalakrishna (TUT) Dmytro Rusanovskyy (Nokia) Miska M. Hannuksela (Nokia) Nokia Research Center Visiokatu 3, 33720 Tampere, Finland | Tel: Email: | Firstname.Lastname@nokia.com |
| Source: | Nokia Corporation |  |  |

_____________________________

# Abstract

This contribution proposes to introduce to the 3DV-HTM software a coding configuration that imposes only high-level (slice level and above) changes to HEVC, i.e. can be considered analogous to the “3DV High Profile” settings in 3DV-ATM. The contribution also proposes to update the Common Test Conditions document to allow comprehensive evaluation of tools targeting such configuration. The contribution also provides simulation results with such a coding configuration, indicating about 8% dBR increase on average for coded and synthesized views when compared to the 3DV-HTM according to the Common Test Conditions.

# Proposal

The current design of the 3DV-HTM includes tools that introduce various changes to the HEVC coding technology. These tools include low-level changes such as intra-modes and new types of transforms as well as high-level changes. A combination of these tools brings a significant coding improvement over the HEVC simulcast coding.

In various discussions during the past MPEG meetings, a depth-enhanced multiview video coding configuration requiring only high-level syntax changes compared to HEVC version 1 has received interest. In order to establish collaborative standardization work towards such a coding configuration, it is proposed in this contribution that a starting point for such configuration is agreed. To be more exact, it is proposed to introduce to the 3DV-HTM software a coding configuration that imposes only high-level (slice level and above) changes to HEVC and to update the Common Test Conditions (CTC) document to include this coding configuration and hence allow comprehensive evaluation of tools targeting this configuration.

This contribution proposes to classify coding tools of the 3DV-HTM on the level of their differentiation from the base HEVC coding technology. Based on such classification, it is proposed to introduce to the reference 3DV-HTM software a coding configuration that would enable only high-level syntax changes to HEVC and address this configuration in Common Test Conditions.

An example of such “High Level Change Only” (HLCO) coding configuration is discussed in the following section. It is believed that the achieved simulation results support the intention of this proposal.

# Simulation results

To evaluate a possible High Level Change Only (HLCO) coding configuration of 3DV-HTM, several parameters have been modified in the CTC configuration settings. The full list of changes introduced to form HLCO is provided in Table 1 and the simulation results produced with these settings are shown in Table 2.

Table 1. 3DV-HTM coding tools disabled in HLCO simulation

| Coding tool | CTC | HLCO |
| --- | --- | --- |
| PredDepthMapGen | 1 | 0 |
| MultiviewMvPred | 7 | 0 |
| MultiviewResPred | 1 | 0 |
| DMM | 1 | 0 |
| MVI | 1 | 0 |

Table 2. Coding results produced with HLCO coding configuration in comparison against the 3DV-HTM anchor

|  | Texture Coding |  | Total (Coded PSNR) |  | Total (Synthesized PSNR) |  | Complexity estimate (ratio to anchor) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | dBR, % | dPSNR,dB | dBR, % | dPSNR,dB | dBR, % | dPSNR,dB | Encoder Time, % | Decoder Time, % | Rendering Time, % |
| S01 | 5.22 | -0.10 | 9.52 | -0.18 | 12.43 | -0.27 | 97.90 | 64.63 | 100.00 |
| S02 | 2.08 | -0.05 | 2.10 | -0.05 | 3.07 | -0.08 | 103.06 | 64.74 | 100.00 |
| S03 | 3.76 | -0.11 | 5.43 | -0.17 | 9.65 | -0.28 | 98.85 | 66.81 | 100.00 |
| S04 | 4.49 | -0.12 | 4.80 | -0.13 | 5.89 | -0.15 | 100.21 | 66.09 | 100.00 |
| S05 | 7.27 | -0.29 | 7.94 | -0.30 | 8.80 | -0.33 | 99.97 | 65.72 | 100.00 |
| S06 | 8.79 | -0.37 | 8.17 | -0.33 | 8.51 | -0.33 | 100.95 | 66.05 | 100.00 |
| S08 | 3.96 | -0.15 | 4.28 | -0.15 | 7.62 | -0.25 | 101.95 | 63.74 | 100.00 |
| Average | 5.08 | -0.17 | 6.03 | -0.19 | 8.00 | -0.24 | 100.41 | 65.40 | 100.00 |

The simulation results in Table 2 show that coding configuration of 3DV-HTM that enables only high level syntax changes to HEVC seems to provide a solid coding performance. Despite some level of penalty on coding gain, it introduces a significant decrease in decoding complexity, about 30-40% of decoding time.