| Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 1st Meeting: Stockholm, SE, 16–20 July 2012 | Document: JCT2-A0114 |
| --- | --- |

| Title: | 3D-CE6.h: Cross check on region-based intra prediction of LG |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Hunn Rhee, Philipp Merkle | Email: | hunn.rhee@hhi.fraunhofer.de, philipp.merkle@hhi.fraunhofer.de |
| Source: | Fraunhofer HHI |  |  |

_____________________________

# Abstract

Cross check results on region-based intra prediction by LG are reported. Performance evaluation results for random access configuration reported by the proponent were verified. Results for intra-only configuration not reported by proponent, but provided in cross check document.

# Introduction

In JCT2-A0068, LG proposes a simple region-intra prediction for depth coding. As cross-checker, we received their source code and simulation results for random access configuration, i.e. common test conditions (CTC). The source code was compiled and the corresponding simulations were conducted within our lab on a clustered system with Linux OS and GCC 4.5.5 compiler.

# Results

## Random Access Configuration (CTC)

Encoding and decoding YUV files (md5sum) matched. More details of the cross check results are summarized in Table 1.

Table 1. Result for 3-view scenario under CTC

|  | video only | synthesized only | coded & synthesized | enc time | dec time | ren time |
| --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.0% | 0.1% | 0.1% | 98.5% | 97.4% | 100.8% |
| Kendo | 0.0% | 0.2% | 0.2% | 95.5% | 95.9% | 98.3% |
| Newspapercc | 0.0% | 0.2% | 0.1% | 92.3% | 103.8% | 97.9% |
| GhostTownFly | 0.0% | 0.0% | 0.0% | 96.5% | 96.8% | 97.8% |
| PoznanHall2 | 0.0% | 0.3% | 0.2% | 97.8% | 95.6% | 100.0% |
| PoznanStreet | 0.0% | 0.1% | 0.0% | 94.5% | 101.1% | 102.0% |
| UndoDancer | 0.0% | -0.1% | -0.1% | 95.8% | 99.5% | 102.8% |
| 1024x768 | 0.0% | 0.2% | 0.1% | 95.4% | 99.0% | 99.0% |
| 1920x1088 | 0.0% | 0.1% | 0.0% | 96.1% | 98.2% | 100.6% |
| average | 0.0% | 0.1% | 0.1% | 95.8% | 98.6% | 99.9% |

## Intra-Only Configuration

Table 2. Result for 3-view scenario under intra-only configuration

|  | video only | synthesized only | coded & synthesized | enc time | dec time | ren time |
| --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.0% | 0.6% | 0.4% | 49.7% | 95.4% | 100.1% |
| Kendo | 0.0% | 0.6% | 0.3% | 54.9% | 95.8% | 99.7% |
| Newspapercc | 0.0% | 1.0% | 0.6% | 39.5% | 96.5% | 100.0% |
| GhostTownFly | 0.0% | -0.1% | -0.1% | 53.8% | 103.2% | 95.0% |
| PoznanHall2 | 0.0% | 0.6% | 0.4% | 57.4% | 98.1% | 101.9% |
| PoznanStreet | 0.0% | 0.3% | 0.2% | 36.1% | 99.0% | 99.3% |
| UndoDancer | 0.0% | -0.4% | -0.3% | 43.9% | 102.0% | 105.6% |
| 1024x768 | 0.0% | 0.7% | 0.5% | 47.6% | 95.9% | 99.9% |
| 1920x1088 | 0.0% | 0.1% | 0.1% | 47.0% | 100.5% | 100.4% |
| average | 0.0% | 0.4% | 0.2% | 47.3% | 98.5% | 100.2% |

# Conclusion

The cross check results on region-based intra prediction by LG were matched to those provided by the proponent. The complexity results differ slightly, because of the different simulation environment. Results for intra-only configuration were not provided by the proponent.