| Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 1st Meeting: Stockholm, SE, 16–20 July 2012 | Document: JCT2-A0115 |
| --- | --- |

| Title: | On NAL Unit Header and Video Parameter Set Design |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Byeongdoo Choi Jaehyun Kim Jeonghoon Park | Email: | b.d.choi@samsung.com jhgim@samsung.com jeonghoon@samsung.com |
| Source: | Samsung Electronics Co., Ltd. |  |  |

_____________________________

# Abstract

NAL unit header and VPS design are proposed for HEVC 3D/scalable extension with consideration of hybrid 3D/scalable CODEC design with non-HEVC video coding standards.

# Introduction

In this document, NAL unit header and VPS design are proposed for HEVC 3D/scalable extension. Without change of the current NAL unit header design, most of scalability type and layer ID can be obtained in systems by parsing only NAL unit header. It is asserted that network elements should be able to get most of useful scalability information from NAL unit headers without parameter set parsing. The proposed VPS design contains additional scalability information for hybrid 3D/scalable CODEC design with non-HEVC video standards.

# Proposal

## NAL unit header design

In m24878, the generic layer_id mapping to specific layer identifiers (dependency_id, view_id, quality_id, and depthmap_flag) was proposed. The scalable_extension_type in NAL unit header specifies how to map the layer_id to several layer identifiers in direct (predefined) or indirect (signaling in parameter set) methods. In this document, the modified version of the NAL unit design utilizing fully the reserved bits in NAL unit and the VPS design is proposed. In the current HEVC design, a VPS is not transmitted for each view/layer. Thus, the NAL unit header of the VPS does not need to contain the layer_id. The reserved_one_5bits can be used for a different purpose, instead of the layer identification. In the proposed design, the reserved_one_5bits specifies the generic layer_id in NAL units of each view/layer, while the reserved_one_5bits specifies the scalability type (scalable_extension_type:SET) in the VPS NAL unit header. After parsing only the NAL unit header of VPS NAL units in network elements, the physical meaning of the generic layer_id in the following NAL units can be interpreted, without parsing any parameter set (including VPS). It could be very helpful for easy manipulation of multi-layered streams for various application and devices. Table 1 shows exemplarily how the scalable_extension_type maps the layer_id to specific layer identifiers. For HEVC 3D extension, scalable_extension_type equal to 1 specifies that the layer_id is mapped to depth_flag (1bit) and the view_id (4bits).

Table 1. Example of mapping method using a predefined table.

| scalable_extension_type (5bits) | layer_id (5bits) |  |  |
| --- | --- | --- | --- |
| 0 | unspecified |  |  |
| 1 | depth_flag (1bit) | view_id (4bits) |  |
| 2 | dependency_id (2bits) |  | quality_id (3bits) |
| 3….31 | reserved |  |  |

## VPS design

For the future extension of multi-standard design (e.g. base : AVC, enhancement : HEVC), syntax changes in VPS are proposed. When multiple_standard_flag is equal to 1, any layer/view can be a bit stream coded by a non-HEVC encoder. The standard_type indicates the video coding standard used for encoding each layer/view bitstream. Except HEVC and AVC, other legacy video standards do not use the NAL unit structure. For easy scalability manipulation of heterogeneous bitstreams in network elements, the layer/view bitstreams codec by non-HEVC encoders (including AVC) can be encapsulated by HEVC NAL header. For the proper decoder allocation to each layer/view, signalling standard_type in the VPS would be helpful to indicate which kind of video coding standard for each layer/view is used.

# Syntax

| nal_unit( NumBytesInNALunit ) { | Descriptor |
| --- | --- |
| forbidden_zero_bit | f(1) |
| nal_ref_flag | u(1) |
| nal_unit_type | u(6) |
| temporal_id | u(3) |
| reserved_one_5bits | u(5) |
| if( nal_unit_type == 25 ) |  |
| scalable_extension_type | u(5) |
| else |  |
| layer_id_plus1 | u(5) |
| NumBytesInRBSP = 0 |  |
| for( i = 2; i < NumBytesInNALunit; i++ ) { |  |
| if( i + 2 < NumBytesInNALunit && next_bits( 24 ) = = 0x000003 ) { |  |
| rbsp_byte[ NumBytesInRBSP++ ] | b(8) |
| rbsp_byte[ NumBytesInRBSP++ ] | b(8) |
| i += 2 |  |
| emulation_prevention_three_byte /* equal to 0x03 */ | f(8) |
| } else |  |
| rbsp_byte[ NumBytesInRBSP++ ] | b(8) |
| } |  |
| } |  |

scalable_extension_type indicates the scalability types of all layers associated with the current video_parameter_set. The value of scalable_extension_type specifies the mapping rule of the layer_id in the NAL unit headers of the following NAL units into scalability identifiers.

layer_id_plus1 minus 1 indicates the layer identification.

| video_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| vps_max_temporal_layers_minus1 | u(3) |
| vps_max_layers_minus1 | u(5) |
| video_parameter_set_id | ue(v) |
| multiple_standard_flag | u(1) |
| If( multiple_codec_flag ) { |  |
| for( i = 0; i <= vps_max_layers_minus1; i++ ) { |  |
| standard _type | u(3) |
| } |  |
| } |  |
| vps_temporal_id_nesting_flag | u(1) |
| for( i = 0; i <= vps_max_temporal_layers_minus1; i++ ) { |  |
| vps_max_dec_pic_buffering[ i ] | ue(v) |
| vps_num_reorder_pics[ i ] | ue(v) |
| vps_max_latency_increase[ i ] | ue(v) |
| } |  |
| vps_extension_flag | u(1) |
| if( vps_extension_flag ) |  |
| while( more_rbsp_data( ) ) |  |
| vps_extension_data_flag | u(1) |
| } |  |
| rbsp_trailing_bits( ) |  |
| } |  |

multiple_standard_flag equal to 1 indicates that one or more layers may not conform to this Recommendation | International Standard. multiple_standard_flag equal to 0 indicates that all layer bitstreams comform to this Recommendation | International Standard.

standard_type equal to 0 indicates that the current layer is conforming to this Recommendation | International Standard. Other values for standard_type are reserved for non-HEVC video coding standards.

# Patent rights declaration(s)

Samsung Electronics Co., Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).