| Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 1st Meeting: Stockholm, SE, 16–20 July 2012 | Document: JCT2-A0117 |
| --- | --- |

| Title: | On Random Access Pictures |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Byeongdoo Choi Jaehyun Kim Jeonghoon Park | Email: | b.d.choi@samsung.com jhgim@samsung.com jeonghoon@samsung.com |
| Source: | Samsung Electronics Co., Ltd. |  |  |

_____________________________

# Abstract

Restrictions on RAP-related pictures (IDR, CRA, BLA, and TFD) are proposed in HEVC 3D, in order to avoid the decoding error, over-/under-flow in HRD, and the POC mismatch.

# Introduction

Without restrictions on the use of RAP-related pictures, the decoding error, over-/under-flow in HRD, or the POC mismatch may occur. To avoid unintended operation in decoder, several restrictions on CRA, BLA, and TFD are proposed. The restriction on the use of IDR was already adopted, in the last Geneva meeting.

# Proposal

## Restrictions on RAP pictures

The NAL unit/picture type of the enhancement (non-base) view picture shall be BLA, when the NAL unit/picture type of the associated base view picture is BLA. The NAL unit/picture type of the enhancement view picture shall be CRA, when the NAL unit/picture type of the associated base view picture is CRA. The NAL unit/picture type of the enhancement (non-base) view picture shall not be CRA, when the NAL unit/picture type of the associated base view picture is BLA. It is because the leading (TFD) pictures associated with the CRA picture in the enhancement view cannot refer to the TFD pictures associated with the BLA picture in the base view after the system discards the TFD pictures associated with the BLA pictures. Moreover, the BLA pictures reset the MSB of POC to zero while the CRA pictures do not reset the MSB of POC. It may result in the POC mismatch between views and the incorrect inter-view reference, when CRA picture and BLA pictures are present in the same access unit. The IDR and BLA pictures cannot be present with different type pictures in the same access unit, due to the POC reset. Additionally, the non-base view pictures shall not be non-RAP pictures, when the corresponding base view picture is the RAP picture, in order to enable the synchronized random accesses of all views.

## Restrictions on TFD pictures

The NAL unit/picture type of the enhancement view picture shall be TFD, when the NAL unit/picture type of the associated base view picture is the TFD picture type. Even though the pictures of enhancement views are not the non-decodable leading pictures associated with a CRA or BLA picture, those pictures are marked as the TFD pictures when the picture type of the base view is TFD. Simultaneously, the pictures referencing the TFD pictures, but not non-decodable leading pictures, shall be marked as the TFD pictures. It is because, if the TFD pictures in the base view are discarded, all pictures dependent on the discarded TFD pictures cannot be correctly decoded. Thus, when the TFD pictures are discarded in the base view, all related pictures in the enhancement views should be discarded. However, the NAL unit/picture type of the enhancement view picture can be TFD, when the NAL unit/picture type of the associated base view picture is the normal (non-RAP, non-TFD) picture type.

## Restrictions on temporal layer

The NAL unit/picture type of the enhancement view picture shall be the TLA, when the NAL unit/picture type of the associated base view picture is the TLA. Whereas, the NAL unit/picture type of the enhancement view picture can be the TLA, when the NAL unit/picture type of the associated base view picture is the normal picture. This restriction is to guarantee that the temporal_id of the enhancement view shall be higher than the temporal_id of the base view in the same access unit, to avoid the error of inter-view prediction.

# Patent rights declaration(s)

Samsung Electronics Co., Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).