| Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 1st Meeting: Stockholm, SE, 16–20 July 2012 | Document: JCT2-A0118 |
| --- | --- |

| Title: | On View Layer Switching |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Byeongdoo Choi Jaehyun Kim Jeonghoon Park | Email: | b.d.choi@samsung.com jhgim@samsung.com jeonghoon@samsung.com |
| Source: | Samsung Electronics Co., Ltd. |  |  |

_____________________________

# Abstract

Similarly to the temporal layer access (TLA) unit and picture for temporal layer switching in HEVC, the view layer access (VLA) unit and pictures are proposed for view layer switching in HEVC 3D extension.

# Introduction

One key feature of multiview video is the interaction which can choose viewpoints of interests freely. In free viewpoint displays, some parts of views can be selectively decoded and displayed interacting to user activity or control. When different viewpoints are displayed by head tracking, the decoded and rendered viewpoint will frequently change. In another application, sometimes users want to change the number of views from 2D to 3D while watching video. Then, the smooth change of view layer switching without decoding error could be a required functionality.

Figure 1: Viewpoint switching.

In HEVC, the TLA picture was adopted to support the temporal layer switching from lower layer to upper layer at non-random access points. Inside a GOP, the temporal layer can be changed at temporal layer switching points. In case of view layer switching, more frequent changes are sometimes necessary to support smooth and fast viewpoint switches.

# Proposal

In this proposal, a new NAL unit type and a picture type are proposed to support the smooth view layer switching. In general, the IDR unit/picture can support all kinds of layer switching, but its frequent use degrades the coding efficiency. The CRA also can support the view layer switching. As reported in m23649, the CRA may result in decoding error of frame loss when a view layer switching is performed because the leading pictures associated with the CRA picture in the switched view cannot be corrected decoded. If a CRA picture in the non-base view is changed to BLA picture when the view layer switching is performed, the POC misalignment between views occurs and may result in the error of the interview prediction.

A new NAL unit type and a new picture type for viewpoint or view layer switching are referred to as view layer access (VLA) unit and picture. The VLA pictures refer to only inter-view reference pictures for inter-view prediction, without temporal prediction. All coded pictures that follow the VLA picture both in decoding order and output order shall not use inter prediction from any picture that precedes the VLA picture either in decoding order or output order, and any picture that precedes the VLA picture in decoding order also precedes the VLA picture in output order. The non-decodable leading pictures, which are associated with the previous VLA picture in decoding order and preceding the associated picture in output order, are marked as the TFD pictures. When the view layer switching is performed, those TFD pictures associated with the VLA at the view layer switching point may not be correctly decoded and output. The TFD pictures in the switched views will be discarded when the view switching is performed in systems. Figure 2 shows an example of view layer switching with the proposed VLA picture when the view layer switching is performed at POC 8.

When the picture type of the non-base view is the VLA, the picture type of the associated base view shall be non-IDR and non-BLA for the POC alignment between views. The VLA unit is an access unit in which the coded picture is a VLA picture.

Figure 2: Example of View Layer Switching with the VLA picture.

# Syntax

Table 7-1 – NAL unit type codes and NAL unit type classes

| nal_unit_type | Content of NAL unit and RBSP syntax structure | NAL unit type class |
| --- | --- | --- |
| 0 | Unspecified | non-VCL |
| 1 | Coded slice of a non-RAP, non-TFD and non-TLA picture slice_layer_rbsp( ) | VCL |
| 2 | Coded slice of a TFD picture slice_layer_rbsp( ) | VCL |
| 3 | Coded slice of a non-TFD TLA picture slice_layer_rbsp( ) | VCL |
| 4, 5 | Coded slice of a CRA picture slice_layer_rbsp( ) | VCL |
| 6, 7 | Coded slice of a BLA picture slice_layer_rbsp( ) | VCL |
| 8 | Coded slice of an IDR picture slice_layer_rbsp( ) | VCL |
| 9..14 | Reserved | n/a |
| 15 | Coded slice of a VLA picture slice_layer_rbsp( ) | VCL |
| 16..24 | Reserved | n/a |
| 25 | Video parameter set video_parameter_set_rbsp( ) | non-VCL |
| 26 | Sequence parameter set seq_parameter_set_rbsp( ) | non-VCL |
| 27 | Picture parameter set pic_parameter_set_rbsp( ) | non-VCL |
| 28 | Adaptation parameter set aps_rbsp( ) | non-VCL |
| 29 | Access unit delimiter access_unit_delimiter_rbsp( ) | non-VCL |
| 30 | Filler data filler_data_rbsp( ) | non-VCL |
| 31 | Supplemental enhancement information (SEI) sei_rbsp( ) | non-VCL |
| 32..47 | Reserved | n/a |
| 48..63 | Unspecified | non-VCL |

view layer access (VLA) picture: A coded picture for which each slice has nal_unit_type equal to 15. VLA pictures refer to only inter-view pictures, without temporal prediction. All coded pictures that follow the VLA picture both in decoding order and output order shall not use inter prediction from any picture that precedes the VLA picture either in decoding order or output order; and any picture that precedes the VLA picture in decoding order also precedes the VLA picture in output order.

View layer access (VLA) unit: An access unit in which the coded picture is a VLA picture.

# Patent rights declaration(s)

Samsung Electronics Co., Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).