| Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 1st Meeting: Stockholm, SE, 16–20 July 2012 | Document: JCT2-A0116 |
| --- | --- |

| Title: | CE8.h Depth distortion metric with a weighted depth fidelity term |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jiwook Jung, Sehoon Yea Seungchul Ryu, Donghyun Kim, Kwanghoon Sohn 221, Yangjae-Dong, Seocho-Gu, Seoul 137-130, Korea C125, The 3rd Engineering Building, Yonsei University 50 Yonsei-ro, Seodaemun-Gu, Seoul 120-749, Korea | Tel: Email: | +82-2-6912-6477 jiwook.jung@lge.com |
| Source: | LG Electronics, Yonsei University |  |  |

_____________________________

# Abstract

Under the Common Test Conditions (CTC) of 3D Video Coding, the quality of synthesized views at specified viewing positions has been used as a sole measure of coded depth fidelity, with which RD-mode decision was performed at the encoder when coding depth. In the current reference encoder implementation, a technique called VSO[3] is being used to synthesize the intermediate views and calculate the distortion of coded depth indirectly in terms of the quality of those synthesized views.

In M25010, it was pointed out that use of such an indirect distortion metric for depth coding may introduce undesirable quality fluctuation of synthesized views along intermediate viewing positions as well as noticeable artifacts in synthesized views when an alternative (but quite standard) view-synthesis algorithm such as VSRS is used at the receiver in place of that included in VSO. The primary reason for such potential problems is that the fidelity of coded-depth itself is not high enough especially around object boundaries and spurious depth values are often created in the coded depth due to the inherent nature of the current depth coding metric.

This contribution proposes to add a depth-fidelity term in the distortion metric currently used for RD-optimization at the encoder for depth coding. Comparisons between the cases with and without the proposed depth-fidelity term are provided to highlight how the proposed change can mitigate the abovementioned potential problems. The average change of luma BD-Rate (BDR) in synthesized result is 0.4% on CTC. And, on 3DVC Cfp conditions using VSRS3.5, the average change of luma BDR is      -2.4%. Especially, the luma BDR of Undo_Dancer and GT_Fly is -9.5% and -5.1%, respectively. The encoding time is increased by 1.8%.

# Introduction

Under the Common Test Conditions (CTC) of 3D Video Coding, depth distortion is measured indirectly in synthesis view domain (i.e., by measuring the view synthesis quality using VSO). Consequently, it is necessary to assume a specific view synthesis algorithm and view synthesis positions. For this reason, the coded-depth property can be biased to specific view synthesis conditions. Moreover, the VSO algorithm capitalizes on the fact that compromised quality of coded-depth in one view can be often compensated by higher quality of coded-depth in the other view under the current CTC. However, such compensation might not be feasible or effective enough depending on view-synthesis position settings (e.g., extrapolation) or when alternative view-rendering algorithm is adopted at the display.

It has been reported that VSO depth distortion metric can cause some problems [1] [2]. Figure 1 shows an example of synthesis results under the CTC of HTM-3.0 at the encoder and VSRS3.5 (i.e., an alternative, but standard renderer) as renderer. In Figure 1-(c), one cannot observe apparent distortion in the synthesized view, while the opposite is true in Figure 1-(d).

(a) Coded depth (Left View, 5th view)           (b) Coded depth (Right View, 9th view)

(c) Synthesis by VSRS1DFast (included in VSO)         (d) Synthesis by VSRS3.5 1D mode

Figure1. Synthesizing View 7 using different renders at the receiver

# Proposed Algorithm

In this proposal, a depth fidelity term which is SAD/SSE in depth domain is combined with the synthesis domain distortion term. As the depth fidelity term is explicitly included into the depth distortion metric, problems caused by using only synthesis domain distortion metric can be mitigated. Specifically, the depth distortion metric function is modified as follows:

Cdepthdistrotion=wsynthCsynth+wdepthCdepth

where Cdepthdistrotion : The total depth distortion for mode decision

Csynth, Cdepth: Distortions in synthesis and depth domains, respectively

wsynth, wdepth : Weights for distortions in synthesis and depth domains, respectively

# Results

Table 3.1 and 3.2 shows the results of BD-rate performance of Anchor and proposed method. Anchor is the A configuration of joint proposal of Samsung and HHI[6]. This proposal also has some possibility to have undesirable artifacts in various synthesis conditions. Table 3.1 shows the result of CTC[4]. Table 3.2 shows the result of the Cfp of 3D Video Coding[5]. It uses VSRS3.5 as the renderer of synthesized views. wsynth: wdepth = 10 : 1 was set in SAD where HHI_VSO metric was used. wsynth: wdepth = 1 : 1 was set in SAD where SAIT_RDO metric was used. In SSE, the weights were squared. The weight factors were determined heuristically.

Table 3.1 BDR performance of proposed method using VSRS1DFast as renderer (not cross-checked)

|  | video 0 | video 1 | video 2 | video only | synthesized only | coded & synthesized | enc time | dec time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.0% | 0.0% | 0.0% | 0.0% | 0.7% | 1.2% | 101.9% | 100.2% |
| Kendo | 0.0% | 0.0% | 0.0% | 0.0% | 1.7% | 2.6% | 103.1% | 101.4% |
| Newspapercc | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | 0.6% | 100.1% | 100.3% |
| GhostTownFly | 0.0% | 0.0% | 0.0% | 0.0% | -0.5% | -0.3% | 100.1% | 99.8% |
| PoznanHall2 | 0.0% | 0.0% | 0.0% | 0.0% | 0.8% | 1.0% | 102.8% | 99.5% |
| PoznanStreet | 0.0% | 0.0% | 0.0% | 0.0% | 0.4% | 0.5% | 102.8% | 99.4% |
| UndoDancer | 0.0% | 0.0% | 0.0% | 0.0% | -0.7% | -0.6% | 101.6% | 99.1% |
| 1024x768 | 0.0% | 0.0% | 0.0% | 0.0% | 0.8% | 1.4% | 101.7% | 100.6% |
| 1920x1088 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | 101.8% | 99.4% |
| average | 0.0% | 0.0% | 0.0% | 0.0% | 0.4% | 0.7% | 101.8% | 100.0% |

Table 3.2 BDR performance of proposed method using VSRS3.5 as renderer (not cross-checked)

|  | video 0 | video 1 | video 2 | video only | synthesized only | coded & synthesized | enc time | dec time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.0% | 0.0% | 0.0% | 0.0% | 0.3% | 0.9% | 101.9% | 100.2% |
| Kendo | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | 1.5% | 103.1% | 101.4% |
| Newspapercc | 0.0% | 0.0% | 0.0% | 0.0% | -3.5% | -1.7% | 100.1% | 100.3% |
| GhostTownFly | 0.0% | 0.0% | 0.0% | 0.0% | -5.1% | -3.3% | 100.1% | 99.8% |
| PoznanHall2 | 0.0% | 0.0% | 0.0% | 0.0% | 0.6% | 0.9% | 102.8% | 99.5% |
| PoznanStreet | 0.0% | 0.0% | 0.0% | 0.0% | 0.5% | 0.5% | 102.8% | 99.4% |
| UndoDancer | 0.0% | 0.0% | 0.0% | 0.0% | -9.5% | -6.6% | 101.6% | 99.1% |
| 1024x768 | 0.0% | 0.0% | 0.0% | 0.0% | -1.0% | 0.3% | 101.7% | 100.6% |
| 1920x1088 | 0.0% | 0.0% | 0.0% | 0.0% | -3.4% | -2.1% | 101.8% | 99.4% |
| average | 0.0% | 0.0% | 0.0% | 0.0% | -2.4% | -1.1% | 101.8% | 100.0% |

Table3.3 represents the example of different synthesis conditions which uses VSRS3.5 as renderer.

Table3.3 Subjective sample of proposed method on CTC

|  | M25896 A Cfg | Proposed method |
| --- | --- | --- |
| Left Coded Depth |  |  |
| VSRS 1D Fast |  |  |
| VSRS3.5 |  |  |

The synthesis result of M25896 using VSRS3.5 has some severe visual artifacts while the synthesis result using VSRS1DFast does not. It is because the distortion of depth can be suppressed by VSRS1DFast algorithm. However, VSRS3.5 couldn’t suppress the distortion of depth. In proposed method, coded depth and synthesis results don’t have severe visual artifacts like those result of M25896.

# Conclusion

The conventional depth distortion metric (e.g. VSO) considers only distortion of synthesis domain. It makes depth map optimized in specific synthesis algorithm and virtual view position. Therefore, coded depth might have some problems in different synthesis conditions. They can be well mitigated by explicitly adding a depth fidelity term in the depth distortion metric function. The weighting factor had been not well adjusted on the results. The subjective and objective results can be improved by tuning the weighting factor.

In this proposal, the utility of adding a depth fidelity term is reported subjectively and objectively.

# Patent rights declaration(s)

LG does not have any current or pending patent rights relating to the technology described in this contribution.

# References

- Eunyong Son, Sehoon Yea, "3D-CE8.h results: On the adequacy of the RD-measure in VSO", ISO/IEC JTC1/SC29/WG11, M25010, Geneva, May 2012.

- Li Wang, Deliang Fu, Yin Zhao, Lu Yu, “3D-CE8.h results on JRDO and VSO with different view synthesis algorithms” , ISO/IEC JTC1/SC29/WG11, M24899, Geneva, May 2012.

- Heiko Schwarz, "Description of 3D Video Coding Technology Proposal by Fraunhofer HHI (HEVC compatible, configuration A)" , ISO/IEC JTC1/SC29/WG11, M22570, Geneva, November 2011.

- Heiko Schwarz, Dmytro Rusanovskyy, "Common Test Conditions for 3DV experimentation”, ISO/IEC JTC1/SC29/WG11, N12745, Geneva, May 2012.

- “Call for Proposals on 3D Video Coding Technology” , ISO/IEC JTC1/SC29/WG11, N12036, Geneva, March 2011.

- Byung Tae Oh, Jaejoon Lee, Du Sik Park, Gerhard Tech, Karsten Muller, Thomas Wiegand, " 3D-CE8.h results on view synthesis optimization", M25896, ISO/IEC JTC1/SC29/WG11