| Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 1st Meeting: Stockholm, SE, 16–20 July 2012 | Document: JCT2-A0122_r1 |
| --- | --- |

| Title: | Maximum depth discontinuity signaling for view synthesis range determination |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Andrey Norkin Martin Pettersson Thomas Rusert Färögatan 6 164 80 Stockholm Sweden | Tel: Email: | +46 10 714 84 02 andrey.norkin@ericsson.com |
| Source: | Ericsson |  |  |

_____________________________

# Abstract

This document presents a proposal to signal maximum discontinuity value for the depth map. In the proposed mechanism, the encoder signals a maximum depth discontinuity value that can be used to determine a maximum disocclusion width when performing view extrapolation. Using the maximum depth discontinuity value, the view renderer can determine a proper view synthesis range depending on its "hole filling" capabilities. The proposal relates in particular to the MVC track since the Working Draft for this track also exists. It is however applicable to all depth-based 3DV tracks. The proposal contains explanations how the maximum depth discontinuity may be used determining the disocclusion width and the proposed syntax to be included in the depth acquisition SEI message.

# Introduction

In order to enable view rendering capabilities in the receiver, a number of views containing texture (and possibly depth) information are transmitted along with their camera parameters. With help of depth information, the receiver can synthesize intermediate views or perform view extrapolation. However, the view renderer is not aware of the quality of such a view reconstruction (since it cannot necessarily synthesize correctly the data that have been disoccluded). This could lead to potential problems when view synthesis equipment renders views at particular camera positions that result in bad visual quality.

Synthesized views

transmitted views

?

?

?

?

OK

Figure 1		The view renderer can synthesize intermediate views with good quality. But it does not know how far the views can be extrapolated without causing bad visual quality.

It is therefore beneficial for the view renderer to determine a proper view synthesis range during view extrapolation (or interpolation) so that the view renderer chooses a proper view synthesis position that would not lead to a poor quality of the synthesized views. Since different view renderers can have very different hole filling capabilities, it is clear that an acceptable view synthesis range differs for different view renderers. It is therefore beneficial to provide a view renderer with information on the possible complexity of view synthesis at a chosen position which would allow the view synthesizer to determine a proper view synthesis range.

OK

proper view synthesis range

OK

Figure 2		Given information on the possible complexity of view synthesis at chosen positions, the view renderer can determine a proper view synthesis range. The view renderer will then know which views can be rendered with good quality and can adjust the rendered depth of the scene accordingly.

It may be argued that the view renderer can determine the view synthesis range based on the first frame(s)/depth maps of a scene. However, it might be difficult for a view renderer to determine incoming changes in the scene. For example, in case some object appears in front of the camera or the camera in the scene is moving, the number or width of disocclusions may increase so that the view renderer would not be capable of concealing the disocclutions properly. Therefore, a view renderer may benefit from knowing the complexity of the view synthesis in the future frames when determining a proper view synthesis range.

# Estimation of disocclusion width

The values of the depth maps can be translated to the depth values by using the following formula

Z=1.0v255.0⋅1.0Znear−1.0Zfar+1.0Zfar

where v is the value of the depth map sample, Znear and Zfar are the near and far depth clipping values. In case of 1D linear camera arrangement, the view synthesis equations can be reduced to

d=f⋅lz+du

where d is the resulted displacement (disparity) value, f is the focal length of the camera, l is the baseline distance and du is the sensor (image) shift. All pixels from the coded view can be mapped to the corresponding pixels in the synthesized view by applying the displacement/disparity d to the horizontal coordinate x of the sample position in the image.

From this formula, we can estimate a disocclusion width. Let’s consider two samples that are located next to each other. Consider, for example, that p1 and p2 are two neighboring pixels, where pixel p1 is located on the left and the pixel p2 is next to the right, their coordinates being (x1,y) and (x2= x1+1,y) respectively and that the synthesized camera position is on the left from the coded camera position. These two pixels will be mapped to the virtual camera position by applying shifts d1 = f*l/z1 + du and d2 = f*l/z2 + du, where z1 and z2 are the depth values associated with the pixel positions p1 and p2, respectively.  Therefore, the difference in the displacement of pixels p1 and p2 after synthesis will be

d2-d1 = f*l*(1/z2-1/z1) = f l2551.0Znear−1.0Zfarv2−v1=K*(v2-v1)  		 (1)

A positive difference in displacement values d2-d1 means that there is a disocclusion with the width of d2-d1, whereas a negative value of d2-d1 indicates an occlusion.

Similar reasoning is applicable to the view synthesis algorithm used in Test Model under Consideration TMuC, where the following formula for the pixels displacement d in view synthesis is used instead

d = ( s * v + o ) >> n,

where v is the depth sample value, s is the transmitted scale factor, o is the transmitted offset, and n is a shift parameter that depends on the required accuracy of the disparity vectors. In this case, the difference in the displacements will be equal to

d2-d1 = s(v2-v1) >> n						(2)

Here, the width of a disocclusion (hole) would also be equal to (d2-d1).

A symmetric reasoning can be applied to the view synthesis extrapolation in the direction to the right from the coded camera view. In this case, the negative values of (d1-d2) would correspond to disocclusions (holes).

It is therefore proposed to send the maximum depth discontinuity to the right and the maximum depth discontinuity to the left. This enables the view renderer to estimate the maximum disocclusion width based on depending on the baseline distance between the coded and the synthesized view.

It is suggested to send the proposed information in the depth acquisition SEI message described in [1]. The maximum depth discontinuity values related to depth maps in the future frames are signaled.

# Proposed SEI syntax

The following changes, marked in yellow are proposed to the depth acquisition SEI message.

Depth acquisition information SEI message syntax

| depth_acquisition_info( payloadSize ) { | C | Descriptor |
| --- | --- | --- |
| all_views_equal_flag | 5 | u(1) |
| if( all_views_equal_flag = = 0 ) |  |  |
| num_views_minus1 | 5 | ue(v) |
| numViews = num_views_minus1 + 1 |  |  |
| Else |  |  |
| num_frames | 5 | ue(v) |
| numViews = 1 |  |  |
| z_near_flag | 5 | u(1) |
| z_far_flag | 5 | u(1) |
| d_min_flag | 5 | u(1) |
| d_max_flag | 5 | u(1) |
| focal_length_x_flag | 5 | u(1) |
| focal_length_y_flag | 5 | u(1) |
| principal_point_x_flag | 5 | u(1) |
| principal_point_y_flag | 5 | u(1) |
| translation_flag | 5 | u(1) |
| max_depth_discontinuity_flag | 5 | u(1) |
| for( i = 0; i < numViews; i++ ) { |  |  |
| if( z_near_flag ) |  |  |
| depth_acquisition_element( i, num_frames, 0, 7, ZNearSign, ZNearExp, ZNearMantissa, ZNearManLen ) |  |  |
| if( z_far_flag ) |  |  |
| depth_acquisition_sei_element( i, num_frames, 0, 7, ZFarSign, ZFarExp, ZFarMantissa, ZFarManLen ) |  |  |
| if( d_min_flag ) |  |  |
| depth_acquisition_sei_element( i, num_frames, 0, 7, DMinSign, DMinExp, DMinMantissa, DMinManLen ) |  |  |
| if( d_max_flag ) |  |  |
| depth_acquisition_sei_element( i, num_frames, 0, 7, DMaxSign, DMaxExp, DMaxMantissa, DMaxManLen ) |  |  |
| if( focal_length_x_flag ) |  |  |
| depth_acquisition_sei_element( i, num_frames, 1, 6, FocalLengthXSign, FocalLengthXExp, FocalLengthXMantissa, FocalLengthXManLen ) |  |  |
| if( focal_length_y_flag ) |  |  |
| depth_acquisition_element( i, num_frames, 1, 6, FocalLengthYSign, FocalLengthYExp, FocalLengthYMantissa, FocalLengthYManLen ) |  |  |
| if( principal_point_x_flag ) |  |  |
| depth_acquisition_element( i, num_frames, 1, 6, PrincipalPointXSign, PrincipalPointXExp, PrincipalPointXMantissa, PrincipalPointXManLen ) |  |  |
| if( principal_point_y_flag ) |  |  |
| depth_acquisition_element( i, num_frames, 1, 6, PrincipalPointYSign, PrincipalPointYExp, PrincipalPointYMantissa, PrincipalPointYManLen ) |  |  |
| if( translation_flag ) |  |  |
| depth_acquisition_element( i, num_frames, 1, 6, TXSign, TXExp, TXMantissa, TXManLen ) |  |  |
| if ( max_depth_discontinuity_flag ) { |  |  |
| max_depth_discontinuity_left[ i ] | 5 | u(8) |
| max_depth_discontinuity_right[ i ] | 5 | u(8) |
| } |  |  |
| } |  |  |
| } |  |  |

Depth acquisition information SEI message semantics

The syntax elements in the depth acquisition information SEI message specifies various parameters for depth views for the purpose of processing decoded texture and depth view components prior to rendering on a 3D display, such as view synthesis. Specifically, camera parameters and depth ranges for depth views are specified. Camera parameters maybe used as camera parameters for the texture views with same VOIdx. When present, the depth acquisition information SEI message may be associated with any access unit. It is recommended, when present, the SEI message is associated with an IDR access unit for the purpose of random access. The information signaled in the SEI message applies to all the access units from the access unit the SEI message is associated with to the next access unit, in decoding order, containing an SEI message o the same type, exclusively, or to the end of the coded video sequence, which ever is earlier in decoding order.

all_views_equal_flag equal to 0 specifies that depth acquisition information may not be identical to respective values for each view in target views. all_views_equal_flag equal to 1 specifies that the depth acquisition information are identical to respective values for all target views.

num_views_minus1 shall be equal to the value of the syntax element num_views_minus1 in the 3DVC sequence parameter set for the coded video sequence. The value of num_views_minus1 shall be in the range of 0 to 1023, inclusive.

num_frames specifies the number of frames from the frame the depth acquisition information SEI message is associated with to the next frame which the depth acquisition information SEI message is associated with, exclusive, or to the end of the coded video sequence.

z_near_flag equal to 0 specifies that the syntax elements specifying the closest depth value are not present in the syntax structure. z_near_flag equal to 1 specifies that the syntax elements specifying the closest depth value are present in the syntax structure.

z_far_flag equal to 0 specifies that the syntax elements specifying the farthest depth value are not present in the syntax structure. z_near_flag equal to 1 specifies that the syntax elements specifying the farthest depth value are present in the syntax structure.

d_min_flag equal to 0 specifies that the syntax elements specifying the minimum disparity value are not present in the syntax structure. d_min_flag equal to 1 specifies that the syntax elements specifying the minimum disparity value are present in the syntax structure.

d_max_flag equal to 0 specifies that the syntax elements specifying the maximum disparity value are not present in the syntax structure. z_near_flag equal to 1 specifies that the syntax elements specifying the maximum disparity value are present in the syntax structure.

focal_length_x_flag equal to 0 specifies that the syntax elements specifying the horizontal focal length are not present in the syntax structure. focal_length_x_flag equal to 1 specifies that the syntax elements specifying the horizontal focal length are present in the syntax structure.

focal_length_y_flag equal to 0 specifies that the syntax elements specifying the vertical focal length are not present in the syntax structure. focal_length_y_flag equal to 1 specifies that the syntax elements specifying the vertical focal length are present in the syntax structure.

principal_point_x_flag equal to 0 specifies that the syntax elements specifying the horizontal principal point are not present in the syntax structure. principal_point_x_flag equal to 1 specifies that the syntax elements specifying the horizontal principal point are present in the syntax structure.

principal_point_y_flag equal to 0 specifies that the syntax elements specifying the vertical principal point are not present in the syntax structure. principal_point_y_flag equal to 1 specifies that the syntax elements specifying the vertical principal point are present in the syntax structure.

translation_flag equal to 0 specifies that the syntax elements specifying the horizontal translation between views are not present in the syntax structure. translation_flag equal to 1 specifies that the syntax elements specifying the horizontal translation between views are present in the syntax structure.

max_depth_discontinuity_flag equal to 0 specifies that the syntax elements specifying the maximum difference between horizontally adjacent depth samples in horizontal direction to the left and the maximum difference between horizontally depth adjacent samples in horizontal direction to the right are present in the bitstream. max_depth_discontinuity_flag equal to 0 specifies that the syntax elements specifying the maximum difference between horizontally adjacent depth samples in horizontal direction to the left and the maximum difference between horizontally depth adjacent samples in horizontal direction to the right are not present in the bistream.

max_depth_discontinuity_left[i] specifies the maximum difference between horizontally adjacent depth samples in direction from right to left in the depth frames corresponding to view i. The max_depth_discontinuity_left[i] value is valid until is it replaced by the value max_depth_discontinuity_left[i] in an access unit succeeding the access unit where the current value was transmitted. The value of max_depth_discontinuity_left[i] equal to 0 indicates that no maximum depth discontinuity to the left direction information is assigned for view i depth component for the current and the following access units.

max_depth_discontinuity_right[i] specifies the maximum difference between horizontally adjacent depth samples in direction from left to right in the depth frames corresponding to view i. The max_depth_discontinuity_right[i] value is valid until is it replaced by another value max_depth_discontinuity_right[i] in an access unit succeeding the access unit where the current value was transmitted. The value of max_depth_discontinuity_right[i] equal to 0 indicates that no maximum depth discontinuity to the right direction information is assigned for view i depth component for the current and the following access units.

# Illustration of depth discontinuity example

An example of using depth discontinuity information is shown below.  Figure 3 shows two sequences, Poznan Street and Undo Dancer, with corresponding depth maps. These sequences have frequently been used for experiments during the 3DV standardization in MPEG.

Figure 3		Poznan Street and Undo Dancer with corresponding depth maps.

In the current version of the 3D view renderer it is not possible to extrapolate views outside the transmitted views. In order to simulate extrapolation of virtual views in the examples below, views between the middle and the leftmost views have been rendered using only the texture from the middle view as rendering reference. The following settings have been used to produce the simulated extrapolated views in the example below: RenderDirection = 1 (extrapolate from left), HoleFillingMode = 0 (No hole filling) and BlendHoleMargin = 0 (Not extending hole sizes).

In  four different extrapolation distances have been tested, 0.25, 0.5, 0.75 and 1 camera distances to the left. For each extrapolation distance the maximum hole size from left for the whole sequence and the maximum hole size from left as averaged over all frames have been determined.

When extrapolating a virtual view, there will be a vertical stripe of holes on one of the borders of the view (see Figure 4 below).The best practise for a view renderer would be to crop (and maybe resize) the view to get rid of this large hole at the border. Therefore, when determining the maximum hole sizes for the example in  the vertical stripe of holes at the border of the view was not considered.

Table 1	Maximum hole size as the maximum for the whole sequence and per frame (averaged over all frames).

| Content | View extrapolation (in terms of camera distance) | Max hole size (for whole sequence) | Max hole size (averaged over frames) |
| --- | --- | --- | --- |
| Poznan Street | 0.25 | 13 | 6 |
| Poznan Street | 0.5 | 21 | 11 |
| Poznan Street | 0.75 | 30 | 15 |
| Poznan Street | 1 | 38 | 19 |
| Undo Dancer | 0.25 | 13 | 8 |
| Undo Dancer | 0.5 | 23 | 16 |
| Undo Dancer | 0.75 | 34 | 23 |
| Undo Dancer | 1 | 44 | 30 |

Note that a view extrapolation of 0.75 camera distances for Poznan Street has a similar maximum hole size as a view extrapolation of 0.5 camera distances for Undo Dancer. This indicates that assuming a certain maximum hole size can be accepted given the hole filling capabilities of the view renderer, virtual views for Poznan Street may be extrapolated further than for Undo Dancer.

Figure 4 shows the Poznan Street and the Undo Dancer sequences where the virtual view is extrapolated one camera distance to the left. To illustrate the size of the holes that need to be filled, no hole filling has been applied in the example.

Figure 4  Poznan Street and Undo Dancer where the views have been extrapolated one camera distance to the left. No hole filling has been performed to show the sizes of the holes that need to be filled.

shows the maximum depth difference for the whole sequence and as averaged over all frames.

In order to avoid the misleading impact of potential blurry object borders, the depth maps have been subsampled by a factor 16 before the maximum depth differences have been calculated.

Table 2	Maximum depth difference for the whole sequences and as averaged over all frames.

| Content | Max depth difference (for whole sequence) | Max depth difference (averaged over frames) |
| --- | --- | --- |
| Poznan Street | 128 | 69 |
| Undo Dancer | 150 | 102 |

Using formula (1) and the following camera parameters

Table 3	Camera parameters for Poznan Street and Undo Dancer

| Content | f | Baseline | Znear | Zfar |
| --- | --- | --- | --- | --- |
| Poznan Street | 1732.875727 | -1.593023 | -34.506386 | -2760.510889 |
| Undo Dancer | 2302.852541609168 | 4*20 | 2289 | 213500 |

we get that

(d2-d1) = f l2551.0Znear−1.0Zfarv2−v1 =>

(d2-d1) = 0.0774*128  = 9.9   ≈ 13 for Poznan Street with view extrapolation 0.25

(d2-d1) = 0.1549*128  = 19.8 ≈ 21 for Poznan Street with view extrapolation 0.5

(d2-d1) = 0.2324*128  = 29.7 ≈ 30 for Poznan Street with view extrapolation 0.75

(d2-d1) = 0.3098*128  = 39.7 ≈ 38 for Poznan Street with view extrapolation 1

(d2-d1) = 0.0780*150 = 11.7 ≈ 13 for Undo dancer with view extrapolation 0.25

(d2-d1) = 0.1561*150 = 23.4 ≈ 23 for Undo dancer with view extrapolation 0.5

(d2-d1) = 0.2342*150 = 35.1 ≈ 34 for Undo dancer with view extrapolation 0.75

(d2-d1) = 0.3122*150 = 46.8 ≈ 44 for Undo dancer with view extrapolation 1

when calculating the maximum hole size for the whole sequence, and

(d2-d1) = 0.0774*69   = 5.3   ≈ 6   for Poznan Street with view extrapolation 0.25

(d2-d1) = 0.1549*69   = 10.7 ≈ 11 for Poznan Street with view extrapolation 0.5

(d2-d1) = 0.2324*69   = 16.0 ≈ 15 for Poznan Street with view extrapolation 0.75

(d2-d1) = 0.3098*69   = 21.4 ≈ 19 for Poznan Street with view extrapolation 1

(d2-d1) = 0.0780*102 = 8.0   ≈ 8 for Undo dancer with view extrapolation 0.25

(d2-d1) = 0.1561*102 = 15.9 ≈ 16 for Undo dancer with view extrapolation 0.5

(d2-d1) = 0.2342*102 = 23.9 ≈ 23 for Undo dancer with view extrapolation 0.75

(d2-d1) = 0.3122*102 = 31.8 ≈ 30 for Undo dancer with view extrapolation 1

when calculating the maximum hole size per frame and averaging over all frames of the sequence.

Thus, the example indicates that the maximum hole size can be consistently derived from the maximum depth difference also in practice.

# Conclusions

The proposal addresses a problem of view extrapolation. In particular, it is proposed to send the maximum value of depth discontinuities in the following depth maps in order to enable the view renderer to estimate the width of the maximum disocclusion at a chosen camera position. Syntax and semantics changes to the depth acquisition SEI message are provided. It is proposed to adopt the changes to the PDAM on 3D extensions to MVC, the WD on AVC compatible video with depth and to the HEVC 3D extension draft when that is available.

# Patent rights declaration(s)

Ericsson may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).