| Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 1st Meeting: Stockholm, SE, 16–20 July 2012 | Document: JCT2-A0132 |
| --- | --- |

| Title: | 3D-CE5.h related: Cross check of independent estimated depth maps of Qualcomm |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Cross check |  |  |
| Author(s) or Contact(s): | Heiko Schwarz Gerhard Tech Einsteinufer 37 10587 Berlin Germany | Tel: Email: | +49-30-31002-226 heiko.schwarz@hhi.fraunhofer.de +49-30-31002-190 gerhard.tech@hhi.fraunhofer.de |
| Source: | Fraunhofer HHI |  |  |

_____________________________

# Abstract

This document reports the crosscheck results for the Qualcomm proposal on independent estimated depth maps of non-base views in 3D-HTM. The description provided by Qualcomm in the document JCT2-A0095 is consistent with the software implementation that was provided to the crosschecker. The same coding results have been obtained as reported by Qualcomm in JCT2-A0095.

# Summary of Qualcomm Proposal

In the current 3D-HTM, the inter-view motion and residual prediction tools can introduce undesired dependencies between the coded view. As an example, for a configuration according to the common test conditions, the left and right view of the 3-view test sequences refer only to the center view for the purpose of disparity-compensated prediction. Hence, a bitstream obtained by discarding the NAL units associated with the left or right view should be decodable. Such functionality is for example required for view switching at the decoder side or a media-aware network device. However, if the inter-view motion and/or residual prediction based on estimated depth data are enabled in the current 3D-HTM, it is not possible to independently decode the base view and the second dependent view. For the correct generation of the estimated depth maps for the second dependent view, the coded motion and disparity vector fo the first dependent view are required. At the last meeting it was agreed (see meeting notes) that the current behaviour is undesirable and that the motion/residual prediction tools should not refer to any views that are not included in the reference picture lists.

Qualcomm proposes to overcome this issue for the common test conditions by treating each view in exactly the same way as the first dependent view (view 1) in the current 3D-HTM. That means for generating the estimated depth maps for the second dependent view, only the coded motion and disparity vector of the base view and the second dependent view are used, etc.

We have analyzed the source code that was provided to us by Qualcomm. In our understanding, the description in the document JCT2-A0095 is consistent with the provided implementation.

# Simulation Results

Coding experiments have been conducted according to the common test conditions. As reference, the 3D-HTM software version 3.1rc1 was used, which is identical to the CE5.h software with MPEG/m24937 disabled. This version was also used by Qualcomm for implementing the described modification.

The following table summarizes the simulation results for the crosscheck. More detailed data are provided in the accompanying Excel document. All data are exactly the same as the data reported by Qualcomm in JCT2-A0095.

As expected, the bit rates and PSNR values for the base view and the first dependent view are exactly the same as for the reference. For the video of the second dependent view, an average bit rate increase of 1.1% has been measured, which results in an overall bit rate increase of about 0.1%. The change in encoding and decoding time are negligible.