| Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 1st Meeting: Stockholm, SE, 16–20 July 2012 | Document: JCT2-A0139 |
| --- | --- |

| Title: | 3D-CE5.h related: Cross check of Inter-view motion prediction of MediaTek |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Li Zhang 5775 Morehouse Drive San Diego, CA 92121 USA | Tel: Email: | 1-858-651-6660 lizhang@qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This document reports the cross-check results of MediaTek proposal JCT2-A0049, “3D-CE5.h related: Inter-view motion prediction for HEVC-based 3D video coding.” The simulation results performed for this cross-verification match those provided by the proponents.

# Introduction and examination of software

In contribution JCT2-A0049, two methods were proposed to simplify and/or improve the coding efficiency of inter-view motion prediction:

- Method 1: Only the motion data of the inter-view picture which is in the reference picture lists (List 0 or List 1) of current picture with the smallest viewId can be used in inter-view motion prediction

- Method 2: Simplify and improve the derivation of the inter-view merging candidate for the merge and skip modes

# Performance of proposed method

The proposed method from JCT2-A0049 [1] was implemented on top of HTM3.1. For this cross verification, simulations using software provided by the proponents were compared to a reference coded with HTM3.1, using the common conditions described in [2].

Three cases are tested:

- Case 1: method #1, with macro ‘MTK_IVMVP_LIMITATION’ set to 1 and ‘MTK_MERGEIV’ set to 0

- Case 2: method #2, with macro ‘MTK_IVMVP_LIMITATION’ set to 0 and ‘MTK_MERGEIV’ set to 1

- Case 3: method #1 and #2, with both macros ‘MTK_IVMVP_LIMITATION’ and ‘MTK_MERGEIV’ set to 1

All software was compiled and run on a Linux cluster. Results are shown in Table 1, Table 2 and Table 3.

Table 1. Performance of Case 1

|  | video 0 | video 1 | video 2 | video only | synthesized only | coded & synthesized | enc time | dec time | ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.0% | 0.0% | -0.3% | -0.1% | 0.0% | 0.0% | 97.9% | 95.9% | 98.9% |
| Kendo | 0.0% | 0.0% | -0.4% | -0.1% | 0.0% | 0.0% | 102.3% | 99.3% | 99.5% |
| Newspapercc | 0.0% | 0.0% | -0.5% | -0.1% | -0.2% | -0.2% | 98.3% | 97.1% | 96.2% |
| GhostTownFly | 0.0% | 0.0% | -0.3% | 0.0% | 0.0% | 0.0% | 103.7% | 101.3% | 102.2% |
| PoznanHall2 | 0.0% | 0.0% | -0.3% | -0.1% | 0.0% | -0.1% | 109.0% | 102.0% | 105.8% |
| PoznanStreet | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% | 102.4% | 96.0% | 101.8% |
| UndoDancer | 0.0% | 0.0% | -0.2% | 0.0% | 0.0% | 0.0% | 106.1% | 99.2% | 100.3% |
| 1024x768 | 0.0% | 0.0% | -0.4% | -0.1% | -0.1% | -0.1% | 99.5% | 97.4% | 98.2% |
| 1920x1088 | 0.0% | 0.0% | -0.2% | 0.0% | 0.0% | 0.0% | 105.3% | 99.6% | 102.5% |
| average | 0.0% | 0.0% | -0.3% | -0.1% | 0.0% | 0.0% | 102.7% | 98.7% | 100.6% |

Table 2. Performance of Case 2

|  | video 0 | video 1 | video 2 | video only | synthesized only | coded & synthesized | enc time | dec time | ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.0% | -1.8% | -2.4% | -0.8% | -0.6% | -0.7% | 100.7% | 100.3% | 101.1% |
| Kendo | 0.0% | -1.4% | -2.1% | -0.7% | -0.5% | -0.6% | 99.7% | 97.2% | 96.0% |
| Newspapercc | 0.0% | -1.5% | -1.6% | -0.6% | -0.4% | -0.5% | 98.6% | 98.8% | 98.4% |
| GhostTownFly | 0.0% | -0.3% | -0.5% | -0.1% | -0.1% | -0.1% | 101.8% | 100.2% | 100.4% |
| PoznanHall2 | 0.0% | -0.4% | -0.8% | -0.2% | -0.1% | -0.1% | 100.1% | 101.7% | 99.4% |
| PoznanStreet | 0.0% | -0.9% | -1.1% | -0.3% | -0.2% | -0.2% | 97.5% | 93.8% | 98.1% |
| UndoDancer | 0.0% | -1.2% | -1.7% | -0.5% | -0.3% | -0.4% | 99.9% | 95.7% | 96.3% |
| 1024x768 | 0.0% | -1.6% | -2.0% | -0.7% | -0.5% | -0.6% | 99.7% | 98.8% | 98.5% |
| 1920x1088 | 0.0% | -0.7% | -1.0% | -0.3% | -0.2% | -0.2% | 99.8% | 97.8% | 98.5% |
| average | 0.0% | -1.1% | -1.4% | -0.5% | -0.3% | -0.4% | 99.8% | 98.2% | 98.5% |

Table 3. Performance of Case 3

|  | video 0 | video 1 | video 2 | video only | synthesized only | coded & synthesized | enc time | dec time | ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.0% | -1.8% | -2.4% | -0.8% | -0.6% | -0.7% | 98.5% | 99.6% | 101.5% |
| Kendo | 0.0% | -1.4% | -2.1% | -0.7% | -0.5% | -0.6% | 97.5% | 99.9% | 96.3% |
| Newspapercc | 0.0% | -1.5% | -1.6% | -0.6% | -0.2% | -0.3% | 98.3% | 98.0% | 100.2% |
| GhostTownFly | 0.0% | -0.3% | -0.4% | -0.1% | -0.1% | -0.1% | 97.8% | 96.3% | 96.2% |
| PoznanHall2 | 0.0% | -0.4% | -0.7% | -0.2% | -0.2% | -0.2% | 100.3% | 99.5% | 100.5% |
| PoznanStreet | 0.0% | -0.9% | -1.1% | -0.3% | -0.3% | -0.3% | 96.4% | 96.2% | 98.2% |
| UndoDancer | 0.0% | -1.2% | -1.7% | -0.5% | -0.3% | -0.3% | 99.3% | 99.2% | 97.1% |
| 1024x768 | 0.0% | -1.6% | -2.0% | -0.7% | -0.5% | -0.5% | 98.1% | 99.2% | 99.3% |
| 1920x1088 | 0.0% | -0.7% | -1.0% | -0.3% | -0.2% | -0.2% | 98.4% | 97.8% | 98.0% |
| average | 0.0% | -1.1% | -1.4% | -0.5% | -0.3% | -0.3% | 98.3% | 98.4% | 98.5% |

# Conclusions

The results obtained during this cross verification match those provided by the proponents.

# References

- Jichen An, et.al., "3D-CE5.h related: Inter-view motion prediction for HEVC-based 3D video coding", JCT2-A0049, Stockholm, July 2012

- Heiko Schwarz, Dmytro Rusanovskyy, "Common Test Conditions for 3DV experimentation", ISO/IEC JTC1/SC29/WG11 document N12745, Geneva, Switzerland, May 2012.