| Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 1st Meeting: Stockholm, SE, 16–20 July 2012 | Document: JCT2-A0140 |
| --- | --- |

| Title: | 3DV-AVC HLS: On depth definition |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Shinya Shimizu, Shiori Sugimoto, Hideaki Kimata | Tel: Email: | +81-46-859-2703 shimizu.shinya@lab.ntt.co.jp |
| Source: | NTT Corporation |  |  |

_____________________________

# Abstract

This contribution proposes a modification on the depth acquisition information SEI message and a definition of depth representation type in order to support many kinds of multiview video plus depth map other than those used in the development. Regarding the depth acquisition information SEI message, it is suggested to support convergence camera setting. The proposed depth representation type includes three kinds of representation: disparity, an inverse of Z value, and Z value. Since there are some coding tools which requires understanding the definition of depth, this contribution proposes to define depth representation type as a part of the sequence parameter set extension. The extension of the depth-range-based weighted prediction, which is one of the coding tool effected by the depth representation type, is also discussed.

# Introduction

There is no definition of the depth in either text of ISO/IEC 14496-10:2012/PDAM2 "MVC extension for inclusion of depth maps" [1] or Working Draft 2 of AVC compatible video with depth [2]. However, it seems necessary to inform what kind of information is transmitted in order to utilize in the appropriate manner. We extend the definition proposed in [3] to include some other popular depth representations. In this contribution, we would like to discuss the better place where the depth representation type defined. In addition, we’d also like to discuss the depth-range-based weighted prediction.

Since camera arrangement is one of the important factors on the definition of the depth representation, we first propose to extend the camera arrangement to be supported in the 3DV activity.

# Camera Arrangement

Since almost all recent stereo cameras use a convergence camera setting, it is necessary to support such convergence camera settings. Actually, there is no restriction on the camera arrangement in the normative parts, but the current activity is not considering the camera arrangement other than 1D parallel. We think it is necessary to develop the specification with considering supporting, at least, convergence camera arrangements.

For the support of convergence camera settings, we propose to extend the Depth acquisition information SEI syntax. The proposed syntax is presented in below. Since we cannot find out a use case which requires either pitch or roll rotations, the proposed syntax supports only a yaw rotation.

I.13.1.3	Depth acquisition information SEI message syntax

| depth_acquisition_info( payloadSize ) { | C | Descriptor |
| --- | --- | --- |
| all_views_equal_flag | 5 | u(1) |
| if( all_views_equal_flag = = 0 ){ |  |  |
| num_views_minus1 | 5 | ue(v) |
| numViews = num_views_minus1 + 1 |  |  |
| }else{ |  |  |
| numViews = 1 |  |  |
| } |  |  |
| num_frames | 5 | ue(v) |
| z_near_flag | 5 | u(1) |
| z_far_flag | 5 | u(1) |
| d_min_flag | 5 | u(1) |
| d_max_flag | 5 | u(1) |
| focal_length_x_flag | 5 | u(1) |
| focal_length_y_flag | 5 | u(1) |
| principal_point_x_flag | 5 | u(1) |
| principal_point_y_flag | 5 | u(1) |
| translation_flag | 5 | u(1) |
| rotation_flag | 5 | u(1) |
| for( i = 0; i < numViews; i++ ) { |  |  |
| if( z_near_flag ) |  |  |
| depth_acquisition_element( i, num_frames, 0, 7, ZNearSign, ZNearExp, ZNearMantissa, ZNearManLen ) |  |  |
| if( z_far_flag ) |  |  |
| depth_acquisition_sei_element( i, num_frames, 0, 7, ZFarSign, ZFarExp, ZFarMantissa, ZFarManLen ) |  |  |
| if( d_min_flag ) |  |  |
| depth_acquisition_sei_element( i, num_frames, 0, 7, DMinSign, DMinExp, DMinMantissa, DMinManLen ) |  |  |
| if( d_max_flag ) |  |  |
| depth_acquisition_sei_element( i, num_frames, 0, 7, DMaxSign, DMaxExp, DMaxMantissa, DMaxManLen ) |  |  |
| if( focal_length_x_flag ) |  |  |
| depth_acquisition_sei_element( i, num_frames, 1, 6, FocalLengthXSign, FocalLengthXExp, FocalLengthXMantissa, FocalLengthXManLen ) |  |  |
| if( focal_length_y_flag ) |  |  |
| depth_acquisition_element( i, num_frames, 1, 6, FocalLengthYSign, FocalLengthYExp, FocalLengthYMantissa, FocalLengthYManLen ) |  |  |
| if( principal_point_x_flag ) |  |  |
| depth_acquisition_element( i, num_frames, 1, 6, PrincipalPointXSign, PrincipalPointXExp, PrincipalPointXMantissa, PrincipalPointXManLen ) |  |  |
| if( principal_point_y_flag ) |  |  |
| depth_acquisition_element( i, num_frames, 1, 6, PrincipalPointYSign, PrincipalPointYExp, PrincipalPointYMantissa, PrincipalPointYManLen ) |  |  |
| if( translation_flag ) |  |  |
| depth_acquisition_element( i, num_frames, 1, 6, TXSign, TXExp, TXMantissa, TXManLen ) |  |  |
| if( rotation_flag ) |  |  |
| depth_acquisition_element( i, num_frames, 1, 6, YawSign, YawExp, YawMantissa, YawManLen ) |  |  |
| } |  |  |
| } |  |  |

I.13.2.3   Depth acquisition information SEI message semantics

rotation_flag equal to 0 specifies that the syntax elements specifying the angle of the yaw rotation between views are not present in the syntax structure. rotation_flag equal to 1 specifies that the syntax elements specifying the angle of the yaw rotation between views are present in the syntax structure.

- Table I-X – Association between depth parameter variables and syntax elements.

| x | s | e | n | v |
| --- | --- | --- | --- | --- |
| ZNear | ZNearSign | ZNearExp | ZNearMantissa | ZNearManLen |
| ZFar | ZFarSign | ZFarExp | ZFarMantissa | ZFarManLen |
| DMax | DMaxSign | DMaxExp | DMaxMantissa | DMaxManLen |
| DMin | DMinSign | DMinExp | DMinMantissa | DMinManLen |
| FocalLengthX | FocalLengthXSign | FocalLengthXExp | FocalLengthXMantissa | FocalLengthXManLen |
| FocalLengthY | FocalLengthYSign | FocalLengthYExp | FocalLengthYMantissa | FocalLengthYManLen |
| PrincipalPointX | PrincipalPointXSign | PrincipalPointXExp | PrincipalPointXMantissa | PrincipalPointXManLen |
| PrincipalPointY | PrincipalPointYSign | PrincipalPointYExp | PrincipalPointYMantissa | PrincipalPointYManLen |
| TX | TXSign | TXExp | TXMantissa | TXManLen |
| Yaw | YawSign | YawExp | YawMantissa | YawManLen |

# Definition of Depth

## Depth representation type

In [3], two depth representation types are proposed to be defined: 1) an inverse of Z value and 2) disparity. The first one is the most familiar type since depth information obtained by using pixel-based estimation is normally defined with this type. The depth information from Microsoft KINECT is also based on this type. Although depth information in the second type can be derived by scaling those in the first one, it might be very useful for disparity-based display systems like multiview auto-stereoscopic display. In addition to these types, this contribution proposes to define distance, i.e. Z value itself as the third type. This type of depth representation is used in the Time-On-Flight (TOF) camera system and the computer graphic. Therefore, this contribution proposes to define a new syntax element, depth_representaion_type, to specify three depth representation types.

Besides the kind of the depth information, it is also necessary to specify some additional information. As proposed in [3], if the depth representation type is disparity, it is necessary to indicate the base view which the displacement is derived from. If the depth representation type is either an inverse of Z value or Z value itself, the definition of Z-axis is necessary. Considering the acquisition method, it is normal to define Z-axis is defined as the optical axis of the own camera/sensor. However, it is also popular to share one axis among multi-camera in order to make it easier to process depth information on the 3D space although direct acquisition of such information is impossible. Therefore, this contribution proposes to define two new syntax elements, all_views_equal_flag and depth_representaion_base_view_id. all_views_equal_flag specifies whether a common base view is used or not. depth_representaion_base_view_id specifies disparity base view when the depth representation type is disparity or z-axis when the depth representation type is either an inverse of Z value or Z value.

The proposed syntax is as follows:

| depth_represention_type( payloadSize ) { | C | Descriptor |
| --- | --- | --- |
| depth_representation_type | 5 | ue(v) |
| all_views_equal_flag | 5 | u(1) |
| if( all_views_equal_flag = = 0 ){ |  |  |
| num_views_minus1 | 5 | ue(v) |
| numViews = num_views_minus1 + 1 |  |  |
| }else{ |  |  |
| numViews = 1 |  |  |
| } |  |  |
| for( i = 0; i < numViews; i++ ) { |  |  |
| depth_representation_base_view_id[i] | 5 | ue(v) |
| } |  |  |
| } |  |  |

depth_representation_type specifies the representation definition of luma pixels in coded frame of depth views as specified in Table I-XX. In Table I-XX, disparity specifies the horizontal displacement between two texture views and Z value specifies the distance from a camera.

Table I-XX Definition of depth_representation_type

| depth_representation_type | Interpretation |
| --- | --- |
| 0 | Each luma pixel value in coded frame of depth views represents an inverse of Z value normalized in range from 0 to 255 |
| 1 | Each luma pixel value in coded frame of depth views represents disparity normalized in range from 0 to 255 |
| 2 | Each luma pixel value in coded frame of depth views represents Z value normalized in range from 0 to 255 |
| Other values | Reserved for future use |

all_views_equal_flag equal to 0 specifies that depth representation base view may not be identical to respective values for each view in target views. all_views_equal_flag equal to 1 specifies that the depth representation base views are identical to respective values for all target views.

num_views_minus1 shall be equal to the value of the syntax element num_views_minus1 in the 3DVC sequence parameter set for the coded video sequence. The value of num_views_minus1 shall be in the range of 0 to 1023, inclusive.

depth_representaion_base_view_id[i] specifies the view identifier for the NAL unit of either base view which the disparity for coded depth frame of i-th view_id is derived from or base view which the Z-axis for the coded depth frame of i-th view_id is defined as the optical axis of.

There are three choices for the position where the proposed depth representation type is placed: 1) SPS, 2) VUI, and 3) SEI message. If all coding tools are free from the depth representation type, a SEI message is a good choice. However, there is at least one coding tool which affected by the depth representation type. depth-range-based weighted prediction is the tool which is developed with the assumption that the depth representation type is an inverse of Z value with a 1D parallel camera arrangement. Therefore, this contribution proposes to define the depth representation type information in the sequence parameter set 3DVC extension.

## Depth-range-based weighted prediction

As described in the subsection 3.1, the depth-ranged-based weighted prediction is designed to one specific depth representation type. Therefore, in order to the depth range information correctly, it is necessary to modify the process based on the depth representation type. When depth_representation_type is equal to 2, wXC and oXC are calculated by the following equations:

Let wXC be equal to round( ( zF1 – zN1 ) ÷ ( zF2 –zN2 ) * 2logWDC )

Let oXC be equal to round( ( zN1 – zN2 ) ÷ ( zF2 –zN2 ) * 255 )

Note that the current equations in I.8.4.1.1 seem too complicated and require some floating point operations, so that the simplification and confirmation are required. Especially, the term, 2*64, on the calculation of z2 seems strange.

Furthermore, it is also necessary to investigate the way to provide a better compensation on non-parallel sequences. The current and above described equations are derived under the assumption that the camera arrangement is 1D-parallel.

# Conclusion

The contribution proposed a definition of depth representation type and a related change on depth-range-based weighted prediction. A modification on depth acquisition information SEI message is also proposed to support one of the most common stereo camera arrangements.

We recommend 1) adopting the proposed modification on depth acquisition information SEI message, 2) discussing the right place to define the depth representation type, and 3) studying the depth-range-based weighted prediction which should be designed based on the depth representation type.

# Patent rights declaration

NTT Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- ISO/IEC JTC1/SC29/WG11, “Text of ISO/IEC 14496-10:2012/PDAM2 MVC extension for inclusion of depth maps”, ISO/IEC JTC1/SC29/WG11 N12731, May 2012, Geneva, Switzerland.

- ISO/IEC JTC1/SC29/WG11, “Working Draft 2 of AVC compatible video with depth”, ISO/IEC JTC1/SC29/WG11 N12732, May 2012, Geneva, Switzerland.

- Sally Hattori, and Yoshitomo Takahashi, "3D-AVC HLS: Modifications on Depth Acquisition Informormation SEI message", ISO/IEC JTC1/SC29/WG11 MPEG2012/M24730, April 2012, Geneva, Switzerland.