| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: Stockholm, SE, 11–20 July 2012 | Document: JCT2-A0142 |
| --- | --- |

| Title: | 3D-CE5.h related: Cross-check report of JCT2-A0095 on bug fix for estimated depth maps |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT2 |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Yoshitomo Takahashi Sally Hattori Teruhiko Suzuki Sony City Osaki. 2-10-1 Osaki Shinagawa-ku , Tokyo, 141-8610 JAPAN | Tel: Email: | +81-50-3750-2746 Yoshitomo.Takahashi@jp.sony.com Shinobu.Hattori@jp.sony.com teruhikos@jp.sony.com |
| Source: | Sony Corporation |  |  |

_____________________________

# Abstract

This contribution reports the cross-checking results on JCT2-A0095 proposed by Qualcomm.

In JCT2-A0095, the depth map estimation process is modified in order to decode two non-base views independently. The proposed method is the same method as anchor for view1, but different method for view2. In random access units, the depth map of view2 is estimated form coded disparity vector of view2, instead of using the depth map warped from view1. In the other access units, the reference view of warping process for view2 is changed from view1 to view0.

The experimental results in 3D-HTM3.1rc1 are reported. The results show that BD-bitrate loss is 1.1% for the video of view2, and is no gain or loss for the video of view0 and view1 compared with anchor on average. The results also shows that BD-bitrate loss is 0.1%, 0.1% and 0.1% for only video views, only synthesized views, and both video views and synthesized views, respectively. It is reported that these schemes does not affect the decoding time at least.

# Cross-checking

The software based on 3D-HTM3.1rc1 and the experimental results were provided by Qualcomm. The software was compiled fine and the cross-checking was performed on 64bit linux environment. The simulation has been conducted under common test conditions [1] for 3 view case.

The results of anchor were generated using the released 3D-HTM3.1rc1. The results of proposal were generated with the macro “#define PDM_REMOVE_DEPENDENCE 1” provided by the proponent.

# Results

The experimental results are summarized in the following table. The detailed results are shown in the attached excel-file.

|  | video 0 | video 1 | video 2 | video only | synthesized only | coded & synthesized | enc time | dec time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.0% | 0.0% | -1.2% | -0.3% | -0.1% | -0.1% | 99.6% | 98.9% |
| Kendo | 0.0% | 0.0% | -0.2% | -0.1% | 0.0% | 0.0% | 124.7% | 100.5% |
| Newspapercc | 0.0% | 0.0% | 0.1% | 0.0% | 0.1% | 0.1% | 111.4% | 99.1% |
| GhostTownFly | 0.0% | 0.0% | 4.4% | 0.6% | 0.5% | 0.5% | 93.3% | 90.8% |
| PoznanHall2 | 0.0% | 0.0% | 0.7% | 0.1% | 0.0% | 0.0% | 94.5% | 91.8% |
| PoznanStreet | 0.0% | 0.0% | 1.0% | 0.1% | 0.2% | 0.1% | 100.7% | 90.9% |
| UndoDancer | 0.0% | 0.0% | 3.0% | 0.4% | 0.4% | 0.4% | 99.4% | 92.1% |
| 1024x768 | 0.0% | 0.0% | -0.4% | -0.1% | 0.0% | 0.0% | 111.4% | 99.5% |
| 1920x1088 | 0.0% | 0.0% | 2.3% | 0.3% | 0.2% | 0.3% | 96.9% | 91.4% |
| average | 0.0% | 0.0% | 1.1% | 0.1% | 0.1% | 0.1% | 102.9% | 94.8% |

# Comment

In JCT2-A0095, the complexity of depth map estimation for view2 is increased in order to remove the dependency between two non-base views. In non-anchor access units, two procedures are added to anchor. First procedure is the warping of the depth from view2 to view0. Second procedure is the motion compensation in view0 using the depth map warped from view2.

At the previous meeting, the similar approach is proposed in m24989. In m24989, the dependency between two non-base views is also removed. In addition, the complexity of depth map estimation is reduced by removing all the warping process of depth map and all the motion compensation of depth map.

# Conclusion

In this contribution, the cross-checking results on JCT2-A0095 are reported. It is confirmed that the results match with the ones provided by Qualcomm.

# References

- ISO/IEC JTC1/SC29/WG11, “Common Test Conditions for 3DV experimentation”, N12745, Geneva, Switzerland, May 2012.

# Patent rights declaration(s)

Sony Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).