| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: Stockholm, SE, 11–20 July 2012 | Document: JCT2-A0143 |
| --- | --- |

| Title: | 3D-CE5.h related: Cross-check report of JCT2-A0014 on simplification of AMVP |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT2 |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Yoshitomo Takahashi Sally Hattori Teruhiko Suzuki Sony City Osaki. 2-10-1 Osaki Shinagawa-ku , Tokyo, 141-8610 JAPAN | Tel: Email: | +81-50-3750-2746 Yoshitomo.Takahashi@jp.sony.com Shinobu.Hattori@jp.sony.com teruhikos@jp.sony.com |
| Source: | Sony Corporation |  |  |

_____________________________

# Abstract

This contribution reports the cross-checking results on JCT2-A0014 proposed by Sharp.

In JCT2-A0014, the motion vector candidate construction is simplified. The proposed method adds zero vector to the candidate list when first predictor (derived by Inter-view motion prediction) is not available and changes the pruning combination to avoid the unnecessary candidate motion vector derivation process.

The verification task has been done successfully and the results match those provided by proponent. It is noted that the encoding time of our cross-checking results may be not reliable since the simulation machines with various specifications were used. The decoding time is more reliable since the machines with same specification were used.

The experimental results in 3D-HTM version 3.1 are reported. The results show that BD-bitrate loss is -0.1% for the video of view1, and is no gain or loss for the video of view0 and view1 compared with anchor on average. The results also shows that BD-bitrate loss is no gain or loss for only video views, only synthesized views, and both video views and synthesized views, respectively. It is reported that these schemes does not affect the decoding time at least.

# Cross-checking

The software based on 3D-HTM version 3.1 and the experimental results were provided by Sharp. The software was compiled fine and the cross-checking was performed on 64bit linux environment. The simulation has been conducted under common test conditions [1] for 3 view case.

The results of anchor were generated using the released 3D-HTM version 3.1. The results of proposal were generated with the macro provided by the proponent, as follows:

#define SHARP_IMVP_RETURN_ZEROMV          1

#define SHARP_IMVP_SHIFT_PRUNING          1

# Results

The experimental results are summarized in the following table. The detailed results are shown in the attached excel-file.

|  | video 0 | video 1 | video 2 | video only | synthesized only | coded & synthesized | enc time | dec time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.0% | 0.1% | 0.1% | 0.0% | 0.0% | 0.0% | 111.9% | 100.1% |
| Kendo | 0.0% | -0.1% | 0.0% | 0.0% | 0.1% | 0.0% | 97.5% | 99.4% |
| Newspapercc | 0.0% | -0.1% | -0.1% | 0.0% | 0.0% | 0.0% | 113.5% | 99.5% |
| GhostTownFly | 0.0% | -0.1% | 0.2% | 0.0% | 0.0% | 0.0% | 101.3% | 100.5% |
| PoznanHall2 | 0.0% | -0.2% | -0.5% | -0.1% | 0.0% | -0.1% | 103.4% | 99.5% |
| PoznanStreet | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% | 0.0% | 102.4% | 99.0% |
| UndoDancer | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% | 103.7% | 100.8% |
| 1024x768 | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% | 0.0% | 107.4% | 99.7% |
| 1920x1088 | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% | 0.0% | 102.7% | 99.9% |
| average | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% | 0.0% | 104.7% | 99.8% |

# Conclusion

In this contribution, the cross-checking results on JCT2-A0014 are reported. It is confirmed that the results match with the ones provided by Sharp.

# References

- ISO/IEC JTC1/SC29/WG11, “Common Test Conditions for 3DV experimentation”, N12745, Geneva, Switzerland, May 2012.

# Patent rights declaration(s)

Sony Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).