INTERNATIONAL ORGANISATION FOR STANDARDISATION

ORGANISATION INTERNATIONALE DE NORMALISATION

ISO/IEC JTC1/SC29/WG11

CODING OF MOVING PICTURES AND AUDIO

ISO/IEC JTC1/SC29/WG11

MPEG2012/M26157

July 2012, Stockholm, Sweden

| Source | Industrial Technology Research Institute |
| --- | --- |
| Status | Technical input |
| Title | 3DV Technical Input: JMVC software integration of coding interlace texture with interlace depth |
| Author | Ching-Chieh Lin, Fang-Chu Chen, Wen-Hao Chung |

- Introduction

The needs and the possibility of 3D interlace coding has been discussed during MPEG 100th meeting in Geneva. In the meeting, Sony’s proposal M24731[1] provided modified JMVC reference code to integrate interlace texture and progressive depth coding. In the same meeting, ITRI contributed a document M25069[2] with experiment data demonstrating the importance of interlace depth coding especially when top and bottom fields of the texture views are captured at different time (for example 1/60 second apart). If depth cannot be field captured and coded, then at least for one field the texture and the depth will not be synchronized. The timing mismatch of the texture and the depth will result in unacceptable synthesized views. Allowing JMVC to be able to encode interlace depth, therefore, is important.

This document is intent to extend Sony’s contribution to add an option for interlace depth coding. It provides a description of the modification to the JMVC software (proposed by Sony in M24731[1]) so that the reference code can encode/decode interlace texture with interlace depth content.

- Integration Details

The contribution follows Sony’s previous specifications as described in M24731, only to add an option for the encoder’s choice of  interlace depth or progressive depth. For this purpose, a new structure for the access unit (AU) composed of interlace texture and interlace depth is shown in Figure 1. The structure shows an AU of the first field containing the first texture field view components and the corresponding field depth view components with same view_id; and an AU of the second field containing the second texture field view components and its corresponding field depth view components with same view_id. By “corresponding” it means the field depth captured at the same time as the field texture.  This structure indicates that the field texture and the corresponding field depth have the POC.

- Figure 1. Access units of field texture and field depth

- The modifications of the Reference Software

- 3.1 Modification on the encoder configuration file

To allow the choice of interlace depth or progressive depth, we add in the encoder configuration file a parameter

- InterlaceDepth

When set to 1, the depth will be encoded as interlace depth; otherwise, it will be encoded as progressive depth.

- 3.2 Modification on the assembler configuration file

To allow assembling of field texture and the corresponding field depth in the same AU, as shown in figure 1, we add in the assembler configuration file a parameter

- InterlaceDepth

When set to 1, field texture and the corresponding field depth will be assembled in the same AU; otherwise, it will regard depth as frame depth and assembled according to M24731[1].

- 3.3 Modifications on the encoder

- The modifications on the encoder are to add a variable InterlaceDepthFlag at the appropriate places so that the flag, reflecting the value of InterlaceDepth parameter in the encoder configuration file, can be checked to see if the depth view component is encoded as frame picture or field picture.  The following is an example of the modification:

- #if SONY_3DV_INTERLACE_VIDEO

- Bool bFieldCoded;

- if(this->getDepthFlag()){

- #if ITRI_3DV_INTERLACE_DEPTH_EXTENSION

- if(m_pcCodingParameter->getInterlaceDepthFlag() != 0 ) {

- bFieldCoded = ( m_pcCodingParameter->getPAff()==1? true : (m_pcCodingParameter->getPAff()==2? (( rand()%2 ) ? true : false) : false));

- }

- else

- #endif

- bFieldCoded = false;

- }

- else{

- bFieldCoded = ( m_pcCodingParameter->getPAff()==1? true :  (m_pcCodingParameter->getPAff()==2? (( rand()%2 ) ? true : false) : false));

- }

- #else

- const Bool bFieldCoded = ( m_pcCodingParameter->getPAff()==1? true : (m_pcCodingParameter->getPAff()==2? (( rand()%2 ) ? true : false) : false));

- #endif

- 3.4 Modifications on the assembler

The modifications on the assembler are to add a variable InterlaceDepthFlag at the appropriate places so that the flag, reflecting the value of InterlaceDepth parameter in the assembler configuration file, can be checked to see if the AU is assembled with the field texture and the corresponding field depth, or assembled the way as described in document M24731[1].

- 3.5 Modifications on the decoder

- There is only one modification in the decoder, that is to increase the size of the POC list so that it can accommodate the interlace depth components. The modified code is shown as below:

- In H264AVCDecoderTest class,

- In xRemovePicBuffer() member function, modified the original code into

- #if SONY_3DV_INTERLACE_VIDEO

- #if ITRI_3DV_INTERLACE_DEPTH_EXTENSION

- for(int i = (m_cActivePicBufferList.size() - m_pcH264AVCDecoder->getMaxEtrDPB() * 14); i > 0; i--)

- #else

- for(int i = (m_cActivePicBufferList.size() - m_pcH264AVCDecoder->getMaxEtrDPB() * 9); i > 0; i--) // takahashi

- #endif

- #else

- for(int i = (m_cActivePicBufferList.size() - m_pcH264AVCDecoder->getMaxEtrDPB() * 4); i > 0; i--)

- #endif

- Verification

The modified JMVC software has been verified on all 7 official 3DV test sequences as defined in the common test conditions[3] for two view input. The encoder, the assembler, and the decoder perform correctly for all three coding configurations: progressive texture with progressive depth, interlace texture with progressive depth, and interlace texture with interlace depth.

- Conclusion

The contribution provides description of the modifications to the version of JMVC software contributed by Sony in MPEG 100th meeting to allow interlace depth coding. Verifications of the modified software on 3DV official sequences have been performed and the results are correct. We recommend the working group to approve the modifications to interlace depth coding in JMVC.

- Reference

- Sally Hattori, Yoshitomo Takahashi,” JMVC software integration of coding interlace texture with progressive depth based on WD on MVC extensions for inclusion of depth maps “, ISO/IEC JTC1/SC29/WG11  M24731, April, Geneva, Switzerland.

- Fang-Chu Chen, Ching-Chieh Lin,” 3DV AVC based Interlace Multiview Plus Depth Coding (revised version 2)”, ISO/IEC JTC1/SC29/WG11 M25069, April, Geneva, Switzerland.

- Heiko Schwarz, Dmytro Rusanovskyy, "Common Test Conditions for HEVC- and AVC-based 3DV", ISO/IEC JTC1/SC29/WG11 MPEG2011/N12352, December 2011, Geneva, Switzerland.