| Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 1st Meeting: Stockholm, SE, 16–20 July 2012 | Document: JCT2-A0147 |
| --- | --- |

| Title: | 3D-CE3.a: Cross check report of JCT2-A0037 on region based adaptive loop filter of Samsung |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to the JCT2 |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Gun Bang, *Kyung Yong Kim, *Young Su Heo, *Gwang Hoon Park, Won-Sik Cheong, Namho Hur ETRI, 218 Gajeong-Ro, Yuseong-Gu, Daejeon, 305-700, Korea *Media Lab., Kyung Hee University 1, Seochun, Kiheung, Youngin, Gyonggi, 446-701, Korea | Tel: Email: | +82-42-860-5069 {gbang, wscheong, namho} @etri.re.kr +82-31-201-3680 {*kimky, *medist, *ghpark} @khu.ac.kr |
| Source: | ETRI and Kyung Hee University |  |  |

_____________________________

# Abstract

This contribution reports cross-check results for CE3 experiment RALF [1-2]. The R-D performance almost matched with the data provided by Samsung. Some parts of the result that have a difference between the data provided by Samsung may be caused by the system environments. Common test conditions specified in W12745 [3] is used for the evaluation. The decoding times were obtained using the same system environments as the anchor (Configuration in decoder.cfg: OutputRecYUVFile = 0). Experimental results shows average 0.99% BD-rate reduction with decoding runtime increased by 12% compared with 3DV-ATM v0.4 (De-blocking filter for depth: ON). And it also shows 1.70% BD-rate reduction with decoding runtime increased by 15% compared with 3DV-ATM v0.4 (De-blocking filter for depth: OFF).

Table 1. Simulation result of RALF (Anchor: 3DV-ATM v0.4 + Depth De-block ON)

|  | Texture Coding |  | Depth Coding |  | Total (Coded PSNR) |  | Total (Synthesed PSNR) |  | Complexity estimate (ratio to anchor) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | dBR, % | dPSNR,dB | dBR, % | dPSNR,dB | dBR, % | dPSNR,dB | dBR, % | dPSNR,dB | Encoder Time, % | Decoder Time, % | Rendering Time, % |
| S01 | -0.59 | 0.02 | 0.40 | -0.01 | -0.49 | 0.01 | -1.01 | 0.03 | 104% | 107% | 100.00 |
| S02 | -0.13 | 0.00 | 0.81 | -0.03 | 0.00 | 0.00 | -0.54 | 0.02 | 99% | 107% | 100.00 |
| S03 | -0.55 | 0.02 | -1.22 | 0.11 | -0.44 | 0.02 | -2.84 | 0.08 | 96% | 111% | 100.00 |
| S04 | -0.16 | 0.01 | -1.58 | 0.09 | -0.14 | 0.01 | -0.70 | 0.02 | 102% | 112% | 100.00 |
| S05 | -0.19 | 0.01 | 2.48 | -0.12 | -0.06 | 0.00 | -0.44 | 0.02 | 96% | 105% | 100.00 |
| S06 | -0.15 | 0.01 | 2.12 | -0.10 | -0.02 | 0.00 | -0.40 | 0.02 | 100% | 104% | 100.00 |
| S08 | -0.13 | 0.01 | 2.04 | -0.09 | 0.00 | 0.00 | -0.98 | 0.04 | 102% | 103% | 100.00 |
| Average | -0.27 | 0.01 | 0.72 | -0.02 | -0.17 | 0.01 | -0.99 | 0.03 | 100% | 107% | 100.00 |

Table 2. Simulation result of RALF (Anchor: 3DV-ATM v0.4 + Depth De-block OFF)

|  | Texture Coding |  | Depth Coding |  | Total (Coded PSNR) |  | Total (Synthesed PSNR) |  | Complexity estimate (ratio to anchor) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | dBR, % | dPSNR,dB | dBR, % | dPSNR,dB | dBR, % | dPSNR,dB | dBR, % | dPSNR,dB | Encoder Time, % | Decoder Time, % | Rendering Time, % |
| S01 | -1.13 | 0.03 | -5.20 | 0.33 | -1.21 | 0.04 | -1.78 | 0.06 | 107% | 112% | 100.00 |
| S02 | -0.12 | 0.00 | -7.22 | 0.36 | -0.17 | 0.01 | -1.08 | 0.03 | 104% | 110% | 100.00 |
| S03 | -1.14 | 0.04 | -7.33 | 0.64 | -1.19 | 0.04 | -3.18 | 0.08 | 102% | 114% | 100.00 |
| S04 | -0.30 | 0.01 | -8.24 | 0.48 | -0.36 | 0.01 | -1.32 | 0.04 | 105% | 115% | 100.00 |
| S05 | -0.47 | 0.02 | -4.06 | 0.20 | -0.48 | 0.02 | -1.38 | 0.06 | 95% | 108% | 100.00 |
| S06 | -0.48 | 0.02 | -5.09 | 0.25 | -0.52 | 0.02 | -1.32 | 0.06 | 100% | 106% | 100.00 |
| S08 | -0.20 | 0.01 | -5.57 | 0.25 | -0.27 | 0.01 | -1.86 | 0.07 | 99% | 106% | 100.00 |
| Average | -0.55 | 0.02 | -6.10 | 0.36 | -0.60 | 0.02 | -1.70 | 0.06 | 102% | 110% | 100.00 |

The following Table 3 shows the simulation result of the de-blocking filter for depth. The de-blocking filter for depth brings a coding gain without complexity.

Table 3. Simulation result of Depth De-block ON (Anchor: 3DV-ATM v0.4 + Depth De-block OFF)

|  | Texture Coding |  | Depth Coding |  | Total (Coded PSNR) |  | Total (Synthesed PSNR) |  | Complexity estimate (ratio to anchor) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | dBR, % | dPSNR,dB | dBR, % | dPSNR,dB | dBR, % | dPSNR,dB | dBR, % | dPSNR,dB | Encoder Time, % | Decoder Time, % | Rendering Time, % |
| S01 | -0.54 | 0.02 | -5.56 | 0.34 | -0.72 | 0.03 | -0.78 | 0.03 | 103% | 103% | 100.00 |
| S02 | 0.00 | 0.00 | -7.88 | 0.39 | -0.17 | 0.00 | -0.54 | 0.02 | 106% | 103% | 100.00 |
| S03 | -0.60 | 0.02 | -6.26 | 0.52 | -0.76 | 0.03 | -0.36 | 0.01 | 106% | 101% | 100.00 |
| S04 | -0.14 | 0.00 | -6.72 | 0.39 | -0.22 | 0.01 | -0.62 | 0.02 | 103% | 101% | 100.00 |
| S05 | -0.28 | 0.01 | -6.44 | 0.32 | -0.42 | 0.02 | -0.95 | 0.04 | 99% | 99% | 100.00 |
| S06 | -0.33 | 0.02 | -7.07 | 0.35 | -0.50 | 0.02 | -0.93 | 0.04 | 100% | 102% | 100.00 |
| S08 | -0.06 | 0.00 | -7.48 | 0.34 | -0.27 | 0.01 | -0.89 | 0.03 | 96% | 101% | 100.00 |
| Average | -0.28 | 0.01 | -6.77 | 0.38 | -0.44 | 0.02 | -0.72 | 0.03 | 102% | 102% | 100.00 |

# Reference

[1]	I. Lim, H. Wey, and J. Lee, “3D-CE4.a results on region based adaptive loop filter by Samsung”, The 100th MPEG meeting, M24822, Geneva, Switzerland, May 2012.

[2]	Ilsoon Lim, Ho-Cheon Wey, Jaejoon Lee, “3D-CE3.a results on region based adaptive loop filter”, JCT2-A0037, Stockholm, July 2012

[3]	H. Schwarz, and D. Rusanovskyy, “Common Test Conditions for 3DV experimentation”, The 100th MPEG meeting, W12745, Geneva, Switzerland, May 2012.