| Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 1st Meeting: Stockholm, SE, 16–20 July 2012 | Document: JCT2-A0152 |
| --- | --- |

| Title: | 3D-CE1.a : Cross check on NTT's adaptive illumination compensation (JCT2-A0022) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | A. Fartukov I. Kovliga M. Mishurovskiy K. Dubkov Jaejoon Lee | Tel: Email: | +7-495-797-25-00 A.fartukov@samsung.com i.kovliga@samsung.com mike.mishky@samsung.com k.dubkov@samsung.com jaejoon1.lee@samsung.com |
| Source: | Samsung Moscow Research Center & Samsung Electronics Co. Ltd. |  |  |

_____________________________

# Abstract

This contribution summarizes cross check results on NTT’s adaptive illumination compensation proposal (JCT2-A0022). Performance evaluation results reported by proponent are confirmed. The simulation results (BD Rates and PSNR’s) received by Samsung - Moscow are matched perfectly with the results provided by NTT.

# Test results

Software provided by the proponent has been used for cross check. Only one configuration has been simulated: adaptive illumination compensation is activated for synthesized reference pictures only; in accordance with NTT description, «compensation model is linear function like the weighted prediction». Simulation results for this configuration are shown in the Table 1.

Table 1. Test results for NTT’s adaptive illumination compensation.

| Sequences | dBR%, PSNR of decoded views, total rate (depths + views) | dBR%, PSNR of synthesized views, total rate (depths + views) |
| --- | --- | --- |
| S01 (Poznan Hall2) | -0.23 | -0.22 |
| S02 (Poznan Street) | -0.08 | -0.04 |
| S03 (Undo Dancer) | 0.01 | 0.02 |
| S04 (GT Fly) | 0.10 | -0.10 |
| S05 (Kendo) | -0.74 | -0.61 |
| S06 (Balloons) | -0.32 | -0.22 |
| S08 (Newspaper) | -0.16 | -0.09 |
| Average | -0.20 | -0.18 |
| Encoding Time (%) | 117.07 |  |
| Decoding Time (%) | 150.67 |  |

# Summary

Cross check of NTT’s adaptive illumination compensation was done by running NTT software. Simulation results including BD-Rates and complexity are matched perfectly with the results provided by NTT.