| Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 1st Meeting: Stockholm, SE, 16–20 July 2012 | Document: JCT2-A0153 |
| --- | --- |

| Title: | 3D-AVC HLS: a show case of m25256 (view synthesis adaptation to display parameters) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yin Zhao and Lu Yu #38 Zheda Road Zhejiang University Hangzhou, China | Tel: Email: | +86-571-87953107 {zhaoyin,yul}@zju.edu.cn |
| Source: | Zhejiang University |  |  |

_____________________________

# Abstract

This document describes the design of a show case for demonstrating the benefits from the SEI messages proposed in m25256.

# Introduction to the show case

In m25256 from Ericsson and Zhejiang Univ., a minimum set of parameters necessary for adjusting view synthesis algorithm to the decoder-side display parameters are proposed as new SEI messages. The SEI includes the reference display width, reference viewing distance, reference baseline and reference horizontal shift value, which helps a decoder-side display to generate a stereo pair that presents the same (or similar) depth sensation as that recommended by content producers (which is signaled in the SEI). The decoder-side adaptation operation is scaling the baseline and horizontal shift values by a factor related to the ratio between “display width” of the reference display and that of the decoder-side display. Note that the “display width” refers to the “the width of the visible part of the display used for showing the video sequence” as mentioned in m25256.

This proposal introduces a show case that demonstrates the benefits from signaling these four parameters. On one stereoscopic display, there are three stereo pairs, as shown in Fig. 1. Stereo pair 1 (P1 in short) is of width W and baseline X, and stereo pairs 2 and 3 (P2 and P3) are of width W/2.

P1 consists of two captured view (view 1 and 3 of Balloons), and is supposed to be shown on a reference display with comfortable 3D sensation after applying horizontal shift of Y pixels. Without the horizontal shift, all objects are visually in front of the screen; the shift operation pushes the scene backwards and all objects appear on or behind the screen, which makes P1 visually more comfortable.

P2 and P3 are half the width and height of P1. P2 is used to simulate when P1 is directly shown on a smaller display of half width of the reference display if the SEI is absent. P3 is used to show when P1 is adapted to the new display by using the SEI. According to m25256 and m24893, the reference baseline and shift parameters should be scaled by a same factor of 2 in this case to obtain the new baseline and shift values:

baseline = ref_baseline * (ref_display_width/display_width) = ref_baseline * 2

shift = ref_shift * (ref_display_width/display_width) = ref_shift * 2

Then, P3 consists of the left view of P1 and a synthesized view, and the two views have a baseline of 2X and are shifted by 2Y pixels.

Viewing P3 in distance V/2 is supposed to give the same depth sensation as when viewing P1 in the reference viewing distance V. In other words, the viewing distances are both N times of the image heights. However, P2 (which is without the adaptation) presents different 3D sensation from P1 and P3. The 3D feeling is confirmed on a ViewSonic VX2268wm stereoscopic display (1680*1050, 120Hz, 22 inch) stereoscopic display with shutter glasses. The view synthesis is done by a modified VSRS (which also performs the horizontal shift operation).

Fig. 1. Three stereo pairs on one stereoscopic display (only the left view is shown), simulating two cases: 1) a stereo pair P1 suitable for a reference display at viewing distance V is shown on a half-width display directly, and 2) P1 is adapted to the half-width display by utilizing the SEI messages and generating a new stereo pair P3 with double baseline and shift values. Viewing P3 in distance V/2 gives the same depth sensation as when viewing P1 in distance V (i.e., viewing distances are both N times of image height).

# Conclusion

We have designed a show case that clearly demonstrates the benefits from display-based stereo pair adaptation with the SEI messages proposed in m25256, and thus recommend adopting m25256 after this show case.

The software (in the attachment) and generated stereo pairs will be taken to the meeting for further evaluation and discussion.

# Patent rights declaration(s)

Zhejiang University may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).