| Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 1st Meeting: Stockholm, SE, 16–20 July 2012 | Document: JCT2-A0157 |
| --- | --- |

| Title: | 3D-CE5.h related: Cross check report on Improved merge mode for inter-view predicted motion of Qualcomm |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Cross check |  |  |
| Author(s) or Contact(s): | Jaewon Sung, Sehoon Yea 221, Yangjae-dong, Seocho-gu, Seoul 137-130, Korea | Tel: Email: | {jw.sung, sehoon.yea}.lge.com |
| Source: | LG Electronics |  |  |

_____________________________

# Abstract

This contribution reports the cross check results for “3D-CE5.h related: Improved merge mode for inter-view predicted motion” (JCT2-A0096, m26021). Qualcomm implemented the improved merge candidate pruning algorithm on top of HTM3.1rc1 and the implementation was consistent with the description in A0096. Experiment results matched with the data provided by Qualcomm.

# Introduction

This contribution reports the verification results of A0096, which is a related contribution to 3D-CE5.h. The proposed algorithm added merge candidate pruning step to remove remaining redundancy in HTM3.1 between inter-view predicted candidate and spatial predicted merge candidate.

Provided software package is based on bug fixed HTM3.1rc1. On top of this, the proposed merge candidate pruning algorithm is implemented. As this software package is different from HTM3.1rc1, new anchor data was generated by turning off two MACROs, QC_MULTI_DIS_CAN and MRG_PRUNING, which were added by Qualcomm. To generate test data, only MRG_PRUNING was turned on and QC_MULTI_DIS_CAN was off. This combination measures the impact of proposed merge pruning algorithm when predicted depth map was used for disparity vector generation.

# Experiment results

Common test condition for coding of texture and depth was used for this experiment. A cluster system with 64-bit linux OS was for this experiment. The bitrates and PSNR values exactly matched with the simulation data provided by Qualcomm. Table 1 shows the coding performance.

Table 1 Coding performance of the A0096.

|  | video 0 | video 1 | video 2 | video only | synthesized only | coded & synthesized | enc time | dec time | ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.0% | -0.3% | -0.2% | -0.2% | -0.1% | -0.1% | 99.7% | 100.5% | 104.5% |
| Kendo | 0.0% | -0.3% | -0.5% | -0.2% | -0.2% | -0.2% | 99.8% | 102.7% | 101.2% |
| Newspapercc | 0.0% | -0.3% | -0.2% | -0.1% | 0.0% | 0.0% | 97.6% | 101.9% | 97.2% |
| GhostTownFly | 0.0% | -0.1% | 0.0% | -0.1% | 0.0% | 0.0% | 100.0% | 99.4% | 89.7% |
| PoznanHall2 | 0.0% | -0.4% | -0.9% | -0.4% | -0.3% | -0.3% | 99.8% | 98.3% | 98.5% |
| PoznanStreet | 0.0% | -0.1% | -0.2% | -0.1% | 0.0% | -0.1% | 99.8% | 102.3% | 99.2% |
| UndoDancer | 0.0% | -0.2% | -0.3% | -0.1% | -0.1% | -0.1% | 100.0% | 102.9% | 96.2% |
| 1024x768 | 0.0% | -0.3% | -0.3% | -0.2% | -0.1% | -0.1% | 99.1% | 101.7% | 100.9% |
| 1920x1088 | 0.0% | -0.2% | -0.4% | -0.2% | -0.1% | -0.1% | 99.9% | 100.7% | 95.8% |
| average | 0.0% | -0.3% | -0.3% | -0.2% | -0.1% | -0.1% | 99.5% | 101.2% | 98.0% |