| Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 1st Meeting: Stockholm, SE, 16–20 July 2012 | Document: JCT2-A0158 |
| --- | --- |

| Title: | JCT2-A0158 3D-CE1.a results on joint proposal of Samsung and NTT |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Seok Lee, Shinya Shimizu, Seungsin Lee, Shiori Sugimoto, Hocheon Wey, Hideaki Kimata, Jinyoung Lee, Jaejoon Lee | Tel: Email: | +82-31-280-6696 lee.seok@samsung.com shimizu.shinya@lab.ntt.co.jp |
| Source: | Samsung Electronics Co. Ltd., NTT Corporation |  |  |

_____________________________

# Abstract

This report is about results on joint proposal of Samsung and NTT. Samsung proposes a dilation based view synthesis prediction method without upsampling in JCT2-A0036 and NTT proposes sub-pel VSP simplification method in JCT2-A0023.

# Introduction

Samsung’s proposal is described in JCT2-A0036. It improves the coding performance by applying dilation to low resolution depth data before using in warping process.

NTT’s proposal is described in JCT2-A0023. It improves the coding performance by simplifying the sub-pel VSP. In the proposed method, the processing direction of the pixel-wise warping is inverted. Besides, the hole-filling and inpainting is also simplified.

These proposals are combined for the improvement of the view synthesis prediction in terms of the coding performance and the complexity.

# Test results

Test results are summarized in Table 2.1. These tables show BD-rate gain in terms of PSNR of decoded views over texture + depth bit rate and BD-rate gain in terms of PSNR of synthesized views over texture + depth bit rate, respectively when Samsung’s dilation based VSP and NTT’s sub-pel VSP simplification are enabled. The ATM v0.4 result with CTC is used as anchor.

Table 2.1 Test results for proposed dilation based view synthesis prediction

| Sequences | BD-rate with PSNR of decoded views over texture + depth bit rate | BD-rate with PSNR of synthesized views over texture + depth bit rate |
| --- | --- | --- |
| Poznan Hall2 | -0.61 | -0.61 |
| Poznan Street | -0.49 | -0.49 |
| Undo Dancer | -0.97 | -1.00 |
| GT Fly | -0.74 | -0.77 |
| Kendo | -0.53 | -0.60 |
| Balloons | -0.74 | -0.80 |
| Newspaper | -0.29 | -0.31 |
| Average | -0.62 | -0.66 |
| Encoding Time (%) | 104.26% |  |
| Decoding Time (%) | 99.79% |  |

# Summary

Joint proposal of Samsung and NTT was integrated on 3D ATM 0.4 and performance evaluation was shown. Proposed method achieves -0.62% BD-rate gain in terms of decoded texture PSNR, and -0.66% BD-rate gain in terms of synthesized PSNR. The decoding time is 99.79% compared to anchor.

# Patent rights declaration(s)

Samsung Electronics Co. Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

NTT Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).