| Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 1st Meeting: Stockholm, SE, 16–20 July 2012 | Document: JCT2-A0163_r1 |
| --- | --- |

| Title: | Show-case and syntax for SEI message on reference display information signaling |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Andrey Norkin Ivana Girdziajuskas Yin Zhao Yu Lu Färögatan 6 164 80 Stockholm Sweden | Tel: Email: | +46 10 714 84 02 andrey.norkin@ericsson.com yul@zju.edu.cn |
| Source: | Ericsson, Zhejiang University |  |  |

_____________________________

# Abstract

This contribution contains a description of a demo and an a semantics for SEI on reference display information parameters which was conditionally adopted at Geneva meeting 2012 subject to a show-case and syntax and semantics clean-up at the following Stockholm meeting.

# Introduction

During the MPEG meeting in Geneva, proposal M25256 [1] was conditionally adopted subject to a demonstration of how the SEI on display information parameters can be used. This document describes the proposed demonstration and contains updated syntax and updated semantics for adopted SEI message. M25256 was a joint proposal from Ericsson and Zhejiang university that was a clean combination of the main elements from [4] and [5] necessary adaptation to the screen size. The main idea of the M25256 is to send the receiver information necessary for choosing the baseline and other parameters such as a horizontal shift necessary for choosing such reference parameters so that the same scene geometry can be reproduced on the display that the renderer generates the views for as on the reference display used by the content provider.

The idea that the screen adaptation is based on is summarized in the  following. Let us first consider the case when the screen width and the viewing distance are scaled proportionally. Suppose that it is required to scale a screen width (W) with a scaling factor b. The viewing distance then also changes with the same factor b. This assumption is supported by an observation that the best viewing distance is usually set as the screen height times the constant multiplication factor (e.g. 3 in case of HD resolution). This constant factor can, however, be different for different classes of resolution (e.g. HD and SD. In this case the perceived depth should be adjusted relative to the screen width (size) in order to avoid changing the ratio between the spatial and the depth dimension in the scene.

Let the original parameter be Wd1 (display/screen width), zd1 (perceived depth relative to the display surface), D1 (viewing distance), and the new parameters be Wd2 (new screen width), zd2 (new perceived depth), d2 (new viewing distance). One should also note that by the screen size we mean the ”active part” of the screen, i.e. part of the screen that is used for displaying the 3D video sequence.

Our goal is to change the perceived depth when the screen size and the viewing distance change proportionally. Figure 1 illustrates the relationship between the perceived depth, the screen parallax (the distance between the points in the left and the right view that correspond to the same point in a 3D space), viewing distance and the inter-ocular distance (distance between the human eyes). One can see that when the viewing distance changes, the perceived depth of the point changes by the same factor as the viewing distance in case the screen parallax does not change. However, when the screen size changes, the screen parallax remains unchanged if the disparity (in the number of pixels) changes reciprocally to the scaling factor of the screen width. For example, if the screen size (width) and the viewing distance are scaled by a factor of two, showing the same video stereo-pair on a larger screen size would result in all sizes of the horizontal sizes of objects in the scene being increased by a factor of two. Therefore, the physical screen parallax for the same object would also increase by a factor of two. If we want to keep the physical screen parallax the same, the parallax in terms of the number of pixels should be decreased by a factor of two.

screen

distance

between observer’s eyes

p

D1

D2

Zd2

Zd1

Figure 1. Relationship between the screen width, viewing distance and the perceived depth.

In general,  the screen parallax (in units of pixels) should change with a factor inversely proportional to the screen width in order to keep the perceived depth constant.

It can be also shown the disparity d (in units of pixels) of  a point in 3D space can be found according to the following formula:

d=tc⋅f⋅(1zconv−1z)                                                           (1)

where tc is the baseline distance, i.e. the distance between the two cameras that form a stereo-pair, f is the focal length, zconv is the z coordinate of the convergence point (plane) and z is the depth coordinate of the point in 3D space. Under the assumption that the depth from the camera and the convergence plane are constant, it follows the disparity (or parallax in unit of pixels) is proportional to the baseline distance.

In the formula above, the convergence plane distance zconv should stay the same in order for the relation between the parallax and the baseline scaling to hold. For that, the displacement of the principal point coordinate (or a sensor shift) should be adjusted. In the parallel camera setup this is typically done when changing the baseline distance. Therefore, changing the baseline distance between the cameras assumes also adjusting the sensor shift in order for the convergence plane to stay at the same distance which would ensure the same distribution of depth in the virtual space in front and behind the screen.

The same observation can be derived from Figure 2. One can see that when changing the baseline distance from tc1 to tc2, the disparity d related to point O (with reference to camera C0) changes from d1 to d2 with the ratio d1/d2 equal to tc1/tc2. Therefore, the screen parallax at the same screen width should also change with the same ratio as the baseline: p1/p2 = tc1/tc2.

C2

C1

C0

d2

d1

tc1

tc2

O

Figure 2. Dependency between change of camera baseline distance and change of disparity. C0, C1, and C2 are camera centers. tc1 and tc2 are baseline distances for camera 1 and camera 2 respectively. d1 and d2 are parallax values for point O in camera 1 and camera 2 respectively (relative to camera 0).

Therefore, the baseline distance should be adjusted reciprocally to the change in screen width relative to the reference screen width (provided that the viewing distance changes proportionally).

This contribution therefore proposes indicating the reference baseline (tcref), and signaling reference screen width (Wdref) and the reference viewing distance to the decoder. This would enable the view synthesis algorithm at the receiver to adjust the baseline (tc) for the chosen screen width (Wd) according to the following formula:

tc=tcref⋅WdrefWd                                                             (2)

Also, in order to provide the correct scene position relative to the screen size, an additional shift for the reference screen may be signaled in case this shift was not signaled as a difference of principal point coordinates of two views. The additional horizontal shift, if present, and the sensor shift should also be scaled with the same scaling factor as the baseline distance in order to provide the same scene position relative to the screen size.

# Demonstration description

To demonstrate the importance of signaling the reference values for the screen width and the viewing distance, a following demonstration was prepared. The demonstration is intended to show that the viewing experience can be significantly affected if the stereo content is used at a different display size and the viewing distance that it was optimized for. Here we give the details of the demonstration setup.

The demonstration is done by using two stereo displays with passive glasses: Hyundai 46”  (model: S465D), and the Hyundai 24” (model: W240S), which have a screen size ratio of approximately 2. The recommended viewing distances for the first and second display are approximately 2.3m and 1.2m respectively. The derivations obtained from (2) suggest that watching stereo content on a larger display with a baseline distance BL and from 2.3m is equivalent, in terms of relative depth perception, to watching the this content on the small display and from 1.2m, however with a larger baseline distance equal to 2BL. Showing the content optimized for the 46” display on the 24” display results in the scene looking more flat.  Namely, the depth range shrinks if the viewer observes the content on a smaller display than the one the content is optimized for. The opposite happens if the viewer observes the content optimized for the 24” display on the 46” display. Namely, the depth range gets extended, which leads to two possible problems. One problem may arise for the close objects since they appear to be even closer now. This may create a serious discomfort as the viewers' eyes are forced to be crossed when observing these objects. Another type of problem can arise for distant objects, as they may appear to be beyond infinity if observed on a bigger display, which causes discomfort as our eyes diverge. A position of the scene relative to the screen surface is also important since the in case the object touches the screen border, a so-called framing effect may appear so that the object look like if cut by the screen frame.

The above-mentioned problems are illustrated in Figure 1, where ZD denotes the viewing distance. Figure 3 (a) shows some points in a scene, A-G, placed at different depths. Figure 3(b) and Figure 3 (c) show the depths of the same points where the same content is shown on displays that have half and double of the original size, respectively.

- (b)                                                     (c)

Figure 3 Dependency of perceived depth on display size

In the demo we plan to show the above-mentioned problems that appear if the content is not optimized for specific viewing conditions. Namely, we both show the effects of displaying the content optimized for a 46” display on a 24” display, as well as the opposite case where the content is optimized for a 24” display but shown on a 46” display. We use the standard MPEG stereo sequences, where the baselines and the shifts between the views correspond to the ones specified in the common test conditions document [2] in case of 46'' display and are increased twice when intended to be shown on a 24'' display.

# Syntax and semantics

## 3D_reference_displays_info SEI message

It is proposed to add an SEI message 3d_reference_displays_info( ). This message contains information about the reference display sizes and reference viewing distances as well as the information about the reference baselines and additional sensor shift if required. The reference display width and viewing distance values are signaled in centimeters. The reference baseline values are given in the same units as the x component of the translation vector in the SEI depth acquisition message. The SEI message 3d_reference_displays_info() is aligned with the IDR access unit and the depth_acquisition_info() SEI message sent for the same access unit. In the following, syntax elements marked with yellow color are taken from proposal M24975 and those from M24893 are marked with green color.

3.2.1.  3DVC reference displays information SEI message syntax

| 3d_reference_displays_info( payloadSize ) { | C | Descriptor |
| --- | --- | --- |
| prec_ref_baseline | 5 | ue(v) |
| prec_ref_display_width | 5 | ue(v) |
| ref_viewing_distance_flag | 5 | u(1) |
| if( ref_viewing_distance_flag ) |  |  |
| prec_ref_viewing_dist | 5 | ue(v) |
| num_ref_displays_minus1 | 5 | ue(v) |
| numRefDisplays = num_ref_displays_minus1 + 1 |  |  |
| for( i = 0; i < numRefDisplays; i++ ) { |  |  |
| exponent_ref_baseline[ i ] | 5 | u(6) |
| mantissa_ref_baseline[ i ] | 5 | u(v) |
| exponent_ref_display_width[ i ] | 5 | u(6) |
| mantissa_ref_display_width[ i ] | 5 | u(v) |
| if(ref_viewing_distance_flag) { |  |  |
| exponent_ref_viewing_distance[ i ] | 5 | u(6) |
| mantissa_ref_viewing_distance[ i ] | 5 | u(v) |
| } |  |  |
| additional_shift_present_flag[ i ] | 5 | u(1) |
| if (additional_shift_present[ i ]) |  |  |
| num_pixel_shift_plus128[ i ] | 5 | u(8) |
| } |  |  |
| } |  |  |

- 3.2.2 3DVC reference displays information SEI message semantics

When present, this SEI message shall be associated with an IDR access unit. A reference displays information message contains information about the reference display size(s) and reference viewing distance(s) as well as information about the corresponding baseline distances and additional horizontal image shift. This information provides a view renderer with information that produces a proper stereo-pair for indicated screen width and the viewing distance.  The reference display width and viewing distance values are signaled in units of centimeters. The reference baseline values shall be signaled in the same units as the x component of the translation vector in the SEI message depth acquisition information that is valid for the same access unit. When a reference displays information SEI message is present in an access unit, the depth acquisition information SEI message shall also be present in the same access unit. The baseline and shift information signaled for the reference display is valid for all access unit they associated with and until the next IDR access unit or the next access unit containing depth acquisition information SEI message or reference displays information SEI message.

NOTE 1 – The reference displays information SEI message specifies display parameters for which the 3D sequence was optimized and the corresponding reference parameters. When camera parameters are updated by a depth acquisition information SEI message in the following frames, the baseline distance for the receiver's display should be modified in order to have the same ratio between the receiver's display baseline and the baseline between the views used in view synthesis process as in the access unit, which the reference displays information SEI belongs to.

The following formula can be used for calculating the baseline distance for the receiver's display when the ratio between the receiver's viewing distance and the reference viewing distance is the same as the ratio between the receiver screen width and the reference screen width:

baseline = ref_baseline*(ref_display_width÷display_width)

In the provided formula, the width of the visible part of the display used for showing the video sequence should be understood under "display width". The same formula can also be used for choosing the baseline distance in cases when the viewing distance is not scaled proportionally to the screen width compared to the reference display parameters. In this case, the effect of applying this formula would be to keep the perceived depth in the same proportion to the viewing distance as in the reference setup.

prec_ref_baseline specifies the exponent of the maximum allowable truncation error for ref_baseline[ i ] as given by 2−prec_ref_baseline. The value of prec_ref_baseline shall be in the range of 0 to 31, inclusive.

prec_ref_display_width specifies the exponent of the maximum allowable truncation error for ref_display_width[ i ] as given by 2−prec_ref_display_width. The value of prec_ref_display_width shall be in the range of 0 to 31, inclusive.

ref_viewing_distance_flag equal to 1 indicates the presence of reference viewing distance. ref_viewing_distance_flag equal to 0 indicates that the reference viewing distance is not present in the bitstream.

prec_ref_viewing_dist specifies the exponent of the maximum allowable truncation error for ref_viewing_dist[ i ] as given by 2−prec_ref_viewing_dist. The value of prec_ref_viewing_dist shall be in the range of 0 to 31, inclusive.

num_ref_displays_minus1 plus 1 specifies the number of reference displays sizes that are signaled in the bitstream. The value of num_ref_displays_minus1 shall be in the range of 0 to 31, inclusive.

exponent_ref_baseline[ i ] specifies the exponent part of the reference baseline of the i-th view. The value of exponent_ref_baseline[ i ] shall be in the range of 0 to 62, inclusive. The value 63 is reserved for future use by ITU-T | ISO/IEC. Decoders shall treat the value 63 as indicating an unspecified reference baseline.

mantissa_ref_baseline[ i ] specifies the mantissa part of the reference baseline of the i-th view. The length of the mantissa_ref_baseline[ i ] syntax element is variable and determined as follows.

–	If exponent_ref_baseline[ i ] = = 0, the length is Max( 0, prec_ref_baseline − 30 ).

–	Otherwise ( 0 < exponent_ref_baseline[ i ] < 63), the length is Max( 0, exponent_ref_baseline[ i ] + prec_ref_baseline − 31 ).

exponent_ref_display_width[ i ] specifies the exponent part of the reference display width of the i-th view. The value of exponent_ref_display_width[ i ] shall be in the range of 0 to 62, inclusive. The value 63 is reserved for future use by ITU-T | ISO/IEC. Decoders shall treat the value 63 as indicating an unspecified reference display width.

mantissa_ref_display_width[ i ] specifies the mantissa part of the reference display width of the i-th view. The length of the mantissa_ref_display_width[ i ] syntax element is variable and determined as follows.

–	If exponent_ref_display_width[ i ] = = 0, the length is Max( 0, prec_ref_display_width − 30 ).

–	Otherwise ( 0 < exponent_ref_display_width[ i ] < 63 ), the length is Max( 0, exponent_ref_display_width[ i ] + prec_ref_display_width − 31 ).

exponent_ref_viewing_distance[ i ] specifies the exponent part of the reference viewing distance of the i-th view. The value of exponent_ref_viewing_distance[ i ] shall be in the range of 0 to 62, inclusive. The value 63 is reserved for future use by ITU-T | ISO/IEC. Decoders shall treat the value 63 as indicating an unspecified reference display width.

mantissa_ref_viewing_distance[ i ] specifies the mantissa part of the reference viewing distance of the i-th view. The length of the mantissa_ref_viewing_distance[ i ] syntax element is variable and determined as follows.

–	If exponent_ref_viewing_distance[ i ]  = = 0, the length is Max( 0, prec_ref_viewing_distance − 30 ).

–	Otherwise ( 0 < exponent_ref_viewing_distance[ i ] < 63 ), the length is Max( 0, exponent_ref_viewing_distance[ i ]  + prec_ref_viewing_distance − 31 ).

The association between the reference display and baseline variables and corresponding syntax elements is specified by Table 1. Each parameter is obtained from the variables specified in Table 1 as the variable x computed as follows.

–	If 0 < e < 63, x = (−1)s * 2e−31 * (1 + n  2v).

–	Otherwise (e is equal to 0), x = (−1)s * 2−(30+v) * n.

NOTE – The above specification is similar to that found in IEC 60559:1989, Binary floating-point arithmetic for microprocessor systems.

Table 1– Association between camera parameter variables and syntax elements.

| x | s | e | n |
| --- | --- | --- | --- |
| refBaseline[ i ] | 0 | exponent_ref_baseline[ i ] | mantissa_ref_baseline[ i ] |
| refDisplayWidth[ i ] | 0 | exponent_ref_display_width[ i ] | mantissa_ref_display_width [ i ] |
| refViewingDistance[ i ] | 0 | exponent_ref_viewing_distance[ i ] | mantissa_ref_viewing_distance[ i ] |

additional_shift_present_flag[ i ] equal to 1 indicates that the information about additional horizontal shift of the left and right views for the reference display i is present in the bitstream. additional_shift_present_flag[ i ] equal to 0 indicates that the information about additional horizontal shift of the left and right views for the reference display i is not present in the bitstream.

num_pixel_shift_plus_128[ i ] indicates the recommended horizontal shift for the stereo-pair i that corresponds to the recommended baseline for the reference display i. If ( num_pixel_shift_plus_128 – 128 ) is less than 0, it is recommended that the left view in the stereo-pair i is shifted in to the left direction by ( 128 – num_pixel_shift_plus_128 ) pixels with respect to the right view of the stereo pair i; if num_pixel_shift_plus_128 is equal to 128, no shifting is applied; if ( num_pixel_shift_plus_128 – 128 ) is larger than 0, it is recommended that the left view in the stereo pair i is shifted in the right direction by ( 128 – num_pixel_shift_plus_128 ) pixels with respect to the right view of the stereo pair i. The value of num_pixel_shift_plus_128 takes values is in the range of 0 to 255, inclusive.

Note: shifting the left view in the left -hand (or right-hand) direction for X pixels with respect to the right view can be performed by the following two-step processing:

- shift the left-view by X/2 pixels in the left-hand (or right hand) direction, and shift the right-view by X/2 pixels in the right-hand (or left-hand) direction;

- fill the left and right image margins of X/2 pixels in width in both the left and right views in background color.

The following pseudo code explains the recommended shifting processing in the case of shifting the left view in the left-hand direction by X pixels with respect to the right view.

for ( i = X/2; i < width – X/2; i++ ){

for  ( j=0;  j < height; j++ ){

left_view[ j ][ i ] = left_view[ j ][ i + X/2 ];

right_view[ j ][ width – 1 – i ] = right_view[ j ][ width – 1 – i– X/2 ];

}

}

for ( i = 0; i < X/2; i++) {

for ( j = 0; j < height; j++ ){

left_view[ j ][ width – 1 – i ] = left_view[ j ][ i ] = BACKGROUND_COLOR;

right_view[ j ][ width – 1 – i ] = right_view[ j ][ i ] = BACKGROUND_COLOR;

}

}

The following pseudo code explains the recommended shifting processing in the case of shifting the left view in the right-hand direction by X pixels with respect to the right view.

for ( i = X/2; i < width – X/2; i++ ){

for ( j = 0; j < height; j++ ){

left_view[ j ][ width – 1 – i ] = left_view[ j ][ width – 1 – i – X/2 ];

right_view[ j ][ i ] = right_view[ j ][ i + X/2 ];

}

}

for ( i=0; i < X/2; i++ ){

for ( j = 0; j < height; j++ ){

left_view[ j ][ width – 1– i ] = left_view[ j ][ i ] = BACKGROUND_COLOR;

right_view[ j ][ width – 1– i ] = right_view[ j ][ i ] = BACKGROUND_COLOR;

}

}

BACKGROUND_COLOR may take different values in different systems, for example black or gray.

# References

- Norkin, Y. Zhao, I. Girdzijauskas, L. Yu," View synthesis adaptation to display parameters", ISO/IEC JTC1/SC29/WG11 Document M25256, May 2012.

- T. Suzuki, M. M. Hannuksela, Y. Chen, ”WD on MVC extensions for inclusion of depth  maps”, ISO/IEC/JTC1/SC29/WG11/N12544, Feb. 2012.

- H. Schwarz, D. Rusanovskyy, “Common test conditions for 3DV experimentation”, ISO/IEC JTC1/SC29/WG11 MPEG 2011/N12745, May 2012, Geneva, Switzerland.

- Y. Zhao and L. Yu, “3D-AVC HLS: recommended stereo pair information signalling”, SO/IEC JTC1/SC29/WG11 M24893, May 2012.

- A. Norkin and I. Girdzijauskas, “View synthesis adaptation to display parameters”, ISO/IEC/JTC1/SC29/WG11/M24975, May 2012.

# Patent rights declaration(s)

Ericsson may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).