| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 10th Meeting: Stockholm, Sweden, 11-20 July 2012 | Document: JCT2-A0164 |
| --- | --- |

| Title: | 3DV: Cross check report of ITRI’s JMVC software integration of coding interlaced texture with interlaced depth (JCT2-A0144) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Chao-Hsiung Hung Hsueh-Ming Hang Department of Electronics Engineering National Chiao-Tung University 1001, Ta-Hsueh Road Hsinchu, Taiwan 30050, R.O.C. | Tel: Email: | +886-3-5712121-54228 +886-3-5731861 morning.ee94g@nctu.edu.tw hmhang@mail.nctu.edu.tw |
| Source: | National Chiao-Tung University |  |  |

_____________________________

# Abstract

This contribution provides cross-check results of “3DV technical input: JMVC software integration of coding interlace texture with interlace depth” (JCT2-A0144). The verification work was performed using the software provided by the proponent. Our simulation results match those reported in JCT2-A0144 well.

# Cross Check Procedure

The purpose of the cross-check is to examine the coded bitstreams produced by the proponent (ITRI) software and the PSNR values of the reconstructed images. The ITRI proposal is described in [1]. The source code of test software was provided by ITRI. We compile the code using GCC 4.3.3 (Ubuntu 4.3.3-5ubuntu4). A PC cluster consisting of 17 Personal Computers is used as the test platform. We examine 7 test sequences: Balloons, GT_Fly, Newspaper1, Kendo, Poznan_Hall2, Poznan_Street, and Undo_Dancer.

The encoded bit streams are in the files with naming “xxxxxx.264”. The files with the name “enc_xxxxxx_xxx.log” include PSNR of every decoded frame. We run the ITRI-provided programs and produce the result files. Then, we compare the generated files with the corresponding files provided by ITRI, bit-by-bit. They are matched if all bits are identical. We show the comparison results in the next section

# Results

TABLE I to  show the comparison results we checked so far. The data provided by ITRI and those we simulated are all very well matched. There is only one frame PSNR value of Poznan_Hall2 is slightly different but the difference is very small (0.005 PSNR).

ITRI’s results in enc_hall2_T7.log

44 B               86  QP 29  bits   12376

44 B               87  QP 29  Y 41.4256 dB  U 46.5191 dB  V 46.5372 dB   bits   11688

NCTU’s results in enc_hall2_T7.log

44 B               86  QP 29  bits   12376

44 B               87  QP 29  Y 41.4261 dB  U 46.5170 dB  V 46.5373 dB   bits   11712

TABLE I Comparison Results of Balloons

TABLE II Comparison Results of GTFly

TABLE III Comparison Results of Newspaper1

TABLE IV Comparison Results of Kendo

TABLE V Comparison Results of Poznan_Hall2

TABLE VI Comparison Results of Poznan_Street2

TABLE VII Comparison Results of Undo_Dancer

# Conclusions

This contribution reports the cross-verification results for JCT2-A0144 on interlace texture with interlace depth coding. It has been confirmed that all the cross-check results match very well the results from the proponent. There are 3 cases that not all bits are identical but only one case has a slightly PSNR difference as reported in the last section.

# Reference

[1] C.-C. Lin, F.-C. Chen, W.-H. Chung, “3DV technical input: JMVC software integration of coding interlace texture with interlace depth,” JCT2-A0144, 10th Meeting: Stockholm, Sweden, 11-20 July 2012.