| Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 1st Meeting: Stockholm, SE, 16–20 July 2012 | Document: JCT2-A0166r1 |
| --- | --- |

| Title: | 3DV-ATM High-Level Syntax: Loss Detection of Camera Parameter Sets |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Miska M. Hannuksela Nokia Research Center Visiokatu 3, 33720Tampere, Finland | Tel: Email: | +358 7180 08000 miska.hannuksela@nokia.com |
| Source: | Nokia Corporation |  |  |

_____________________________

# Abstract

Contributions JCT2-A0007, JCT2-A0008, and JCT2-A0053 propose the use of camera parameter sets to convey information for the closest and farthest depth as well as parameters related to view synthesis prediction. While parameter sets enable loss-resilient transmission of parameters, for example through retransmission possibility, it is asserted in this contribution that the mentioned proposals do not propose means to detect a loss of a camera parameter set. More specifically, if the camera_param_set_id value of the lost camera parameter set NAL unit has been used previously by another camera parameter set NAL unit, it is claimed that in such a case the decoder would continue decoding using the previously received camera parameter set NAL unit, which would be likely to result into clearly visible artifacts. It is asserted that this problem is being discussed also in JCT-VC related to adaptation parameter sets, which may be frequently updated similarly to camera parameter sets. This contribution proposes a parameter set loss detection approach based on constraints on camera_param_set_id value assignment.

Revision 1 of the contribution includes proposed specification text (see Section 4 of the contribution).

# Problem Setting

If the camera_param_set_id value of the lost camera parameter set NAL unit has been used previously by another camera parameter set NAL unit, it is claimed that in such a case the decoder would continue decoding using the previously received camera parameter set NAL unit, which would be likely to result into clearly visible artifacts.

There are currently no means to detect a loss of a parameter set when the lost parameter set re-used an parameter set ID value that had already been used earlier by another parameter set. Consequently, any reference to the parameter set ID value used in a lost parameter set NAL unit points to the previous parameter set NAL unit using the same ID value. Therefore wrong syntax element values would be used in the decoding process.

# Earlier Approaches

One option to overcome the loss resilience problem presented in the previous paragraph could be to avoid re-using of ID values in parameter set NAL units. However, this could potentially lead to a need for requiring decoders to support a large value range for ID values and hence increased memory consumption, as decoders must be prepared for storing a parameter set for each value in the allowed parameter set ID value range.

The problem described above was identified in the context of adaptation parameter sets (APS) (JCTVC-I0069). In JCTVC-I0069, we proposed a constraint to the aps_id values that enabled to detect losses of APS NAL units. In the discussion about JCTVC-I0069, it was remarked that the constraint is analogous to the maximum POC difference constraint. In the discussion the following comments were also made:

- It was remarked that such a scheme would have a problem for splicing, as the range in which the APS IDs would fall after the splice point is not under the control of the encoder.

- An alternative loss detection suggestion was to send a parameter set checksum in an SEI message, which reportedly would not have the splicing issue. It was remarked that this checksum idea had actually been previously proposed for AVC, but in that case it was thought that since the parameter sets are relatively small, it did not seem necessary, as they could be repeated. This argument would not be valid for APSs.

# Proposal

In contribution JCTVC-J0072, we re-propose the solution of JCTVC-I0069 with an updated specification text, which is not tied with a partial updating mechanism as in JCTVC-I0069.

If camera parameter sets are adopted in 3DV-ATM, it is proposed that the method JCTVC-J0072 is adopted for camera parameter sets in 3DV-ATM.

For details of the proposed method, we refer to the JCTVC-J0072 document and slides included in the zip file of this contribution.

## Discussion

The issue with splicing has been solved similarly to what is done for POC, i.e. the aps_id constraint is “reset” at IDR and BLA access units.

We think that JCTVC-J0072 has different strengths from those of a parameter set checksum method (see JVT-S037 for a reference). The difference between the approaches can be summarized as follows:

- The parameter set checksum SEI message is an optional mechanism in both encoding and decoding ends, and it might end up not being used often as both ends have to support it. In our opinion, it would be good to have basic loss resilience tools mandatory, similarly as with the maximum POC difference constraint or reference picture sets. We think that the proposed constraints on aps_id usage provide basic loss resilience practically with no implementation or rate-distortion cost.

- On the other hand, the parameter set checksum SEI message has benefits that the proposed aps_id constraints cannot provide, such as:

  - No APS NAL unit loss can be detected with the proposed aps_id constraint before the aps_id of the lost APS NAL unit is referred from a coded slice NAL unit. In the parameter set checksum SEI, also those parameter sets that have already been transmitted but not yet activated can be included. Consequently, retransmission can be requested prior to the activation of adaptation parameter sets, for example.

  - The parameter set checksum SEI message can list only those parameter sets that may be activated in the future and omit other parameter sets. Hence, particularly software decoders could use the parameter set checksum SEI message to free dynamically allocated memory for parameter sets that won’t be activated subsequently in the bitstream.

# Specification text

The proposed specification text changes are indicated with green highlights on top of the proposed specification text of JCT-A0008.

JCT-A0008 proposes activation of depth range parameter sets per texture or depth view, which we consider unnecessary and confusing, since a depth range parameter set documents characteristics for multiple views and across views (such as camera translation). Changes removing per-view activation of depth range parameter set RBSPs are indicated with strikethroughs having green highlights.

Modified sequence parameter set 3DVC extension syntax

| seq_parameter_set_3dvc_extension( ) { | C | Descriptor |
| --- | --- | --- |
| depth_info_present_flag | 0 | u(1) |
| if( depth_info_present_flag ) { |  |  |
| 3dv_acquisition_idc | 0 | ue(v) |
| if( 3dv_acquisition_idc != 1 ) |  |  |
| max_drps_id | 0 | ue(v) |
| max_drps_id_diff | 0 | ue(v) |
| } |  |  |
| if( ( 3dv_acquisition_idc ) && ( num_views_minus1 > 0 ) ) |  |  |
| for( i = 0; i < num_views_minus1; i++ ) |  |  |
| view_id_3dv[ i ] | 0 | ue(v) |
| if( 3dv_acquisition_idc = = 1 ¦ ¦ 3dv_acquisition_idc = = 2 ) { |  |  |
| depth_ranges( 2, SPS ) |  |  |
| if( ( 3dv_acquisition_idc ) && ( num_views_minus1 > 0 ) ) { |  |  |
| depth_ranges( 2, 0 ) |  |  |
| if( profile_idc = = 139 ) |  |  |
| vsp_param( 2, SPS ) |  |  |
| vsp_param( 2, 0 ) |  |  |
| } |  |  |
| } |  |  |
| ...... |  |  |
| } |  |  |

Modified semantics:

3dv_acquisition_idc equal to 0 indicates that no 3DV acquisition parameters (the closest and farthest depth value) are present in the sequence parameter set and at least one 3DV parameter update NAL unit shall be present in each access unit. 3dv_acquisition_idc equal to 1 indicates that 3DV acquisition parameters are present in the sequence parameter set and valid for the entire coded video sequence. 3dv_acquisition_idc equal to 2 indicates that 3DV acquisition parameters are present in the sequence parameter set and depth range parameter update NAL units may be present. 3dv_acquisition_idc values greater than 2 are reserved.

max_drps_id specifies the maximum allowed depth_range_parameter_set_id value.

max_drps_id_diff specifies the value range of depth_range_parameter_set_id values of depth range parameter sets marked as “used”. max_drps_id_diff * 2 shall be less than max_drps_id.

I.7.4.1.2.1  Order of 3DVC sequence parameter set RBSPs and picture parameter set RBSPs and their activation

- NOTE 1 – The sequence, picture and depth range parameter set mechanism decouples the transmission of infrequently changing information from the transmission of coded macroblock data. Sequence, picture and depth range parameter sets may, in some applications, be conveyed "out-of-band" using a reliable transport mechanism.

- A depth range parameter set RBSP includes parameters that can be referred to by the coded slice NAL units of one or more texture view or depth view components of one or more coded pictures.

- Each depth range parameter set RBSP is initially considered not active at the start of the operation of the decoding process and the variable MaxUsedDrpsId is initially set equal to “no value” and the variable UsedDrpsIdSet is initially an empty set of depth_range_parameter_set_id values. At most one depth range parameter set RBSP is considered as the active depth range parameter set RBSP at any given moment during the operation of the decoding process, and when any particular depth range parameter set RBSP becomes the active depth range parameter set RBSP, the previously-active depth range parameter set RBSP (if any) is deactivated.

- When the current access unit is an IDR access unit, MaxUsedDrpsId is set equal to “no value”, UsedDrpsIdSet is an empty set of depth_range_parameter_set_id values, and all depth range parameter set RBSPs included in the bitstream or made available to the decoding process through external means prior to the access unit containing the IDR picture are marked unavailable to the decoding process.

- When MaxUsedDrpsId is not equal to “no value”, the value of depth_range_parameter_set_id of the slice header is constrained and the variable updateMaxUsedDrpsIdFlag is set as follows.

- –	If MaxUsedDrpsId >= max_drps_id_diff and MaxUsedDrpsId <= max_drps_id – max_drps_id_diff, depth_range_parameter_set_id shall be in the range of MaxUsedDrpsId – max_drps_id_diff to MaxUsedDrpsId + max_drps_id_diff, inclusive. The variable updateMaxUsedDrpsIdFlag is set equal to ( depth_range_parameter_set_id > MaxUsedDrpsId ).

- –	Otherwise, if MaxUsedDrpsId < max_drps_id_diff, depth_range_parameter_set_id shall be either in the range of 0 to MaxUsedDrpsId + max_drps_id_diff, inclusive, or in the range of max_drps_id – ( max_drps_id_diff – MaxUsedDrpsId – 1 ) to max_drps_id, inclusive. The updateMaxUsedDrpsIdFlag is set equal to ( depth_range_parameter_set_id > MaxUsedDrpsId  &&  depth_range_parameter_set_id < max_drps_id – ( max_drps_id_diff – MaxUsedDrpsId – 1 ) ).

- –	Otherwise (MaxUsedDrpsId > max_drps_id – max_drps_id_diff), depth_range_parameter_set_id shall be either in the range of 0 to max_drps_id_diff – ( max_drps_id – MaxUsedDrpsId ) – 1, inclusive, or in the range of MaxUsedDrpsId – max_drps_id_diff to max_drps_id, inclusive. The updateMaxUsedDrpsIdFlag is set equal to ( depth_range_parameter_set_id > MaxUsedDrpsId  ||  depth_range_parameter_set_id < max_drps_id_diff – ( max_drps_id – MaxUsedDrpsId ) ).

- In addition to the active depth range parameter set RBSP, zero or more depth range parameter set RBSPs may be specifically active for texture view components (with a particular value of VOIdx less than or equal to VOIdxMax) that belong to the target output views or that may be referred to through inter-view prediction in decoding texture view components belonging to the target output views. Such a depth range parameter set RBSP is referred to as the active texture view component depth range parameter set RBSP for the particular value of VOIdx. The restrictions on active depth range parameter set RBSPs also apply to active texture view component depth range parameter set RBSPs for a particular value of VOIdx.

- Furthermore, zero or more depth range parameter set RBSPs may be specifically active for depth view components (with a particular value of VOIdx less than VOIdxMax) that belong to the target output views or that may be referred to through inter-view prediction in decoding depth view components belonging to the target output views. Such a depth range parameter set RBSP is referred to as the active depth view component depth range parameter set RBSP for the particular value of VOIdx. The restrictions on active depth range parameter set RBSPs also apply to active depth view component depth range parameter set RBSPs for a particular value of VOIdx less than VOIdxMax.

- When a depth range parameter set RBSP (with a particular value of depth_range_parameter_set_id equal to activeDrpsId) is not the active depth range parameter set RBSP and it is referred to by a coded slice NAL unit belonging to a depth view component (i.e., with nal_unit_type equal to 21) and with VOIdx equal to VOIdxMax (using that value of depth_range_parameter_set_id), it is activated. This depth range parameter set RBSP is called the active depth range parameter set RBSP until it is deactivated when another depth range parameter set RBSP becomes the active depth range parameter set RBSP. A depth range parameter set RBSP, with that particular value of depth_range_parameter_set_id equal to activeDrpsId, shall be available to the decoding process prior to its activation. When a depth range parameter set RBSP is activated, the same depth range parameter set RBSP shall remain active for subsequent coded slice NAL units of the same access unit.

- When updateMaxUsedDrpsIdFlag is equal to 1, depth range parameter set RBSPs are marked as unavailable to decoding process as follows.

i = MaxUsedDrpsId
do {
	Mark depth range parameter set RBSP with depth_range_parameter_set_id equal to i, if present, as unavailable to decoding process.
	i = ( i + 1 ) % ( max_drps_id + 1 )
} while( i != depth_range_parameter_set_id )

- When updateMaxUsedDrpsIdFlag is equal to 1 or MaxUsedDrpsId is equal to “no value”, the following applies.

–	MaxUsedDrpsId is set equal to depth_range_parameter_set_id.

–	If MaxUsedDrpsId is greater than or equal to max_drps_id_diff, UsedDrpsIdSet is set to the depth_parameter_set_id values in the range of MaxUsedDrpsId – max_drps_id_diff to MaxUsedDrpsId, inclusive.

–	Otherwise (MaxUsedDrpsId is smaller than max_drps_id_diff), UsedDrpsIdSet is set to the drps_id values in the range of 0 to MaxUsedDrpsId, inclusive, and in the range of max_drps_id – ( max_drps_id_diff – MaxUsedDrpsId ) to max_drps_id, inclusive.

- Any depth range parameter set RBSP included in the bitstream or made available to the decoding process through external means and having an depth_range_parameter_set_id value that is included in UsedDrpsIdSet shall have the same content as the previous depth range parameter set RBSP included in the bitstream or made available to the decoding process through external means having the same depth_range_parameter_id value.

- When a depth range parameter set RBSP (with a particular value of depth_range_parameter_set_id) is not the active depth view component depth range parameter set for a particular value of VOIdx less than VOIdxMax and it is referred to by a coded slice NAL unit belonging to a depth view component (i.e., with nal_unit_type equal to 21) and with the particular value of VOIdx (using that value of depth_range_parameter_set_id), it is activated for view components with the particular value of VOIdx. This depth range parameter set RBSP is called the active depth view component depth range parameter set RBSP for the particular value of VOIdx until it is deactivated when another depth range parameter set RBSP becomes the active depth view component depth range parameter set RBSP for the particular value of VOIdx. A depth range parameter set RBSP, with that particular value of depth_range_parameter_set_id, shall be available to the decoding process prior to its activation.

- When a depth range parameter set RBSP (with a particular value of depth_range_parameter_set_id) is not the active texture view component depth range parameter set for a particular value of VOIdx less than or equal to VOIdxMax and it is referred to by a coded slice NAL unit belonging to a texture view component (i.e., with nal_unit_type equal to 1, 5 or 20) and with the particular value of VOIdx (using that value of pic_parameter_set_id), it is activated for texture view components with the particular value of VOIdx. This depth range parameter set RBSP is called the active texture view component depth range parameter set RBSP for the particular value of VOIdx until it is deactivated when another depth range parameter set RBSP becomes the active texture view component depth range parameter set RBSP for the particular value of VOIdx. A depth range parameter set RBSP, with that particular value of depth_range_parameter_set_id, shall be available to the decoding process prior to its activation.

- Any depth range parameter set NAL unit containing the value of depth_range_parameter_set_id for the active depth range parameter set RBSP for a coded picture shall have the same content as that of the active depth range parameter set RBSP for this coded picture unless it follows the last VCL NAL unit of this coded picture and precedes the first VCL NAL unit of another coded picture. Any depth range parameter set NAL unit containing the value of depth_range_parameter_set_id for the active depth view component depth range parameter set RBSP for a particular value of VOIdx less than VOIdxMax for a coded picture shall have the same content as that of the active depth view component depth range parameter set RBSP for the particular value of VOIdx for this coded picture unless it follows the last VCL NAL unit of this coded picture and precedes the first VCL NAL unit of another coded picture. Any depth range parameter set NAL unit containing the value of depth_range_parameter_set_id for the active texture view component depth range parameter set RBSP for a particular value of VOIdx for a coded picture shall have the same content as that of the active texture view component depth range parameter set RBSP for the particular value of VOIdx for this coded picture unless it follows the last VCL NAL unit of this coded picture and precedes the first VCL NAL unit of another coded picture.

For view components with VOIdx equal to VOIdxMax, all constraints that are expressed on the relationship among the values of the syntax elements (and the values of variables derived from those syntax elements) in 3DVC sequence parameter sets, picture parameter sets and depth range parameter sets and other syntax elements are expressions of constraints that apply only to the active 3DVC sequence parameter set, the active picture parameter set and the active depth range parameter set. For view components with a particular value of VOIdx less than VOIdxMax, all constraints that are expressed on the relationship among the values of the syntax elements (and the values of variables derived from those syntax elements) in 3DVC sequence parameter sets, picture parameter sets and depth range parameter sets and other syntax elements are expressions of constraints that apply only to the active view 3DVC sequence parameter set, the active view picture parameter set and the active depth range parameter set for the particular value of VOIdx. If any 3DVC sequence parameter set RBSP having profile_idc equal to the value of one of the profile_idc values specified in Annex Error: Reference source not found or Annex Error: Reference source not found or Annex I is present that is never activated in the bitstream (i.e., it never becomes the active 3DVC sequence parameter set or an active view 3DVC sequence parameter set), its syntax elements shall have values that would conform to the specified constraints if it were activated by reference in an otherwise-conforming bitstream. If any picture parameter set RBSP is present that is never activated in the bitstream (i.e., it never becomes the active picture parameter set or an active view picture parameter set), its syntax elements shall have values that would conform to the specified constraints if it were activated by reference in an otherwise-conforming bitstream. If any depth range parameter set RBSP is present that is never activated in the bitstream (i.e., it never becomes the active depth range parameter set or an active view depth range parameter set), its syntax elements shall have values that would conform to the specified constraints if it were activated by reference in an otherwise-conforming bitstream.

During operation of the decoding process (see subclause I.8), for view components with VOIdx equal to VOIdxMax, the values of parameters of the active depth range parameter set, the active picture parameter set and the active 3DVC sequence parameter set shall be considered in effect. For view components with a particular value of VOIdx less than VOIdxMax, the values of the parameters of the active view depth range parameter set, the active view picture parameter set and the active view 3DVC sequence parameter set for the particular value of VOIdx shall be considered in effect. For interpretation of SEI messages that apply to the entire access unit or the view component with VOIdx equal to VOIdxMax, the values of the parameters of the active depth range parameter set, the active picture parameter set and the active 3DVC sequence parameter set for the same access unit shall be considered in effect unless otherwise specified in the SEI message semantics. For interpretation of SEI messages that apply to view components with a particular value of VOIdx less than VOIdxMax, the values of the parameters of the active view depth parameter set, the active view picture parameter set and the active view 3DVC sequence parameter set for the particular value of VOIdx for the same access unit shall be considered in effect unless otherwise specified in the SEI message semantics.

# Patent rights declaration(s)

Nokia Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).