| Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 1st Meeting: Stockholm, SE, 16–20 July 2012 | Document: JCT2-A0169 |
| --- | --- |

| Title: | 3D-CE3.a related: The simplification of the de-blocking scheme for PSIP and DISP |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to the JCT2 |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Gun Bang, *Kyung Yong Kim, *Young Su Heo, *Gwang Hoon Park, Won-Sik Cheong, Namho Hur ETRI, 218 Gajeong-Ro, Yuseong-Gu, Daejeon, 305-700, Korea *Media Lab., Kyung Hee University 1, Seochun, Kiheung, Youngin, Gyonggi, 446-701, Korea | Tel: Email: | +82-42-860-5069 {gbang, wscheong, namho} @etri.re.kr +82-31-201-3680 {*kimky, *medist, *ghpark} @khu.ac.kr |
| Source: | ETRI and Kyung Hee University |  |  |

_____________________________

# Abstract

This contribution proposes to reduce the simplification of the de-blocking scheme for PSIP and DISP mode. Simulation result achieves a 0.06% bit rate reduction on the coded data, and a 0.03% bit rate reduction on the synthesized views. The maximum gain has almost 1.0% bit rate reduction on the depth coded data for the GT_Fly sequence. And the simulation result achieves almost 7.0% bit rate reduction on the coded data, and a 0.75% bit rate reduction on the synthesized views compared with 3DV-ATM v0.4 (De-blocking filter for depth: OFF). We recommend keeping the de-blocking scheme for depth coding, because it has still a chance to improve a performance for depth coding.

# Introduction

The depth intra skip prediction (DISP) and plane segmentation based intra prediction (PSIP) was adopted at last meeting [1]. PSIP mode is applied for DC mode of I16x16, I8x8, and I4x4. For I8x8 and I4x4, the most probable mode is set as DC when PSIP is enabled. DISP mode is applied for I16x16. The derivation process for the luma content dependent boundary filtering strength is modified for the DISP mode and PSIP mode.

The following Table 1, Figure 1 and Figure 2 show the derivation process for the luma content dependent boundary filtering strength for DISP mode and PISP mode.

Table 1. Derivation process for the luma content dependent boundary filtering strength for DISP mode and PISP mode

– The following paragraph is added after the paragraph starting with “Output of this process is the variable bS”: The variable bS is derived as follows if psip_flag is equal to 1. – If psip_flag is equal to 1, both of the samples p0 and q0 is in a macroblock coded using a I16x16 Mode and both of the samples p0 and q0 is in a macroblock coded using an Intra_16x16_DC_PSIP_MODE, a value of bS equal to 0 is the output: – Otherwise, if psip_flag is equal to 1, both of the samples p0 and q0 is in a macroblock coded as I8x8 Mode and both of the samples p0 and q0 is in a macroblock coded using an Intra_8x8_DC_PSIP_MODE, a value of bS equal to 0 is the output: – Otherwise, if psip_flag is equal to 1, both of the samples p0 and q0 is in a macroblock coded as I4x4 Mode and both of the samples p0 and q0 is in a macroblock coded using an Intra_4x4_DC_PSIP_MODE, a value of bS equal to 0 is the output: – Otherwise, if psip_flag is equal to 1, either the samples p0 or q0 is in a macroblock coded as I16x16 Mode with Intra_16x16_DC_PSIP_MODE, the following applies – If the block edge is also a macroblock edge, a value of bS equal to 1 is the output: – Otherwise, a value of bS equal to 0 is the output: – Otherwise, if psip_flag is equal to 1, either the samples p0 or q0 is in a macroblock coded as I8x8 Mode with Intra_8x8_DC_PSIP_MODE, intra prediction mode, the following applies – If the block edge is also a macroblock edge, a value of bS equal to 1 is the output: – Otherwise, a value of bS equal to 0 is the output: – Otherwise, if psip_flag is equal to 1, either the samples p0 or q0 is in a macroblock coded as I4x4 Mode with Intra_4x4_DC_PSIP_MODE intra prediction mode, the following applies – If the block edge is also a macroblock edge, a value of bS equal to 1 is the output: – Otherwise, a value of bS equal to 0 is the output: The variable bS is derived as follows if disp_flag is equal to 1. mb_type is DISPMODE if mb_disp_flag is equal to 1. – If verticalEdgeFlag is euqal to 1, both of the samples p0 and q0 is in a macroblock coded using a DISPMODE and both of the samples p0 and q0 is in a macroblock coded using an Intra_16x16_Horizontal intra prediction mode, a value of bS equal to 0 is the output: – Otherwise, if verticalEdgeFlag is euqal to 0, both of the samples p0 and q0 is in a macroblock coded using a DISPMODE and both of the samples p0 and q0 is in a macroblock coded using an Intra_16x16_Vertical intra prediction mode, a value of bS equal to 0 is the output: – Otherwise, if both of the samples p0 and q0 is in a macroblock coded using a DISPMODE, the following applies – If the block edge is also a macroblock edge, a value of bS equal to 2 is the output: – Otherwise, a value of bS equal to 0 is the output: – Otherwise, if either the samples p0 or q0 is in a macroblock coded using a DISPMODE, the following applies – If the block edge is also a macroblock edge, a value of bS equal to 4 is the output: – Otherwise, a value of bS equal to 3 is the output:

Figure 1. Flow chart of the derivation process for the luma content dependent boundary filtering strength for PISP mode

Figure 2. Flow chart of the derivation process for the luma content dependent boundary filtering strength for DISP mode

# Simplification

We reduced the derivation process for the luma content dependent boundary filtering strength for DISP mode and PISP mode. The following Figure 3 and Figure 4 show the proposed method for DISP mode and PISP mode.

Figure 3. Proposed method for PSIP mode

Figure 4. Proposed method for DISP mode

# Simulation Results

Common test conditions specified in W12745 [2] is used for the evaluation. The following Table 2 shows the simulation result of the proposed method of the de-blocking filter for depth. The proposed method brings little coding gain.

Table 2. Simulation result of the proposed method (Anchor: 3DV-ATM v0.4)

|  | Texture Coding |  | Depth Coding |  | Total (Coded PSNR) |  | Total (Synthesed PSNR) |  | Complexity estimate (ratio to anchor) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | dBR, % | dPSNR,dB | dBR, % | dPSNR,dB | dBR, % | dPSNR,dB | dBR, % | dPSNR,dB | Encoder Time, % | Decoder Time, % | Rendering Time, % |
| S01 | -0.19 | 0.00 | 0.02 | 0.00 | -0.17 | 0.00 | 0.01 | 0.00 | 100% | 102% | 100.00 |
| S02 | -0.02 | 0.00 | -0.17 | 0.01 | -0.01 | 0.00 | 0.03 | 0.00 | 101% | 99% | 100.00 |
| S03 | -0.03 | 0.00 | -0.06 | 0.01 | -0.04 | 0.00 | -0.06 | 0.00 | 99% | 102% | 100.00 |
| S04 | -0.01 | 0.00 | -0.96 | 0.05 | -0.07 | 0.00 | -0.11 | 0.00 | 102% | 101% | 100.00 |
| S05 | -0.03 | 0.00 | -0.08 | 0.00 | -0.06 | 0.00 | -0.07 | 0.00 | 101% | 101% | 100.00 |
| S06 | -0.07 | 0.00 | 0.07 | 0.00 | -0.08 | 0.00 | -0.09 | 0.00 | 105% | 101% | 100.00 |
| S08 | -0.01 | 0.00 | -0.27 | 0.01 | -0.01 | 0.00 | 0.10 | 0.00 | 118% | 100% | 100.00 |
| Average | -0.05 | 0.00 | -0.21 | 0.01 | -0.06 | 0.00 | -0.03 | 0.00 | 104% | 101% | 100.00 |

The following Table 3 shows the simulation result of the proposed method compared with ATM0.4 + Depth De-block OFF.

Table 3. Simulation result of the proposed method (Anchor: 3DV-ATM v0.4 + Depth De-block OFF)

|  | Texture Coding |  | Depth Coding |  | Total (Coded PSNR) |  | Total (Synthesed PSNR) |  | Complexity estimate (ratio to anchor) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | dBR, % | dPSNR,dB | dBR, % | dPSNR,dB | dBR, % | dPSNR,dB | dBR, % | dPSNR,dB | Encoder Time, % | Decoder Time, % | Rendering Time, % |
| S01 | -0.74 | 0.02 | -5.54 | 0.34 | -0.89 | 0.03 | -0.77 | 0.03 | 104% | 105% | 100.00 |
| S02 | -0.01 | 0.00 | -8.07 | 0.40 | -0.18 | 0.01 | -0.52 | 0.02 | 107% | 102% | 100.00 |
| S03 | -0.62 | 0.02 | -6.33 | 0.53 | -0.80 | 0.03 | -0.42 | 0.01 | 105% | 103% | 100.00 |
| S04 | -0.15 | 0.01 | -7.64 | 0.45 | -0.28 | 0.01 | -0.73 | 0.02 | 105% | 103% | 100.00 |
| S05 | -0.31 | 0.02 | -6.51 | 0.32 | -0.48 | 0.02 | -1.02 | 0.04 | 100% | 100% | 100.00 |
| S06 | -0.40 | 0.02 | -7.00 | 0.34 | -0.57 | 0.03 | -1.02 | 0.05 | 104% | 103% | 100.00 |
| S08 | -0.08 | 0.00 | -7.74 | 0.36 | -0.27 | 0.01 | -0.79 | 0.03 | 113% | 101% | 100.00 |
| Average | -0.33 | 0.01 | -6.97 | 0.39 | -0.50 | 0.02 | -0.75 | 0.03 | 106% | 102% | 100.00 |

# Conclusion

We propose to reduce the simplification of the de-blocking scheme for PSIP and DISP mode. The de-blocking filter for depth brings a coding gain just using simplification of derivation process for de-blocking.

Then, we recommend keeping the de-blocking scheme for depth coding, because it has still a chance to improve a performance for depth coding.

# Reference

[1]	K. J Oh, B. Tae Oh, J. Y Lee, J. J Lee, H. C Wey, D. S Park, G. Bang, W.S Cheong, N. H Hur, K. Y Kim and G. H Park, “3D-CE6 results on depth intra prediction by Samsung and ETRI-KHU”, The 100th MPEG meeting, M24825, Geneva, Switzerland, May 2012.

[2]	H. Schwarz, and D. Rusanovskyy, “Common Test Conditions for 3DV experimentation”, The 100th MPEG meeting, W12745, Geneva, Switzerland, May 2012.

# Patent rights declaration(s)

ETRI and KyungHee University may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Annex:	Proposed text changes based on w12732

J.8.9.2.1 Derivation process for the luma content dependent boundary filtering strength

The specifications in 8.7.2.1 apply with the following changes:

–	The following paragraph is added after the paragraph starting with “Output of this process is the variable bS”:

The variable bS is derived as follows if psip_flag is equal to 1.

–	If psip_flag is equal to 1, the samples q0 is in a macroblock coded as I16x16 Mode with Intra_16x16_DC_PSIP_MODE, the following applies

–	If the block edge is also a macroblock edge, a value of bS equal to 1 is the output:

–	Otherwise, a value of bS equal to 0 is the output:

–	Otherwise, if psip_flag is equal to 1, the samples q0 is in a macroblock coded as I8x8 Mode with Intra_8x8_DC_PSIP_MODE, intra prediction mode, the following applies

–	If the block edge is also a macroblock edge, a value of bS equal to 1 is the output:

–	Otherwise, a value of bS equal to 0 is the output:

–	Otherwise, if psip_flag is equal to 1, the samples q0 is in a macroblock coded as I4x4 Mode with Intra_4x4_DC_PSIP_MODE intra prediction mode, the following applies

–	If the block edge is also a macroblock edge, a value of bS equal to 1 is the output:

–	Otherwise, a value of bS equal to 0 is the output:

–	If psip_flag is equal to 1, both of the samples p0 and q0 is in a macroblock coded using a I16x16 Mode and both of the samples p0 and q0 is in a macroblock coded using an Intra_16x16_DC_PSIP_MODE, a value of bS equal to 0 is the output:

–	Otherwise, if psip_flag is equal to 1, both of the samples p0 and q0 is in a macroblock coded as I8x8 Mode and both of the samples p0 and q0 is in a macroblock coded using an Intra_8x8_DC_PSIP_MODE, a value of bS equal to 0 is the output:

–	Otherwise, if psip_flag is equal to 1, both of the samples p0 and q0 is in a macroblock coded as I4x4 Mode and both of the samples p0 and q0 is in a macroblock coded using an Intra_4x4_DC_PSIP_MODE, a value of bS equal to 0 is the output:

–	Otherwise, if psip_flag is equal to 1, either the samples p0 or q0 is in a macroblock coded as I16x16 Mode with Intra_16x16_DC_PSIP_MODE, the following applies

–	If the block edge is also a macroblock edge, a value of bS equal to 1 is the output:

–	Otherwise, a value of bS equal to 0 is the output:

–	Otherwise, if psip_flag is equal to 1, either the samples p0 or q0 is in a macroblock coded as I8x8 Mode with Intra_8x8_DC_PSIP_MODE, intra prediction mode, the following applies

–	If the block edge is also a macroblock edge, a value of bS equal to 1 is the output:

–	Otherwise, a value of bS equal to 0 is the output:

–	Otherwise, if psip_flag is equal to 1, either the samples p0 or q0 is in a macroblock coded as I4x4 Mode with Intra_4x4_DC_PSIP_MODE intra prediction mode, the following applies

–	If the block edge is also a macroblock edge, a value of bS equal to 1 is the output:

–	Otherwise, a value of bS equal to 0 is the output:

The variable bS is derived as follows if disp_flag is equal to 1. mb_type is DISPMODE if mb_disp_flag is equal to 1.

–	If verticalEdgeFlag is euqal to 1, both of the samples p0 and q0 is in a macroblock coded using a DISPMODE and both of the samples p0 and q0 is in a macroblock coded using an Intra_16x16_Horizontal intra prediction mode, a value of bS equal to 0 is the output:

–	Otherwise, if verticalEdgeFlag is euqal to 0, both of the samples p0 and q0 is in a macroblock coded using a DISPMODE and both of the samples p0 and q0 is in a macroblock coded using an Intra_16x16_Vertical intra prediction mode, a value of bS equal to 0 is the output:

–	Otherwise, if either the samples p0 or q0 is in a macroblock coded using a DISPMODE, the following applies

–	If the block edge is also a macroblock edge, a value of bS equal to 4 is the output:

–	Otherwise, a value of bS equal to 0 is the output:

–	Otherwise, if both of the samples p0 and q0 is in a macroblock coded using a DISPMODE, the following applies

–	If the block edge is also a macroblock edge, a value of bS equal to 2 is the output:

–	Otherwise, a value of bS equal to 0 is the output:

–	Otherwise, if either the samples p0 or q0 is in a macroblock coded using a DISPMODE, the following applies

–	If the block edge is also a macroblock edge, a value of bS equal to 4 is the output:

–	Otherwise, a value of bS equal to 3 is the output: