| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 10th Meeting: Stockholm, Sweden, 11-20 July 2012 | Document: JCT2-A0170 |
| --- | --- |

| Title: | 3DV: Proposed modifications to MVC+depth WD to include interlaced depth (JCT2-A0170) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-3V |  |  |
| Purpose: | For standard |  |  |
| Author(s) or Contact(s): | Fang-Chu Chen Ching-Chieh Lin, | Tel: Email: | +886-3-5914782 fcchen@itri.org.tw |
| Source: | Industrial Technology Research Institute |  |  |

_____________________________

# Abstract

This document complements the contribution JCT2-A0144 [1] and provides modified text to the working draft W12731[2] to include interlace depth coding in the working draft.

# Introduction

The current working draft W12331 for MVC extensions for inclusion of depth maps is targeted for progressive depth. However, the existing syntax and functionalities support interlace depth coding. The only thing which is lacking is the definition of the interlace depth component. We propose the following modifications to the text to add in the definitions, and include the new terms in the text where necessary, so that the working draft include interlace depth coding in addition to frame depth coding.

# The modifications

In session I.3 add the following definitions after depth view component

I.3.x	depth field view component: A depth view component of a field.

I.3.x	depth frame view component: A depth view component of a frame.

In session I.8.2, add bold text in the paragraph

The specification of subclause H.8.2 apply with substituting "view component" as either "texture view component" or "depth view component ", and "frame view component"" as either "depth frame view component" or "texture frame view component ", and "field view component" as "texture field view component" or “depth field view component”.

In session I.8.3, add the bold text in the following paragraph

The process specified in this subclause is invoked for a particular texture view or depth view with view order index VOIdx. The specifications in subclause H.8.3 apply with "view component" being replaced by either "texture view component" or "depth view component", "frame view component" being replaced by either "texture frame view component" or "depth frame view component", and "field view component" being replaced by either "texture field view component" or “depth field view component”. During the invocation of the process for a particular texture view, only texture view components of the particular view are considered. During the invocation of the process for a particular depth view, only depth view components of the particular view are considered. The marking of view components of other views is not changed.

# References

[1] C.-C. Lin, F.-C. Chen, W.-H. Chung, “3DV technical input: JMVC software integration of coding interlace texture with interlace depth,” JCT2-A0144, JCT Meeting: Stockholm, Sweden, July 2012.

[2] Teruhiko Suzuki, Miska M. Hannuksela, Ying Chen, Shinobu Hattori , “Text of ISO/IEC 14496-10:2012/PDAM 2 MVC extensions for inclusion of depth maps”, W12731, MPEG meeting, Geneva, May 2012.

# Patent rights declaration(s)

Industrial Technology Research Institute may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).