| Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 1st Meeting: Stockholm, SE, 16–20 July 2012 | Document: JCT2-A0175 |
| --- | --- |

| Title: | JCT-3V BoG Report: Preparation of Text on an Exploration Experiment on Warp Coding and Synthesis |  |  |
| --- | --- | --- | --- |
| Status: |  |  |  |
| Purpose: |  |  |  |
| Author(s) or Contact(s): | Nikolce Stefanoski | Tel: Email: | stefanos@disneyresearch.com |
| Source: | BoG |  |  |

# Abstract

A BoG meeting with the aim to draft text for an Exploration Experiment on Warp Coding and Synthesis was held Wednesday, July 18, 2012 from 14:00 to 15:00. Approximately 9 delegates were present. Below the text which was drafted can be found.

# Introduction

The mandates of this Exploration Experiment (EE) are to

- study similarities and differences between the warp representation and the existing depth/disparity representation,

- explore warp coding technologies and their implications on the bit rate and synthesis quality.

Coordinators:

xxx

# Transmission and Rendering System under Consideration

In this EE, the transmission and rendering system shown in Fig. 1 is considered. First, warp data is generated from Multiview Video plus Depth (MVD) data. Multiview Video plus Warp (MVW ) data is then encoded and decoded. Decoded MVW data is used as input to the View Synthesizer to generate new views. To be able to conduct experiments in the scope of this EE, Warp data, which is obtained as output of the Warp Generation Module, and the View Synthesis Software are provided.

Fig. 1: Transmission and Rendering System under Consideration.

# Available Data and Software

Image-domain-warping-based Rendering (IDWR) software and original warp data is provided here:

http://zurich.disneyresearch.com/videodata/niko/MPEG/WarpData

In particular, in subdirectory

http://zurich.disneyresearch.com/videodata/niko/MPEG/WarpData/scripts

a script is contained which calls the IDWR executable with example parameters (i.e. sequence names, warp names, input/output view positions). In the same directory, a MATLAB script which reads warp files is provided. It describes how these binary files are parsed and how the values are interpreted. Warp data is provided for all 2-view input test sequences specified in the Common Test Conditions for 3DV Experimentation N12745. More details on the provided data and software can be found in JCT-3V-A0003.

The complete set of multi-view video test sequences with depth maps that are being used for the experiment along with ftp access information and copyright statements can be found in the Call for Proposals on 3D-Video technology N12036.

# Experiments and Test Definition

To study the warp representation according to the mandates reported in Section , the following experiments are considered:

- EE1 (Warp to Depth Conversion): The aim of this experiment is to develop and explore technologies which approximate warp data by a depth representation. Warp data, which is provided in Section 3, is used as input in this experiment.  Sequences synthesized by i) IDWR using provided warp data and ii)  sequences synthesized with VSRS 1D Fast Mode using converted depth data will be generated and brought to the meeting. Synthesis quality will be assessed by viewing during the meeting, if this is requested. Proponents are encouraged to analyze the impact of the proposed conversion technology on the synthesis. Note that generating PSNR results as it is proposed in the CTC is not considered as appropriate in this experiment, since different synthesizers would be used to generate a synthesized reference.

- EE2 (Depth to Warp Conversion): The aim of this experiment is to develop and explore technologies which approximate depth data by a warp representation. Original depth data is used as input in this experiment.  Sequences synthesized with i)  VSRS 1D Fast Mode using original depth data and ii) sequences synthesized by IDWR using converted warp data will be generated and brought to the meeting. Synthesis quality will be assessed by viewing during the meeting, if requested. Proponents are encouraged to analyze the impact of the proposed conversion technology on the synthesis. Note that generating PSNR results as it is proposed in the CTC is not considered as appropriate in this experiment, since different synthesizers would be used to generate a synthesized reference.

- EE3 (Warp Coding): The aim of this experiment is to develop and explore warp coding methods. Warps data, which is obtained by conversion according to EE2 or which is provided in Section 3, is coded. Exact warp coding bit rates and corresponding view synthesis distortion have to be reported. View synthesis distortion shall be measured against a synthesized reference, i.e. uncoded warps and original texture video is used for the reference synthesis. View synthesis distortion will be reported for each sequence specified in Table 1. In addition, if requested, quality can be assessed by viewing during meeting.

- EE4 (Comparison of depth-based and warp-based view synthesis):  The aim of this experiment is to compare the synthesis quality achieved by i) using original depth maps and original textures with VSRS 1D Fast Mode and ii) by using original warp data and original textures with IDWR. In this experiment, warp data can be used which is provided in Section 3 or which is generated according to EE2. Subjective testing will be conducted during the meeting. Please note that only uncoded original data is used in this experiment.

EE4 is considered to have highest priority. The following table shows all test sequences (2-view input test sequences according to the Common Test Conditions for 3DV Experimentation (N12745)) as well as the corresponding views which have to be synthesized:

| Seq. ID | Test Sequence | Input views | Views to be synthesized | Stereo pair |
| --- | --- | --- | --- | --- |
| S01 | Poznan_Hall2 | 7-6 | 6.75-6.50-6.25 | 6.5-6 |
| S02 | Poznan_Street | 5-4 | 4.75-4.50-4.25 | 4.5-4 |
| S03 | Undo_Dancer | 1-5 | 2-3-4 | 3-5 |
| S04 | GT_Fly | 9-5 | 8-7-6 | 7-5 |
| S05 | Kendo | 1-3 | 1.5-2.0-2.5 | 2-3 |
| S06 | Balloons | 1-3 | 1.5-2.0-2.5 | 2-3 |
| S08 | Newspaper | 2-4 | 2.5-3.0-3.5 | 3-4 |

Table 1: Sequences and views to be synthesized.

All participants need to deliver synthesized sequences to the next meeting on an USB-disk. Stereo pairs must be packaged as a single AVI file for each sequence with the Stereo2AVI utility.