| Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 1st Meeting: Stockholm, SE, 16–20 July 2012 | Document: JCT2-A1001 |
| --- | --- |

| Title: | MVC Extension for Inclusion of Depth Maps Draft Text 4 |  |  |
| --- | --- | --- | --- |
| Status: | Output Document Approved by the JCT2 |  |  |
| Purpose: | Draft |  |  |
| Author(s) or Contact(s): | T. Suzuki, S. Hattori 2-10-1 Osaki, Shinagawa-ku Tokyo, 141-8610, Japan M. Hannuksela Visiokatu 3, 33720Tampere, Finland Ying Chen 5775 Morehouse Drive San Diego, CA 92121, USA Gary Sullivan 1 Microsoft Way Redmond, WA 98052 USA | Tel: Email: | +81-50-3750-2740 teruhikos@jp.sony.com +358 7180 08000 miska.hannuksela@nokia.com +1-858-845-6589 cheny@qualcomm.com +1 425 703 5308 garysull@microsoft.com |
| Source: | Editors |  |  |

_____________________________

This amendment of ISO/IEC 14496-10 defines a new profile appropriate for inclusion of depth maps. The new profile is specified as the Multiview Depth High profile.

The amendment extends the Stereo High profile to accommodate 3D enhancement while maintaining MVC stereo compatibility.

ISO/IEC JTC 1/SC 29 N

Date:   2012-07-27

ISO/IEC 14496-10:2012/DAM 2

ISO/IEC JTC 1/SC 29/WG 11

Secretariat:

Information technology — Coding of audio-visual objects — Part 10: Advanced Video Coding, AMENDMENT 2: MVC extensions for inclusion of depth maps

Élément introductif — Élément central — Partie 10: Titre de la partie

Warning

This document is not an ISO International Standard. It is distributed for review and comment. It is subject to change without notice and may not be referred to as an International Standard.

Recipients of this draft are invited to submit, with their comments, notification of any relevant patent rights of which they are aware and to provide supporting documentation.

Copyright notice

This ISO document is a working draft or committee draft and is copyright-protected by ISO. While the reproduction of working drafts or committee drafts in any form for use by participants in the ISO standards development process is permitted without prior permission from ISO, neither this document nor any extract from it may be reproduced, stored or transmitted in any form for any other purpose without prior written permission from ISO.

Requests for permission to reproduce this document for the purpose of selling it should be addressed as shown below or to ISO's member body in the country of the requester:

[Indicate the full address, telephone number, fax number, telex number, and electronic mail address, as appropriate, of the Copyright Manger of the ISO member body responsible for the secretariat of the TC or SC within the framework of which the working document has been prepared.]

Reproduction for sales purposes may be subject to royalty payments or a licensing agreement.

Violators may be prosecuted.

Foreword

ISO (the International Organization for Standardization) and IEC (the International Electrotechnical Commission) form the specialized system for worldwide standardization. National bodies that are members of ISO or IEC participate in the development of International Standards through technical committees established by the respective organization to deal with particular fields of technical activity. ISO and IEC technical committees collaborate in fields of mutual interest. Other international organizations, governmental and non-governmental, in liaison with ISO and IEC, also take part in the work. In the field of information technology, ISO and IEC have established a joint technical committee, ISO/IEC JTC 1.

International Standards are drafted in accordance with the rules given in the ISO/IEC Directives, Part 2.

The main task of the joint technical committee is to prepare International Standards. Draft International Standards adopted by the joint technical committee are circulated to national bodies for voting. Publication as an International Standard requires approval by at least 75 % of the national bodies casting a vote.

Attention is drawn to the possibility that some of the elements of this document may be the subject of patent rights. ISO and IEC shall not be held responsible for identifying any or all such patent rights.

Amendment 2 to ISO/IEC 14496-10:2012 was prepared by Joint Technical Committee ISO/IEC JTC 1, Information technology, Subcommittee SC 29, Coding of audio, picture, multimedia and hypermedia information.

Information technology — Coding of audio-visual objects — Part 10: Advanced Video Coding, AMENDMENT 2: MVC extensions for inclusion of depth maps

In 0.6, add the following paragraph after the paragraph that starts with “Multiview video coding”:

An extension of multiview video coding that also supports the inclusion of depth maps is specified in Annex I allowing the construction of bitstreams that represent multiple views with corresponding depth views. Similar to multiview video coding, bitstreams that include multiple depth views may also contain sub-bitstreams that conform to this specification. For temporal bitstream scalability, i.e., the presence of a sub-bitstream with a smaller temporal sampling rate than the bitstream, complete access units are removed from the bitstream when deriving the sub-bitstream. For view bitstream scalability, i.e. the presence of a sub-bitstream with fewer views than those included in the bitstream, NAL units are removed from the bitstream when deriving the sub-bitstream. In this case, inter-view prediction, i.e., the prediction of one view by data of another view signal, is typically used for efficient coding.

In 0.7, add the following paragraph after the paragraph that starts with “Annex H specifies”:

Annex I specifies MVC extensions for inclusion of depth maps, referred to as 3D Video Coding (3DVC). The reader is referred to Annex I for the entire decoding process for 3DVC, which is specified there with references being made to clauses 2-9 and Annexes A-E and Annex H. Subclause I.1Error: Reference source not found specifies one profile for 3DVC (Multiview and Depth).

In 7.3.1, replace the syntax table with:

| nal_unit( NumBytesInNALunit ) { | C | Descriptor |
| --- | --- | --- |
| forbidden_zero_bit | All | f(1) |
| nal_ref_idc | All | u(2) |
| nal_unit_type | All | u(5) |
| NumBytesInRBSP = 0 |  |  |
| nalUnitHeaderBytes = 1 |  |  |
| if( nal_unit_type = = 14 ¦ ¦ nal_unit_type = = 20 ¦ ¦ nal_unit_type = = 21) { |  |  |
| svc_extension_flag | All | u(1) |
| if( !svc_extension_flag ¦ ¦ nal_unit_type = = 21 ) ) |  |  |
| nal_unit_header_mvc_extension( ) /* specified in Annex Error: Reference source not found */ | All |  |
| else |  |  |
| nal_unit_header_svc_extension( ) /* specified in Annex Error: Reference source not found */ | All |  |
| nalUnitHeaderBytes += 3 |  |  |
| } |  |  |
| for( i = nalUnitHeaderBytes; i < NumBytesInNALunit; i++ ) { |  |  |
| if( i + 2 < NumBytesInNALunit && next_bits( 24 ) = = 0x000003 ) { |  |  |
| rbsp_byte[ NumBytesInRBSP++ ] | All | b(8) |
| rbsp_byte[ NumBytesInRBSP++ ] | All | b(8) |
| i += 2 |  |  |
| emulation_prevention_three_byte /* equal to 0x03 */ | All | f(8) |
| } else |  |  |
| rbsp_byte[ NumBytesInRBSP++ ] | All | b(8) |
| } |  |  |
| } |  |  |

In 7.3.2.1.3, replace the syntax table with:

| subset_seq_parameter_set_rbsp( ) { | C | Descriptor |
| --- | --- | --- |
| seq_parameter_set_data( ) | 0 |  |
| if( profile_idc = = 83 ¦ ¦ profile_idc = = 86 ) { |  |  |
| seq_parameter_set_svc_extension( ) /* specified in Annex Error: Reference source not found */ | 0 |  |
| svc_vui_parameters_present_flag | 0 | u(1) |
| if( svc_vui_parameters_present_flag = = 1 ) |  |  |
| svc_vui_parameters_extension( ) /* specified in Annex Error: Reference source not found */ | 0 |  |
| } else if( profile_idc = = 118 ¦ ¦ profile_idc = = 128 ) { |  |  |
| bit_equal_to_one /* equal to 1 */ | 0 | f(1) |
| seq_parameter_set_mvc_extension( ) /* specified in Annex Error: Reference source not found */ | 0 |  |
| mvc_vui_parameters_present_flag | 0 | u(1) |
| if( mvc_vui_parameters_present_flag = = 1 ) |  |  |
| mvc_vui_parameters_extension( ) /* specified in Annex Error: Reference source not found */ | 0 |  |
| } |  |  |
| if( profile_idc = = 138 ) { |  |  |
| bit_equal_to_one /* equal to 1 */ | 0 | f(1) |
| seq_parameter_set_mvc_extension( ) /* specified in Annex Error: Reference source not found */ | 0 |  |
| seq_parameter_set_3dvc_extension( ) | 0 |  |
| } |  |  |
| additional_extension3_flag | 0 | u(1) |
| if( additional_extension3_flag = = 1 ) |  |  |
| while( more_rbsp_data( ) ) |  |  |
| additional_extension3_data_flag | 0 | u(1) |
| rbsp_trailing_bits( ) | 0 |  |
| } |  |  |

In 7.3.3, replace the syntax table with:

| slice_header( ) { | C | Descriptor |
| --- | --- | --- |
| first_mb_in_slice | 2 | ue(v) |
| slice_type | 2 | ue(v) |
| pic_parameter_set_id | 2 | ue(v) |
| if( separate_colour_plane_flag = = 1 ) |  |  |
| colour_plane_id | 2 | u(2) |
| frame_num | 2 | u(v) |
| if( !frame_mbs_only_flag ) { |  |  |
| field_pic_flag | 2 | u(1) |
| if( field_pic_flag ) |  |  |
| bottom_field_flag | 2 | u(1) |
| } |  |  |
| if( IdrPicFlag ) |  |  |
| idr_pic_id | 2 | ue(v) |
| if( pic_order_cnt_type = = 0 ) { |  |  |
| pic_order_cnt_lsb | 2 | u(v) |
| if( bottom_field_pic_order_in_frame_present_flag && !field_pic_flag ) |  |  |
| delta_pic_order_cnt_bottom | 2 | se(v) |
| } |  |  |
| if( pic_order_cnt_type = = 1 && !delta_pic_order_always_zero_flag ) { |  |  |
| delta_pic_order_cnt[ 0 ] | 2 | se(v) |
| if( bottom_field_pic_order_in_frame_present_flag && !field_pic_flag ) |  |  |
| delta_pic_order_cnt[ 1 ] | 2 | se(v) |
| } |  |  |
| if( redundant_pic_cnt_present_flag ) |  |  |
| redundant_pic_cnt | 2 | ue(v) |
| if( slice_type = = B ) |  |  |
| direct_spatial_mv_pred_flag | 2 | u(1) |
| if( slice_type = = P ¦ ¦ slice_type = = SP ¦ ¦ slice_type = = B ) { |  |  |
| num_ref_idx_active_override_flag | 2 | u(1) |
| if( num_ref_idx_active_override_flag ) { |  |  |
| num_ref_idx_l0_active_minus1 | 2 | ue(v) |
| if( slice_type = = B ) |  |  |
| num_ref_idx_l1_active_minus1 | 2 | ue(v) |
| } |  |  |
| } |  |  |
| if( nal_unit_type = = 20 ¦ ¦ nal_unit_type = = 21 ) |  |  |
| ref_pic_list_mvc_modification( ) /* specified in Annex Error: Reference source not found */ | 2 |  |
| else |  |  |
| ref_pic_list_modification( ) | 2 |  |
| if( ( weighted_pred_flag && ( slice_type = = P ¦ ¦ slice_type = = SP ) ) ¦ ¦ ( weighted_bipred_idc = = 1 && slice_type = = B ) ) |  |  |
| pred_weight_table( ) | 2 |  |
| if( nal_ref_idc != 0 ) |  |  |
| dec_ref_pic_marking( ) | 2 |  |
| if( entropy_coding_mode_flag && slice_type != I && slice_type != SI ) |  |  |
| cabac_init_idc | 2 | ue(v) |
| slice_qp_delta | 2 | se(v) |
| if( slice_type = = SP ¦ ¦ slice_type = = SI ) { |  |  |
| if( slice_type = = SP ) |  |  |
| sp_for_switch_flag | 2 | u(1) |
| slice_qs_delta | 2 | se(v) |
| } |  |  |
| if( deblocking_filter_control_present_flag ) { |  |  |
| disable_deblocking_filter_idc | 2 | ue(v) |
| if( disable_deblocking_filter_idc != 1 ) { |  |  |
| slice_alpha_c0_offset_div2 | 2 | se(v) |
| slice_beta_offset_div2 | 2 | se(v) |
| } |  |  |
| } |  |  |
| if( num_slice_groups_minus1 > 0 && slice_group_map_type >= 3 && slice_group_map_type <= 5) |  |  |
| slice_group_change_cycle | 2 | u(v) |
| } |  |  |

Replace Table 7-1 with:

| nal_unit_type | Content of NAL unit and RBSP syntax structure | C | Annex Error: Reference source not found NAL unit type class | Annex Error: Reference source not found and Annex Error: Reference source not found NAL unit type class | Annex Error: Reference source not found NAL unit type class |
| --- | --- | --- | --- | --- | --- |
| 0 | Unspecified |  | non-VCL | non-VCL | non-VCL |
| 1 | Coded slice of a non-IDR picture slice_layer_without_partitioning_rbsp( ) | 2, 3, 4 | VCL | VCL | VCL |
| 2 | Coded slice data partition A slice_data_partition_a_layer_rbsp( ) | 2 | VCL | not applicable | not applicable |
| 3 | Coded slice data partition B slice_data_partition_b_layer_rbsp( ) | 3 | VCL | not applicable | not applicable |
| 4 | Coded slice data partition C slice_data_partition_c_layer_rbsp( ) | 4 | VCL | not applicable | not applicable |
| 5 | Coded slice of an IDR picture slice_layer_without_partitioning_rbsp( ) | 2, 3 | VCL | VCL | VCL |
| 6 | Supplemental enhancement information (SEI) sei_rbsp( ) | 5 | non-VCL | non-VCL | non-VCL |
| 7 | Sequence parameter set seq_parameter_set_rbsp( ) | 0 | non-VCL | non-VCL | non-VCL |
| 8 | Picture parameter set pic_parameter_set_rbsp( ) | 1 | non-VCL | non-VCL | non-VCL |
| 9 | Access unit delimiter access_unit_delimiter_rbsp( ) | 6 | non-VCL | non-VCL | non-VCL |
| 10 | End of sequence end_of_seq_rbsp( ) | 7 | non-VCL | non-VCL | non-VCL |
| 11 | End of stream end_of_stream_rbsp( ) | 8 | non-VCL | non-VCL | non-VCL |
| 12 | Filler data filler_data_rbsp( ) | 9 | non-VCL | non-VCL | non-VCL |
| 13 | Sequence parameter set extension seq_parameter_set_extension_rbsp( ) | 10 | non-VCL | non-VCL | non-VCL |
| 14 | Prefix NAL unit prefix_nal_unit_rbsp( ) | 2 | non-VCL | suffix dependent | suffix dependent |
| 15 | Subset sequence parameter set subset_seq_parameter_set_rbsp( ) | 0 | non-VCL | non-VCL | non-VCL |
| 16..18 | Reserved |  | non-VCL | non-VCL | non-VCL |
| 19 | Coded slice of an auxiliary coded picture without partitioning slice_layer_without_partitioning_rbsp( ) | 2, 3, 4 | non-VCL | non-VCL | non-VCL |
| 20 | Coded slice extension slice_layer_extension_rbsp( ) | 2, 3, 4 | non-VCL | VCL | VCL |
| 21 | Coded slice extension for depth view components /*specified in Annex I */ slice_layer_extension_rbsp( ) /* specified in Annex I */ | 2, 3, 4 | non-VCL | VCL | VCL |
| 22..23 | Reserved |  | non-VCL | non-VCL | VCL |
| 24..31 | Unspecified |  | non-VCL | non-VCL | non-VCL |

In 7.4.1, add the following paragraph in the semantics of svc_extension_flag just before the semantics of rbsp_byte[ i ].

The value of svc_extension_flag shall be equal to 0 for coded video sequences conforming to one or more profiles specified in Annex I. Decoders conforming to one or more profiles specified in Annex I shall ignore (remove from the bitstream and discard) NAL units for which nal_unit_type is equal to 14, 20, or 21 and for which svc_extension_flag is equal to 1.

In 7.4.2.1.3, make the following changes:

Replace the sentence following text in the semantics of chroma_format_idc after “inclusive.” with:

When chroma_format_idc is not present and when profile_idc is equal to 138, it shall be inferred to be equal to 0 (4:0:0 chroma format), otherwise, it shall be inferred to be equal to 1 (4:2:0 chroma format).

Substitute each occurrence of “additional_extension2_flag” with “additional_extension3_flag”.

Replace the semantics of additional_extension2_data_flag with the following:

additional_extension3_data_flag may have any value. It shall not affect the conformance to profiles specified in Annex Error: Reference source not found, Error: Reference source not found, Error: Reference source not found, or I.

Replace Annex C with:

- Annex C\
Hypothetical reference decoder

- (This annex forms an integral part of this Recommendation | International Standard)

This annex specifies the hypothetical reference decoder (HRD) and its use to check bitstream and decoder conformance.

Two types of bitstreams are subject to HRD conformance checking for this Recommendation | International Standard. The first such type of bitstream, called Type I bitstream, is a NAL unit stream containing only the VCL NAL units and filler data NAL units for all access units in the bitstream. The second type of bitstream, called a Type II bitstream, contains, in addition to the VCL NAL units and filler data NAL units for all access units in the bitstream, at least one of the following:

–	additional non-VCL NAL units other than filler data NAL units,

–	all leading_zero_8bits, zero_byte, start_code_prefix_one_3bytes, and trailing_zero_8bits syntax elements that form a byte stream from the NAL unit stream (as specified in Annex Error: Reference source not found).

Figure C-1 shows the types of bitstream conformance points checked by the HRD.

- Figure C-1 – Structure of byte streams and NAL unit streams for HRD conformance checks

The syntax elements of non-VCL NAL units (or their default values for some of the syntax elements), required for the HRD, are specified in the semantic subclauses of clause Error: Reference source not found, Annexes Error: Reference source not found and Error: Reference source not found, and subclauses Error: Reference source not found, Error: Reference source not found, Error: Reference source not found, I.7, Error: Reference source not found, I.14, I.7, Error: Reference source not found, and I.14.

Two types of HRD parameter sets (NAL HRD parameters and VCL HRD parameters) are used. The HRD parameter sets are signalled as follows:

–	When the coded video sequence conforms to one or more of the profiles specified in Annex Error: Reference source not found and the decoding process specified in clauses Error: Reference source not found-Error: Reference source not found is applied, the HRD parameter sets are signalled through video usability information as specified in subclauses Error: Reference source not found and Error: Reference source not found, which is part of the sequence parameter set syntax structure.

–	When the coded video sequence conforms to one or more of the profiles specified in Annex Error: Reference source not found and the decoding process specified in Annex Error: Reference source not found is applied, the HRD parameter sets are signalled through the SVC video usability information extension as specified in subclauses Error: Reference source not found and Error: Reference source not found, which is part of the subset sequence parameter set syntax structure.

NOTE 1 – For coded video sequences that conform to both, one or more of the profiles specified in Annex Error: Reference source not found and one or more of the profiles specified in Annex Error: Reference source not found, the signalling of the applicable HRD parameter sets is depending on whether the decoding process specified in clauses Error: Reference source not found-Error: Reference source not found or the decoding process specified in Annex Error: Reference source not found is applied.

–	When the coded video sequence conforms to one or more of the profiles specified in Annex Error: Reference source not found and the decoding process specified in Annex Error: Reference source not found is applied, the HRD parameter sets are signalled through the MVC video usability information extension as specified in subclauses Error: Reference source not found and Error: Reference source not found, which is part of the subset sequence parameter set syntax structure.

NOTE 2 – For coded video sequences that conform to both, one or more of the profiles specified in Annex Error: Reference source not found and one or more of the profiles specified in Annex Error: Reference source not found, the signalling of the applicable HRD parameter sets is depending on whether the decoding process specified in clauses Error: Reference source not found-Error: Reference source not found or the decoding process specified in Annex Error: Reference source not found is applied.

–	When the coded video sequence conforms to one or more of the profiles specified in Annex I and the decoding process specified in Annex I is applied, the HRD parameter sets are signalled through the MVC video usability information extension as specified in subclauses I.14, which is part of the subset sequence parameter set syntax structure.

NOTE 3 – For coded video sequences that conform to one or more of the profiles specified in Annex Error: Reference source not found, one or more of the profiles specified in Annex H and one or more of the profiles specified in Annex I, the signalling of the applicable HRD parameter sets is depending on whether the decoding process specified in clauses Error: Reference source not found-Error: Reference source not found, the decoding process specified in Annex Error: Reference source not found or the decoding process specified in Annex I is applied.

All sequence parameter sets and picture parameter sets referred to in the VCL NAL units, and corresponding buffering period and picture timing SEI messages shall be conveyed to the HRD, in a timely manner, either in the bitstream (by non-VCL NAL units), or by other means not specified in this Recommendation | International Standard.

In Annexes Error: Reference source not found, Error: Reference source not found, and Error: Reference source not found and subclauses Error: Reference source not found, Error: Reference source not found, Error: Reference source not found, I.12, Error: Reference source not found, I.14, I.12, Error: Reference source not found, and I.14, the specification for "presence" of non-VCL NAL units is also satisfied when those NAL units (or just some of them) are conveyed to decoders (or to the HRD) by other means not specified by this Recommendation | International Standard. For the purpose of counting bits, only the appropriate bits that are actually present in the bitstream are counted.

NOTE 3 – As an example, synchronization of a non-VCL NAL unit, conveyed by means other than presence in the bitstream, with the NAL units that are present in the bitstream, can be achieved by indicating two points in the bitstream, between which the non-VCL NAL unit would have been present in the bitstream, had the encoder decided to convey it in the bitstream.

When the content of a non-VCL NAL unit is conveyed for the application by some means other than presence within the bitstream, the representation of the content of the non-VCL NAL unit is not required to use the same syntax specified in this annex.

NOTE 4 – When HRD information is contained within the bitstream, it is possible to verify the conformance of a bitstream to the requirements of this subclause based solely on information contained in the bitstream. When the HRD information is not present in the bitstream, as is the case for all "stand-alone" Type I bitstreams, conformance can only be verified when the HRD data is supplied by some other means not specified in this Recommendation | International Standard.

The HRD contains a coded picture buffer (CPB), an instantaneous decoding process, a decoded picture buffer (DPB), and output cropping as shown in Figure C-2.

- Figure C-2 – HRD buffer model

The CPB size (number of bits) is CpbSize[ SchedSelIdx ]. The DPB size (number of frame buffers) is Max( 1, max_dec_frame_buffering ). When the coded video sequence conforms to one or more of the profiles specified in Annex Error: Reference source not found and the decoding process specified in Annex Error: Reference source not found is applied, the DPB size is specified in units of view components. When the coded video sequence conforms to one or more of the profiles specified in Annex I and the decoding process specified in Annex I is applied, the DPB size is specified in units of texture view components and depth view components.

The HRD operates as follows. Data associated with access units that flow into the CPB according to a specified arrival schedule are delivered by the HSS. The data associated with each access unit are removed and decoded instantaneously by the instantaneous decoding process at CPB removal times. Each decoded picture is placed in the DPB at its CPB removal time unless it is output at its CPB removal time and is a non-reference picture. When a picture is placed in the DPB it is removed from the DPB at the later of the DPB output time or the time that it is marked as "unused for reference".

For each picture in the bitstream, the variable OutputFlag for the decoded picture and, when applicable, the reference base picture is set as follows:

–	If the coded video sequence containing the picture conforms to one or more of the profiles specified in Annex Error: Reference source not found and the decoding process specified in clauses Error: Reference source not found-Error: Reference source not found is applied, OutputFlag is set equal to 1.

–	Otherwise, if the coded video sequence containing the picture conforms to one or more of the profiles specified in Annex Error: Reference source not found and the decoding process specified in Annex Error: Reference source not found is applied, the following applies:

–	For a reference base picture, OutputFlag is set equal to 0.

–	For a decoded picture, OutputFlag is set equal to the value of the output_flag syntax element of the target layer representation.

–	Otherwise (the coded video sequence containing the picture conforms to one or more of the profiles specified in Annex Error: Reference source not found and the decoding process specified in Annex Error: Reference source not found is applied), the following applies:

–	For the decoded view components of the target output views, OutputFlag is set equal to 1.

–	For the decoded view components of other views, OutputFlag is set equal to 0.

–	Otherwise (the coded video sequence containing the picture conforms to one or more of the profiles specified in Annex I and the decoding process specified in Annex I is applied), the following applies:

–	For the decoded texture view components and depth view component with same VOIdx of the target output views, OutputFlag is set equal to 1.

–	For the decoded texture view components and depth view component with same VOIdx of other views, OutputFlag is set equal to 0.

The operation of the CPB is specified in subclause C.1. The instantaneous decoder operation is specified in clauses Error: Reference source not found-Error: Reference source not found (for coded video sequences conforming to one or more of the profiles specified in Annex Error: Reference source not found) and in Annex Error: Reference source not found (for coded video sequences conforming to one or more of the profiles specified in Annex Error: Reference source not found) and in Annex Error: Reference source not found (for coded video sequences conforming to one or more of the profiles specified in Annex Error: Reference source not found) and in Annex I (for coded video sequences conforming to one or more of the profiles specified in Annex I). The operation of the DPB is specified in subclause C.2. The output cropping is specified in subclause C.2.2.

NOTE 5 – Coded video sequences that conform to both, one or more of the profiles specified in Annex Error: Reference source not found and one or more of the profiles specified in Annex Error: Reference source not found, can be decoded either by the decoding process specified in clauses Error: Reference source not found-Error: Reference source not found or by the decoding process specified in Annex Error: Reference source not found. The decoding result and the HRD operation may be depending on which of the decoding processes is applied.

NOTE 6 – Coded video sequences that conform to both, one or more of the profiles specified in Annex Error: Reference source not found and one or more of the profiles specified in Annex Error: Reference source not found, can be decoded either by the decoding process specified in clauses Error: Reference source not found-Error: Reference source not found or by the decoding process specified in Annex Error: Reference source not found. The decoding result and the HRD operation may be depending on which of the decoding processes is applied.

NOTE 7 – Coded video sequences that conform to one or more of the profiles specified in Annex Error: Reference source not found, one or more of the profiles specified in Annex H and one or more of the profiles specified in Annex I, can be decoded either by the decoding process specified in clauses Error: Reference source not found-Error: Reference source not found, by the decoding process specified in Annex H or by the decoding process specified in Annex I. The decoding result and the HRD operation may be depending on which of the decoding processes is applied.

HSS and HRD information concerning the number of enumerated delivery schedules and their associated bit rates and buffer sizes is specified in subclauses Error: Reference source not found, Error: Reference source not found, Error: Reference source not found, Error: Reference source not found, Error: Reference source not found, Error: Reference source not found, Error: Reference source not found, Error: Reference source not found and I.14. The HRD is initialised as specified by the buffering period SEI message as specified in subclauses Error: Reference source not found and Error: Reference source not found. The removal timing of access units from the CPB and output timing from the DPB are specified in the picture timing SEI message as specified in subclauses Error: Reference source not found and Error: Reference source not found. All timing information relating to a specific access unit shall arrive prior to the CPB removal time of the access unit.

When the coded video sequence conforms to one or more of the profiles specified in Annex Error: Reference source not found and the decoding process specified in Annex Error: Reference source not found is applied, the following is specified:

- When an access unit contains one or more buffering period SEI messages that are included in scalable nesting SEI messages and are associated with values of DQId in the range of ( ( DQIdMax >> 4) << 4 ) to ( ( ( DQIdMax >> 4 ) << 4 ) + 15 ), inclusive, the last of these buffering period SEI messages in decoding order is the buffering period SEI message that initialises the HRD. Let hrdDQId be the largest value of 16 * sei_dependency_id[ i ] + sei_quality_id[ i ] that is associated with the scalable nesting SEI message containing the buffering period SEI message that initialises the HRD, let hrdDId and hrdQId be equal to hrdDQId >> 4 and hrdDQId & 15, respectively, and let hrdTId be the value of sei_temporal_id that is associated with the scalable nesting SEI message containing the buffering period SEI message that initialises the HRD.

- The picture timing SEI messages that specify the removal timing of access units from the CPB and output timing from the DPB are the picture timing SEI messages that are included in scalable nesting SEI messages associated with values of sei_dependency_id[ i ], sei_quality_id[ i ], and sei_temporal_id equal to hrdDId, hrdQId, and hrdTId, respectively.

- The HRD parameter sets that are used for conformance checking are the HRD parameter sets, included in the SVC video usability information extension of the active SVC sequence parameter set, that are associated with values of vui_ext_dependency_id[ i ], vui_ext_quality_id[ i ], and vui_ext_temporal_id[ i ] equal to hrdDId, hrdQId, and hrdTId, respectively. For the specification in this annex, num_units_in_tick, time_scale, fixed_frame_rate_flag, nal_hrd_parameters_present_flag, vcl_hrd_parameters_present_flag, low_delay_hrd_flag, and pic_struct_present_flag are substituted with the values of vui_ext_num_units_in_tick[ i ], vui_ext_time_scale[ i ], vui_ext_fixed_frame_rate_flag[ i ], vui_ext_nal_hrd_parameters_present_flag[ i ], vui_ext_vcl_hrd_parameters_present_flag[ i ], vui_ext_low_delay_hrd_flag[ i ], and vui_ext_pic_struct_present_flag[ i ], respectively, with i being the value for which vui_ext_dependency_id[ i ], vui_ext_quality_id[ i ], and vui_ext_temporal_id[ i ] are equal to hrdDId, hrdQId, and hrdTId, respectively.

When the coded video sequence conforms to one or more of the profiles specified in Annex Error: Reference source not found and the decoding process specified in Annex Error: Reference source not found is applied, the following is specified:

- When an access unit contains one or more buffering period SEI messages that are included in MVC scalable nesting SEI messages, the buffering period SEI message that is associated with the operation point being decoded is the buffering period SEI message that initialises the HRD. Let hrdVId[ i ] be equal to sei_op_view_id[ i ] for all i in the range of 0 to num_view_components_op_minus1, inclusive, and let hrdTId be the value of sei_op_temporal_id, that are associated with the MVC scalable nesting SEI message containing the buffering period SEI message that initialises the HRD.

- The picture timing SEI messages that specify the removal timing of access units from the CPB and output timing from the DPB are the picture timing SEI messages that are included in MVC scalable nesting SEI messages associated with values of sei_op_view_id[ i ] equal to hrdVId[ i ] for all i in the range of 0 to num_view_components_op_minus1, inclusive, and sei_temporal_id equal to hrdTId.

- The HRD parameters that are used for conformance checking are the HRD parameters, included in the MVC video usability information extension of the active MVC sequence parameter set, that are associated with values of vui_mvc_view_id[ i ][ j ] for all j in the range of 0 to vui_mvc_num_target_output_views_minus1[ i ], inclusive, equal to hrdVId[ j ], and the value of vui_mvc_temporal_id[ i ] equal to hrdTId. For the specification in this annex, num_units_in_tick, time_scale, fixed_frame_rate_flag, nal_hrd_parameters_present_flag, vcl_hrd_parameters_present_flag, low_delay_hrd_flag, and pic_struct_present_flag are substituted with the values of vui_mvc_num_units_in_tick[ i ], vui_mvc_time_scale[ i ], vui_mvc_fixed_frame_rate_flag[ i ], vui_mvc_nal_hrd_parameters_present_flag[ i ], vui_mvc_vcl_hrd_parameters_present_flag[ i ], vui_mvc_low_delay_hrd_flag[ i ], and vui_mvc_pic_struct_present_flag[ i ], respectively, with i being the value for which vui_mvc_view_id[ i ] is equal to hrdVId[ j ] for all j in the range of 0 to vui_mvc_num_traget_output_views_minus1[ i ], inclusive, and vui_mvc_temporal_id[ i ] equal to hrdTId.

When the coded video sequence conforms to one or more of the profiles specified in Annex I and the decoding process specified in Annex I is applied, the following is specified:

- When an access unit contains one or more buffering period SEI messages that are included in 3DVC scalable nesting SEI messages, the buffering period SEI message that is associated with the operation point being decoded is the buffering period SEI message that initialises the HRD. Let hrdVId[ i ] be equal to sei_op_view_id[ i ] for all i in the range of 0 to num_view_components_op_minus1, inclusive, and let hrdTId be the value of sei_op_temporal_id, that are associated with the 3DVC scalable nesting SEI message containing the buffering period SEI message that initialises the HRD.

- The picture timing SEI messages that specify the removal timing of access units from the CPB and output timing from the DPB are the picture timing SEI messages that are included in 3DVC scalable nesting SEI messages associated with values of sei_op_view_id[ i ] equal to hrdVId[ i ] for all i in the range of 0 to num_view_components_op_minus1, inclusive, and sei_temporal_id equal to hrdTId.

- The HRD parameter sets that are used for conformance checking are the HRD parameter sets, included in the MVC video usability information extension of the active 3DVC sequence parameter set, that are associated with values of vui_mvc_view_id[ i ][ j ] for all j in the range of 0 to vui_mvc_num_target_output_views_minus1[ i ], inclusive, equal to hrdVId[ j ], and the value of vui_mvc_temporal_id[ i ] equal to hrdTId. For the specification in this annex, num_units_in_tick, time_scale, fixed_frame_rate_flag, nal_hrd_parameters_present_flag, vcl_hrd_parameters_present_flag, low_delay_hrd_flag, and pic_struct_present_flag are substituted with the values of vui_mvc_num_units_in_tick[ i ], vui_mvc_time_scale[ i ], vui_mvc_fixed_frame_rate_flag[ i ], vui_mvc_nal_hrd_parameters_present_flag[ i ], vui_mvc_vcl_hrd_parameters_present_flag[ i ], vui_mvc_low_delay_hrd_flag[ i ], and vui_mvc_pic_struct_present_flag[ i ], respectively, with i being the value for which vui_mvc_view_id[ i ] is equal to hrdVId[ j ] for all j in the range of 0 to vui_mvc_num_traget_output_views_minus1[ i ], inclusive, and vui_mvc_temporal_id[ i ] equal to hrdTId.

The HRD is used to check conformance of bitstreams and decoders as specified in subclauses C.3 and C.4, respectively.

NOTE 7 – While conformance is guaranteed under the assumption that all frame-rates and clocks used to generate the bitstream match exactly the values signalled in the bitstream, in a real system each of these may vary from the signalled or specified value.

All the arithmetic in this annex is done with real values, so that no rounding errors can propagate. For example, the number of bits in a CPB just prior to or after removal of an access unit is not necessarily an integer.

The variable tc is derived as follows and is called a clock tick:

- tc = num_units_in_tick  time_scale		(C-1)

The following is specified for expressing the constraints in this annex:

–	Let access unit n be the n-th access unit in decoding order with the first access unit being access unit 0.

–	Let picture n be the primary coded picture or the decoded primary picture of access unit n.

  - Operation of coded picture buffer (CPB)

The specifications in this subclause apply independently to each set of CPB parameters that is present and to both the Type I and Type II conformance points shown in Figure C-1.

    - Timing of bitstream arrival

The HRD may be initialised at any one of the buffering period SEI messages. Prior to initialisation, the CPB is empty.

NOTE – After initialisation, the HRD is not initialised again by subsequent buffering period SEI messages.

- Each access unit is referred to as access unit n, where the number n identifies the particular access unit. The access unit that is associated with the buffering period SEI message that initialises the CPB is referred to as access unit 0. The value of n is incremented by 1 for each subsequent access unit in decoding order.

The time at which the first bit of access unit n begins to enter the CPB is referred to as the initial arrival time tai( n ).

The initial arrival time of access units is derived as follows:

–	If the access unit is access unit 0, tai( 0 ) = 0,

–	Otherwise (the access unit is access unit n with n > 0), the following applies:

–	If cbr_flag[ SchedSelIdx ] is equal to 1, the initial arrival time for access unit n, is equal to the final arrival time (which is derived below) of access unit n − 1, i.e.,

- tai( n ) = taf( n − 1 )		(C-2)

–	Otherwise (cbr_flag[ SchedSelIdx ] is equal to 0), the initial arrival time for access unit n is derived by

- tai( n ) = Max( taf( n − 1 ), tai,earliest( n ) )		(C-3)

where tai,earliest( n ) is derived as follows:

–	If access unit n is not the first access unit of a subsequent buffering period, tai,earliest( n ) is derived as

- tai,earliest( n ) = tr,n( n ) − ( initial_cpb_removal_delay[ SchedSelIdx ] +
                                       initial_cpb_removal_delay_offset[ SchedSelIdx ] )  90000	(C-4)

with tr,n( n ) being the nominal removal time of access unit n from the CPB as specified in subclause C.1.2 and initial_cpb_removal_delay[ SchedSelIdx ] and initial_cpb_removal_delay_offset[ SchedSelIdx ] being specified in the previous buffering period SEI message.

–	Otherwise (access unit n is the first access unit of a subsequent buffering period), tai,earliest( n ) is derived as

- tai,earliest( n ) = tr,n( n ) − ( initial_cpb_removal_delay[ SchedSelIdx ]  90000 )	(C-5)

with initial_cpb_removal_delay[ SchedSelIdx ] being specified in the buffering period SEI message associated with access unit n.

The final arrival time for access unit n is derived by

- taf( n ) = tai( n ) + b( n )  BitRate[ SchedSelIdx ]		(C-6)

- where b( n ) is the size in bits of access unit n, counting the bits of the VCL NAL units and the filler data NAL units for the Type I conformance point or all bits of the Type II bitstream for the Type II conformance point, where the Type I and Type II conformance points are as shown in Figure C-1.

The values of SchedSelIdx, BitRate[ SchedSelIdx ], and CpbSize[ SchedSelIdx ] are constrained as follows:

–	If the content of the active sequence parameter sets for access unit n and access unit n − 1 differ, the HSS selects a value SchedSelIdx1 of SchedSelIdx from among the values of SchedSelIdx provided in the active sequence parameter set for access unit n that results in a BitRate[ SchedSelIdx1 ] or CpbSize[ SchedSelIdx1 ] for access unit n. The value of BitRate[ SchedSelIdx1 ] or CpbSize[ SchedSelIdx1 ] may differ from the value of BitRate[ SchedSelIdx0 ] or CpbSize[ SchedSelIdx0 ] for the value SchedSelIdx0 of SchedSelIdx that was in use for access unit n − 1.

–	Otherwise, the HSS continues to operate with the previous values of SchedSelIdx, BitRate[ SchedSelIdx ] and CpbSize[ SchedSelIdx ].

When the HSS selects values of BitRate[ SchedSelIdx ] or CpbSize[ SchedSelIdx ] that differ from those of the previous access unit, the following applies:

–	the variable BitRate[ SchedSelIdx ] comes into effect at time tai( n )

–	the variable CpbSize[ SchedSelIdx ] comes into effect as follows:

–	If the new value of CpbSize[ SchedSelIdx ] exceeds the old CPB size, it comes into effect at time tai( n ),

–	Otherwise, the new value of CpbSize[ SchedSelIdx ] comes into effect at the time tr( n ).

    - Timing of coded picture removal

When an access unit n is the access unit with n equal to 0 (the access unit that initialises the HRD), the nominal removal time of the access unit from the CPB is specified by

- tr,n( 0 ) = initial_cpb_removal_delay[ SchedSelIdx ]  90000	(C-7)

When an access unit n is the first access unit of a buffering period that does not initialise the HRD, the nominal removal time of the access unit from the CPB is specified by

- tr,n( n ) = tr,n( nb ) + tc * cpb_removal_delay( n )		(C-8)

where tr,n( nb ) is the nominal removal time of the first access unit of the previous buffering period and cpb_removal_delay( n ) is the value of cpb_removal_delay specified in the picture timing SEI message associated with access unit n.

The nominal removal time tr,n(n) of an access unit n that is not the first access unit of a buffering period is given by

- tr,n( n ) = tr,n( nb ) + tc * cpb_removal_delay( n )		(C-9)

where tr,n( nb ) is the nominal removal time of the first access unit of the current buffering period and cpb_removal_delay( n ) is the value of cpb_removal_delay specified in the picture timing SEI message associated with access unit n.

The removal time of access unit n is specified as follows:

–	If low_delay_hrd_flag is equal to 0 or tr,n( n ) >= taf( n ), the removal time of access unit n is specified by

- tr( n ) = tr,n( n )		(C-10)

–	Otherwise (low_delay_hrd_flag is equal to 1 and tr,n( n ) < taf( n )), the removal time of access unit n is specified by

- tr( n ) = tr,n( n ) + tc * Ceil( ( taf( n ) − tr,n( n ) ) tc ) 		(C-11)

NOTE – The latter case indicates that the size of access unit n, b( n ), is so large that it prevents removal at the nominal removal time.

When an access unit n is the first access unit of a buffering period, nb is set equal to n at the removal time tr( n ) of the access unit n.

  - Operation of the decoded picture buffer (DPB)

The decoded picture buffer contains frame buffers. When a coded video sequence conforming to one or more of the profiles specified in Annex Error: Reference source not found is decoded by applying the decoding process specified in clauses Error: Reference source not found-Error: Reference source not found, each of the frame buffers may contain a decoded frame, a decoded complementary field pair or a single (non-paired) decoded field that is marked as "used for reference" (reference pictures) or is held for future output (reordered or delayed pictures). When a coded video sequence conforming to one or more of the profiles specified in Annex Error: Reference source not found is decoded by applying the decoding process specified in Annex Error: Reference source not found, each frame buffer may contain a decoded frame, a decoded complementary field pair, a single (non-paired) decoded field, a decoded reference base frame, a decoded reference base complementary field pair or a single (non-paired) decoded reference base field that is marked as "used for reference" (reference pictures) or is held for future output (reordered or delayed pictures). When a coded video sequence conforming to one or more of the profiles specified in Annex Error: Reference source not found is decoded by applying the decoding process specified in Annex Error: Reference source not found, each of the frame buffers may contain a decoded frame view component, a decoded complementary field view component pair, or a single (non-paired) decoded field view component that is marked as "used for reference" (reference pictures) or is held for future output (reordered or delayed pictures) or is held as reference for inter-view prediction (inter-view only reference components). When a coded video sequence conforming to one or more of the profiles specified in Annex I is decoded by applying the decoding process specified in Annex I, each of the frame buffers may contain a decoded texture frame view component, a decoded depth frame view component, a decoded complementary texture field view component pair, or a single (non-paired) decoded texture field view component that is marked as "used for reference" (reference pictures) or is held for future output (reordered or delayed pictures) or is held as reference for inter-view prediction (inter-view only reference components).

Prior to initialisation, the DPB is empty (the DPB fullness is set to zero). The following steps specified in this subclause all happen instantaneously at tr( n ) and in the order listed. When the decoding process specified in Annex Error: Reference source not found or Annex I is applied, the view components of the current primary coded picture are processed by applying the ordered steps to each view component in increasing order of the associated view order index VOIdx. During the invocation of the process for a particular texture view, only the texture view components of the particular view are considered and during the invocation of the process for a particular depth view, only the depth view components of the particular view are considered. Prior to initialisation, the DPB is empty (the DPB fullness is set to zero). Inside each view component of the current primary coded picture, depth view component if present, is processed after the texture view component within the same view component.

- The process of decoding gaps in frame_num and storing "non-existing" frames as specified in subclause C.2.1 is invoked.

- The picture decoding and output process as specified in subclause C.2.2 is invoked.

- The process of removing pictures from the DPB before possible insertion of the current picture as specified in subclause C.2.3 is invoked.

- The process of marking and storing the current decoded picture as specified in subclause C.2.4 is invoked.

NOTE – When the decoding process specified in Annex Error: Reference source not found is applied, the DPB is only operated for decoded pictures and reference base pictures associated with decoded pictures. The DPB is not operated for layer pictures with dependency_id less than DependencyIdMax (and associated reference base pictures). All decoded pictures and associated reference base pictures are decoded pictures and associated reference base pictures for dependency_id equal to DependencyIdMax, which represent the results of the decoding process specified in subclause Error: Reference source not found.

    - Decoding of gaps in frame_num and storage of "non-existing" frames

When the decoding process specified in Annex Error: Reference source not found is applied, the process specified in this subclause is invoked for a particular view with view order index VOIdx, with "picture" being replaced by "view component", "frame" being replaced by "frame view component", and "field" being replaced by "field view component". During the invocation of the process for a particular view, only view components of the particular view are considered and view components of other views are not marked as "unused for reference" or removed from the DPB. The DPB fullness represents the total number of non-empty frame buffers, including frame buffers that contain view components of other views. When the decoding process specified in Annex I is applied, the process specified in this subclause for Annex Error: Reference source not found is invoked for particular texture view or depth view with view order index VOIdx, with each "view component" being replaced by "texture view component" or "depth view component", "frame view component" being replaced by "texture frame view component" or "depth frame view component", and "field view component" being replaced by "texture field view component". During the invocation of the process for a particular texture view, only the texture view components of the particular view are considered and during the invocation of the process for a particular depth view, only the depth view components of the particular view are considered and view components of other views are not marked as "unused for reference" or removed from the DPB. The DPB fullness represents the total number of non-empty frame buffers, including frame buffers that contain view components of other views.

When applicable, gaps in frame_num are detected by the decoding process and the generated frames are marked and inserted into the DPB as specified below.

Gaps in frame_num are detected by the decoding process and the generated frames are marked as specified in subclauses Error: Reference source not found and Error: Reference source not found.

After the marking of each generated frame, each picture m marked by the "sliding window" process as "unused for reference" is removed from the DPB when it is also marked as "non-existing" or its DPB output time is less than or equal to the CPB removal time of the current picture n; i.e., to,dpb( m ) <= tr( n ), or it has OutputFlag equal to 0. When a frame or the last field in a frame buffer is removed from the DPB, the DPB fullness is decremented by one. The "non-existing" generated frame is inserted into the DPB and the DPB fullness is incremented by one.

    - Picture decoding and output

When the decoding process specified in Annex Error: Reference source not found is applied, the process specified in this subclause is invoked for a particular view with view order index VOIdx.

When the decoding process specified in Annex I is applied, the process specified in this subclause in a particular texture view or depth view with view order index VOIdx.

The decoding of the current picture or view component (when applying the decoding process specified in Annex Error: Reference source not found or Annex I) and the derivation of the DPB output time (if applicable) is specified as follows:

–	If the decoding process specified in clause Error: Reference source not found or Annex Error: Reference source not found is applied, the following applies:

–	The current primary coded picture n is decoded.

–	When picture n has OutputFlag equal to 1, its DPB output time to,dpb( n ) is derived by

- to,dpb( n ) = tr( n ) + tc * dpb_output_delay( n )	(C-12)

where dpb_output_delay( n ) is the value of dpb_output_delay specified in the picture timing SEI message associated with access unit n.

–	Otherwise (the decoding process specified in Annex Error: Reference source not found or Annex I is applied), the following applies:

–	The view component with view order index VOIdx of the current primary coded picture n is decoded.

–	When VOIdx is equal to VOIdxMin and any of the view components of picture n has OutputFlag equal to 1, the DPB output time to,dpb( n ) for picture n is derived by Equation C-12, where dpb_output_delay( n ) is the value of dpb_output_delay specified in the picture timing SEI message associated with access unit n.

The output of the current picture or view component (when applying the decoding process specified in Annex Error: Reference source not found or Annex I) is specified as follows:

–	If OutputFlag is equal to 1 and to,dpb( n ) = tr( n ), the current picture or view component is output.

NOTE 1 – When the current picture or view component has nal_ref_idc greater than 0 (when using the decoding process specified in Annex Error: Reference source not found, nal_ref_idc is the syntax element of the target layer representation), it will be stored in the DPB.

–	Otherwise, if OutputFlag is equal to 0, the current picture or view component is not output, but it may be stored in the DPB as specified in subclause C.2.4.

–	Otherwise (OutputFlag is equal to 1 and to,dpb( n ) > tr( n ) ), the current picture or view component is output later and will be stored in the DPB (as specified in subclause C.2.4) and is output at time to,dpb( n ) unless indicated not to be output by the decoding or inference of no_output_of_prior_pics_flag equal to 1 at a time that precedes to,dpb( n ).

NOTE 2 – When the coded video sequence conforms to a profile specified in Annex Error: Reference source not found and the decoding process specified in Annex Error: Reference source not found is used, the view components of all the target output views of a picture are output at the same time instant and in increasing order of the view order index VOIdx.

NOTE 3 – When the coded video sequence conforms to a profile specified in Annex I and the decoding process specified in Annex I is used, the view components of all the target output views of a picture are output at the same time instant and in increasing order of the view order index VOIdx. A depth view component, if present, follows the texture view component within the same view component.

When output, the picture or view component shall be cropped, using the cropping rectangle specified in the active sequence parameter set for the picture or view component.

When the decoding process specified in clause Error: Reference source not found or Annex Error: Reference source not found is applied, the current picture n is a picture that is output and is not the last picture of the bitstream that is output, the value of to,dpb( n ) is derived by

- to,dpb( n ) = to,dpb( nn ) − to,dpb( n )		(C-13)

where nn indicates the picture that follows after picture n in output order and has OutputFlag equal to 1.

When the decoding process specified in Annex Error: Reference source not found or Annex I is applied, the current picture n is a picture that contains at least one view component that is output and the current picture is not the last picture of the bitstream that contains at least one view component that is output and VOIdx is equal to VOIdxMin, the value of to,dpb( n ) is derived by Equation C-13, where nn indicates the picture that follows after picture n in output order and contains at least one any view component with OutputFlag equal to 1.

The decoded picture or view component is temporarily stored (not in the DPB).

    - Removal of pictures from the DPB before possible insertion of the current picture

When the decoding process specified in Annex Error: Reference source not found is applied, the process specified in this subclause is invoked for a particular view with view order index VOIdx, with "picture" being replaced by "view component", "frame" being replaced by "frame view component", and "field" being replaced by "field view component".

When the decoding process specified in Annex I is applied, the process specified in this subclause for Annex Error: Reference source not found is invoked for particular texture view and depth view with view order index VOIdx, with each "view component" being replaced by "texture view component" or "depth view component", "frame view component" being replaced by "texture frame view component" or "depth frame view component", and "field view component" being replaced by "texture field view component". During the invocation of the process for a particular texture view, only the texture view components of the particular view are considered and during the invocation of the process for a particular depth view, only the depth view components of the particular view are considered.

When the decoding process specified in Annex Error: Reference source not found or Annex I is applied, the following process is specified for removing inter-view only reference components of the current access unit from the DPB. By this process, view components of the current view with view order index VOIdx are not removed from the DPB, but inter-view only reference components of other views may be removed. The removal of inter-view only reference components is specified as follows:

–	If the view order index VOIdx of the current view is equal to VOIdxMax, all inter-view only reference components m for which any of the following conditions are true are removed from the DPB:

–	OutputFlag is equal to 0,

–	The DPB output time to,dpb( m ) of the picture containing the view component m is less than or equal to the CPB removal time tr( n ) of the current picture.

–	Otherwise (the view order index VOIdx of the current view is less than VOIdxMax), all inter-view only reference components m for which both of the following conditions are true are removed from the DPB:

–	OutputFlag is equal to 0 or the DPB output time to,dpb( m ) of the picture containing the view component m is less than or equal to the CPB removal time tr( n ) of the current picture,

–	One of the following conditions is true:

–	the current view component is a view component of an anchor picture and the view_id of the inter-view only reference component m is not equal to any value of anchor_ref_lX[ k ][ j ], with X being equal to 0 or 1, k being any integer value greater than the view order index VOIdx of the current view, and j being any integer value in the range of 0 to Max( 0, num_anchor_refs_lX[ k ] − 1 ), inclusive,

–	the current view component is not a view component of an anchor picture and the view_id of the inter-view only reference component m is not equal to any value of non_anchor_ref_lX[ k ][ j ], with X being equal to 0 or 1, k being any integer value greater than the view order index VOIdx of the current view, and j being any integer value in the range of 0 to Max( 0, num_non_anchor_refs_lX[ k ] − 1 ), inclusive.

When the decoding process specified in Annex Error: Reference source not found is applied, for the following processes specified in this subclause, only view components of the particular view for which this subclause is invoked are considered, and view components of other views are not marked as "unused for reference" or removed from the DPB. When the decoding process specified in Annex I is applied, for the following processes specified for Annex I in this subclause, during the invocation of the process for a particular texture view, only texture view components of the particular texture view are considered and during the invocation of the process for a particular depth view, only depth view components of the particular depth view  are considered, and view components of other views are not marked as "unused for reference" or removed from the DPB. The DPB fullness represents the total number of non-empty frame buffers, including frame buffers that contain texture or depth view components of the current view and other views.

The removal of pictures from the DPB before possible insertion of the current picture proceeds as follows:

–	If the decoded picture is an IDR picture the following applies:

- All reference pictures in the DPB are marked as "unused for reference" as specified in subclause Error: Reference source not found when a coded video sequence conforming to one or more of the profiles specified in Annex Error: Reference source not found is decoded by applying the decoding process specified in clauses Error: Reference source not found-Error: Reference source not found, or as specified in subclause Error: Reference source not found when a coded video sequence conforming to one or more of the profiles specified in Annex Error: Reference source not found is decoded by applying the decoding process specified in Annex Error: Reference source not found, or as specified in subclause I.8.3 when a coded video sequence conforming to one or more of the profiles specified in Annex Error: Reference source not found is decoded by applying the decoding process specified in Annex Error: Reference source not found, or as specified in subclause I.8.3 when a coded video sequence conforming to one or more of the profiles specified in Annex I is decoded by applying the decoding process specified in Annex I.

- When the IDR picture is not the first IDR picture decoded and the value of PicWidthInMbs or FrameHeightInMbs or max_dec_frame_buffering derived from the active sequence parameter set is different from the value of PicWidthInMbs or FrameHeightInMbs or max_dec_frame_buffering derived from the sequence parameter set that was active for the preceding picture, respectively, no_output_of_prior_pics_flag is inferred to be equal to 1 by the HRD, regardless of the actual value of no_output_of_prior_pics_flag.

NOTE – Decoder implementations should try to handle frame or DPB size changes more gracefully than the HRD in regard to changes in PicWidthInMbs or FrameHeightInMbs.

- When no_output_of_prior_pics_flag is equal to 1 or is inferred to be equal to 1, all frame buffers in the DPB are emptied without output of the pictures they contain, and DPB fullness is set to 0.

–	Otherwise (the decoded picture is not an IDR picture), the following applies:

–	If the slice header of the current picture includes memory_management_control_operation equal to 5, all reference pictures in the DPB are marked as "unused for reference".

–	Otherwise (the slice header of the current picture does not include memory_management_control_operation equal to 5), the decoded reference picture marking process specified in subclause Error: Reference source not found is invoked when a coded video sequence conforming to one or more of the profiles specified in Annex Error: Reference source not found is decoded by applying the decoding process specified in clauses Error: Reference source not found-Error: Reference source not found, or the decoded reference picture marking process specified in subclause Error: Reference source not found is invoked when a coded video sequence conforming to one or more of the profiles specified in Annex Error: Reference source not found is decoded by applying the decoding process specified in Annex Error: Reference source not found, or the decoded reference picture marking process specified in subclause I.8.3 is invoked when a coded video sequence conforming to one or more of the profiles specified in Annex Error: Reference source not found is decoded by applying the decoding process specified in Annex Error: Reference source not found, or the decoded reference picture marking process specified in subclause I.8.3 is invoked when a coded video sequence conforming to one or more of the profiles specified in Annex I is decoded by applying the decoding process specified in Annex I.

All pictures m in the DPB, for which all of the following conditions are true, are removed from the DPB:

–	picture m is marked as "unused for reference" or picture m is a non-reference picture. When a picture is a reference frame, it is considered to be marked as "unused for reference" only when both of its fields have been marked as "unused for reference",

–	picture m is marked as "non-existing" or it has OutputFlag equal to 0 or its DPB output time to,dpb( m ) is less than or equal to the CPB removal time tr( n ) of the current picture n.

When a frame or the last field in a frame buffer is removed from the DPB, the DPB fullness is decremented by one.

    - Current decoded picture marking and storage

When the decoding process specified in Annex Error: Reference source not found is applied, the process specified in this subclause is invoked for a particular view with view order index VOIdx, with "picture" being replaced by "view component", "frame" being replaced by "frame view component", and "field" being replaced by "field view component". When the decoding process specified in Annex I is applied, the process specified in this subclause for Annex Error: Reference source not found is invoked for particular texture view and depth view with view order index VOIdx, with each "view component" being replaced by "texture view component" and "depth view component", "frame view component" being replaced by "texture frame view component" and "depth frame view component", and "field view component" being replaced by "texture field view component".In subclause C.2.4.2, the DPB output time to,dpb(n) and the CPB removal time tr(n) of a view component are the DPB output time and the CPB removal time of the picture n containing the view component, depth view component or texture view component.

The marking and storage of the current decoded picture is specified as follows:

–	If the current picture is a reference picture, the marking and storage process for reference pictures as specified in subclause C.2.4.1 is invoked.

–	Otherwise (the current picture is a non-reference picture), the storage process for non-reference pictures as specified in subclause C.2.4.2 is invoked.

      - Marking and storage of a reference picture into the DPB

The current picture is stored in the DPB as follows:

–	If the current decoded picture is a second field (in decoding order) of a complementary reference field pair, and the first field of the pair is still in the DPB, the current decoded picture is stored in the same frame buffer as the first field of the pair.

–	Otherwise, the current decoded picture is stored in an empty frame buffer, and the DPB fullness is incremented by one.

When the coded video sequence conforms to one or more of the profiles specified in Annex Error: Reference source not found and the decoding process specified in Annex Error: Reference source not found is applied and the current picture has store_ref_base_pic_flag equal to 1 (i.e., the current picture is associated with a reference base picture), the associated reference base picture is stored in the DPB as follows:

–	If the reference base picture is a second field (in decoding order) of a complementary reference base field pair, and the first field of the pair is still in the DPB, the reference base picture is stored in the same frame buffer as the first field of the pair.

–	Otherwise, the reference base picture is stored in an empty frame buffer, and the DPB fullness is incremented by one.

      - Storage of a non-reference picture into the DPB

The variable storePicFlag is derived as follows:

–	If any of the following conditions are true, storePicFlag is set equal to 1:

–	the current picture n has OutputFlag equal to 1 and to,dpb(n) > tr(n),

–	the decoding process specified in Annex Error: Reference source not found or Annex I is used and the current view component has a view order index VOIdx less than VOIdxMax and inter_view_flag equal to 1.

–	Otherwise, storePicFlag is set equal to 0.

When storePicFlag is equal to 1, the current picture is stored in the DPB as follows:

–	If the current decoded picture is a second field (in decoding order) of a complementary non-reference field pair, and the first field of the pair is still in the DPB, the current decoded picture is stored in the same frame buffer as the first field of the pair.

–	Otherwise, the current decoded picture is stored in an empty frame buffer, and the DPB fullness is incremented by one.

  - Bitstream conformance

A bitstream of coded data conforming to this Recommendation | International Standard fulfils the following requirements.

The bitstream is constructed according to the syntax, semantics, and constraints specified in this Recommendation | International Standard outside of this annex.

The bitstream is tested by the HRD as specified below:

For Type I bitstreams, the number of tests carried out is equal to cpb_cnt_minus1 + 1 where cpb_cnt_minus1 is either the syntax element of hrd_parameters( ) following the vcl_hrd_parameters_present_flag or is determined by the application by other means not specified in this Recommendation | International Standard. One test is carried out for each bit rate and CPB size combination specified by hrd_parameters( ) following the vcl_hrd_parameters_present_flag. Each of these tests is conducted at the Type I conformance point shown in Figure C-1.

For Type II bitstreams there are two sets of tests. The number of tests of the first set is equal to cpb_cnt_minus1 + 1 where cpb_cnt_minus1 is either the syntax element of hrd_parameters( ) following the vcl_hrd_parameters_present_flag or is determined by the application by other means not specified in this Recommendation | International Standard. One test is carried out for each bit rate and CPB size combination. Each of these tests is conducted at the Type I conformance point shown in Figure C-1. For these tests, only VCL and filler data NAL units are counted for the input bit rate and CPB storage.

The number of tests of the second set, for Type II bitstreams, is equal to cpb_cnt_minus1 + 1 where cpb_cnt_minus1 is either the syntax element of hrd_parameters( ) following the nal_hrd_parameters_present_flag or is determined by the application by other means not specified in this Recommendation | International Standard. One test is carried out for each bit rate and CPB size combination specified by hrd_parameters( ) following the nal_hrd_parameters_present_flag. Each of these tests is conducted at the Type II conformance point shown in Figure C-1. For these tests, all NAL units (of a Type II NAL unit stream) or all bytes (of a byte stream) are counted for the input bit rate and CPB storage.

NOTE 1 – NAL HRD parameters established by a value of SchedSelIdx for the Type II conformance point shown in Figure C-1 are sufficient to also establish VCL HRD conformance for the Type I conformance point shown in Figure C-1 for the same values of initial_cpb_removal_delay[ SchedSelIdx ], BitRate[ SchedSelIdx ], and CpbSize[ SchedSelIdx ] for the VBR case (cbr_flag[ SchedSelIdx ] equal to 0). This is because the data flow into the Type I conformance point is a subset of the data flow into the Type II conformance point and because, for the VBR case, the CPB is allowed to become empty and stay empty until the time a next picture is scheduled to begin to arrive. For example, when a coded video sequence conforming to one or more of the profiles specified in Annex Error: Reference source not found is decoded by applying the decoding process specified in clauses Error: Reference source not found-Error: Reference source not found, when NAL HRD parameters are provided for the Type II conformance point that not only fall within the bounds set for NAL HRD parameters for profile conformance in item Error: Reference source not found of subclause Error: Reference source not found or item Error: Reference source not found of subclause Error: Reference source not found (depending on the profile in use) but also fall within the bounds set for VCL HRD parameters for profile conformance in item Error: Reference source not found of subclause Error: Reference source not found or item Error: Reference source not found of subclause Error: Reference source not found (depending on the profile in use), conformance of the VCL HRD for the Type I conformance point is also assured to fall within the bounds of item Error: Reference source not found of subclause Error: Reference source not found.

For conforming bitstreams, all of the following conditions shall be fulfilled for each of the tests:

- For each access unit n, with n>0, associated with a buffering period SEI message, with tg,90( n ) specified by

- tg,90( n ) = 90000 * ( tr,n( n ) − taf( n − 1 ) )	(C-14)

the value of initial_cpb_removal_delay[ SchedSelIdx ] shall be constrained as follows:

–	If cbr_flag[ SchedSelIdx ] is equal to 0,

- initial_cpb_removal_delay[ SchedSelIdx ] <= Ceil( tg,90( n ) )	(C-15)

–	Otherwise (cbr_flag[ SchedSelIdx ] is equal to 1),

- Floor( tg,90( n ) ) <= 	initial_cpb_removal_delay[ SchedSelIdx ] <= Ceil( tg,90( n ) )	(C-16)

- NOTE 2 – The exact number of bits in the CPB at the removal time of each picture may depend on which buffering period SEI message is selected to initialise the HRD. Encoders must take this into account to ensure that all specified constraints must be obeyed regardless of which buffering period SEI message is selected to initialise the HRD, as the HRD may be initialised at any one of the buffering period SEI messages.

- A CPB overflow is specified as the condition in which the total number of bits in the CPB is larger than the CPB size. The CPB shall never overflow.

- A CPB underflow is specified as the condition in which tr,n( n ) is less than taf( n ). When low_delay_hrd_flag is equal to 0, the CPB shall never underflow.

- The nominal removal times of pictures from the CPB (starting from the second picture in decoding order), shall satisfy the constraints on tr,n( n ) and tr( n ) expressed in subclauses Error: Reference source not found through Error: Reference source not found for the profile and level specified in the bitstream when a coded video sequence conforming to one or more of the profiles specified in Annex Error: Reference source not found is decoded by applying the decoding process specified in clauses Error: Reference source not found-Error: Reference source not found, and they shall satisfy the constraints on tr,n( n ) and tr( n ) expressed in subclauses Error: Reference source not found and Error: Reference source not found for profile and level specified in the bitstream when a coded video sequence conforming to one or more of the profiles specified in Annex Error: Reference source not found is decoded by applying the decoding process specified in Annex Error: Reference source not found, and they shall satisfy the constraints on tr,n( n ) and tr( n ) expressed in subclause I.10.2 for the profile and level specified in the bitstream when a coded video sequence conforming to one or more of the profiles specified in Annex Error: Reference source not found is decoded by applying the decoding process specified in Annex Error: Reference source not found, and they shall satisfy the constraints on tr,n( n ) and tr( n ) expressed in subclause I.10.2 for the profile and level specified in the bitstream when a coded video sequence conforming to one or more of the profiles specified in Annex I is decoded by applying the decoding process specified in Annex I.

- Immediately after any decoded picture is added to the DPB, the fullness of the DPB shall be less than or equal to the DPB size as constrained by Annexes Error: Reference source not found, Error: Reference source not found, and Error: Reference source not found and subclauses Error: Reference source not found, Error: Reference source not found, Error: Reference source not found, I.10, Error: Reference source not found, I.14, and I.14 for the profile and level specified in the bitstream.

- All reference pictures shall be present in the DPB when needed for prediction. Each picture shall be present in the DPB at its DPB output time unless it is not stored in the DPB at all, or is removed from the DPB before its output time by one of the processes specified in subclause C.2.

- The value of to,dpb( n ) as given by Equation C-13, which is the difference between the output time of a picture and that of the first picture following it in output order and having OutputFlag equal to 1, shall satisfy the constraint expressed in subclause Error: Reference source not found for the profile and level specified in the bitstream when a coded video sequence conforming to one or more of the profiles specified in Annex Error: Reference source not found is decoded by applying the decoding process specified in clauses Error: Reference source not found-Error: Reference source not found, and it shall satisfy the constraint expressed in subclause Error: Reference source not found for profile and level specified in the bitstream when a coded video sequence conforming to one or more of the profiles specified in Annex Error: Reference source not found is decoded by applying the decoding process specified in Annex Error: Reference source not found, and it shall satisfy the constraints expressed in subclause I.10.2 for the profile and level specified in the bitstream when a coded video sequence conforming to one or more of the profiles specified in Annex Error: Reference source not found is decoded by applying the decoding process specified in Annex Error: Reference source not found, and it shall satisfy the constraints expressed in subclause I.10.2 for the profile and level specified in the bitstream when a coded video sequence conforming to one or more of the profiles specified in Annex I is decoded by applying the decoding process specified in Annex I.

  - Decoder conformance

A decoder conforming to this Recommendation | International Standard fulfils the following requirements.

A decoder claiming conformance to a specific profile and level shall be able to decode successfully all conforming bitstreams specified for decoder conformance in subclause C.3, provided that all sequence parameter sets and picture parameter sets referred to in the VCL NAL units, and appropriate buffering period and picture timing SEI messages are conveyed to the decoder, in a timely manner, either in the bitstream (by non-VCL NAL units), or by external means not specified by this Recommendation | International Standard.

There are two types of conformance that can be claimed by a decoder: output timing conformance and output order conformance.

To check conformance of a decoder, test bitstreams conforming to the claimed profile and level, as specified in subclause C.3 are delivered by a hypothetical stream scheduler (HSS) both to the HRD and to the decoder under test (DUT). All pictures output by the HRD shall also be output by the DUT and, for each picture output by the HRD, the values of all samples that are output by the DUT for the corresponding picture shall be equal to the values of the samples output by the HRD.

For output timing decoder conformance, the HSS operates as described above, with delivery schedules selected only from the subset of values of SchedSelIdx for which the bit rate and CPB size are restricted as specified in Annex Error: Reference source not found, Annex Error: Reference source not found, Annex Error: Reference source not found and Annex I for the specified profile and level, or with "interpolated" delivery schedules as specified below for which the bit rate and CPB size are restricted as specified in Annex Error: Reference source not found, Annex Error: Reference source not found, Annex Error: Reference source not found, and Annex I. The same delivery schedule is used for both the HRD and DUT.

When the HRD parameters and the buffering period SEI messages are present with cpb_cnt_minus1 greater than 0, the decoder shall be capable of decoding the bitstream as delivered from the HSS operating using an "interpolated" delivery schedule specified as having peak bit rate r, CPB size c( r ), and initial CPB removal delay ( f( r )r ) as follows:

-  = ( r − BitRate[ SchedSelIdx − 1 ] )  ( BitRate[ SchedSelIdx ] − BitRate[ SchedSelIdx − 1 ] ),	(C-17)

- c( r ) =  * CpbSize[ SchedSelIdx ] + (1 −  * CpbSize[ SchedSelIdx−1 ],	(C-18)

- f( r ) = initial_cpb_removal_delay[ SchedSelIdx ] * BitRate[ SchedSelIdx ] + 
		( 1 −  initial_cpb_removal_delay[ SchedSelIdx − 1 ] * BitRate[ SchedSelIdx − 1 ]	(C-19)

for any SchedSelIdx > 0 and r such that BitRate[ SchedSelIdx − 1 ] <= r <= BitRate[ SchedSelIdx ] such that r and c( r ) are within the limits as specified in Annex Error: Reference source not found, Annex Error: Reference source not found, Annex Error: Reference source not found, and Annex I for the maximum bit rate and buffer size for the specified profile and level.

NOTE 1 – initial_cpb_removal_delay[ SchedSelIdx ] can be different from one buffering period to another and have to be re-calculated.

For output timing decoder conformance, an HRD as described above is used and the timing (relative to the delivery time of the first bit) of picture output is the same for both HRD and the DUT up to a fixed delay.

For output order decoder conformance, the HSS delivers the bitstream to the DUT "by demand" from the DUT, meaning that the HSS delivers bits (in decoding order) only when the DUT requires more bits to proceed with its processing.

NOTE 2 – This means that for this test, the coded picture buffer of the DUT could be as small as the size of the largest access unit.

A modified HRD as described below is used, and the HSS delivers the bitstream to the HRD by one of the schedules specified in the bitstream such that the bit rate and CPB size are restricted as specified in Annex Error: Reference source not found, Annex Error: Reference source not found, Annex Error: Reference source not found, and Annex I. The order of pictures output shall be the same for both HRD and the DUT.

For output order decoder conformance, the HRD CPB size is equal to CpbSize[ SchedSelIdx ] for the selected schedule and the DPB size is equal to MaxDpbFrames. Removal time from the CPB for the HRD is equal to final bit arrival time and decoding is immediate. The operation of the DPB of this HRD is specified in subclause C.4.1.

    - Operation of the output order DPB

The decoded picture buffer contains frame buffers. When a coded video sequence conforming to one or more of the profiles specified in Annex Error: Reference source not found is decoded by applying the decoding process specified in clauses Error: Reference source not found-Error: Reference source not found, each of the frame buffers may contain a decoded frame, a decoded complementary field pair or a single (non-paired) decoded field that is marked as "used for reference" or is held for future output (reordered pictures). When a coded video sequence conforming to one or more of the profiles specified in Annex Error: Reference source not found is decoded by applying the decoding process specified in Annex Error: Reference source not found, each frame buffer may contain a decoded frame, a decoded complementary field pair, a single (non-paired) decoded field, a decoded reference base frame, a decoded reference base complementary field pair or a single (non-paired) decoded reference base field that is marked as "used for reference" (reference pictures) or is held for future output (reordered or delayed pictures). When a coded video sequence conforming to one or more of the profiles specified in Annex Error: Reference source not found is decoded by applying the decoding process specified in Annex Error: Reference source not found, each of the frame buffers may contain a decoded frame view component, a decoded complementary field view component pair, or a single (non-paired) decoded field view component that is marked as "used for reference" (reference pictures) or is held for future output (reordered or delayed pictures) or is held for inter-view prediction (inter-view only reference components). When a coded video sequence conforming to one or more of the profiles specified in Annex I is decoded by applying the decoding process specified in Annex I, each of the frame buffers may contain a decoded texture frame view component, a decoded depth frame view component, a decoded complementary texture field view component pair, a single (non-paired) decoded texture field view component that is marked as "used for reference" (reference pictures) or is held for future output (reordered or delayed pictures) or is held for inter-view prediction (inter-view only reference components).

At HRD initialisation, the DPB fullness, measured in non-empty frame buffers, is set equal to 0. The following steps all happen instantaneously when an access unit is removed from the CPB, and in the order listed. When the decoding process specified in Annex Error: Reference source not found or Annex I is applied, the view components of the current primary coded picture are processed by applying the ordered steps to each view component in increasing order of the associated view order index VOIdx. The invocation of the process for a depth view component, if present, follows the invocation of the process for the texture view component within the same view component.

- The process of decoding gaps in frame_num and storing "non-existing" frames as specified in subclause C.4.2 is invoked.

- The picture decoding and output process as specified in subclause C.4.3 is invoked.

- The process of removing pictures from the DPB before possible insertion of the current picture as specified in subclause C.4.4 is invoked.

- The process of marking and storing the current decoded picture as specified in subclause C.4.5 is invoked.

NOTE – When the decoding process specified in Annex Error: Reference source not found is applied, the DPB is only operated for decoded pictures and reference base pictures associated with decoded pictures. The DPB is not operated for layer pictures with dependency_id less than DependencyIdMax (and associated reference base pictures). All decoded pictures and associated reference base pictures are decoded pictures and associated reference base pictures for dependency_id equal to DependencyIdMax, which represent the results of the decoding process specified in subclause Error: Reference source not found.

    - Decoding of gaps in frame_num and storage of "non-existing" pictures

When the decoding process specified in Annex Error: Reference source not found is applied, the process specified in this subclause is invoked for a particular view with view order index VOIdx, with "picture" being replaced by "view component", "frame" being replaced by "frame view component", and "field" being replaced by "field view component". During the invocation of the process for a particular view, only view components of the particular view are considered and view components of other views are not marked as "unused for reference" or removed from the DPB. The DPB fullness represents the total number of non-empty frame buffers, including frame buffers that contain texture or depth view components of the current view and other views.

When the decoding process specified in Annex I is applied, the process specified in this subclause for Annex Error: Reference source not found is invoked for particular texture view and depth view with view order index VOIdx, with each "view component" being replaced by "texture view component" or "depth view component", "frame view component" being replaced by "texture frame view component" or "depth frame view component", and "field view component" being replaced by "texture field view component". During the invocation of the process for a particular texture view, only the texture view components of the particular view are considered and during the invocation of the process for a particular depth view, only the depth view components of the particular view are considered and view components of other views are not marked as "unused for reference" or removed from the DPB.

When applicable, gaps in frame_num are detected by the decoding process and the necessary number of "non-existing" frames are inferred in the order specified by the generation of values of UnusedShortTermFrameNum in Equation Error: Reference source not found and are marked as specified in subclauses Error: Reference source not found and Error: Reference source not found. Frame buffers containing a frame or a complementary field pair or a non-paired field which are marked as "not needed for output" and "unused for reference" are emptied (without output), and the DPB fullness is decremented by the number of frame buffers emptied. Each "non-existing" frame is stored in the DPB as follows:

–	When there is no empty frame buffer (i.e., DPB fullness is equal to DPB size), the "bumping" process specified in subclause C.4.5.3 is invoked repeatedly until there is an empty frame buffer in which to store the "non-existing" frame.

–	The "non-existing" frame is stored in an empty frame buffer and is marked as "not needed for output", and the DPB fullness is incremented by one.

    - Picture decoding

When the decoding process specified in Annex Error: Reference source not found is applied, the process specified in this subclause is invoked for a particular view with view order index VOIdx.

When the decoding process specified in Annex I is applied, the process specified for Annex Error: Reference source not found in this subclause is invoked for particular texture view and depth view with view order index VOIdx.

The decoding of the current picture or view component (when applying the decoding process specified in Annex Error: Reference source not found or Annex I) is specified as follows:

–	If the decoding process specified in clause Error: Reference source not found or Annex Error: Reference source not found is applied, the current primary coded picture n is decoded and is temporarily stored (not in the DPB).

–	Otherwise (the decoding process specified in Annex Error: Reference source not found or Annex I is applied), the view component with view order index VOIdx of the current primary coded picture n is decoded and is temporarily stored (not in the DPB).

    - Removal of pictures from the DPB before possible insertion of the current picture

When the decoding process specified in Annex Error: Reference source not found is applied, the process specified in this subclause is invoked for a particular view with view order index VOIdx, with "picture" being replaced by "view component", "frame" being replaced by "frame view component", and "field" being replaced by "field view component".

When the decoding process specified in Annex I is applied, the process specified in this subclause for Annex Error: Reference source not found is invoked for particular texture view and depth view with view order index VOIdx, with each "view component" being replaced by "texture view component" or "depth view component", "frame view component" being replaced by "texture frame view component" or "depth frame view component", and "field view component" being replaced by "texture field view component". During the invocation of the process for a particular texture view, only the texture view components of the particular view are considered and during the invocation of the process for a particular depth view, only the depth view components of the particular view are considered.

When the decoding process specified in Annex Error: Reference source not found or Annex I is applied, the following process is specified for emptying frame buffers containing inter-view only reference components of the current access unit. By this process, frame buffers that contain view components of the current view with view order index VOIdx are not emptied, but frame buffers that contain inter-view only reference components of other views may be emptied. The process is specified as follows:

–	If the view order index VOIdx of the current view is equal to VOIdxMax, all frame buffers containing a frame or a complementary field pair or a non-paired field which are marked as "not needed for output" and "unused for reference" are emptied (without output), and the DPB fullness is decremented by the number of frame buffers emptied.

NOTE 1 – At this stage of the process, all frame buffers that contain a frame or a complementary field pair or a non-paired field marked as "not needed for output" and "unused for reference" are frame buffers that contain an inter-view only reference component (of the current access unit and a view with view order index less than VOIdx) with OutputFlag equal to 0.

–	Otherwise (the view order index VOIdx of the current view is less than VOIdxMax), frame buffers containing a frame or a complementary field pair or a non-paired field for which both of the following conditions are true are emptied (without output), and the DPB fullness is decremented by the number of frame buffers emptied:

–	the frame or complementary field pair or non-paired field is marked as "not needed for output" and "unused for reference",

NOTE 2 – At this stage of the process, all frame buffers that contain a frame or a complementary field pair or a non-paired field marked as "not needed for output" and "unused for reference" are frame buffers that contain an inter-view only reference component (of the current access unit and a view with view order index less than VOIdx) with OutputFlag equal to 0.

–	one of the following conditions is true:

–	the current view component is a view component of an anchor picture and the view_id of the frame or complementary field pair or non-paired field is not equal to any value of anchor_ref_lX[ k ][ j ], with X being equal to 0 or 1, k being any integer value greater than the view order index VOIdx of the current view, and j being any integer value in the range of 0 to Max( 0, num_anchor_refs_lX[ k ] − 1 ), inclusive,

–	the current view component is not a view component of an anchor picture and the view_id of the frame or complementary field pair or non-paired field is not equal to any value of non_anchor_ref_lX[ k ][ j ], with X being equal to 0 or 1, k being any integer value greater than the view order index VOIdx of the current view, and j being any integer value in the range of 0 to Max( 0, num_non_anchor_refs_lX[ k ] − 1 ), inclusive.

When the decoding process specified in Annex Error: Reference source not found or Annex I is applied, for the following processes specified in this subclause, only view components of the particular view for which this subclause is invoked are considered, and frame buffers containing view components of other views are not emptied. The DPB fullness represents the total number of non-empty frame buffers, including frame buffers that contain view components of other views.

The removal of pictures from the DPB before possible insertion of the current picture proceeds as follows:

–	If the decoded picture is an IDR picture the following applies:

- All reference pictures in the DPB are marked as "unused for reference" as specified in subclause Error: Reference source not found when a coded video sequence conforming to one or more of the profiles specified in Annex Error: Reference source not found is decoded by applying the decoding process specified in clauses Error: Reference source not found-Error: Reference source not found, or as specified in subclause Error: Reference source not found when a coded video sequence conforming to one or more of the profiles specified in Annex Error: Reference source not found is decoded by applying the decoding process specified in Annex Error: Reference source not found, or as specified in subclause I.8.3 when a coded video sequence conforming to one or more of the profiles specified in Annex Error: Reference source not found is decoded by applying the decoding process specified in Annex Error: Reference source not found, or as specified in subclause I.8.3 when a coded video sequence conforming to one or more of the profiles specified in Annex I is decoded by applying the decoding process specified in Annex I.

- When the IDR picture is not the first IDR picture decoded and the value of PicWidthInMbs or FrameHeightInMbs or max_dec_frame_buffering derived from the active sequence parameter set is different from the value of PicWidthInMbs or FrameHeightInMbs or max_dec_frame_buffering derived from the sequence parameter set that was active for the preceding picture, respectively, no_output_of_prior_pics_flag is inferred to be equal to 1 by the HRD, regardless of the actual value of no_output_of_prior_pics_flag.

NOTE 3 – Decoder implementations should try to handle changes in the value of PicWidthInMbs or FrameHeightInMbs or max_dec_frame_buffering more gracefully than the HRD.

- When no_output_of_prior_pics_flag is equal to 1 or is inferred to be equal to 1, all frame buffers in the DPB are emptied without output of the pictures they contain, and DPB fullness is set to 0.

–	Otherwise (the decoded picture is not an IDR picture), the decoded reference picture marking process is invoked as specified in subclause Error: Reference source not found when a coded video sequence conforming to one or more of the profiles specified in Annex Error: Reference source not found is decoded by applying the decoding process specified in clauses Error: Reference source not found-Error: Reference source not found, or as specified in subclause Error: Reference source not found when a coded video sequence conforming to one or more of the profiles specified in Annex Error: Reference source not found is decoded by applying the decoding process specified in Annex Error: Reference source not found, or as specified in subclause I.8.3 when a coded video sequence conforming to one or more of the profiles specified in Annex Error: Reference source not found is decoded by applying the decoding process specified in Annex Error: Reference source not found, or as specified in subclause I.8.3 when a coded video sequence conforming to one or more of the profiles specified in Annex I is decoded by applying the decoding process specified in Annex I. Frame buffers containing a frame or a complementary field pair or a non-paired field which are marked as "not needed for output" and "unused for reference" are emptied (without output), and the DPB fullness is decremented by the number of frame buffers emptied.

When the current picture has a memory_management_control_operation equal to 5 or is an IDR picture for which no_output_of_prior_pics_flag is not equal to 1 and is not inferred to be equal to 1, the following two steps are performed.

1.	Frame buffers containing a frame or a complementary field pair or a non-paired field which are marked as "not needed for output" and "unused for reference" are emptied (without output), and the DPB fullness is decremented by the number of frame buffers emptied.

2.	All non-empty frame buffers in the DPB are emptied by repeatedly invoking the "bumping" process specified in subclause C.4.5.3, and the DPB fullness is set to 0.

    - Current decoded picture marking and storage

When the decoding process specified in Annex Error: Reference source not found is applied, the process specified in this subclause is invoked for a particular view with view order index VOIdx, with "picture" being replaced by "view component", "frame" being replaced by "frame view component", and "field" being replaced by "field view component". During the invocation of the process for a particular view, only view components of the particular view are considered and frame buffers containing view components of other views are not emptied. The DPB fullness represents the total number of non-empty frame buffers, including frame buffers that contain texture view components and depth view components of the current and other views.

When the decoding process specified in Annex I is applied, the process specified in this subclause for Annex Error: Reference source not found is invoked for particular texture view and depth view with view order index VOIdx, with each "view component" being replaced by "texture view component" or "depth view component", "frame view component" being replaced by "texture frame view component" or "depth frame view component", and "field view component" being replaced by "texture field view component". During the invocation of the process for a particular texture view, only the texture view components of the particular view are considered and during the invocation of the process for a particular depth view, only the depth view components of the particular view are considered and frame buffers containing view components of other views are not emptied.

The marking and storage of the current decoded picture is specified as follows:

–	If the current picture is a reference picture, the storage and marking process for decoded reference pictures as specified in subclause C.4.5.1 is invoked.

–	Otherwise (the current picture is a non-reference picture), the storage and marking process for decoded non-reference pictures as specified in subclause C.4.5.2 is invoked.

      - Storage and marking of a reference decoded picture into the DPB

The current picture is stored in the DPB as follows:

–	If the current decoded picture is the second field (in decoding order) of a complementary reference field pair, and the first field of the pair is still in the DPB, the current picture is stored in the same frame buffer as the first field of the pair and the following applies:

–	If the current decoded picture has OutputFlag equal to 1, it is marked as "needed for output".

–	Otherwise (the current decoded picture has OutputFlag equal to 0), it is marked as "not needed for output".

–	Otherwise, the following operations are performed:

- When there is no empty frame buffer (i.e., DPB fullness is equal to DPB size), the "bumping" process specified in subclause C.4.5.3 is invoked repeatedly until there is an empty frame buffer in which to store the current decoded picture.

- The current decoded picture is stored in an empty frame buffer, the DPB fullness is incremented by one, and the following applies:

–	If the current decoded picture has OutputFlag equal to 1, it is marked as "needed for output".

–	Otherwise (the current decoded picture has OutputFlag equal to 0), it is marked as "not needed for output".

When the coded video sequence conforms to one or more of the profiles specified in Annex Error: Reference source not found and the decoding process specified in Annex Error: Reference source not found is applied and the current picture has store_ref_base_pic_flag equal to 1 (i.e., the current picture is associated with a reference base picture), the associated reference base picture is stored in the DPB as follows:

–	If the reference base picture is a second field (in decoding order) of a complementary reference base field pair, and the first field of the pair is still in the DPB, the reference base picture is stored in the same frame buffer as the first field of the pair and marked as "not needed for output".

–	Otherwise, the following operations are performed:

- When there is no empty frame buffer (i.e., DPB fullness is equal to DPB size), the "bumping" process specified in subclause C.4.5.3 is invoked repeatedly until there is an empty frame buffer in which to store the reference base picture.

- The reference base picture is stored in an empty frame buffer and marked as "not needed for output" and the DPB fullness is incremented by one.

      - Storage and marking of a non-reference decoded picture into the DPB

The current picture is associated with a variable StoreInterViewOnlyRefFlag, which is derived as follows:

–	If the decoding process specified in Annex Error: Reference source not found or Annex I is applied, the current view component has a view order index VOIdx less than VOIdxMax and inter_view_flag equal to 1, StoreInterViewOnlyRefFlag is set equal to 1.

–	Otherwise, StoreInterViewOnlyRefFlag is set equal to 0.

The current picture is stored in the DPB or output as follows:

–	If the current decoded picture is the second field (in decoding order) of a complementary non-reference field pair and the first field of the pair is still in the DPB, the current picture is stored in the same frame buffer as the first field of the pair and the following applies:

–	If the current decoded picture has OutputFlag equal to 1, it is marked as "needed for output".

–	Otherwise (the current decoded picture has OutputFlag equal to 0), it is marked as "not needed for output".

–	Otherwise, if the current picture has OutputFlag equal to 0 and StoreInterViewOnlyRefFlag equal to 0, the DPB is not modified and the current picture is not output.

–	Otherwise, if the current picture has StoreInterViewOnlyRefFlag equal to 1, the following operations are performed:

- When there is no empty frame buffer (i.e., DPB fullness is equal to DPB size), the "bumping" process specified in subclause C.4.5.3 is invoked repeatedly until there is an empty frame buffer in which to store the current decoded picture.

- The current decoded picture is stored in an empty frame buffer, the DPB fullness is incremented by one, and the following applies:

–	If the current decoded picture has OutputFlag equal to 1, it is marked as "needed for output".

–	Otherwise (the current decoded picture has OutputFlag equal to 0), it is marked as "not needed for output".

–	Otherwise, the following operations are performed repeatedly until the current decoded picture has been cropped and output or has been stored in the DPB:

–	If there is no empty frame buffer (i.e., DPB fullness is equal to DPB size), the following applies:

–	If the current picture does not have a lower value of PicOrderCnt( ) than all pictures in the DPB that are marked as "needed for output", the "bumping" process described in subclause C.4.5.3 is performed.

–	Otherwise (the current picture has a lower value of PicOrderCnt( ) than all pictures in the DPB that are marked as "needed for output"), the current picture is cropped, using the cropping rectangle specified in the active sequence parameter set for the picture and the cropped picture is output.

–	Otherwise (there is an empty frame buffer, i.e., DPB fullness is less than DPB size), the current decoded picture is stored in an empty frame buffer and is marked as "needed for output", and the DPB fullness is incremented by one.

      - "Bumping" process

When the decoding process specified in Annex Error: Reference source not found is applied, the process specified in this subclause is invoked for a particular view with view order index VOIdx, with "picture" being replaced by "view component", "frame" being replaced by "frame view component", and "field" being replaced by "field view component". During the invocation of the process for a particular view, only view components of the particular view are considered and frame buffers containing view components of other views are not emptied. The DPB fullness represents the total number of non-empty frame buffers, including frame buffers that contain texture view components and depth view components of the current view and other views.

When the decoding process specified in Annex I is applied, the process specified in this subclause for Annex Error: Reference source not found is invoked for particular texture view and depth view with view order index VOIdx, with each "view component" being replaced by "texture view component" or "depth view component", "frame view component" being replaced by "texture frame view component" or "depth frame view component", and "field view component" being replaced by "texture field view component". During the invocation of the process for a particular texture view, only the texture view components of the particular view are considered and during the invocation of the process for a particular depth view, only the depth view components of the particular view are considered and frame buffers containing view components of other views are not emptied. The DPB fullness represents the total number of non-empty frame buffers, including frame buffers that contain view components of other views.

The "bumping" process is invoked in the following cases.

–	There is no empty frame buffer (i.e., DPB fullness is equal to DPB size) and an empty frame buffer is needed for storage of an inferred "non-existing" frame, as specified in subclause C.4.2.

–	The current picture is an IDR picture and no_output_of_prior_pics_flag is not equal to 1 and is not inferred to be equal to 1, as specified in subclause C.4.4.

–	The current picture has memory_management_control_operation equal to 5, as specified in subclause C.4.4.

–	There is no empty frame buffer (i.e., DPB fullness is equal to DPB size) and an empty frame buffer is needed for storage of a decoded (non-IDR) reference picture or a reference base picture, as specified in subclause C.4.5.1.

–	There is no empty frame buffer (i.e., DPB fullness is equal to DPB size) and the current picture is a non-reference picture that is not the second field of a complementary non-reference field pair and the current picture has OutputFlag equal to 1 and there are pictures in the DPB that are marked as "needed for output" that precede the current non-reference picture in output order, as specified in subclause C.4.5.2, so an empty buffer is needed for storage of the current picture.

–	There is no empty frame buffer (i.e., DPB fullness is equal to DPB size) and the current picture is a non-reference picture that is not the second field of a complementary non-reference field pair and the current picture has StoreInterViewOnlyRefFlag equal to 1, as specified in subclause C.4.5.2, so an empty buffer is needed for storage of the current picture.

The "bumping" process consists of the following ordered steps:

- The picture or complementary reference field pair that is considered first for output is selected as follows:

- The frame buffer is selected that contains the picture having the smallest value of PicOrderCnt( ) of all pictures in the DPB marked as "needed for output".

- Depending on the frame buffer, the following applies:

–	If this frame buffer contains a complementary non-reference field pair with both fields marked as "needed for output" and both fields have the same PicOrderCnt( ), the first of these two fields in decoding order is considered first for output.

–	Otherwise, if this frame buffer contains a complementary reference field pair with both fields marked as "needed for output" and both fields have the same PicOrderCnt( ), the entire complementary reference field pair is considered first for output.

NOTE – When the two fields of a complementary reference field pair have the same value of PicOrderCnt( ), this "bumping" process will output these pictures together, although the two fields have different output times from a decoder that satisfies output timing conformance criteria (as specified in subclause C.2.2).

–	Otherwise, the picture in this frame buffer that has the smallest value of PicOrderCnt( ) is considered first for output.

- Depending on whether a single picture or a complementary reference field pair is considered for output, the following applies:

–	If a single picture is considered first for output, this picture is cropped, using the cropping rectangle specified in the active sequence parameter set for the picture, the cropped picture is output, and the picture is marked as "not needed for output".

–	Otherwise (a complementary reference field pair is considered first for output), the two fields of the complementary reference field pair are both cropped, using the cropping rectangle specified in the active sequence parameter set for the pictures, the two fields of the complementary reference field pair are output together, and both fields of the complementary reference field pair are marked as "not needed for output".

- The frame buffer that included the picture or complementary reference field pair that was cropped and output is checked, and when any of the following conditions are true, the frame buffer is emptied and the DPB fullness is decremented by 1:

–	The frame buffer contains a non-reference non-paired field.

–	The frame buffer contains a non-reference frame.

–	The frame buffer contains a complementary non-reference field pair with both fields marked as "not needed for output".

–	The frame buffer contains a non-paired reference field marked as "unused for reference".

–	The frame buffer contains a reference frame with both fields marked as "unused for reference".

–	The frame buffer contains a complementary reference field pair with both fields marked as "unused for reference" and "not needed for output".

In Table D.1, add the following after “display_orientation (payloadSize)”:

| else if( payloadType = = 48 ) |  |  |
| --- | --- | --- |
| 3dvc_scalable_nesting( payloadSize ) |  |  |
| else if ( payloadType = = 49 ) |  |  |
| 3dvc_view_scalability_info( payloadSize ) |  |  |
| else if (payloadType = = 50 ) |  |  |
| depth_acquisition_info ( payloadSize ) |  |  |
| else if (payloadType = = 51 ) |  |  |
| depth_representation_information ( payloadSize ) |  |  |
| else if (payloadType = = 52 ) |  |  |
| 3d_reference_displays_info ( payloadSize ) |  |  |

Add Annex I:

- Annex I

Multiview and depth video coding

(This annex forms an integral part of this Recommendation | International Standard)

This annex specifies multiview and depth video coding, referred to as 3DVC.

  - Scope

Bitstreams and decoders conforming to the profile specified in this annex are completely specified in this annex with reference made to clauses Error: Reference source not found-Error: Reference source not found and Annexes Error: Reference source not found-H.

  - Normative references

The specifications in clause Error: Reference source not found apply.

  - Definitions

For the purpose of this annex, the following definitions apply in addition to the definitions in clause H.Error: Reference source not found. These definitions are either not present in clause H.Error: Reference source not found or replace definitions in clause H.Error: Reference source not found.

- stereoscopic texture bitstream: A bitstream containing two texture views and conforming to one of the profiles specified in Annex H.

- depth field view: A depth view component of a field.

- depth frame view: A depth view component of a field.

- depth view: A sequence of depth view components associated with an identical value of view_id.

- depth view component: A coded representation of the depth of a view in a single access unit.

- inter-view only reference component: A view component, texture view component, or depth view component coded with nal_ref_idc equal to 0 and inter_view_flag equal to 1. An inter-view only reference component contains samples that may be used for inter-view prediction in the decoding process of subsequent view components in decoding order, but are not used for inter prediction by any view components. Inter-view only reference components are non-reference pictures.

- inter-view reference component: A view component, texture view component, or depth view component coded with nal_ref_idc greater than 0 and inter_view_flag equal to 1. An inter-view reference component contains samples that may be used for inter prediction of subsequent pictures in decoding order and inter-view prediction of subsequent view components in decoding order. Inter-view reference components are reference pictures.

- reference picture: A view component, texture view component, or depth view component coded with nal_ref_idc greater than 0. A reference picture contains samples that may be used for inter prediction in the decoding process of subsequent view components in decoding order. A reference picture may be an inter-view reference component, in which case the samples contained in the reference picture may also be used for inter-view prediction in the decoding process of subsequent view components in decoding order.

- texture field view component: A texture view component of a field.

- texture frame view component: A texture view component of a frame.

- texture view: A sequence of texture view components associated with an identical value of view_id.

- texture view component: A coded representation of the texture of a view in a single access unit.

- view: a texture view and a depth view of the same value of view_id, unless explicitly limited to either texture view or depth view.

- view component: A coded representation of a view in a single access unit. A view component may consist of a texture view component and a depth view component.

- view component pair: A texture view component and a depth view component of the same view within the same access unit.

- 3DVC sequence parameter set: A collective term for sequence parameter set or subset sequence parameter set.

- 3DVC sequence parameter set RBSP: A collective term for sequence parameter set RBSP or subset sequence parameter set RBSP.

  - Abbreviations

The specifications in clause Error: Reference source not found apply.

  - Conventions

The specifications in clause Error: Reference source not found apply.

  - Source, coded, decoded and output data formats, scanning processes, and neighbouring relationships

The specifications in clause Error: Reference source not found apply with substitution of 3DVC sequence parameter set for sequence parameter set.

  - Syntax and semantics

This clause specifies syntax and semantics for coded video sequences that conform to one or more of the profiles specified in this annex.

    - Method of specifying syntax in tabular form

The specifications in subclause H.Error: Reference source not found apply.

    - Specification of syntax functions, categories, and descriptors

The specifications in subclause H.Error: Reference source not found apply.

    - Syntax in tabular form

      - NAL unit syntax

The syntax table is specified in subclause H.Error: Reference source not found.

        - NAL unit header MVC extension syntax

The syntax table is specified in subclause H.Error: Reference source not found.1.

      - Raw byte sequence payloads and RBSP trailing bits syntax

        - Sequence parameter set RBSP syntax

The syntax table is specified in subclause H.Error: Reference source not found.

          - Sequence parameter set data syntax

The syntax table is specified in subclause H.Error: Reference source not found.

            - Scaling list syntax

The syntax table is specified in subclause H.Error: Reference source not found.

          - Sequence parameter set extension RBSP syntax

The syntax table is specified in subclause H.Error: Reference source not found.

          - Subset sequence parameter set RBSP syntax

The syntax table is specified in subclause H.Error: Reference source not found.

          - Sequence parameter set MVC extension syntax

The syntax table is specified in subclause H.7.3.2.1.4.

When profile_idc is equal to 138 in active sequence parameter, the seq_parameter_set_mvc_extension( ) is applicable to the depth views and 3DV operation points.

          - Sequence parameter set 3DVC extension syntax

| seq_parameter_set_3dvc_extension( ) { | C | Descriptor |
| --- | --- | --- |
| for( i = 0; i <= num_level_values_signalled_minus1; i++ ) |  |  |
| for( j = 0; j <= num_applicable_ops_minus1[ i ]; j++ ) |  |  |
| applicable_op_num_depth_views_minus1[ i ][ j ] | 0 | ue(v) |
| 3dvc_vui_parameters_present_flag | 0 | u(1) |
| if( 3dvc_vui_parameters_present_flag = = 1 ) |  |  |
| mvc_vui_parameters_extension( ) | 0 |  |
| texture_vui_parameters_present_flag | 0 | u(1) |
| if( texture_vui_parameters_present_flag = = 1 ) |  |  |
| mvc_vui_parameters_extension( ) | 0 |  |
| } |  |  |

        - Picture parameter set RBSP syntax

The syntax table is specified in subclause H.Error: Reference source not found.

        - Supplemental enhancement information RBSP syntax

The syntax table is specified in subclause H.Error: Reference source not found.

          - Supplemental enhancement information message syntax

The syntax table is specified in subclause H.Error: Reference source not found.

        - Access unit delimiter RBSP syntax

The syntax table is specified in subclause H.Error: Reference source not found.

        - End of sequence RBSP syntax

The syntax table is specified in subclause H.Error: Reference source not found.

        - End of stream RBSP syntax

The syntax table is specified in subclause H.Error: Reference source not found.

        - Filler data RBSP syntax

The syntax table is specified in subclause H.Error: Reference source not found.

        - Slice layer without partitioning RBSP syntax

The syntax table is specified in subclause H.Error: Reference source not found.

        - Slice data partition RBSP syntax

Slice data partition syntax is not present in coded video sequences conforming to one or more of the profiles specified in this annex.

        - RBSP slice trailing bits syntax

The syntax table is specified in subclause H.Error: Reference source not found.

        - RBSP trailing bits syntax

The syntax table is specified in subclause H.Error: Reference source not found.

        - Prefix NAL unit RBSP syntax

The syntax table is specified in subclause H.Error: Reference source not found.

        - Slice layer extension RBSP syntax

The syntax table is specified in subclause H.Error: Reference source not found.

      - Slice header syntax

The syntax table is specified in subclause H.Error: Reference source not found.

        - Reference picture list modification syntax

The syntax table is specified in subclause H.Error: Reference source not found.

          - Reference picture list MVC modification syntax

The syntax table is specified in subclause H.7.3.3.1.1

        - Prediction weight table syntax

The syntax table is specified in subclause H.Error: Reference source not found.

        - Decoded reference picture marking syntax

The syntax table is specified in subclause H.Error: Reference source not found.

      - Slice data syntax

The syntax table is specified in subclause H.Error: Reference source not found.

      - Macroblock layer syntax

The syntax table is specified in subclause H.Error: Reference source not found.

        - Macroblock prediction syntax

The syntax table is specified in subclause H.Error: Reference source not found.

        - Sub-macroblock prediction syntax

The syntax table is specified in subclause H.Error: Reference source not found.

        - Residual data syntax

The syntax table is specified in subclause H.Error: Reference source not found.

          - Residual luma syntax

The syntax table is specified in subclause H.Error: Reference source not found.

          - Residual block CAVLC syntax

The syntax table is specified in subclause H.Error: Reference source not found.

          - Residual block CABAC syntax

The syntax table is specified in subclause H.Error: Reference source not found.

    - Semantics

Semantics associated with the syntax structures and syntax elements within these structures (in subclause I.7.3 and in subclause I.7.3 by reference in subclause I.7.3) are specified in this subclause and by reference to subclause I.7.4. When the semantics of a syntax element are specified using a table or a set of tables, any values that are not specified in the table(s) shall not be present in the bitstream unless otherwise specified in this Recommendation | International Standard.

      - NAL unit semantics

The semantics for the syntax elements in subclause I.7.3.1 are specified in subclause I.7.3.1.

        - NAL unit header MVC extension semantics

The semantics for the syntax elements in subclause I.7.3.1.1 are specified in subclause I.7.3.1.1.

        - Order of NAL units and association to coded pictures, access units, and video sequences

This subclause specifies constraints on the order of NAL units in the bitstream. Any order of NAL units in the bitstream obeying these constraints is referred to in the text as the decoding order of NAL units. Within a NAL unit, the syntax in subclauses Error: Reference source not found, Error: Reference source not found, Error: Reference source not found, I.7.3, Error: Reference source not found, Error: Reference source not found, I.13.1 and I.14.1 specifies the decoding order of syntax elements. Decoders shall be capable of receiving NAL units and their syntax elements in decoding order.

          - Order of 3DVC sequence parameter set RBSPs and picture parameter set RBSPs and their activation

- NOTE 1 – The sequence and picture parameter set mechanism decouples the transmission of infrequently changing information from the transmission of coded macroblock data. Sequence and picture parameter sets may, in some applications, be conveyed "out-of-band" using a reliable transport mechanism.

- A picture parameter set RBSP includes parameters that can be referred to by the coded slice NAL units of one or more texture view or depth view components of one or more coded pictures.

- Each picture parameter set RBSP is initially considered not active at the start of the operation of the decoding process. At most one picture parameter set RBSP is considered as the active picture parameter set RBSP at any given moment during the operation of the decoding process, and when any particular picture parameter set RBSP becomes the active picture parameter set RBSP, the previously-active picture parameter set RBSP (if any) is deactivated.

- In addition to the active picture parameter set RBSP, zero or more picture parameter set RBSPs may be specifically active for texture view components (with a particular value of VOIdx less than or equal to VOIdxMax) that belong to the target output views or that may be referred to through inter-view prediction in decoding texture view components belonging to the target output views. Such a picture parameter set RBSP is referred to as the active texture picture parameter set RBSP for the particular value of VOIdx. The restrictions on active picture parameter set RBSPs also apply to active texture picture parameter set RBSPs for a particular value of VOIdx.

- Furthermore, zero or more picture parameter set RBSPs may be specifically active for depth view components (with a particular value of VOIdx less than VOIdxMax) that belong to the target output views or that may be referred to through inter-view prediction in decoding depth view components belonging to the target output views. Such a picture parameter set RBSP is referred to as the active depth picture parameter set RBSP for the particular value of VOIdx. The restrictions on active picture parameter set RBSPs also apply to active depth picture parameter set RBSPs for a particular value of VOIdx less than VOIdxMax.

- When a picture parameter set RBSP (with a particular value of pic_parameter_set_id) is not the active picture parameter set RBSP and it is referred to by a coded slice NAL unit belonging to a depth view component (i.e., with nal_unit_type equal to 21) and with VOIdx equal to VOIdxMax (using that value of pic_parameter_set_id), it is activated. This picture parameter set RBSP is called the active picture parameter set RBSP until it is deactivated when another picture parameter set RBSP becomes the active picture parameter set RBSP. A picture parameter set RBSP, with that particular value of pic_parameter_set_id, shall be available to the decoding process prior to its activation.

- When a picture parameter set RBSP (with a particular value of pic_parameter_set_id) is not the active depth picture parameter set for a particular value of VOIdx less than VOIdxMax and it is referred to by a coded slice NAL unit belonging to a depth view component (i.e., with nal_unit_type equal to 21) and with the particular value of VOIdx (using that value of pic_parameter_set_id), it is activated for view components with the particular value of VOIdx. This picture parameter set RBSP is called the active depth picture parameter set RBSP for the particular value of VOIdx until it is deactivated when another picture parameter set RBSP becomes the active depth picture parameter set RBSP for the particular value of VOIdx. A picture parameter set RBSP, with that particular value of pic_parameter_set_id, shall be available to the decoding process prior to its activation.

- When a picture parameter set RBSP (with a particular value of pic_parameter_set_id) is not the active texture picture parameter set for a particular value of VOIdx less than or equal to VOIdxMax and it is referred to by a coded slice NAL unit belonging to a texture view component (i.e., with nal_unit_type equal to 1, 5 or 20) and with the particular value of VOIdx (using that value of pic_parameter_set_id), it is activated for texture view components with the particular value of VOIdx. This picture parameter set RBSP is called the active texture picture parameter set RBSP for the particular value of VOIdx until it is deactivated when another picture parameter set RBSP becomes the active texture picture parameter set RBSP for the particular value of VOIdx. A picture parameter set RBSP, with that particular value of pic_parameter_set_id, shall be available to the decoding process prior to its activation.

- Any picture parameter set NAL unit containing the value of pic_parameter_set_id for the active picture parameter set RBSP for a coded picture shall have the same content as that of the active picture parameter set RBSP for this coded picture unless it follows the last VCL NAL unit of this coded picture and precedes the first VCL NAL unit of another coded picture. Any picture parameter set NAL unit containing the value of pic_parameter_set_id for the active depth picture parameter set RBSP for a particular value of VOIdx less than VOIdxMax for a coded picture shall have the same content as that of the active view picture parameter set RBSP for the particular value of VOIdx for this coded picture unless it follows the last VCL NAL unit of this coded picture and precedes the first VCL NAL unit of another coded picture. Any picture parameter set NAL unit containing the value of pic_parameter_set_id for the active texture picture parameter set RBSP for a particular value of VOIdx for a coded picture shall have the same content as that of the active texture picture parameter set RBSP for the particular value of VOIdx for this coded picture unless it follows the last VCL NAL unit of this coded picture and precedes the first VCL NAL unit of another coded picture.

- A 3DVC sequence parameter set RBSP includes parameters that can be referred to by one or more picture parameter set RBSPs or one or more buffering period SEI messages.

- Each 3DVC sequence parameter set RBSP is initially considered not active at the start of the operation of the decoding process. At most one 3DVC sequence parameter set RBSP is considered as the active 3DVC sequence parameter set RBSP at any given moment during the operation of the decoding process, and when any particular 3DVC sequence parameter set RBSP becomes the active 3DVC sequence parameter set RBSP, the previously-active 3DVC sequence parameter set RBSP (if any) is deactivated.

- In addition to the active 3DVC sequence parameter set RBSP, zero or more 3DVC sequence parameter set RBSPs may be specifically active for view components (with a particular value of VOIdx less than VOIdxMax) that belong to the target output views or that may be referred to through inter-view prediction in decoding view components belonging to the target output views. Such a 3DVC sequence parameter set RBSP is referred to as the active view 3DVC sequence parameter set RBSP for the particular value of VOIdx. The restrictions on active 3DVC sequence parameter set RBSPs also apply to active view 3DVC sequence parameter set RBSPs for a particular value of VOIdx less than VOIdxMax.

- Furthermore, zero or more 3DVC sequence parameter set RBSPs may be specifically active for texture view components (with a particular value of VOIdx less than or equal to VOIdxMax) that belong to the target output views or that may be referred to through inter-view prediction in decoding texture view components belonging to the target output views. Such a 3DVC sequence parameter set RBSP is referred to as the active texture 3DVC sequence parameter set RBSP for the particular value of VOIdx. The restrictions on active 3DVC sequence parameter set RBSPs also apply to active texture 3DVC sequence parameter set RBSPs for a particular value of VOIdx.

- For the following specification, the activating buffering period SEI message is specified as follows.

–	If VOIdxMax is equal to VOIdxMin and the access unit contains a buffering period SEI message not included in an MVC scalable nesting SEI message and not included in a 3DVC scalable nesting SEI message, this buffering period SEI message is the activating buffering period SEI message.

–	Otherwise if VOIdxMax is not equal to VOIdxMin and the access unit contains a buffering period SEI message included in a 3DVC scalable nesting SEI message and associated with the operation point being decoded, this buffering period SEI message is the activating buffering period SEI message.

–	Otherwise, the access unit does not contain an activating buffering period SEI message.

- When a sequence parameter set RBSP (nal_unit_type is equal to 7) with a particular value of seq_parameter_set_id is not already the active 3DVC sequence parameter set RBSP and it is referred to by activation of a picture parameter set RBSP (using that value of seq_parameter_set_id) and the picture parameter set RBSP is activated by a coded slice NAL unit with nal_unit_type equal to 1 or 5 (the picture parameter set RBSP becomes the active picture parameter set RBSP and VOIdxMax is equal to VOIdxMin and there is no depth view component in any access unit) and the access unit does not contain an activating buffering period SEI message, it is activated. This sequence parameter set RBSP is called the active 3DVC sequence parameter set RBSP until it is deactivated when another 3DVC sequence parameter set RBSP becomes the active 3DVC sequence parameter set RBSP. A sequence parameter set RBSP, with that particular value of seq_parameter_set_id, shall be available to the decoding process prior to its activation.

- When a sequence parameter set RBSP (nal_unit_type is equal to 7) with a particular value of seq_parameter_set_id is not already the active 3DVC sequence parameter set RBSP and it is referred to by an activating buffering period SEI message (using that value of seq_parameter_set_id) that is not included in a 3DVC scalable nesting SEI message and VOIdxMax is equal to VOIdxMin and there is no depth view component in the access unit, it is activated. This sequence parameter set RBSP is called the active 3DVC sequence parameter set RBSP until it is deactivated when another 3DVC sequence parameter set RBSP becomes the active 3DVC sequence parameter set RBSP. A sequence parameter set RBSP, with that particular value of seq_parameter_set_id, shall be available to the decoding process prior to its activation.

- When a subset sequence parameter set RBSP (nal_unit_type is equal to 15) with a particular value of seq_parameter_set_id is not already the active 3DVC sequence parameter set RBSP and it is referred to by activation of a picture parameter set RBSP (using that value of seq_parameter_set_id) and the picture parameter set RBSP is activated by a coded slice depth extension NAL unit with nal_unit_type equal to 21 and with VOIdx equal to VOIdxMax (the picture parameter set RBSP becomes the active picture parameter set RBSP) and the access unit does not contain an activating buffering period SEI message, it is activated. This subset sequence parameter set RBSP is called the active 3DVC sequence parameter set RBSP until it is deactivated when another 3DVC sequence parameter set RBSP becomes the active 3DVC sequence parameter set RBSP. A subset sequence parameter set RBSP, with that particular value of seq_parameter_set_id, shall be available to the decoding process prior to its activation.

- When a subset sequence parameter set RBSP (nal_unit_type is equal to 15) with a particular value of seq_parameter_set_id is not already the active 3DVC sequence parameter set RBSP and it is referred to by an activating buffering period SEI message (using that value of seq_parameter_set_id) that is included in a 3DVC scalable nesting SEI message, it is activated. This subset sequence parameter set RBSP is called the active 3DVC sequence parameter set RBSP until it is deactivated when another 3DVC sequence parameter set RBSP becomes the active 3DVC sequence parameter set RBSP. A subset sequence parameter set RBSP, with that particular value of seq_parameter_set_id, shall be available to the decoding process prior to its activation.

NOTE 2 – The active 3DVC sequence parameter set RBSP is either a sequence parameter set RBSP or a subset sequence parameter set RBSP. Sequence parameter set RBSPs are activated by coded slice NAL units with nal_unit_type equal to 1 or 5 or buffering period SEI messages that are not included in an MVC scalable nesting SEI message or a 3DVC scalable nesting SEI message. Subset sequence parameter sets are activated by coded slice depth extension NAL units (nal_unit_type equal to 21) or buffering period SEI messages that are included in a 3DVC scalable nesting SEI message. A sequence parameter set RBSP and a subset sequence parameter set RBSP may have the same value of seq_parameter_set_id.

- For the following specification, the activating texture buffering period SEI message for a particular value of VOIdx is specified as follows.

–	If the access unit contains one or more than one buffering period SEI message included in an MVC scalable nesting SEI message and associated with an operation point for which the greatest VOIdx in the associated bitstream subset is equal to the particular value of VOIdx, the first of these buffering period SEI messages, in decoding order, is the activating texture buffering period SEI message for the particular value of VOIdx.

–	Otherwise, if the access unit contains a buffering period SEI message not included in an MVC scalable nesting SEI message or a 3DVC scalable nesting SEI message, this buffering period SEI message is the activating texture buffering period SEI message for the particular value of VOIdx equal to VOIdxMin.

–	Otherwise, the access unit does not contain an activating texture buffering period SEI message for the particular value of VOIdx.

- When a sequence parameter set RBSP (nal_unit_type is equal to 7) with a particular value of seq_parameter_set_id is not already the active texture 3DVC sequence parameter set RBSP for VOIdx equal to VOIdxMin and it is referred to by activation of a picture parameter set RBSP (using that value of seq_parameter_set_id) and the picture parameter set RBSP is activated by a coded slice NAL unit with nal_unit_type equal to 1 or 5 (the picture parameter set RBSP becomes the active texture picture parameter set RBSP for VOIdx equal to VOIdxMin), it is activated for texture view components with VOIdx equal to VOIdxMin. This sequence parameter set RBSP is called the active texture 3DVC sequence parameter set RBSP for VOIdx equal to VOIdxMin until it is deactivated when another 3DVC sequence parameter set RBSP becomes the active texture 3DVC sequence parameter set RBSP for VOIdx equal to VOIdxMin. A sequence parameter set RBSP, with that particular value of seq_parameter_set_id, shall be available to the decoding process prior to its activation.

- When a sequence parameter set RBSP (nal_unit_type is equal to 7) with a particular value of seq_parameter_set_id is not already the active texture 3DVC sequence parameter set RBSP for VOIdx equal to VOIdxMin and it is referred to by an activating texture buffering period SEI message (using that value of seq_parameter_set_id) that is not included in an MVC scalable nesting SEI message or a 3DVC scalable nesting SEI message, the sequence parameter set RBSP is activated for texture view components with VOIdx equal to VOIdxMin. This sequence parameter set RBSP is called the active texture 3DVC sequence parameter set RBSP for VOIdx equal to VOIdxMin until it is deactivated when another 3DVC sequence parameter set RBSP becomes the active texture 3DVC sequence parameter set RBSP for VOIdx equal to. A sequence parameter set RBSP, with that particular value of seq_parameter_set_id, shall be available to the decoding process prior to its activation.

- When a subset sequence parameter set RBSP (nal_unit_type is equal to 15) with a particular value of seq_parameter_set_id is not already the active texture 3DVC sequence parameter set RBSP for a particular value of VOIdx less than or equal to VOIdxMax and it is referred to by activation of a picture parameter set RBSP (using that value of seq_parameter_set_id) and the picture parameter set RBSP is activated by a coded slice MVC extension NAL unit (nal_unit_type equal to 20) with the particular value of VOIdx (the picture parameter set RBSP becomes the active texture picture parameter set RBSP for the particular value of VOIdx), it is activated for texture view components with the particular value of VOIdx. This subset sequence parameter set RBSP is called the active texture 3DVC sequence parameter set RBSP for the particular value of VOIdx until it is deactivated when another 3DVC sequence parameter set RBSP becomes the active texture 3DVC sequence parameter set RBSP for the particular value of VOIdx. A subset sequence parameter set RBSP, with that particular value of seq_parameter_set_id, shall be available to the decoding process prior to its activation.

- When a subset sequence parameter set RBSP (nal_unit_type is equal to 15) with a particular value of seq_parameter_set_id is not already the active texture 3DVC sequence parameter set RBSP for a particular value of VOIdx less than or equal to VOIdxMax and it is referred to by an activating texture buffering period SEI message (using that value of seq_parameter_set_id) that is included in an MVC scalable nesting SEI message and associated with the particular value of VOIdx, this subset sequence parameter set RBSP is activated for texture view components with the particular value of VOIdx. This subset sequence parameter set RBSP is called the active texture 3DVC sequence parameter set RBSP for the particular value of VOIdx until it is deactivated when another 3DVC sequence parameter set RBSP becomes the active texture 3DVC sequence parameter set RBSP for the particular value of VOIdx. A subset sequence parameter set RBSP, with that particular value of seq_parameter_set_id, shall be available to the decoding process prior to its activation.

- For the following specification, the activating view buffering period SEI message for a particular value of VOIdx is specified as follows.

–	If the access unit contains one or more than one buffering period SEI message included in a 3DVC scalable nesting SEI message and associated with an operation point for which the greatest VOIdx in the associated bitstream subset is equal to the particular value of VOIdx, the first of these buffering period SEI messages, in decoding order, is the activating view buffering period SEI message for the particular value of VOIdx.

–	Otherwise, the access unit does not contain an activating view buffering period SEI message for the particular value of VOIdx.

- When a subset sequence parameter set RBSP (nal_unit_type is equal to 15) with a particular value of seq_parameter_set_id is not already the active view 3DVC sequence parameter set RBSP for a particular value of VOIdx less than VOIdxMax and it is referred to by activation of a picture parameter set RBSP (using that value of seq_parameter_set_id) and the picture parameter set RBSP is activated by a coded slice NAL unit with nal_unit_type equal to 21 and with the particular value of VOIdx (the picture parameter set RBSP becomes the active view picture parameter set RBSP for the particular value of VOIdx), it is activated for view components with the particular value of VOIdx. This subset sequence parameter set RBSP is called the active view 3DVC sequence parameter set RBSP for the particular value of VOIdx until it is deactivated when another 3DVC sequence parameter set RBSP becomes the active view 3DVC sequence parameter set RBSP for the particular value of VOIdx or when decoding an access unit with VOIdxMax less than or equal to the particular value of VOIdx. A subset sequence parameter set RBSP, with that particular value of seq_parameter_set_id, shall be available to the decoding process prior to its activation.

- When a subset sequence parameter set RBSP (nal_unit_type is equal to 15) with a particular value of seq_parameter_set_id is not already the active view 3DVC sequence parameter set RBSP for a particular value of VOIdx less than VOIdxMax and it is referred to by an activating view buffering period SEI message (using that value of seq_parameter_set_id) that is included in a 3DVC scalable nesting SEI message and associated with the particular value of VOIdx, this subset sequence parameter set RBSP is activated for view components with the particular value of VOIdx. This subset sequence parameter set RBSP is called the active view 3DVC sequence parameter set RBSP for the particular value of VOIdx until it is deactivated when another 3DVC sequence parameter set RBSP becomes the active view 3DVC sequence parameter set RBSP for the particular value of VOIdx or when decoding an access unit with VOIdxMax less than or equal to the particular value of VOIdx. A subset sequence parameter set RBSP, with that particular value of seq_parameter_set_id, shall be available to the decoding process prior to its activation.

- A 3DVC sequence parameter set RBSP that includes a value of profile_idc not specified in Annex Error: Reference source not found or Annex Error: Reference source not found or Annex I shall not be referred to by activation of a picture parameter set RBSP as the active picture parameter set RBSP or as active view picture parameter set RBSP or as active texture picture parameter set RBSP (using that value of seq_parameter_set_id) or referred to by a buffering period SEI message (using that value of seq_parameter_set_id). A 3DVC sequence parameter set RBSP including a value of profile_idc not specified in Annex Error: Reference source not found or Annex Error: Reference source not found or Annex I is ignored in the decoding for profiles specified in Annex Error: Reference source not found or Annex Error: Reference source not found or Annex I.

- It is a requirement of bitstream conformance that the following constraints are obeyed:

–	For each particular value of VOIdx, all coded slice NAL units (with nal_unit_type equal to 1, 5, 20, or 21) of a coded video sequence shall refer to the same value of seq_parameter_set_id (via the picture parameter set RBSP that is referred to by the value of pic_parameter_set_id).

–	The value of seq_parameter_set_id in a buffering period SEI message that is not included in an MVC scalable nesting SEI message shall be identical to the value of seq_parameter_set_id in the picture parameter set RBSP that is referred to by coded slice NAL units with nal_unit_type equal to 1 or 5 (via the value of pic_parameter_set_id) in the same access unit.

–	The value of seq_parameter_set_id in a buffering period SEI message that is included in an MVC scalable nesting SEI message and is associated with a particular value of VOIdx shall be identical to the value of seq_parameter_set_id in the picture parameter set RBSP that is referred to by coded slice NAL units with nal_unit_type equal to 1, 5 or 20 with the particular value of VOIdx (via the value of pic_parameter_set_id) in the same access unit.

–	The value of seq_parameter_set_id in a buffering period SEI message that is included in a 3DVC scalable nesting SEI message and is associated with a particular value of VOIdx shall be identical to the value of seq_parameter_set_id in the picture parameter set RBSP that is referred to by coded slice NAL units with nal_unit_type equal to 21 with the particular value of VOIdx (via the value of pic_parameter_set_id) in the same access unit.

- The active view 3DVC sequence parameter set RBSPs for different values of VOIdx may be the same 3DVC sequence parameter set RBSP. The active 3DVC sequence parameter set RBSP and an active view 3DVC sequence parameter set RBSP for a particular value of VOIdx may be the same 3DVC sequence parameter set RBSP.

- The active texture 3DVC sequence parameter set RBSPs for different values of VOIdx may be the same 3DVC sequence parameter set RBSP. The active 3DVC sequence parameter set RBSP and an active texture 3DVC sequence parameter set RBSP for a particular value of VOIdx may be the same 3DVC sequence parameter set RBSP. An active view 3DVC sequence parameter set RBSP for a particular value of VOIdx and an active texture 3DVC sequence parameter set RBSP for a particular value of VOIdx may be the same 3DVC sequence parameter set RBSP.

- When the active 3DVC sequence parameter set RBSP for a coded picture is a sequence parameter set RBSP, any sequence parameter set RBSP in the coded video sequence containing this coded picture and with the value of seq_parameter_set_id for the active 3DVC sequence parameter set RBSP shall have the same content as that of the active 3DVC sequence parameter set RBSP.

- When the active 3DVC sequence parameter set RBSP for a coded picture is a subset sequence parameter set RBSP, any subset sequence parameter set RBSP in the coded video sequence containing this coded picture and with the value of seq_parameter_set_id for the active 3DVC sequence parameter set RBSP shall have the same content as that of the active 3DVC sequence parameter set RBSP.

- For each particular value of VOIdx, the following applies:

–	When the active texture 3DVC sequence parameter set RBSP for a coded picture is a sequence parameter set RBSP, any sequence parameter set RBSP in the coded video sequence containing this coded picture and with the value of seq_parameter_set_id for the active texture 3DVC sequence parameter set RBSP shall have the same content as that of the active texture 3DVC sequence parameter set RBSP.

–	When the active texture 3DVC sequence parameter set RBSP for a coded picture is a subset sequence parameter set RBSP, any subset sequence parameter set RBSP in the coded video sequence containing this coded picture and with the value of seq_parameter_set_id for the active texture 3DVC sequence parameter set RBSP shall have the same content as that of the active texture 3DVC sequence parameter set RBSP.

–	The active view 3DVC sequence parameter set RBSP for a coded picture is a subset sequence parameter set RBSP, and any subset sequence parameter set RBSP in the coded video sequence containing this coded picture and with the value of seq_parameter_set_id for the active view 3DVC sequence parameter set RBSP shall have the same content as that of the active view 3DVC sequence parameter set RBSP.

- NOTE 3 – If picture parameter set RBSPs or 3DVC sequence parameter set RBSPs are conveyed within the bitstream, these constraints impose an order constraint on the NAL units that contain the picture parameter set RBSPs or 3DVC sequence parameter set RBSPs, respectively. Otherwise (picture parameter set RBSPs or 3DVC sequence parameter set RBSPs are conveyed by other means not specified in this Recommendation | International Standard), they must be available to the decoding process in a timely fashion such that these constraints are obeyed.

When present, a sequence parameter set extension RBSP includes parameters having a similar function to those of a sequence parameter set RBSP. For purposes of establishing constraints on the syntax elements of the sequence parameter set extension RBSP and for purposes of determining activation of a sequence parameter set extension RBSP, the sequence parameter set extension RBSP shall be considered part of the preceding sequence parameter set RBSP with the same value of seq_parameter_set_id. When a sequence parameter set RBSP is present that is not followed by a sequence parameter set extension RBSP with the same value of seq_parameter_set_id prior to the activation of the sequence parameter set RBSP, the sequence parameter set extension RBSP and its syntax elements shall be considered not present for the active 3DVC sequence parameter set RBSP. The contents of sequence parameter set extension RBSPs only apply when the base texture view, which conforms to one or more of the profiles specified in Annex Error: Reference source not found, of a coded video sequence conforming to one or more profiles specified in Annex I is decoded. Subset sequence parameter set RBSPs shall not be followed by a sequence parameter set extension RBSP.

- NOTE 4 – Sequence parameter sets extension RBSPs are not considered to be part of a subset sequence parameter set RBSP and subset sequence parameter set RBSPs must not be followed by a sequence parameter set extension RBSP.

For view components with VOIdx equal to VOIdxMax, all constraints that are expressed on the relationship between the values of the syntax elements (and the values of variables derived from those syntax elements) in 3DVC sequence parameter sets and picture parameter sets and other syntax elements are expressions of constraints that apply only to the active 3DVC sequence parameter set and the active picture parameter set. For view components with a particular value of VOIdx less than VOIdxMax, all constraints that are expressed on the relationship between the values of the syntax elements (and the values of variables derived from those syntax elements) in 3DVC sequence parameter sets and picture parameter sets and other syntax elements are expressions of constraints that apply only to the active view 3DVC sequence parameter set and the active view picture parameter set for the particular value of VOIdx. If any 3DVC sequence parameter set RBSP having profile_idc equal to the value of one of the profile_idc values specified in Annex Error: Reference source not found or Annex Error: Reference source not found or Annex I is present that is never activated in the bitstream (i.e., it never becomes the active 3DVC sequence parameter set or an active view 3DVC sequence parameter set), its syntax elements shall have values that would conform to the specified constraints if it were activated by reference in an otherwise-conforming bitstream. If any picture parameter set RBSP is present that is never activated in the bitstream (i.e., it never becomes the active picture parameter set or an active view picture parameter set), its syntax elements shall have values that would conform to the specified constraints if it were activated by reference in an otherwise-conforming bitstream.

During operation of the decoding process (see subclause I.8), for view components with VOIdx equal to VOIdxMax, the values of parameters of the active picture parameter set and the active 3DVC sequence parameter set shall be considered in effect. For view components with a particular value of VOIdx less than VOIdxMax, the values of the parameters of the active view picture parameter set and the active view 3DVC sequence parameter set for the particular value of VOIdx shall be considered in effect. For interpretation of SEI messages that apply to the entire access unit or the view component with VOIdx equal to VOIdxMax, the values of the parameters of the active picture parameter set and the active 3DVC sequence parameter set for the same access unit shall be considered in effect unless otherwise specified in the SEI message semantics. For interpretation of SEI messages that apply to view components with a particular value of VOIdx less than VOIdxMax, the values of the parameters of the active view picture parameter set and the active view 3DVC sequence parameter set for the particular value of VOIdx for the same access unit shall be considered in effect unless otherwise specified in the SEI message semantics.

- For any active MVC sequence parameter set or active view MVC sequence parameter set, part of the syntax elements in the MVC sequence parameter set extension applies only to the depth views referring to this sequence parameter set, while the some other parts of the syntax elements in the MVC sequence parameter set extension collectively apply to both the depth views referring to this sequence parameter set and the corresponding texture views. More specifically, the view dependency information of the MVC sequence parameter set extension applies only to the depth views, and the level definitions collectively apply to operation points, each of which contains both depth views and their corresponding texture views.

          - Order of access units and association to coded video sequences

The specification of subclause H.Error: Reference source not found apply.

          - Order of NAL units and coded pictures and association to access units

The specification of subclause H.Error: Reference source not found applies with the following modifications.

- NOTE – Some bitstreams that conform to one or more profiles specified in this annex do not conform to any profile specified in Annex Error: Reference source not found (prior to operation of the base view extraction process specified in subclause I.8.5.4). As specified in subclauses Error: Reference source not found and Error: Reference source not found, for the profiles specified in Annex Error: Reference source not found, NAL units with nal_unit_type equal to 21 are classified as non-VCL NAL units that must be preceded within each access unit by at least one NAL unit with nal_unit_type in the range of 1 to 5, inclusive. For this reason, any bitstream that conforms to one or more profiles specified in this annex does not conform to any profile specified in Annex Error: Reference source not found when it contains any of the following:

–	any access unit that does not contain any NAL units with nal_unit_type equal to 1 or 5, but contains one or more NAL units with nal_unit_type equal to 6, 7, 8, 9, or 15;

–	any access unit in which one or more NAL units with nal_unit_type equal to 7, 8, or 15 is present after the last NAL unit in the access unit with nal_unit_type equal to 1 or 5.

The association of VCL NAL units to primary or redundant coded pictures is specified in subclause I.7.4.1.2.5.

The constraints for the detection of the first VCL NAL unit of a primary coded picture are specified in subclause I.7.4.1.2.4.

The constraint expressed in subclause H.Error: Reference source not found on the order of a buffering period SEI message is replaced by the following constraints.

–	When an SEI NAL unit containing a buffering period SEI message is present, the following applies:

–	If the buffering period SEI message is the only buffering period SEI message in the access unit and it is not included in an MVC scalable nesting SEI message or a 3DVC scalable nesting SEI message, the buffering period SEI message shall be the first SEI message payload of the first SEI NAL unit in the access unit.

–	Otherwise (the buffering period SEI message is not the only buffering period SEI message in the access unit or it is included in an MVC scalable nesting SEI message or it is included in a 3DVC scalable nesting SEI message), the following constraints are specified:

–	When a buffering period SEI message that is not included in either an MVC scalable nesting SEI message or a 3DVC scalable nesting SEI message is present, this buffering period SEI message shall be the only SEI message payload of the first SEI NAL unit in the access unit.

–	An MVC scalable nesting SEI message that includes a buffering period SEI message shall not include any other SEI messages and shall be the only SEI message inside the SEI NAL unit.

–	A 3DVC scalable nesting SEI message that includes a buffering period SEI message shall not include any other SEI messages and shall be the only SEI message inside the SEI NAL unit.

–	All SEI NAL units that precede an SEI NAL unit that contains an MVC scalable nesting SEI message with a buffering period SEI message as payload, or a 3DVC scalable nesting SEI message with a buffering period SEI message as payload in an access unit shall only contain buffering period SEI messages or MVC scalable nesting SEI messages with a buffering period SEI message as payload, or 3DVC scalable nesting SEI messages with a buffering period SEI message.

          - Detection of the first VCL NAL unit of a primary coded picture

The specification of subclause H.7.4.1.2.4 applies.

          - Order of VCL NAL units and association to coded pictures

The specification of subclause H.7.4.1.2.5 applies with following modifications.

Each VCL NAL unit is part of a coded picture.

Let voIdx be the value of VOIdx of any particular VCL NAL unit. The order of the VCL NAL units within a coded picture is constrained as follows:

- For all VCL NAL units following this particular VCL NAL unit, the value of VOIdx shall be greater than or equal to voIdx.

- All VCL NAL units for a depth view component, if present, shall follow any VCL NAL unit of a texture view component with a same value of VOIdx.

For each set of VCL NAL units within a texture or depth view component, the following applies:

–	If arbitrary slice order, as specified in Annex Error: Reference source not found, subclause Error: Reference source not found or subclause I.10, is allowed, coded slice NAL units of a view component may have any order relative to each other.

- Otherwise (arbitrary slice order is not allowed), coded slice NAL units of a slice group shall not be interleaved with coded slice NAL units of another slice group and the order of coded slice NAL units within a slice group shall be in the order of increasing macroblock address for the first macroblock of each coded slice NAL unit of the same slice group.

If a coded texture frame view component with a particular view_id is present in an access unit, a depth frame view component with the same view_id shall be present in the same access unit.

If a coded texture field view component with a particular view_id is either the first field view component of a complementary field pair or a non-paired field, the first depth field view component of complementary field pair, the first depth field view component of non- paired field or a depth frame view component with the same view_id shall be present in the access unit of that coded texture field view component. If a coded texture field view component with a particular view_id is the second field view component of a complementary field pair or non-paired field, a first depth field view component of complementary field pair or a non-paired field with the same view_id shall not be present in the access unit of that coded texture field view component. If a coded texture view component with a particular view_id is a frame view component, a depth frame view component with the same view_id shall be present in the access unit of that coded texture frame view component.

NAL units having nal_unit_type equal to 12 may be present in the access unit but shall not precede the first VCL NAL unit of the primary coded picture within the access unit.

NAL units having nal_unit_type equal to 0 or in the range of 24 to 31, inclusive, which are unspecified, may be present in the access unit but shall not precede the first VCL NAL unit of the primary coded picture within the access unit.

NAL units having nal_unit_type in the range of 22 to 23, inclusive, which are reserved, shall not precede the first VCL NAL unit of the primary coded picture within the access unit (when specified in the future by ITU-T | ISO/IEC).

      - Raw byte sequence payloads and RBSP trailing bits semantics

        - Sequence parameter set RBSP semantics

The semantics specified in subclause Error: Reference source not found apply.

          - Sequence parameter set data semantics

The semantics specified in subclause H.Error: Reference source not found apply with the substitution of 3DVC sequence parameter set for MVC sequence parameter set. All constraints specified in subclause H.Error: Reference source not found apply only to the texture view components for which the 3DVC sequence parameter set is the active texture MVC sequence parameter set or to the depth view components for which the 3DVC sequence parameter set is the active view MVC sequence parameter set as specified in subclause I.7.4.1.2.1.

            - Scaling list semantics

The semantics specified in subclause H.Error: Reference source not found apply.

          - Sequence parameter set extension RBSP semantics

The semantics specified in subclause Error: Reference source not found apply. Additionally, the following applies.

Sequence parameter set extension RBSPs can only follow sequence parameter set RBSPs in decoding order. Subset sequence parameter set RBSPs shall not be followed by a sequence parameter set extension RBSP. The contents of sequence parameter set extension RBSPs only apply when the base view, which conforms to one or more of the profiles specified in Annex Error: Reference source not found, of a coded video sequence conforming to one or more profiles specified in Annex I is decoded.

          - Subset sequence parameter set RBSP semantics

The semantics specified in subclause Error: Reference source not found apply with the following additions.

3dvc_vui_parameters_present_flag equal to 0 specifies that the syntax structure mvc_vui_parameters_extension( ) corresponding to 3DVC VUI parameters extension is not present. 3dvc_vui_parameters_present_flag equal to 1 specifies that the syntax structure mvc_vui_parameters_extension( ) is present and referred to as 3DVC VUI parameters extension.

texture_vui_parameters_present_flag equal to 0 specifies that the syntax structure mvc_vui_parameters_extension( ) corresponding to 3DVC texture sub-bitstream VUI parameters extension is not present. texture_vui_parameters_present_flag equal to 1 specifies that the syntax structure mvc_vui_parameters_extension( ) is present and referred to as 3DVC texture sub-bitstream VUI parameters extension.

          - Sequence parameter set MVC extension semantics

The semantics specified in subclause H.7.4.2.1.4 apply with the substitution of texture view component or depth view component for view component

          - Sequence parameter set 3DVC extension semantics

applicable_op_num_depth_views_minus1[ i ][ j ] plus 1 specifies the number of depth views required for decoding the target output views corresponding to the j-th operation point to which the level indicated by level_idc[ i ] applies. The number of depth views specified by applicable_op_num_depth_views_minus1 includes the target output views and the depth views that the depth views of the target output views depend on as specified by sub-bitstream extraction process in I.8.5 with tIdTarget equal to applicable_op_temporal_id[ i ][ j ] and viewIdTargetList equal to the list of applicable_op_target_view_id[ i ][ j ][ k ] for all k in the range of 0 to applicable_op_num_target_views_minus1[ i ][ j ], inclusive, as inputs. The value of applicable_op_num_depth_views_minus1[ i ][ j ] shall be in the range of 0 to 1023, inclusive.

        - Picture parameter set RBSP semantics

The semantics specified in subclause H.Error: Reference source not found apply with substituting 3DVC sequence parameter set for MVC sequence parameter set. All constraints specified in subclause H.Error: Reference source not found apply only to the texture or depth view components for which the picture parameter set is the active picture parameter set or the active view picture parameter set or the active texture picture parameter set as specified in subclause I.7.4.1.2.1.

        - Supplemental enhancement information RBSP semantics

The semantics specified in subclause H.Error: Reference source not found apply.

          - Supplemental enhancement information message semantics

The semantics specified in subclause H.Error: Reference source not found apply.

        - Access unit delimiter RBSP semantics

The semantics specified in subclause H.Error: Reference source not found apply.

NOTE – The value of primary_pic_type applies to the slice_type values in all slice headers of the primary coded picture, including the slice_type syntax elements in all NAL units with nal_unit_type equal to 1, 5, 20 or 21. NAL units with nal_unit_type equal to 2 are not present in bitstreams conforming to any of the profiles specified in this annex.

        - End of sequence RBSP semantics

The semantics specified in subclause H.Error: Reference source not found apply.

        - End of stream RBSP semantics

The semantics specified in subclause H.Error: Reference source not found apply.

        - Filler data RBSP semantics

The semantics specified in subclause H.Error: Reference source not found apply with the following modifications.

Filler data NAL units shall be considered to contain the syntax elements priority_id, view_id, and temporal_id with values that are inferred as follows:

- Let prevMvcNalUnit be the most recent NAL unit in decoding order that has nal_unit_type equal to 14, 20 or 21.

NOTE – The most recent NAL unit in decoding order with nal_unit_type equal to 14, 20 or 21 always belongs to the same access unit as the filler data NAL unit.

- The values of priority_id, view_id, and temporal_id for the filler data NAL unit are inferred to be equal to the values of priority_id, view_id, and temporal_id, respectively, of the NAL unit prevMvcNalUnit.

        - Slice layer without partitioning RBSP semantics

The semantics specified in subclause H.Error: Reference source not found apply.

        - Slice data partition RBSP semantics

Slice data partition syntax is not present in bitstreams conforming to one or more of the profiles specified in Annex I.

        - RBSP slice trailing bits semantics

        - The semantics specified in subclause H.Error: Reference source not found apply.RBSP trailing bits semantics

The semantics specified in subclause H.Error: Reference source not found apply.

        - Prefix NAL unit RBSP semantics

The semantics specified in subclause H.Error: Reference source not found apply.

        - Slice layer extension RBSP semantics

The semantics specified in subclause H.Error: Reference source not found apply.

      - Slice header semantics

The semantics specified in subclause H.Error: Reference source not found apply with the substitution of texture view component (for nal_unit_type 1, 5, and 20) or depth view component (for nal_unit_type 21) for view component and with the following modifications.

When nal_unit_type is equal to 1, 5, or 20, all constraints specified in subclause H.Error: Reference source not found apply only to the texture view components with the same value of VOIdx. When nal_unit_type is equal to 21, all constraints specified in subclause H.Error: Reference source not found apply only to the depth view components with the same value of VOIdx.

The value of the following 3DVC sequence parameter set syntax elements shall be the same across all coded slice NAL units of nal_unit_type 1, 5, and 20 of an access unit: chroma_format_idc.

The value of the following slice header syntax elements shall be the same across all coded slice NAL units of nal_unit_type 1, 5, and 20 of an access unit: field_pic_flag and bottom_field_flag.

        - Reference picture list modification semantics

The semantics specified in subclause H.Error: Reference source not found apply.

          - Reference picture list MVC modification semantics

The semantics specified in subclause H.Error: Reference source not found apply.

        - Prediction weight table semantics

The semantics specified in subclause H.Error: Reference source not found apply.

        - Decoded reference picture marking semantics

The semantics specified in subclause  Error: Reference source not found apply to each view independently, with " sequence parameter set" being replaced by "3DVC sequence parameter set", and "primary coded picture" being replaced by "texture view component” for nal_unit_type 1, 5, and 20, and by “depth view component” for nal_unit_type 21.

      - Slice data semantics

The semantics specified in subclause H.Error: Reference source not found apply.

      - Macroblock layer semantics

The semantics specified in subclause H.Error: Reference source not found apply.

        - Macroblock prediction semantics

The semantics specified in subclause H.Error: Reference source not found apply.

        - Sub-macroblock prediction semantics

The semantics specified in subclause H.Error: Reference source not found apply.

        - Residual data semantics

The semantics specified in subclause H.Error: Reference source not found apply.

          - Residual luma semantics

The semantics specified in subclause H.Error: Reference source not found apply.

          - Residual block CAVLC semantics

The semantics specified in subclause H.Error: Reference source not found apply.

          - Residual block CABAC semantics

The semantics specified in subclause H.Error: Reference source not found apply.

  - 3DVC decoding process

This subclause specifies the decoding process for an access unit of a coded video sequence conforming to one or more of the profiles specified in Annex I. Specifically, this subclause specifies how the decoded picture with multiple texture view components and multiple depth view components is derived from syntax elements and global variables that are derived from NAL units in an access unit when the decoder is decoding the operation point identified by the target temporal level and the target output views.

The decoding process is specified such that all decoders shall produce numerically identical results for the target output views. Any decoding process that produces identical results for the target output views to the process described here conforms to the decoding process requirements of this Recommendation | International Standard.

Unless stated otherwise, the syntax elements and derived upper-case variables that are referred to by the decoding process specified in this subclause and all child processes invoked from the process specified in this subclause are the syntax elements and derived upper-case variables for the current access unit.

The target output views are either specified by external means not specified in this Specification, or, when not specified by external means, there shall be one target output view which is the base view. Let OutputVOIdxList be the list of VOIdx values, in increasing order of VOIdx, of all the target output views in one access unit. The list OutputVOIdxList shall not change within a coded video sequence.

For profiles specified in Annex I, a target output view includes both the texture view and the depth view, which have the same VOIdx.

All sub-bitstreams that can be derived using the sub-bitstream extraction process with depthPresentFlagTarget equal to 0 or 1, pIdTarget equal to any value in the range of 0 to 63, inclusive, tIdTarget equal to any value in the range of 0 to 7, inclusive, viewIdTargetList consisting of any one or more viewIdTarget's identifying the views in the bitstream as inputs as specified in subclause I.8.5 shall result in a set of coded video sequences, with each coded video sequence conforming to one or more of the profiles specified in Annex Error: Reference source not found, Annex Error: Reference source not found and Annex I.

Let vOIdxList be a list of integer values specifying the VOIdx values of the view components of the access unit. The variable VOIdxMax is set equal to the maximum value of the entries in the list vOIdxList, and the variable vOIdxMin is set to the minimum value of the entries in the list vOIdxList. VOIdxMax shall be the same for all access units within a coded video sequence. vOIdxMin shall be the same for all anchor access units within a coded video sequence. When the current access unit is an anchor access unit, the variable VOIdxMin is set to vOIdxMin.

The 3DVC video decoding process specified in this subclause is repeatedly invoked for each texture and depth view component with VOIdx from vOIdxMin to VOIdxMax, inclusive, which is present in the list vOIdxList, in increasing order of VOIdx and in decoding order of texture or depth view components as specified in subclause I.7.4.1.2.5.

Outputs of the 3DVC video decoding process are decoded samples of the current primary coded picture including all decoded texture and depth view components.

For each texture view component and each depth view component, the specifications in clause H.Error: Reference source not found apply, with the decoding processes for picture order count, reference picture lists construction and decoded reference picture marking being modified in subclauses I.8.1, I.8.2 and I.8.3, respectively. The 3DVC inter prediction and inter-view prediction process is specified in subclause I.8.4.

    - 3DVC decoding process for picture order count

The specifications in subclause 8.2.1 apply independently for each texture view or depth view.

    - MVC decoding process for reference picture lists construction

The specification of subclause H.8.2 apply with substituting "view component" as either "texture view component" or "depth view component ", and "frame view component"" as either "depth view component" or "texture frame view component ", and "field view component" as "texture field view component" or "depth field view component".

Additionally, the following applies.

An inter-view reference component or the inter-view only reference component is identified by the view_id and a depth view component if the current slice is a part of a coded depth view component or a texture view component if the current slice is a part of a coded texture view component.

      - Initialisation process for reference picture list for inter-view prediction references

The specifications of subclause H.8.2.1 apply.

      - Modification process for reference picture lists

The specifications of subclause H.8.2.2 apply.

        - Modification process of reference picture lists for short-term reference pictures for inter prediction

The specifications of subclause H.8.2.2.1 apply.

        - Modification process of reference picture lists for long-term reference pictures for inter prediction

The specifications of subclause H.8.2.2.2 apply.

        - Modification process for reference picture lists for inter-view prediction references

The specifications of subclause H.8.2.2.3 apply.

    - 3DVC decoded reference picture marking process

The specifications of subclause H.8.3 apply. Additionaly, following applies.

The process specified in this subclause is invoked for a particular texture view or depth view with view order index VOIdx. The specifications in subclause H.8.3 apply with "view component" being replaced by either "texture view component" or "depth view component", "frame view component" being replaced by either "texture frame view component" or "depth view component", and "field view component" being replaced by either "texture field view component" or "depth field view component". During the invocation of the process for a particular texture view, only texture view components of the particular view are considered. During the invocation of the process for a particular depth view, only depth view components of the particular view are considered.The marking of view components of other views is not changed.

NOTE – A texture view component of a picture may have a different marking status than other texture view components of the same picture. A depth view component of a picture may have a different marking status than other depth view components of the same picture. A texture view component of a picture may have a different marking status than a depth view component.

    - 3DVC inter prediction and inter-view prediction process

The specifications of subclause H.8.4 apply.

    - Specification of bitstream subsets

The specifications of subclause H.8.5 apply.

      - Derivation process for required anchor view components

The specification of subclause H.8.5.1 apply with substituting "view component" with either "depth view component" or "texture view component" and "view" with either "depth view" or "texture view".

      - Derivation process for required non-anchor view components

The specification of subclause H.8.5.2 apply with substituting "view component" with either "depth view component" or "texture view component" and "view" with either "depth view" or "texture view".

      - Sub-bitstream extraction process

It is requirement of bitstream conformance that any sub-bitstream that is the output of the process specified in this subclause with depthPresentFlagTarget equal to 0 or 1, pIdTarget equal to any value in the range of 0 to 63, inclusive, tIdTarget equal to any value in the range of 0 to 7, inclusive, viewIdTargetList consisting of any one or more values of viewIdTarget identifying the views in the bitstream, shall be conforming to this Recommendation | International Standard.

NOTE 1 – A conforming bitstream contains one or more coded slice NAL units with priority_id equal to 0 and temporal_id equal to 0.

NOTE 2 – It is possible that not all operation points of sub-bitstreams resulting from the sub-bitstream extraction process have an applicable level_idc or level_idc[ i ]. In this case, each coded video sequence in a sub-bitstream must still conform to one or more of the profiles specified in Annex Error: Reference source not found, Annex Error: Reference source not found and Annex I, but may not satisfy the level constraints specified in subclauses Error: Reference source not found, I.10.2 and I.10.2, respectively.

Inputs to this process are:

–	a variable depthPresentFlagTarget (when present),

–	a variable pIdTarget (when present),

–	a variable tIdTarget (when present),

–	a list viewIdTargetList consisting of one or more values of viewIdTarget (when present).

Outputs of this process are a sub-bitstream and a list of VOIdx values VOIdxList.

When depthPresentFlagTarget is not present as input to this subclause, depthPresentFlagTarget is inferred to be equal to 0.

When pIdTarget is not present as input to this subclause, pIdTarget is inferred to be equal to 63.

When tIdTarget is not present as input to this subclause, tIdTarget is inferred to be equal to 7.

When viewIdTargetList is not present as input to this subclause, there shall be one value of viewIdTarget inferred in viewIdTargetList and the value of viewIdTarget is inferred to be equal to view_id of the base view.

The sub-bitstream is derived by applying the following operations in sequential order:

- Let VOIdxList be empty and minVOIdx be the VOIdx value of the base view.

- For each value of viewIdTarget included in viewIdTargetList, invoke the process specified in subclause I.8.5.1 for texture views with the viewIdTarget as input.

- If depthPresentFlagTarget is equal to 1, for each value of viewIdTarget included in viewIdTargetList, invoke the process specified in subclause I.8.5.1 for depth views with the viewIdTarget as input.

- For each value of viewIdTarget included in viewIdTargetList, invoke the process specified in subclause I.8.5.2 for texture views with the value of viewIdTarget as input.

- If depthPresentFlagTarget is equal to 1, for each value of viewIdTarget included in viewIdTargetList, invoke the process specified in subclause I.8.5.2 for depth views with the viewIdTarget as input.

- Mark all VCL NAL units and filler data NAL units for which any of the following conditions are true as "to be removed from the bitstream":

–	priority_id is greater than pIdTarget,

–	temporal_id is greater than tIdTarget,

–	anchor_pic_flag is equal to 1 and view_id is not marked as "required for anchor",

–	anchor_pic_flag is equal to 0 and view_id is not marked as "required for non-anchor",

–	nal_ref_idc is equal to 0 and inter_view_flag is equal to 0 and view_id is not equal to any value in the list viewIdTargetList,

–	nal_unit_type is equal to 21 and depthPresentFlagTarget is equal to 0.

- Remove all access units for which all VCL NAL units are marked as "to be removed from the bitstream".

- Remove all VCL NAL units and filler data NAL units that are marked as "to be removed from the bitstream".

- When VOIdxList contains only one value of VOIdx that is equal to minVOIdx, remove the following NAL units:

–	all NAL units with nal_unit_type equal to 14 or 15,

–	all NAL units with nal_unit_type equal to 6 in which the first SEI message has payloadType in the range of 36 to 44, inclusive.

NOTE 3 – When VOIdxList contains only one value of VOIdx equal to minVOIdx, the sub-bitstream contains only the base view or only a temporal subset of the base view.

- Remove all NAL units with nal_unit_type equal to 6 in which the first SEI message has payloadType equal to 0 or 1, or the first SEI message has payloadType equal to equal to 37 (MVC scalable nesting SEI message) and operation_point_flag in the first SEI message is equal to 1.

NOTE – The buffering period SEI and picture timing SEI messages, when not nested or nested in the MVC scalable nesting SEI message, apply for a sub-bitstream obtained with the sub-bitstream extraction process of subclause H.8.5.3, which does not process NAL units of nal_unit_type equal to 21.

- When depthPresentFlagTarget is equal to 0, the following applies in sequential order.

–	Replace all NAL units with nal_unit_type equal to 6 in which payloadType indicates a 3DVC texture scalable nesting SEI message with sei_op_texture_only_flag equal to 0 with a MVC scalable nesting SEI message with the same values of num_view_components_op_minus1, sei_op_view_id[ i ] and sei_op_temporal_id and the same nested SEI messges.

–	Remove all NAL units with nal_unit_type equal to 6 in which payloadType indicates a 3DVC texture scalable nesting SEI message.

–	The following applies for each active texture 3DVC sequence parameter set RBSP.

–	Replace mvc_vui_parameters_extension( ) syntax structure in an active texture 3DVC sequence parameter set RBSPs with the mvc_vui_parameters_extension( ) syntax structure of the 3DVC texture sub-bitstream VUI parameters extension, if both mvc_vui_parameters_extension( ) syntax structures apply to the same views.

–	Otherwise, remove mvc_vui_parameters_extension( ) syntax structure in an active texture 3DVC sequence parameter set RBSP.

–	When depthPresentFlagTarget is equal to 0, remove all NAL units with nal_unit_type equal to 6 in which the first SEI message has payloadType in the range of 48 to 52, inclusive.

- Let maxTId be the maximum temporal_id of all the remaining VCL NAL units. Remove all NAL units with nal_unit_type equal to 6 that only contain SEI messages that are part of an MVC scalable nesting SEI message or 3DVC scalable nesting SEI message with any of the following properties:

–	operation_point_flag is equal to 0 and all_view_components_in_au_flag is equal to 0 and none of sei_view_id[ i ] for all i in the range of 0 to num_view_components_minus1, inclusive, corresponds to a VOIdx value included in VOIdxList,

–	operation_point_flag is equal to 1 and either sei_op_temporal_id is greater than maxTId or the list of sei_op_view_id[ i ] for all i in the range of 0 to num_view_components_op_minus1, inclusive, is not a subset of viewIdTargetList (i.e., it is not true that sei_op_view_id[ i ] for any i in the range of 0 to num_view_components_op_minus1, inclusive, is equal to a value in viewIdTargetList).

- Remove each view scalability information SEI message and each operation point not present SEI message, when present.

- When VOIdxList does not contain a value of VOIdx equal to minVOIdx, the view with VOIdx equal to the minimum VOIdx value included in VOIdxList is converted to the base view of the extracted sub-bitstream. An informative procedure that outlines key processing steps to create a base view is described in subclause Error: Reference source not found.

NOTE 4 – When VOIdxList does not contain a value of VOIdx equal to minVOIdx, the resulting sub-bitstream according to the operation steps 1-13 above does not contain a base view that conforms to one or more profiles specified in Annex Error: Reference source not found. In this case, by this operation step, the remaining view with the new minimum VOIdx value is converted to be the new base view that conforms to one or more profiles specified in Annex Error: Reference source not found and Annex H.

      - Specification of the base view bitstream

A bitstream that conforms to one or more profiles as specified in Annex I shall contain a base view bitstream that conforms to one or more of the profiles specified in Annex Error: Reference source not found. This base view bitstream is derived by invoking the sub-bitstream extraction process as specified in subclause I.8.5.3 with no input and the base view bitstream being the output.

NOTE – Although all multiview bitstreams that conform to one or more of the profiles specified in this annex contain a base view bitstream that conforms to one or more of the profiles specified in Annex Error: Reference source not found, the complete multiview bitstream (prior to operation of the base view extraction process specified in this subclause) may not conform to any profile specified in Annex Error: Reference source not found.

      - Specification of the stereoscopic texture bitstream

A bitstream that conforms to a profile as specified in Annex I shall contain a sub-bitstream that conforms to one or more of the profiles specified in Annex H with number of views equal to 2. This stereoscopic texture bitstream is derived by invoking the sub-bitstream extraction process as specified in subclause I.8.5.3 with depthPresentFlagTarget equal to 0 and viewIdTargetList containing the view_id values of the base view and the texture view having VOIdx equal to 1 as inputs and the stereoscopic texture bitstream being the output.

  - Parsing process

The specifications in clause Error: Reference source not found apply.

  - Profiles and levels

The specifications in Annex H apply. Additional profiles and specific values of profile_idc are specified in the following.

The profiles that are specified in subclause I.10.1 are also referred to as the profiles specified in Annex I.

    - Profiles

All constraints for picture parameter sets that are specified in the following are constraints for picture parameter sets that become the active picture parameter set or an active view picture parameter set inside the bitstream. All constraints for 3DVC sequence parameter sets that are specified in the following are constraints for 3DVC sequence parameter sets that become the active 3DVC sequence parameter set or an active view 3DVC sequence parameter set inside the bitstream.

      - Multiview Depth High Profile

Bitstreams conforming to the Multiview Depth High profile shall obey the following constraints:

–	The base view bitstream as specified in subclause I.8.5.4 shall obey all constraints of the High profile specified in subclause Error: Reference source not found and all active sequence parameter sets shall fulfill one of the following conditions:

–	profile_idc is equal to 77 or constraint_set1_flag is equal to 1,

–	profile_idc is equal to 100.

–	The sub-bitstream of stereoscopic texture bitstream as specified in subclause I.8.5.5 shall obey all constraints of the Stereo High profile specified in subclause H.10.2 and all active MVC sequence parameter sets shall fulfill one of the following conditions:

–	profile_idc is equal to 128,

–	profile_idc is equal to 118 and constraint_set5_flag is equal to 1,

–	profile_idc is equal to 100,

–	profile_idc is equal to 77 or constraint_set1_flag is equal to 1.

–	Only I, P, and B slice types may be present.

–	NAL unit streams shall not contain nal_unit_type values in the range of 2 to 4, inclusive.

–	Arbitrary slice order is not allowed.

–	Picture parameter sets shall have num_slice_groups_minus1 equal to 0 only.

–	Picture parameter sets shall have redundant_pic_cnt_present_flag equal to 0 only.

–	When frame_mbs_only_flag is equal to 1 in an active sequence parameter set for a texture view, frame_mbs_only_flag shall be equal to 1 in the active sequence parameter set for the depth view having the same view_id.

–	3DVC sequence parameter sets for the depth views shall have chroma_format_idc equal to 0 only.

–	3DVC sequence parameter sets shall have bit_depth_luma_minus8 equal to 0 only.

–	3DVC sequence parameter sets shall have bit_depth_chroma_minus8 equal to 0 only.

–	3DVC sequence parameter sets shall have qpprime_y_zero_transform_bypass_flag equal to 0 only.

–	For each access unit, the value of level_idc for all active view 3DVC sequence parameter set RBSPs shall be the same as the value of level_idc for the active 3DVC sequence parameter set RBSP.

–	The level constraints specified for the Multiview Depth High profile in subclause I.10.2 shall be fulfilled.

Conformance of a bitstream to the Multiview Depth High profile is indicated by profile_idc being equal to 138.

Decoders conforming to the Multiview Depth High profile at a specific level shall be capable of decoding all bitstreams in which both of the following conditions are true:

- All active 3DVC sequence parameter sets have one or more of the following conditions fulfilled:

–	profile_idc is equal to 138,

–	profile_idc is equal to 128,

–	profile_idc is equal to 118 and constraint_set5_flag is equal to 1,

–	profile_idc is equal to 100,

–	profile_idc is equal to 77 or constraint_set1_flag is equal to 1.

- All active 3DVC sequence parameter sets have one or more of the following conditions fulfilled:

–	level_idc or (level_idc and constraint_set3_flag) represent a level less than or equal to the specific level,

–	level_idc[ i ] or (level_idc[ i ] and constraint_set3_flag) represent a level less than or equal to the specific level.

    - Levels

The specification of subclause H.10.2 apply.

      - Level limits common to Multiview Depth High profiles

The specification of subclause H.10.2.1 apply.

      - Profile specific level limits

- In bitstreams conforming to the Multiview Depth High profile, 3DVC sequence parameter sets shall have frame_mbs_only_flag equal to 1 for the levels specified in Error: Reference source not found.

  - Byte stream format

The specifications in Annex Error: Reference source not found apply.

  - 3DVC hypothetical reference decoder

The specifications in Annex Error: Reference source not found apply with substituting 3DVC sequence parameter set for MVC sequence parameter set.

  - 3DVC SEI messages

The specifications in Annex Error: Reference source not found together with the extensions and modifications specified in this subclause apply.

    - SEI message syntax

      - 3DVC view scalability information SEI message syntax

| 3dvc_view_scalability_info( payloadSize ) { | C | Descriptor |
| --- | --- | --- |
| num_operation_points_minus1 | 5 | ue(v) |
| for( i = 0; i <= num_operation_points_minus1; i++ ) { |  |  |
| operation_point_id[ i ] | 5 | ue(v) |
| priority_id[ i ] | 5 | u(5) |
| temporal_id[ i ] | 5 | u(3) |
| num_target_output_views_minus1[ i ] | 5 | ue(v) |
| op_with_depth_flag[ i ] | 5 | u(1) |
| for( j = 0; j <= num_target_output_views_minus1[ i ]; j++ ) |  |  |
| view_id[ i ][ j ] | 5 | ue(v) |
| profile_level_info_present_flag[ i ] | 5 | u(1) |
| bitrate_info_present_flag[ i ] | 5 | u(1) |
| frm_rate_info_present_flag[ i ] | 5 | u(1) |
| if( !num_target_output_views_minus1[ i ] ) |  |  |
| view_dependency_info_present_flag[ i ] | 5 | u(1) |
| parameter_sets_info_present_flag[ i ] | 5 | u(1) |
| bitstream_restriction_info_present_flag[ i ] | 5 | u(1) |
| if ( profile_level_info_present_flag[ i ] ) |  |  |
| op_profile_level_idc[ i ] | 5 | u(24) |
| if( bitrate_info_present_flag[ i ] ) { |  |  |
| avg_bitrate[ i ] | 5 | u(16) |
| max_bitrate[ i ] | 5 | u(16) |
| max_bitrate_calc_window[ i ] | 5 | u(16) |
| } |  |  |
| if( frm_rate_info_present_flag[ i ] ) { |  |  |
| constant_frm_rate_idc[ i ] | 5 | u(2) |
| avg_frm_rate[ i ] | 5 | u(16) |
| } |  |  |
| if( view_dependency_info_present_flag[ i ] ) { |  |  |
| num_directly_dependent_views[ i ] | 5 | ue(v) |
| for( j = 0; j < num_directly_dependent_views[ i ]; j++ ) { |  |  |
| directly_dependent_view_id[ i ][ j ] | 5 | ue(v) |
| if( op_with_depth_flag[ i ] ) { |  |  |
| num_directly_dependent_depth_views[ i ] | 5 | ue(v) |
| for( j = 0; j < num_directly_dependent_views[ i ]; j++ ) { |  |  |
| directly_dependent_depth_view_id[ i ][ j ] | 5 | ue(v) |
| } |  |  |
| } else |  |  |
| view_dependency_info_src_op_id[ i ] | 5 | ue(v) |
| if( parameter_sets_info_present_flag[ i ] ) { |  |  |
| num_seq_parameter_set_minus1[ i ] | 5 | ue(v) |
| for( j = 0; j <= num_seq_parameter_set_minus1[ i ]; j++ ) |  |  |
| seq_parameter_set_id_delta[ i ][ j ] | 5 | ue(v) |
| num_subset_seq_parameter_set_minus1[ i ] | 5 | ue(v) |
| for( j = 0; j <= num_subset_seq_parameter_set_minus1[ i ]; j++ ) |  |  |
| subset_seq_parameter_set_id_delta[ i ][ j ] | 5 | ue(v) |
| num_pic_parameter_set_minus1[ i ] | 5 | ue(v) |
| for( j = 0; j <= num_init_pic_parameter_set_minus1[ i ]; j++ ) |  |  |
| pic_parameter_set_id_delta[ i ][ j ] | 5 | ue(v) |
| } else |  |  |
| parameter_sets_info_src_op_id[ i ] | 5 | ue(v) |
| if( bitstream_restriction_info_present_flag[ i ] ) { |  |  |
| motion_vectors_over_pic_boundaries_flag[ i ] | 5 | u(1) |
| max_bytes_per_pic_denom[ i ] | 5 | ue(v) |
| max_bits_per_mb_denom[ i ] | 5 | ue(v) |
| log2_max_mv_length_horizontal[ i ] | 5 | ue(v) |
| log2_max_mv_length_vertical[ i ] | 5 | ue(v) |
| num_reorder_frames[ i ] | 5 | ue(v) |
| max_dec_frame_buffering[ i ] | 5 | ue(v) |
| } |  |  |
| } |  |  |
| } |  |  |

      - 3DVC scalable nesting SEI message syntax

| 3dvc_scalable_nesting( payloadSize ) { | C | Descriptor |
| --- | --- | --- |
| operation_point_flag | 5 | u(1) |
| if ( !operation_point_flag ) { |  |  |
| all_view_components_in_au_flag | 5 | u(1) |
| if( !all_view_components_in_au_flag ) { |  |  |
| num_view_components_minus1 | 5 | ue(v) |
| for( i = 0; i <= num_view_components_minus1; i++ ) { |  |  |
| sei_view_id[ i ] | 5 | u(10) |
| sei_view_applicability_flag[ i ] | 5 | u(1) |
| } |  |  |
| } |  |  |
| } else { |  |  |
| sei_op_texture_only_flag | 5 | u(1) |
| num_view_components_op_minus1 | 5 | ue(v) |
| for( i = 0; i <= num_view_components_op_minus1; i++ ) { |  |  |
| sei_op_view_id[ i ] | 5 | u(10) |
| } |  |  |
| sei_op_temporal_id | 5 | u(3) |
| } |  |  |
| while( !byte_aligned( ) ) |  |  |
| sei_nesting_zero_bit /* equal to 0 */ | 5 | f(1) |
| sei_message( ) | 5 |  |
| } |  |  |

      - Depth acquisition information SEI message syntax

| depth_acquisition_info( payloadSize ) { | C | Descriptor |
| --- | --- | --- |
| all_views_equal_flag | 5 | u(1) |
| if( all_views_equal_flag = = 0 ) { |  |  |
| num_views_minus1 | 5 | ue(v) |
| numViews = num_views_minus1 + 1 |  |  |
| } else { |  |  |
| numViews = 1 |  |  |
| num_frames | 5 | ue(v) |
| } |  |  |
| z_near_flag | 5 | u(1) |
| z_far_flag | 5 | u(1) |
| d_min_flag | 5 | u(1) |
| d_max_flag | 5 | u(1) |
| focal_length_x_flag | 5 | u(1) |
| focal_length_y_flag | 5 | u(1) |
| principal_point_x_flag | 5 | u(1) |
| principal_point_y_flag | 5 | u(1) |
| translation_flag | 5 | u(1) |
| rotation_flag | 5 | u(1) |
| for( i = 0; i < numViews; i++ ) { |  |  |
| if( z_near_flag ) |  |  |
| depth_acquisition_element( i, num_frames, 0, 7, ZNearSign, ZNearExp, ZNearMantissa, ZNearManLen ) |  |  |
| if( z_far_flag ) |  |  |
| depth_acquisition_sei_element( i, num_frames, 0, 7, ZFarSign, ZFarExp, ZFarMantissa, ZFarManLen ) |  |  |
| if( d_min_flag ) |  |  |
| depth_acquisition_sei_element( i, num_frames, 0, 7, DMinSign, DMinExp, DMinMantissa, DMinManLen ) |  |  |
| if( d_max_flag ) |  |  |
| depth_acquisition_sei_element( i, num_frames, 0, 7, DMaxSign, DMaxExp, DMaxMantissa, DMaxManLen ) |  |  |
| if( focal_length_x_flag ) |  |  |
| depth_acquisition_sei_element( i, num_frames, 1, 6, FocalLengthXSign, FocalLengthXExp, FocalLengthXMantissa, FocalLengthXManLen ) |  |  |
| if( focal_length_y_flag ) |  |  |
| depth_acquisition_element( i, num_frames, 1, 6, FocalLengthYSign, FocalLengthYExp, FocalLengthYMantissa, FocalLengthYManLen ) |  |  |
| if( principal_point_x_flag ) |  |  |
| depth_acquisition_element( i, num_frames, 1, 6, PrincipalPointXSign, PrincipalPointXExp, PrincipalPointXMantissa, PrincipalPointXManLen ) |  |  |
| if( principal_point_y_flag ) |  |  |
| depth_acquisition_element( i, num_frames, 1, 6, PrincipalPointYSign, PrincipalPointYExp, PrincipalPointYMantissa, PrincipalPointYManLen ) |  |  |
| if( translation_flag ) |  |  |
| depth_acquisition_element( i, num_frames, 1, 6, TXSign, TXExp, TXMantissa, TXManLen ) |  |  |
| if( rotation_flag ) |  |  |
| depth_acquisition_element( i, num_frames, 1, 6, YawSign, YawExp, YawMantissa, YawManLen ) |  |  |
| } |  |  |
| } |  |  |

      - Depth acquisition element syntax

| depth_acquisition_element( viewIdc, numFrames, precMode, expLen, OutSign, OutExp, OutMantissa, OutManLen ) { | C | Descriptor |
| --- | --- | --- |
| all_frames_equal_flag | 5 | u(1) |
| if( all_frames_equal_flag = = 0 ) |  |  |
| numFrames = 1 |  |  |
| for( i = 0; i < numFrames; i++ ) { |  |  |
| sign0 | 5 | u(1) |
| OutSign[ i, viewIdc ] = sign0 |  |  |
| exponent0 | 5 | u(expLen) |
| OutExp[ i, viewIdc ] = exponent0 |  |  |
| if( precMode = = 0 ) { |  |  |
| matissa_len_minus1 | 5 | u(5) |
| OutManLen[ i, viewIdc ] = manLen = mantissa_len_minus + 1 |  |  |
| } else { |  |  |
| prec | 5 | u(5) |
| if( exponent0 = = 0 ) |  |  |
| OutManLen[ i, viewIdc ] = manLen = Max( 0, prec – 30 ) |  |  |
| else |  |  |
| OutManLen[ i, viewIdc ] = manLen = Max( 0, exponent0 + prec – 31 ) |  |  |
| } |  |  |
| mantissa0 | 5 | u(manLen) |
| OutMantissa[ i, viewIdc ] = mantissa0 |  |  |
| } |  |  |
| } |  |  |

      - Depth representation information SEI message syntax

| depth_represention_information( payloadSize ) { | C | Descriptor |
| --- | --- | --- |
| depth_representation_type | 5 | ue(v) |
| all_views_equal_flag | 5 | u(1) |
| if( all_views_equal_flag = = 0 ){ |  |  |
| num_views_minus1 | 5 | ue(v) |
| numViews = num_views_minus1 + 1 |  |  |
| }else{ |  |  |
| numViews = 1 |  |  |
| } |  |  |
| for( i = 0; i < numViews; i++ ) { |  |  |
| depth_representation_base_view_id[i] | 5 | ue(v) |
| } |  |  |
| if ( depth_representation_type == 3 ) { |  |  |
| depth_nonlinear_representation_num_minus1 |  | ue(v) |
| depth_nonlinear_representation_num = depth_nonlinear_representation_num_minus1+1 |  |  |
| for( i = 1; i <= depth_nonlinear_representation_num; i++ ) |  |  |
| depth_nonlinear_representation_model[ i ] |  | ue(v) |
| } |  |  |
| } |  |  |

      - 3D reference displays information SEI message syntax

| 3d_reference_displays_info( payloadSize ) { | C | Descriptor |
| --- | --- | --- |
| prec_ref_baseline | 5 | ue(v) |
| prec_ref_display_width | 5 | ue(v) |
| ref_viewing_distance_flag | 5 | u(1) |
| if( ref_viewing_distance_flag ) |  |  |
| prec_ref_viewing_dist | 5 | ue(v) |
| num_ref_displays_minus1 | 5 | ue(v) |
| numRefDisplays = num_ref_displays_minus1 + 1 |  |  |
| for( i = 0; i < numRefDisplays; i++ ) { |  |  |
| exponent_ref_baseline[ i ] | 5 | u(6) |
| mantissa_ref_baseline[ i ] | 5 | u(v) |
| exponent_ref_display_width[ i ] | 5 | u(6) |
| mantissa_ref_display_width[ i ] | 5 | u(v) |
| if( ref_viewing_distance_flag ) { |  |  |
| exponent_ref_viewing_distance[ i ] | 5 | u(6) |
| mantissa_ref_viewing_distance[ i ] | 5 | u(v) |
| } |  |  |
| additional_shift_present_flag[ i ] | 5 | u(1) |
| if ( additional_shift_present[ i ] ) |  |  |
| num_pixel_shift_plus128[ i ] | 5 | u(8) |
| } |  |  |
| } |  |  |

    - SEI message semantics

Depending on payloadType, the corresponding SEI message semantics are extended as follows:

–	If payloadType is equal to 2, 3, 6, 7, 8, 9, 10, 11, 12, 13, 14, 15, 16, 17, 18, 19, 20, 21, 22, 23, 45 or 47, the following applies:

–	If the SEI message is not included in an MVC scalable nesting SEI message or a 3DVC scalable nesting SEI message, it applies to the texture view component of the current access unit with VOIdx equal to VOIdxMin.

–	Otherwise, if included in an MVC scalable nesting SEI message and not included in a 3DVC scalable nesting SEI message, it applies to all texture view components of the current access unit when all_view_components_in_au_flag is equal to 1, or it applies to all texture view components of the current access unit with view_id equal to sei_view_id[ i ] for any i in the range of 0 to num_view_components_minus1, inclusive, when all_view_components_in_au_flag is equal to 0. When payloadType is equal to 10 for the SEI message that is included in an MVC scalable nesting SEI message, the semantics for sub_seq_layer_num of the sub-sequence information SEI message is modified as follows:

sub_seq_layer_num specifies the sub-sequence layer number of the current picture. When the current picture resides in a sub-sequence for which the first picture in decoding order is an IDR picture, the value of sub_seq_layer_num shall be equal to 0. For a non-paired reference field, the value of sub_seq_layer_num shall be equal to 0. sub_seq_layer_num shall be in the range of 0 to 255, inclusive.

–	Otherwise, if not included in an MVC scalable nesting SEI message and included in an 3DVC scalable nesting SEI message, it applies to all depth view components or view component pairs of the current access unit when all_view_components_in_au_flag is equal to 1, or it applies to all depth view components or view component pairs of the current access unit with view_id equal to sei_view_id[ i ] for any i in the range of 0 to num_view_components_minus1, inclusive, when all_view_components_in_au_flag is equal to 0. When payloadType is equal to 10 for the SEI message that is included in an 3DVC scalable nesting SEI message, the semantics for sub_seq_layer_num of the sub-sequence information SEI message is modified as follows:

sub_seq_layer_num specifies the sub-sequence layer number of the current picture. When the current picture resides in a sub-sequence for which the first picture in decoding order is an IDR picture, the value of sub_seq_layer_num shall be equal to 0. For a non-paired reference field, the value of sub_seq_layer_num shall be equal to 0. sub_seq_layer_num shall be in the range of 0 to 255, inclusive.

–	Otherwise, if payloadType is equal to 41, 42 or 43, the following applies:

–	If the SEI message is not included in 3DVC scalable nesting SEI message, it applies to texture views only and NAL units having nal_unit_type equal to 21 are non-VCL NAL units.

–	Otherwise (the SEI message is included in 3DVC scalable nesting SEI message), the SEI message applies to depth views, to texture views or both texture views all depth views, depending on the values of the syntax elements of the 3DVC scalable nesting SEI message.

–	Otherwise, if payloadType is equal to 0 or 1, the following applies:

–	If the SEI message is not included in an MVC scalable nesting SEI message or a 3DVC scalable nesting SEI message or a 3DVC texture sub-bitstream HRD nesting SEI message, the following applies. When the SEI message and all other SEI messages with payloadType equal to 0 or 1 not included in an MVC scalable nesting SEI message or a 3DVC scalable nesting SEI message or a 3DVC texture sub-bitstream HRD nesting SEI message are used as the buffering period and picture timing SEI messages for checking the bitstream conformance according to Annex Error: Reference source not found and the decoding process specified in clauses Error: Reference source not found-Error: Reference source not found is used, the bitstream shall be conforming to this Recommendation | International Standard.

–	Otherwise, if the SEI message is included in an MVC scalable nesting SEI message and not included in a 3DVC scalable nesting SEI message or a 3DVC texture sub-bitstream HRD nesting SEI message, the following applies. When the SEI message and all other SEI messages with payloadType equal to 0 or 1 included in an MVC scalable nesting SEI message with identical values of sei_op_temporal_id and sei_op_view_id[ i ] for all i in the range of 0 to num_view_components_op_minus1, inclusive, are used as the buffering period and picture timing SEI messages for checking the bitstream conformance according to Annex Error: Reference source not found, the bitstream that would be obtained by invoking the bitstream extraction process as specified in subclause I.8.3 with tIdTarget equal to sei_op_temporal_id and viewIdTargetList equal to sei_op_view_id[ i ] for all i in the range of 0 to num_view_components_op_minus1, inclusive, shall be conforming to this Recommendation | International Standard.

In the semantics of subclauses Error: Reference source not found and Error: Reference source not found, the syntax elements num_units_in_tick, time_scale, fixed_frame_rate_flag, nal_hrd_parameters_present_flag, vcl_hrd_parameters_present_flag, low_delay_hrd_flag, and pic_struct_present_flag and the derived variables NalHrdBpPresentFlag, VclHrdBpPresentFlag, and CpbDpbDelaysPresentFlag are substituted with the syntax elements vui_mvc_num_units_in_tick[ i ], vui_mvc_time_scale[ i ], vui_mvc_fixed_frame_rate_flag[ i ], vui_mvc_nal_hrd_parameters_present_flag[ i ], vui_mvc_vcl_hrd_parameters_present_flag[ i ], vui_mvc_low_delay_hrd_flag[ i ], and vui_mvc_pic_struct_present_flag[ i ] and the derived variables VuiMvcNalHrdBpPresentFlag[ i ], VuiMvcVclHrdBpPresentFlag[ i ], and VuiMvcCpbDpbDelaysPresentFlag[ i ].

The values of seq_parameter_set_id's in all buffering period SEI messages included in MVC scalable nesting SEI messages and associated with operation points for which the greatest VOIdx values in the associated bitstream subsets are identical shall be identical.

–	Otherwise, if the SEI message is included in a 3DVC scalable nesting SEI message and not included in an MVC scalable nesting SEI message or a 3DVC texture sub-bitstream HRD nesting SEI message, the following applies. When the SEI message and all other SEI messages with payloadType equal to 0 or 1 included in a 3DVC scalable nesting SEI message with identical values of sei_op_temporal_id and sei_op_view_id[ i ] for all i in the range of 0 to num_view_components_op_minus1, inclusive, are used as the buffering period and picture timing SEI messages for checking the bitstream conformance according to Annex Error: Reference source not found, the bitstream that would be obtained by invoking the bitstream extraction process as specified in subclause I.8.5 with depthPresentTargetFlag equal to 1, tIdTarget equal to sei_op_temporal_id and viewIdTargetList equal to sei_op_view_id[ i ] for all i in the range of 0 to num_view_components_op_minus1, inclusive, shall be conforming to this Recommendation | International Standard.

In the semantics of subclauses Error: Reference source not found and Error: Reference source not found, the syntax elements num_units_in_tick, time_scale, fixed_frame_rate_flag, nal_hrd_parameters_present_flag, vcl_hrd_parameters_present_flag, low_delay_hrd_flag, and pic_struct_present_flag and the derived variables NalHrdBpPresentFlag, VclHrdBpPresentFlag, and CpbDpbDelaysPresentFlag are substituted with the syntax elements vui_mvc_num_units_in_tick[ i ], vui_mvc_time_scale[ i ], vui_mvc_fixed_frame_rate_flag[ i ], vui_mvc_nal_hrd_parameters_present_flag[ i ], vui_mvc_vcl_hrd_parameters_present_flag[ i ], vui_mvc_low_delay_hrd_flag[ i ], and vui_mvc_pic_struct_present_flag[ i ] and the derived variables VuiMvcNalHrdBpPresentFlag[ i ], VuiMvcVclHrdBpPresentFlag[ i ], and VuiMvcCpbDpbDelaysPresentFlag[ i ] for the 3DVC VUI parameters extension.

The values of seq_parameter_set_id's in all buffering period SEI messages included in 3DVC scalable nesting SEI messages and not included in either MVC scalable nesting SEI messages or 3DVC texture sub-bitstream HRD nesting SEI messages and associated with operation points for which the greatest VOIdx values in the associated bitstream subsets are identical shall be identical.

–	Otherwise, if the SEI message is included in a 3DVC texture sub-bitstream HRD nesting SEI message, the following applies. When the SEI message and all other SEI messages included in a 3DVC texture sub-bitstream HRD nesting SEI message with identical values of texture_subbitstream_temporal_id and texture_subbitstream_view_id[ i ] for all i in the range of 0 to num_texture_subbitstream_view_components_minus1, inclusive, are used as the buffering period and picture timing SEI messages for checking the bitstream conformance according to Annex Error: Reference source not found, the bitstream that would be obtained by invoking the bitstream extraction process as specified in subclause I.8.5 with depthPresentTargetFlag equal to 0, tIdTarget equal to texture_subbitstream_temporal_id and viewIdTargetList equal to texture_subbitstream_view_id[ i ] for all i in the range of 0 to num_texture_subbitstream_view_components_minus1, inclusive, shall be conforming to this Recommendation | International Standard.

In the semantics of subclauses Error: Reference source not found and Error: Reference source not found, the syntax elements num_units_in_tick, time_scale, fixed_frame_rate_flag, nal_hrd_parameters_present_flag, vcl_hrd_parameters_present_flag, low_delay_hrd_flag, and pic_struct_present_flag and the derived variables NalHrdBpPresentFlag, VclHrdBpPresentFlag, and CpbDpbDelaysPresentFlag are substituted with the syntax elements vui_mvc_num_units_in_tick[ i ], vui_mvc_time_scale[ i ], vui_mvc_fixed_frame_rate_flag[ i ], vui_mvc_nal_hrd_parameters_present_flag[ i ], vui_mvc_vcl_hrd_parameters_present_flag[ i ], vui_mvc_low_delay_hrd_flag[ i ], and vui_mvc_pic_struct_present_flag[ i ] and the derived variables VuiMvcNalHrdBpPresentFlag[ i ], VuiMvcVclHrdBpPresentFlag[ i ], and VuiMvcCpbDpbDelaysPresentFlag[ i ] for the 3DVC texture sub-bitstream VUI parameters extension.

The values of seq_parameter_set_id's in all buffering period SEI messages included in 3DVC texture sub-bitstream HRD nesting SEI messages and associated with operation points for which the greatest VOIdx values in the associated bitstream subsets are identical shall be identical.

–	Otherwise (all remaining payloadType values), the corresponding SEI message semantics are not extended.

For the semantics of SEI messages with payloadType in the range of 0 to 23, inclusive, which are specified in subclause Error: Reference source not found, 3DVC sequence parameter set is substituted for sequence parameter set; the parameters of 3DVC sequence parameter set RBSP and picture parameter set RBSP that are in effect are specified in subclauses I.7.4.2.1and I.7.4.2.2, respectively.

Coded video sequences conforming to one or more of the profiles specified in Annex I shall not include SEI NAL units that contain SEI messages with payloadType in the range of 24 to 35, inclusive.

When an SEI NAL unit contains an SEI message with payloadType in the range of 36 to 44, inclusive, it shall not contain any SEI messages with payloadType less than 36 and the first SEI message in the SEI NAL unit shall have payloadType in the range of 36 to 44, inclusive.

When an MVC scalable nesting SEI message (payloadType equal to 37) or a view scalability information SEI message (payloadType equal to 38) or an operation point not present SEI message (payloadType equal to 43) or a 3DVC scalable nesting SEI message (payloadType equal to 48) is present in an SEI NAL unit, it shall be the only SEI message in the SEI NAL unit.

      - 3DVC view scalability information SEI message semantics

The syntax elements in the view scalability information SEI (except num_directly_dependent_depth_views[ i ] and directly_dependent_depth_view_id[ i ][ j ]) have the same semantics as the ones with the same names and present in the view scalability information SEI message in Annex H, but apply to operation points that may potentially contain depth view components.

op_with_depth_flag[ i ] equal to 1 indicates that the current operation point contains depth views. op_with_depth_flag[ i ] equal to 0 indicates that the current operation point doesn’t contain any depth views.

num_directly_dependent_views[ i ] and directly_dependent_view_id[ i ][ j ] apply only to texture view components of an operation point if it contains both texture and depth (op_with_depth_flag[ i ] equal to 1).

num_directly_dependent_depth_views[ i ] specifies the number of depth views that the target output view of the current operation point is directly dependent on within the representation of the current operation point. The value of num_directly_dependent_depth_views[ i ] shall be in the range of 0 to 16, inclusive.

directly_dependent_depth_view_id[ i ][ j ] specifies the view_id of the j-th depth view that the depth view of the target output view of the current operation point is directly dependent on within the representation of the current operation point. The value of directly_dependent_depth_view_id[ i ][ j ] shall be in the range of 0 to 1023, inclusive.

      - 3DVC scalable nesting SEI message semantics

The syntax elements in the 3DVC scalable nesting SEI message have the same semantics as the ones with the same names and present in the MVC scalable nesting SEI message in Annex H.

sei_view_applicability_flag[ i ] equal to 1 indicates that the nested SEI message applies to both the texture view component and the depth view component of the view with view_id equal to sei_view_id[ i ]. sei_view_applicability_flag[ i ] equal to 0 indicates that the nested SEI message applies only to the depth view component of the view with view_id equal to sei_view_id[ i ].

sei_op_texture_only_flag equal to 0 specifies that the semantics of sei_op_view_id[ i ] and sei_op_temporal_id apply to both texture and depth views, if present. sei_op_texture_only_flag equal to 1 specifies that the nested SEI message as well as the semantics of sei_op_view_id[ i ] and sei_op_temporal_id apply to the sub-bitstream obtained by the sub-bitstream extraction process of subclause I.8.5.3 with depthPresentFlagTarget equal to 0, tIdTarget equal to sei_op_temporal_id, and viewIdTargetList equal to sei_op_view_id[ i ] for all values of i in the range of 0 to num_view_components_op_minus1, inclusive, as inputs.

NOTE 1 – MVC scalable nesting SEI message should be used for nesting SEI messages, when depth views may or may not be present in the bitstream, the nested SEI messages apply only to indicated texture view components and the semantics of the nested SEI messages apply when VCL and non-VCL NAL units are classified according to Annex H NAL unit type class of Table 7-1.

NOTE 2 – 3DVC scalable nesting SEI message with sei_op_texture_only_flag equal to 1 should be used when the nested SEI messages concern a sub-bitstream from which depth views have been excluded. For example, 3DVC scalable nesting SEI message with sei_op_texture_only_flag equal to 1 may include buffering period and picture timing SEI messages which apply only to a sub-bitstream containing texture views from which depth views have been removed using the sub-bitstream extraction process of subclause I.8.5.3 with depthPresentFlagTarget equal to 0.

      - Depth acquisition information SEI message semantics

The syntax elements in the depth acquisition information SEI message specifies various parameters for depth views for the purpose of processing decoded texture and depth view components prior to rendering on a 3D display, such as view synthesis. Specifically, camera parameters and depth ranges for depth views are specified. Camera parameters maybe used as camera parameters for the texture views with same VOIdx. When present, the depth acquisition information SEI message may be associated with any access unit. It is recommended, when present, the SEI message is associated with an IDR access unit for the purpose of random access. The information signaled in the SEI message applies to all the access units from the access unit the SEI message is associated with to the next access unit, in decoding order, containing an SEI message of the same type, exclusively, or to the end of the coded video sequence, which ever is earlier in decoding order.

all_views_equal_flag equal to 0 specifies that depth acquisition information may not be identical to respective values for each view in target views. all_views_equal_flag equal to 1 specifies that the depth acquisition information are identical to respective values for all target views.

num_views_minus1 shall be equal to the value of the syntax element num_views_minus1 in the 3DVC sequence parameter set for the coded video sequence. The value of num_views_minus1 shall be in the range of 0 to 1023, inclusive.

num_frames specifies the number of frames from the frame the depth acquisition information SEI message is associated with to the next frame which the depth acquisition information SEI message is associated with, exclusive, or to the end of the coded video sequence.

z_near_flag equal to 0 specifies that the syntax elements specifying the nearest depth value are not present in the syntax structure. z_near_flag equal to 1 specifies that the syntax elements specifying the nearest depth value are present in the syntax structure.

z_far_flag equal to 0 specifies that the syntax elements specifying the farthest depth value are not present in the syntax structure. z_near_flag equal to 1 specifies that the syntax elements specifying the farthest depth value are present in the syntax structure.

d_min_flag equal to 0 specifies that the syntax elements specifying the minimum disparity value are not present in the syntax structure. d_min_flag equal to 1 specifies that the syntax elements specifying the minimum disparity value are present in the syntax structure.

d_max_flag equal to 0 specifies that the syntax elements specifying the maximum disparity value are not present in the syntax structure. z_near_flag equal to 1 specifies that the syntax elements specifying the maximum disparity value are present in the syntax structure.

focal_length_x_flag equal to 0 specifies that the syntax elements specifying the horizontal focal length are not present in the syntax structure. focal_length_x_flag equal to 1 specifies that the syntax elements specifying the horizontal focal length are present in the syntax structure.

focal_length_y_flag equal to 0 specifies that the syntax elements specifying the vertical focal length are not present in the syntax structure. focal_length_y_flag equal to 1 specifies that the syntax elements specifying the vertical focal length are present in the syntax structure.

principal_point_x_flag equal to 0 specifies that the syntax elements specifying the horizontal principal point are not present in the syntax structure. principal_point_x_flag equal to 1 specifies that the syntax elements specifying the horizontal principal point are present in the syntax structure.

principal_point_y_flag equal to 0 specifies that the syntax elements specifying the vertical principal point are not present in the syntax structure. principal_point_y_flag equal to 1 specifies that the syntax elements specifying the vertical principal point are present in the syntax structure.

translation_flag equal to 0 specifies that the syntax elements specifying the horizontal translation between views are not present in the syntax structure. translation_flag equal to 1 specifies that the syntax elements specifying the horizontal translation between views are present in the syntax structure.

rotation_flag equal to 0 specifies that the syntax elements specifying the angle of the yaw rotation between views are not present in the syntax structure. rotation_flag equal to 1 specifies that the syntax elements specifying the angle of the yaw rotation between views are present in the syntax structure.

      - Depth acquisition element semantics

The syntax structure specifies the value of an element in depth acquisition information.

The contents of the syntax structure are controlled through input variables viewIdc, numFrames, precMode, and expLen the semantics of which are as follows.

–	viewIdc specifies the view indicator

–	numFrames specifies num_frames

–	precMode equal to 0 specifies that the number of bits in the mantissa syntax element is in 32-bit variables. precMode equal to 1 specifies that the number of bits in the mantissa syntax element is in 64-bit variables.

–	expLen specifies the number of bits in the exponent syntax element.

The syntax structure uses OutSign, OutExp, and OutMantissa variables for both input and output, where each variable is indexed by [ frame_num, viewIdc ], frame_num being a i-th frame number and viewIdc being a view indicator.

–	Set num_frames to numFrames if all_frames_equal_flag is equal to 0. Otherwise, set numFrames to 1.

- Set preDirection to pred_direction.

Some of the views for which the depth acquisition information is included in the depth_acquisition_info( ) syntax structure may not be present in the coded video sequence.

The variables x in Table I-1 are derived as follows from the respective variables f, s, e, n, and v indicated in Table I-1.

–	If 0 < e < 127, x = (−1)s * 2e−31 * (1 + n ÷ 2v).

–	Otherwise (e is equal to 0), x = (−1)s * 2−(30+v) * n.

NOTE – The above specification is similar to that found in IEC 60559:1989, Binary floating-point arithmetic for microprocessor systems.

- Table I-1 – Association between depth parameter variables and syntax elements.

| x | s | e | n | v |
| --- | --- | --- | --- | --- |
| ZNear | ZNearSign | ZNearExp | ZNearMantissa | ZNearManLen |
| ZFar | ZFarSign | ZFarExp | ZFarMantissa | ZFarManLen |
| DMax | DMaxSign | DMaxExp | DMaxMantissa | DMaxManLen |
| DMin | DMinSign | DMinExp | DMinMantissa | DMinManLen |
| FocalLengthX | FocalLengthXSign | FocalLengthXExp | FocalLengthXMantissa | FocalLengthXManLen |
| FocalLengthY | FocalLengthYSign | FocalLengthYExp | FocalLengthYMantissa | FocalLengthYManLen |
| PrincipalPointX | PrincipalPointXSign | PrincipalPointXExp | PrincipalPointXMantissa | PrincipalPointXManLen |
| PrincipalPointY | PrincipalPointYSign | PrincipalPointYExp | PrincipalPointYMantissa | PrincipalPointYManLen |
| TX | TXSign | TXExp | TXMantissa | TXManLen |
| Yaw | YawSign | YawExp | YawMantissa | YawManLen |

The variablex x in Table I-1 are defined as follows:

- ZNear: The closes depth values.

- Zfar: The farthest depth values.

- DMax: The maximum disparity values.

- DMin: The minimum disparity values.

- FocalLengthX: The horizontal focal length values.

- FocalLenghY: The vertical focal length values.

- PrincipalPointX: The horizontal principal point values.

- PrincipalPointY: The vertical principal point values.

- TX: The horizontal translation delta values.

- AbsTX: The horizontal translation values.

The variable AbsTX is specified as follows:

AbsTX[ 0 ] = 0
for( i = 1; i < num_views_minus1; i ++ )
	AbsTX[ i ] = AbsTX[ i – 1 ] + TX[ i – 1 ]

- Yaw: The rotation angle around the vertical axis.

all_frames_equal_flag equal to 0 specifies that 3DVC multiview acquisition information may not be identical to respective values for each frame in i-th view. all_frames_equal_flag equal to 1 specifies that the 3DVC multiview acquisition information are identical to respective values for all frames in i-th view.

mantissa_len_minus1 + 1 specifies the number of bits in the mantissa syntax element. The value of mantissa_len_minus1 shall be in the range of 0 to 31, inclusive.

prec specifies the exponent of the maximum allowable truncation error for the value represented by the sign, exponent, and mantissa given by 2−prec. The value of prec shall be in the range of 0 to 31, inclusive.

pred_direction equal to 0 specifies that the 3DV acquisition variables for the base view may be predicted from the respective variables of two pictures. pred_direction equal to 1 specifies that the 3DV acquisition variables for the base view may be predicted from the respective variables of one picture. pred_direction equal to 2 specifies that the 3DV acquisition variables for the base view are not predicted. The value of pred_direction shall be in the range of 0 to 2, inclusive.

sign0 equal to 0 indicates that the sign of the value provided in the loop entry is positive. sign0 equal to 1 indicates that the sign is negative.

exponent0 specifies the exponent of the value provided by the loop entry. The value of exponent0 shall be in the range of 0 to 2expLen – 2, inclusive. The value 2expLen – 1 is reserved for future use by ITU-T | ISO/IEC. Decoders shall treat the value 2expLen – 1 as indicating an unspecified value.

mantissa0 specifies the mantissa of the value provided by the loop entry.

      - Depth representation information SEI message semantics

The syntax elements in the depth representation information SEI message specifies various depth representation for depth views for the purpose of processing decoded texture and depth view components prior to rendering on a 3D display, such as view synthesis. It is recommended, when present, the SEI message is associated with an IDR access unit for the purpose of random access. The information signaled in the SEI message applies to all the access units from the access unit the SEI message is associated with to the next access unit, in decoding order, containing an SEI message of the same type, exclusively, or to the end of the coded video sequence, whichever is earlier in decoding order.

depth_representation_type specifies the representation definition of luma pixels in coded frame of depth views as specified in Table I-2. In Table I-2, disparity specifies the horizontal displacement between two texture views and Z value specifies the distance from a camera.

Table I-2 Definition of depth_representation_type

| depth_representation_type | Interpretation |
| --- | --- |
| 0 | Each luma pixel value in coded frame of depth views represents an inverse of Z value normalized in range from 0 to 255 |
| 1 | Each luma pixel value in coded frame of depth views represents disparity normalized in range from 0 to 255 |
| 2 | Each luma pixel value in coded frame of depth views represents Z value normalized in range from 0 to 255 |
| 3 | Each luma pixel value in coded frame of depth views represents nonlinearly mapped disparity, normalized in range from 0 to 255. |
| Other values | Reserved for future use |

all_views_equal_flag equal to 0 specifies that depth representation base view may not be identical to respective values for each view in target views. all_views_equal_flag equal to 1 specifies that the depth representation base views are identical to respective values for all target views.

num_views_minus1 shall be equal to the value of the syntax element num_views_minus1 in the 3DVC sequence parameter set for the coded video sequence. The value of num_views_minus1 shall be in the range of 0 to 1023, inclusive.

depth_representaion_base_view_id[i] specifies the view identifier for the NAL unit of either base view which the disparity for coded depth frame of i-th view_id is derived from (depth_representation_type equal to 1 or 3) or base view which the Z-axis for the coded depth frame of i-th view_id is defined as the optical axis of (depth_representation_type equal to 0 or 2).

depth_nonlinear_representation_num_minus1 + 2 specifies the number of piecewise linear segments for mapping of depth values to a scale that is uniformly quantized in terms of disparity.

depth_nonlinear_representation_model[ i ] specifies the piecewise linear segments for mapping of depth values to a scale that is uniformly quantized in terms of disparity.

NOTE – When depth_representation_type  is equal to 3, depth view component contains nonlinearly transformed depth samples. Variable DepthLUT [ i ], as specified below, is used to transform coded depth sample values from nonlinear representation to the linear representation – disparity normalized in range from 0 to 255. The shape of this transform is defined by means of line-segment-approximation in two-dimensional linear-disparity-to-nonlinear-disparity space. The first (0, 0) and the last (255, 255) nodes of the curve are predefined. Positions of additional nodes are transmitted in form of deviations (depth_nonlinear_representation_model[ i ]) from the straight-line curve. These deviations are uniformly distributed along the whole range of 0 to 255, inclusive, with spacing depending on the value of nonlinear_depth_representation_num.

Variable DepthLUT[ i ] for i in the range of 0 to 255, inclusive, is specified as follows.

depth_nonlinear_representation_model[ 0 ] = 0
depth_nonlinear_representation_model[depth_nonlinear_representation_num + 1 ] = 0
for( k=0; k<= depth_nonlinear_representation_num; ++k )
{
	pos1 = ( 255 * k ) / (depth_nonlinear_representation_num + 1 )
	dev1 = depth_nonlinear_representation_model[ k ]
	pos2 = ( 255 * ( k+1 ) ) / (depth_nonlinear_representation_num + 1 ) )
	dev2 = depth_nonlinear_representation_model[ k+1 ]

	x1 = pos1 - dev1
	y1 = pos1 + dev1
	x2 = pos2 - dev2
	y2 = pos2 + dev2

	for ( x = max( x1, 0 ); x <= min( x2, 255 ); ++x )
		DepthLUT[ x ] = Clip3( 0, 255, Round( ( ( x - x1 ) * ( y2 - y1 ) ) ÷ ( x2 - x1 ) + y1 ) )
}

      - 3D reference displays information SEI message semantics

When present, this SEI message shall be associated with an IDR access unit. A reference displays information message contains information about the reference display width(s) and reference viewing distance(s) as well as information about the corresponding baseline distance(s) and additional horizontal image shift(s), which form a stereo-pair for the reference display width and the reference viewing distance. This information enables a view renderer to produce a proper stereo-pair for the target screen width and the viewing distance. The reference display width and viewing distance values are signaled in units of centimeters. The reference baseline values shall be signaled in the same units as the x component of the translation vector in the SEI message depth acquisition information that is valid for the same access unit. When a reference displays information SEI message is present in an access unit, the depth acquisition information SEI message shall also be present in the same access unit. The baseline and shift information signaled for the reference display is valid for all access units they associated with and until the next IDR access unit or the next access unit containing depth acquisition information SEI message or reference displays information SEI message.

NOTE – The reference displays information SEI message specifies display parameters for which the 3D sequence was optimized and the corresponding reference parameters. Each reference display (i.e. a reference display width and possibly a corresponding viewing distance) is associated with one reference baseline distance.

The following formulas can be used for calculating the baseline distance and horizontal shift for the receiver's display when the ratio between the receiver's viewing distance and the reference viewing distance is the same as the ratio between the receiver screen width and the reference screen width:

baseline = ref_baseline * ( ref_display_width ÷ display_width )

shift = ref_shift * ( ref_display_width ÷ display_width )

In the provided formulas, the width of the visible part of the display used for showing the video sequence should be understood under "display width". The same formulas can also be used for choosing the baseline distance and horizontal shift in cases when the viewing distance is not scaled proportionally to the screen width compared to the reference display parameters. In this case, the effect of applying these formulas would be to keep the perceived depth in the same proportion to the viewing distance as in the reference setup.

When camera parameters are updated by a depth acquisition information SEI message in a following access unit and the baseline between the views used in the view synthesis process in the following access unit changes relative to that in the in the access unit which the reference displays information SEI belongs to, the baseline and the horizontal shift for the receiver's display in the following access unit should be modified accordingly. Let the scaling factor s be equal to the ratio of the baseline between two views in the following access unit and the baseline between the same two views in the access unit, which the reference displays information SEI message belongs to, where the two views are used in the view synthesis process. Then the baseline distance for the receiver's display in the following access unit should be modified with the scaling factor s relative to the baseline distance for the receiver's display in the access unit which the reference displays information SEI belongs to. The hortiontal shift for the receiver’s display should also be modified by scaling it with the same factor as that used to scale the baseline distance.

prec_ref_baseline specifies the exponent of the maximum allowable truncation error for ref_baseline[ i ] as given by 2−prec_ref_baseline. The value of prec_ref_baseline shall be in the range of 0 to 31, inclusive.

prec_ref_display_width specifies the exponent of the maximum allowable truncation error for ref_display_width[ i ] as given by 2−prec_ref_display_width. The value of prec_ref_display_width shall be in the range of 0 to 31, inclusive.

ref_viewing_distance_flag equal to 1 indicates the presence of reference viewing distance. ref_viewing_distance_flag equal to 0 indicates that the reference viewing distance is not present in the bitstream.

prec_ref_viewing_dist specifies the exponent of the maximum allowable truncation error for ref_viewing_dist[ i ] as given by 2−prec_ref_viewing_dist. The value of prec_ref_viewing_dist shall be in the range of 0 to 31, inclusive.

num_ref_displays_minus1 plus 1 specifies the number of reference displays that are signaled in the bitstream. The value of num_ref_displays_minus1 shall be in the range of 0 to 31, inclusive.

exponent_ref_baseline[ i ] specifies the exponent part of the reference baseline of the for the i-th reference display. The value of exponent_ref_baseline[ i ] shall be in the range of 0 to 62, inclusive. The value 63 is reserved for future use by ITU-T | ISO/IEC. Decoders shall treat the value 63 as indicating an unspecified reference baseline.

mantissa_ref_baseline[ i ] specifies the mantissa part of the reference baseline of the for the i-th reference display. The length of the mantissa_ref_baseline[ i ] syntax element is variable and determined as follows.

–	If exponent_ref_baseline[ i ] = = 0, the length is Max( 0, prec_ref_baseline − 30 ).

–	Otherwise ( 0 < exponent_ref_baseline[ i ] < 63), the length is Max( 0, exponent_ref_baseline[ i ] + prec_ref_baseline − 31 ).

exponent_ref_display_width[ i ] specifies the exponent part of the reference display width of the i-th reference display. The value of exponent_ref_display_width[ i ] shall be in the range of 0 to 62, inclusive. The value 63 is reserved for future use by ITU-T | ISO/IEC. Decoders shall treat the value 63 as indicating an unspecified reference display width.

mantissa_ref_display_width[ i ] specifies the mantissa part of the reference display width of the i-th reference display. The length of the mantissa_ref_display_width[ i ] syntax element is variable and determined as follows.

–	If exponent_ref_display_width[ i ] = = 0, the length is Max( 0, prec_ref_display_width − 30 ).

–	Otherwise ( 0 < exponent_ref_display_width[ i ] < 63 ), the length is Max( 0, exponent_ref_display_width[ i ] + prec_ref_display_width − 31 ).

exponent_ref_viewing_distance[ i ] specifies the exponent part of the reference viewing distance of the i-th reference display. The value of exponent_ref_viewing_distance[ i ] shall be in the range of 0 to 62, inclusive. The value 63 is reserved for future use by ITU-T | ISO/IEC. Decoders shall treat the value 63 as indicating an unspecified reference display width.

mantissa_ref_viewing_distance[ i ] specifies the mantissa part of the reference viewing distance of the i-th reference display. The length of the mantissa_ref_viewing_distance[ i ] syntax element is variable and determined as follows.

–	If exponent_ref_viewing_distance[ i ]  = = 0, the length is Max( 0, prec_ref_viewing_distance − 30 ).

–	Otherwise ( 0 < exponent_ref_viewing_distance[ i ] < 63 ), the length is Max( 0, exponent_ref_viewing_distance[ i ]  + prec_ref_viewing_distance − 31 ).

The association between the reference display and baseline variables and corresponding syntax elements is specified by Table I-3. Each parameter is obtained from the variables specified in Table I-3 as the variable x computed as follows.

–	If 0 < e < 63, x = (−1)s * 2e−31 * (1 + n ÷2v).

–	Otherwise (e is equal to 0), x = (−1)s * 2−(30+v) * n.

NOTE – The above specification is similar to that found in IEC 60559:1989, Binary floating-point arithmetic for microprocessor systems.

Table I-3– Association between camera parameter variables and syntax elements.

| x | s | e | n |
| --- | --- | --- | --- |
| refBaseline[ i ] | 0 | exponent_ref_baseline[ i ] | mantissa_ref_baseline[ i ] |
| refDisplayWidth[ i ] | 0 | exponent_ref_display_width[ i ] | mantissa_ref_display_width [ i ] |
| refViewingDistance[ i ] | 0 | exponent_ref_viewing_distance[ i ] | mantissa_ref_viewing_distance[ i ] |

additional_shift_present_flag[ i ] equal to 1 indicates that the information about additional horizontal shift of the left and right views for the i-th reference display is present in the bitstream. additional_shift_present_flag[ i ] equal to 0 indicates that the information about additional horizontal shift of the left and right views for the i-th reference display is not present in the bitstream.

num_pixel_shift_plus128[ i ] indicates the recommended additional horizontal shift for a stereo-pair corresponding to the i-th reference baseline and the i-th reference display. If ( num_pixel_shift_plus128[ i ] − 128 ) is less than 0, it is recommended that the left view of the stereo-pair corresponding to the i-th reference baseline and the i-th reference display is shifted in the left direction by ( 128 − num_pixel_shift_plus128[ i ] ) pixels with respect to the right view of the stereo-pair; if num_pixel_shift_plus128[ i ] is equal to 128, it is recommended that shifting is not applied; if ( num_pixel_shift_plus128[ i ] − 128 ) is greater than 0, it is recommended that the left view of the stereo-pair corresponding to the i-th reference baseline and the i-th reference display should be shifted in the right direction by ( num_pixel_shift_plus128[ i ] − 128 ) pixels with respect to the right view of the stereo-pair. The value of num_pixel_shift_plus128[ i ] shall be in the range of 0 to 255, inclusive.

NOTE – Shifting the left view in the left (or right) direction by X pixels with respect to the right view can be performed by the following two-step processing:

- shift the left-view by X/2 pixels in the left (or right) direction, and shift the right-view by X/2 pixels in the right (or left) direction;

- fill the left and right image margins of X/2 pixels in width in both the left and right views in background color.

The following pseudo code explains the recommended shifting processing in the case of shifting the left view in the left direction by X pixels with respect to the right view.

for ( i = X/2; i < width – X/2; i++ ){

for  ( j=0;  j < height; j++ ){

left_view[ j ][ i ] = left_view[ j ][ i + X/2 ];

right_view[ j ][ width – 1 – i ] = right_view[ j ][ width – 1 – i– X/2 ];

}

}

for ( i = 0; i < X/2; i++) {

for ( j = 0; j < height; j++ ){

left_view[ j ][ width – 1 – i ] = left_view[ j ][ i ] = BACKGROUND_COLOR;

right_view[ j ][ width – 1 – i ] = right_view[ j ][ i ] = BACKGROUND_COLOR;

}

}

The following pseudo code explains the recommended shifting processing in the case of shifting the left view in the right direction by X pixels with respect to the right view.

for ( i = X/2; i < width – X/2; i++ ){

for ( j = 0; j < height; j++ ){

left_view[ j ][ width – 1 – i ] = left_view[ j ][ width – 1 – i – X/2 ];

right_view[ j ][ i ] = right_view[ j ][ i + X/2 ];

}

}

for ( i=0; i < X/2; i++ ){

for ( j = 0; j < height; j++ ){

left_view[ j ][ width – 1– i ] = left_view[ j ][ i ] = BACKGROUND_COLOR;

right_view[ j ][ width – 1– i ] = right_view[ j ][ i ] = BACKGROUND_COLOR;

}

}

BACKGROUND_COLOR may take different values in different systems, for example black or gray.

  - Video usability information

The specifications in Annex H.14 apply with substituting 3DVC sequence parameter set for MVC sequence parameter set. The mvc_vui_parameters_extension() contained in 3DVC sequence parameter set apply for the 3DV operation points.