| Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 1st Meeting: Stockholm, SE, 16–20 July 2012 | Document: JCT3V-A1002 |
| --- | --- |

| Title: | 3D-AVC Draft Text 3 |  |  |
| --- | --- | --- | --- |
| Status: | Output Document of JCT-3V |  |  |
| Purpose: | Draft text |  |  |
| Author(s) or Contact(s): | Miska M. Hannuksela Nokia Corporation Ying Chen Qualcomm Teruhiko Suzuki Sony | Tel: Email: | miska.hannuksela@nokia.com cheny@qualcomm.com teruhikos@jp.sony.com |
| Source: | Editors |  |  |

_____________________________

- Abstract

This document is the working draft 3 of the 3D-AVC specification. The document is based on MPEG output document w12732, “Working Draft 2 of AVC compatible video with depth information”, with adoptions in JCT-3V#1 meeting integrated.

The following adoptions of JCT-3V #1 meeting have been integrated in this version:

- Tools/text moved from MVC+D to AVC-3D

Move depth-range-based weighted prediction from MVC+D PDAM text to AVC-3D WD3 text.

Move znear and zfar signaling from MVC+D PDAM text to AVC-3D WD3 text.

[Editors]

- JCT2-A0008 3D-AVC HLS: Depth Range Parameter Set for AVC Compatible 3D Video Coding

(+ removal the strikethrough green highlighted parts in JCT3V-A0166r1.doc)

- JCT2-A0045/m25916 3D-CE5.a results on motion vector competition-based Skip/Direct mode with explicit signaling.

- JCT2-A0112/m26074 Calculation process for parameters of Depth Range-based Weighted Prediction with fixed-point/integer operations

- JCT2-A0102/m26057 3D-HLS: On reference picture list construction for AVC based 3DV.

[Editors]: WD and the software are aligned, no action.

- JCT2-A0023/m26094 CE1.a related results on the modification of in-loop view synthesis.

Information technology – Coding of audio-visual objects – Part 10: Advanced Video Coding

[Ed. Note(CY): to check if the above title is precise.]

In 0.6, add the following paragraph after the paragraph that starts with “Multiview video coding”:

MVC extensions for inclusion of depth maps video coding is specified in Annex I allowing the construction of bitstreams that represent multiple views with related multiple depth views. AVC-compatible multiview video coding extension with depth information is specified in Annex J. This extension requires the presence of an AVC-compatible texture base view and additionally allows the presence of zero or more MVC-compatible texture views. Moreover, enhanced texture view coding that utilizes the associated depth views is specified for this extension. Similar to multiview video coding, bitstreams that include multiple depth views may also contain sub-bitstreams that conform to this specification. For temporal bitstream scalability, i.e., the presence of a sub-bitstream with a smaller temporal sampling rate than the bitstream, complete access units are removed from the bitstream when deriving the sub-bitstream. In this case, high-level syntax and inter prediction reference pictures in the bitstream are constructed accordingly. For view bitstream scalability, i.e. the presence of a sub-bitstream with fewer views than the bitstream, NAL units are removed from the bitstream when deriving the sub-bitstream. In this case, inter-view prediction, i.e., the prediction of one view signal by data of another view signal, is typically used for efficient coding.

In 0.7, add the following paragraph after the paragraph that starts with “Annex H specifies”:

Annex I specifies MVC extensions for inclusion of depth video coding. Annex J specifies AVC-compatible multiview video coding extension with depth information. Annex I and J are jointly referred to as 3D Video Coding (3DVC). The reader is referred to Annex I and Annex J for the entire decoding process for MVC extensions with depth video and AVC-compatible multiview extension with depth video, respectively, which is specified there with references being made to clauses 2-9 and Annexes A-E and Annex H. Subclause I.1Error: Reference source not found specifies one profile for MVC extension of multiview and depth coding. Subclause J.10 specifies one profile for AVC-compatible multiview video coding extension with depth information.

In 7.3.1Nal unit syntax, replace the syntax table with:

| nal_unit( NumBytesInNALunit ) { | C | Descriptor |
| --- | --- | --- |
| forbidden_zero_bit | All | f(1) |
| nal_ref_idc | All | u(2) |
| nal_unit_type | All | u(5) |
| NumBytesInRBSP = 0 |  |  |
| nalUnitHeaderBytes = 1 |  |  |
| if( nal_unit_type = = 14 ¦ ¦ nal_unit_type = = 20 ¦ ¦ nal_unit_type = = 21 ) { |  |  |
| svc_extension_flag | All | u(1) |
| if( svc_extension_flag && ( nal_unit_type = = 14 ¦ ¦ nal_unit_type = = 20 ) ) { |  |  |
| nal_unit_header_svc_extension( ) /* specified in Annex Error: Reference source not found */ | All |  |
| nalUnitHeaderBytes += 3 |  |  |
| } |  |  |
| else if( svc_extension_flag && nal_unit_type = = 21 ) { |  |  |
| nal_unit_header_3dvc_extension( ) /* specified in Annex J */ |  |  |
| nalUnitHeaderBytes += 2 |  |  |
| } |  |  |
| else { |  |  |
| nal_unit_header_mvc_extension( ) /* specified in Annex Error: Reference source not found */ | All |  |
| nalUnitHeaderBytes += 3 |  |  |
| } |  |  |
| } |  |  |
| for( i = nalUnitHeaderBytes; i < NumBytesInNALunit; i++ ) { |  |  |
| if( i + 2 < NumBytesInNALunit && next_bits( 24 ) = = 0x000003 ) { |  |  |
| rbsp_byte[ NumBytesInRBSP++ ] | All | b(8) |
| rbsp_byte[ NumBytesInRBSP++ ] | All | b(8) |
| i += 2 |  |  |
| emulation_prevention_three_byte /* equal to 0x03 */ | All | f(8) |
| } else |  |  |
| rbsp_byte[ NumBytesInRBSP++ ] | All | b(8) |
| } |  |  |
| } |  |  |

In 7.3.2.1.3,Subset sequence parameter set data syntax, replace the syntax table with:

| subset_seq_parameter_set_rbsp( ) { | C | Descriptor |
| --- | --- | --- |
| seq_parameter_set_data( ) | 0 |  |
| if( profile_idc = = 83 ¦ ¦ profile_idc = = 86 ) { |  |  |
| seq_parameter_set_svc_extension( ) /* specified in Annex Error: Reference source not found */ | 0 |  |
| svc_vui_parameters_present_flag | 0 | u(1) |
| if( svc_vui_parameters_present_flag = = 1 ) |  |  |
| svc_vui_parameters_extension( ) /* specified in Annex Error: Reference source not found */ | 0 |  |
| } else if( profile_idc = = 118 ¦ ¦ profile_idc = = 128 ¦ ¦ profile_idc = = 138 ¦ ¦ profile_idc = = 139) { |  |  |
| bit_equal_to_one /* equal to 1 */ | 0 | f(1) |
| seq_parameter_set_mvc_extension( ) /* specified in Annex Error: Reference source not found */ | 0 |  |
| mvc_vui_parameters_present_flag | 0 | u(1) |
| if( mvc_vui_parameters_present_flag = = 1 ) |  |  |
| mvc_vui_parameters_extension( ) /* specified in Annex Error: Reference source not found */ | 0 |  |
| } |  |  |
| if( profile_idc = = 138 ¦ ¦ profile_idc = = 139 ) { |  |  |
| bit_equal_to_one /* equal to 1 */ | 0 | f(1) |
| seq_parameter_set_3dvc_extension( ) /* specified in Annex I */ | 0 |  |
| } |  |  |
| additional_extension3_flag | 0 | u(1) |
| if( additional_extension3_flag = = 1 ) |  |  |
| while( more_rbsp_data( ) ) |  |  |
| additional_extension3_data_flag | 0 | u(1) |
| rbsp_trailing_bits( ) | 0 |  |
| } |  |  |

In 7.3.2.13 slice layer extension RBSP syntax, replace the table with the following:

| slice_layer_extension_rbsp( ) { | C | Descriptor |
| --- | --- | --- |
| if( svc_extension_flag && nal_unit_type != 21 ) { |  |  |
| slice_header_in_scalable_extension( ) /* specified in Annex Error: Reference source not found */ | 2 |  |
| if( !slice_skip_flag ) |  |  |
| slice_data_in_scalable_extension( ) /* specified in Annex Error: Reference source not found */ | 2 ¦ 3 ¦ 4 |  |
| } else { |  |  |
| slice_header( ) | 2 |  |
| slice_data( ) | 2 ¦ 3 ¦ 4 |  |
| } |  |  |
| rbsp_slice_trailing_bits( ) | 2 |  |
| } |  |  |

In 7.3.3,Slice header syntax, replace the syntax table with:

| slice_header( ) { | C | Descriptor |
| --- | --- | --- |
| first_mb_in_slice | 2 | ue(v) |
| slice_type | 2 | ue(v) |
| pic_parameter_set_id | 2 | ue(v) |
| if( nal_unit_type = = 21 && svc_extension_flag && slice_header_prediction_flag != 0 ) { |  |  |
| pre_slice_header_src | 2 | u(2) |
| if( slice_type = = P ¦ ¦ slice_type = = SP ¦ ¦ slice_type = = B ) { |  |  |
| pre_ref_lists_src | 2 | u(2) |
| if ( !pre_ref_lists_src ) { |  |  |
| num_ref_idx_active_override_flag | 2 | u(1) |
| if( num_ref_idx_active_override_flag ) { |  |  |
| num_ref_idx_l0_active_minus1 | 2 | ue(v) |
| if( slice_type = = B ) |  |  |
| num_ref_idx_l1_active_minus1 | 2 | ue(v) |
| } |  |  |
| if( nal_unit_type = = 20 ¦ ¦ ( nal_unit_type = = 21 && DepthFlag ) ) |  |  |
| ref_pic_list_mvc_modification( ) /* specified in Annex Error: Reference source not found */ | 2 |  |
| else if( nal_unit_type = = 21 && DepthFlag = = 0 ) |  |  |
| ref_pic_list_3dvc_modification( ) | 2 |  |
| } |  |  |
| } |  |  |
| if( ( weighted_pred_flag && ( slice_type = = P ¦ ¦ slice_type = = SP ) ) ¦ ¦ ( weighted_bipred_idc = = 1 && slice_type = = B ) ) { |  |  |
| pre_pred_weight_table_src | 2 | u(2) |
| if( !pre_pred_weight_table_src ) |  |  |
| pred_weight_table( ) | 2 |  |
| if(nal_ref_idc != 0 ) { |  |  |
| pre_dec_ref_pic_marking_src | 2 | u(2) |
| if( !pre_dec_ref_pic_marking_src ) |  |  |
| dec_ref_pic_marking( ) | 2 |  |
| } |  |  |
| slice_qp_delta | 2 | se(v) |
| } else { |  |  |
| if( separate_colour_plane_flag = = 1 ) |  |  |
| colour_plane_id | 2 | u(2) |
| frame_num | 2 | u(v) |
| if( !frame_mbs_only_flag ) { |  |  |
| field_pic_flag | 2 | u(1) |
| if( field_pic_flag ) |  |  |
| bottom_field_flag | 2 | u(1) |
| } |  |  |
| if( IdrPicFlag ) |  |  |
| idr_pic_id | 2 | ue(v) |
| if( pic_order_cnt_type = = 0 ) { |  |  |
| pic_order_cnt_lsb | 2 | u(v) |
| if( bottom_field_pic_order_in_frame_present_flag && !field_pic_flag ) |  |  |
| delta_pic_order_cnt_bottom | 2 | se(v) |
| } |  |  |
| if( pic_order_cnt_type = = 1 && !delta_pic_order_always_zero_flag ) { |  |  |
| delta_pic_order_cnt[ 0 ] | 2 | se(v) |
| if( bottom_field_pic_order_in_frame_present_flag && !field_pic_flag ) |  |  |
| delta_pic_order_cnt[ 1 ] | 2 | se(v) |
| } |  |  |
| if( redundant_pic_cnt_present_flag ) |  |  |
| redundant_pic_cnt | 2 | ue(v) |
| if( slice_type = = B ) |  |  |
| direct_spatial_mv_pred_flag | 2 | u(1) |
| if( slice_type = = P ¦ ¦ slice_type = = SP ¦ ¦ slice_type = = B ) { |  |  |
| num_ref_idx_active_override_flag | 2 | u(1) |
| if( num_ref_idx_active_override_flag ) { |  |  |
| num_ref_idx_l0_active_minus1 | 2 | ue(v) |
| if( slice_type = = B ) |  |  |
| num_ref_idx_l1_active_minus1 | 2 | ue(v) |
| } |  |  |
| } |  |  |
| if( nal_unit_type = = 20 ¦ ¦ ( nal_unit_type = = 21 && DepthFlag ) ) |  |  |
| ref_pic_list_mvc_modification( ) /* specified in Annex Error: Reference source not found */ | 2 |  |
| else if( nal_unit_type = = 21 && DepthFlag = = 0 ) |  |  |
| ref_pic_list_3dvc_modification( ) |  |  |
| else |  |  |
| ref_pic_list_modification( ) | 2 |  |
| if( ( weighted_pred_flag && ( slice_type = = P ¦ ¦ slice_type = = SP ) ) ¦ ¦ ( weighted_bipred_idc = = 1 && slice_type = = B ) ) |  |  |
| pred_weight_table( ) | 2 |  |
| if( nal_ref_idc != 0 ) |  |  |
| dec_ref_pic_marking( ) | 2 |  |
| if( entropy_coding_mode_flag && slice_type != I && slice_type != SI ) |  |  |
| cabac_init_idc | 2 | ue(v) |
| slice_qp_delta | 2 | se(v) |
| if( slice_type = = SP ¦ ¦ slice_type = = SI ) { |  |  |
| if( slice_type = = SP ) |  |  |
| sp_for_switch_flag | 2 | u(1) |
| slice_qs_delta | 2 | se(v) |
| } |  |  |
| if( deblocking_filter_control_present_flag ) { |  |  |
| disable_deblocking_filter_idc | 2 | ue(v) |
| if( disable_deblocking_filter_idc != 1 ) { |  |  |
| slice_alpha_c0_offset_div2 | 2 | se(v) |
| slice_beta_offset_div2 | 2 | se(v) |
| } |  |  |
| } |  |  |
| if( num_slice_groups_minus1 > 0 && slice_group_map_type >= 3 && slice_group_map_type <= 5) |  |  |
| slice_group_change_cycle | 2 | u(v) |
| if ( nal_unit_type = = 21 && ( slice_type != I && slice_type != SI )) { |  |  |
| if( DepthFlag ) |  |  |
| depth_weighted_pred_flag | 2 | u(1) |
| else if( svc_extension_flag && ViewCompOrder( 0, view_idx ) > ViewCompOrder( 1, view_idx ) ) |  |  |
| dmvp_flag | 2 | u(1) |
| if ( 3dv_acquisition_idc != 1 && (depth_weighted_pred_flag = = 1 ¦¦ dmvp_flag = = 1) ) |  |  |
| dps_id | 2 | ue(v) |
| } |  |  |
| } |  |  |
| } |  |  |

In 7.3.4,Slice data syntax, replace the syntax table with:

| slice_data( ) { | C | Descriptor |
| --- | --- | --- |
| if( entropy_coding_mode_flag ) |  |  |
| while( !byte_aligned( ) ) |  |  |
| cabac_alignment_one_bit | 2 | f(1) |
| CurrMbAddr = first_mb_in_slice * ( 1 + MbaffFrameFlag ) |  |  |
| moreDataFlag = 1 |  |  |
| prevMbSkipped = 0 |  |  |
| do { |  |  |
| if( slice_type != I && slice_type != SI ) |  |  |
| if( !entropy_coding_mode_flag ) { |  |  |
| mb_skip_run | 2 | ue(v) |
| prevMbSkipped = ( mb_skip_run > 0 ) |  |  |
| for( i=0; i<mb_skip_run; i++ ) |  |  |
| CurrMbAddr = NextMbAddress( CurrMbAddr ) |  |  |
| if( nal_unit_type = = 21 && !DepthFlag && mb_skip_run > 0 && VspRefExist ) |  |  |
| mb_skip_type_flag | 2 | u(1) ¦ ae(v) |
| if( mb_skip_run > 0 ) |  |  |
| moreDataFlag = more_rbsp_data( ) |  |  |
| } else { |  |  |
| if( nal_unit_type = = 21 && !DepthFlag && VspRefExist && leftMbVSSkipped && upMbVSSkipped ) { |  |  |
| mb_vsskip_flag | 2 | u(1) ¦ ae(v) |
| moreDataFlag = !mb_vsskip_flag |  |  |
| if( !mb_vsskip_flag ) { |  |  |
| mb_skip_flag | 2 | u(1) ¦ ae(v) |
| moreDataFlag = !mb_skip_flag |  |  |
| } |  |  |
| } else { |  |  |
| mb_skip_flag | 2 | u(1) ¦ ae(v) |
| moreDataFlag = !mb_skip_flag |  |  |
| if( nal_unit_type = = 21 && !DepthFlag && VspRefExist && !mb_skip_flag ) { |  |  |
| mb_vsskip_flag | 2 | u(1) ¦ ae(v) |
| moreDataFlag = !mb_vsskip_flag |  |  |
| } |  |  |
| } |  |  |
| } |  |  |
| if( moreDataFlag ) { |  |  |
| if( MbaffFrameFlag && ( CurrMbAddr % 2 = = 0 ¦ ¦ ( CurrMbAddr % 2 = = 1 && prevMbSkipped ) ) ) |  |  |
| mb_field_decoding_flag | 2 | u(1) ¦ ae(v) |
| macroblock_layer( ) | 2 ¦ 3 ¦ 4 |  |
| } |  |  |
| if( !entropy_coding_mode_flag ) |  |  |
| moreDataFlag = more_rbsp_data( ) |  |  |
| else { |  |  |
| if( slice_type != I && slice_type != SI ) |  |  |
| prevMbSkipped = mb_skip_flag ¦ ¦ mb_vsskip_flag |  |  |
| if( MbaffFrameFlag && CurrMbAddr % 2 = = 0 ) |  |  |
| moreDataFlag = 1 |  |  |
| else { |  |  |
| end_of_slice_flag | 2 | ae(v) |
| moreDataFlag = !end_of_slice_flag |  |  |
| } |  |  |
| } |  |  |
| CurrMbAddr = NextMbAddress( CurrMbAddr ) |  |  |
| } while( moreDataFlag ) |  |  |
| } |  |  |

In 7.3.5,Macroblock layer syntax, replace the syntax table with:

| macroblock_layer( ) { | C | Descriptor |
| --- | --- | --- |
| if ( IvmpEnabledFlag && CheckIVMPAvailable ) |  |  |
| mb_ivmp_flag |  | u(1) ¦ ae(v) |
| if (!mb_ivmp_flag) { |  |  |
| if( DepthFlag && disp_flag ) |  |  |
| mb_disp_flag |  | u(1) ¦ ae(v) |
| if(!mb_disp_flag){ |  |  |
| mb_type | 2 | ue(v) ¦ ae(v) |
| if(nal_unit_type = = 21 && !DepthFlag && slice_type = = B && direct_spatial_mv_pred_flag && VspRefExist && mb_type = = B_Direct_16x16 ) |  |  |
| mb_direct_type_flag | 2 | u(1) ¦ ae(v) |
| } |  |  |
| } |  |  |
| if( !mb_disp_flag ) { |  |  |
| if( mb_type = = I_PCM ) { |  |  |
| while( !byte_aligned( ) ) |  |  |
| pcm_alignment_zero_bit | 3 | f(1) |
| for( i = 0; i < 256; i++ ) |  |  |
| pcm_sample_luma[ i ] | 3 | u(v) |
| for( i = 0; i < 2 * MbWidthC * MbHeightC; i++ ) |  |  |
| pcm_sample_chroma[ i ] | 3 | u(v) |
| } else { |  |  |
| noSubMbPartSizeLessThan8x8Flag = 1 |  |  |
| if( mb_type != I_NxN && MbPartPredMode( mb_type, 0 ) != Intra_16x16 && NumMbPart( mb_type ) = = 4 ) { |  |  |
| if( !mb_ivmp_flag ) |  |  |
| sub_mb_pred( mb_type ) | 2 |  |
| for( mbPartIdx = 0; mbPartIdx < 4; mbPartIdx++ ) |  |  |
| if( sub_mb_type[ mbPartIdx ] != B_Direct_8x8 ) { |  |  |
| if( NumSubMbPart( sub_mb_type[ mbPartIdx ] ) > 1 ) |  |  |
| noSubMbPartSizeLessThan8x8Flag = 0 |  |  |
| } else if( !direct_8x8_inference_flag ) |  |  |
| noSubMbPartSizeLessThan8x8Flag = 0 |  |  |
| } else { |  |  |
| if( transform_8x8_mode_flag && mb_type = = I_NxN ) |  |  |
| transform_size_8x8_flag | 2 | u(1) ¦ ae(v) |
| if( !mb_ivmp_flag ) |  |  |
| mb_pred( mb_type ) | 2 |  |
| } |  |  |
| if( MbPartPredMode( mb_type, 0 ) != Intra_16x16 ) { |  |  |
| coded_block_pattern | 2 | me(v) ¦ ae(v) |
| if( CodedBlockPatternLuma > 0 && transform_8x8_mode_flag && mb_type != I_NxN && noSubMbPartSizeLessThan8x8Flag && ( mb_type != B_Direct_16x16 ¦ ¦ direct_8x8_inference_flag ) ) |  |  |
| transform_size_8x8_flag | 2 | u(1) ¦ ae(v) |
| } |  |  |
| if( CodedBlockPatternLuma > 0 ¦ ¦ CodedBlockPatternChroma > 0 ¦ ¦ MbPartPredMode( mb_type, 0 ) = = Intra_16x16 ) { |  |  |
| mb_qp_delta | 2 | se(v) ¦ ae(v) |
| residual( 0, 15 ) | 3 ¦ 4 |  |
| } |  |  |
| } |  |  |
| } |  |  |
| } |  |  |

Replace Table 7-1 with:

| nal_unit_type | Content of NAL unit and RBSP syntax structure | C | Annex A NAL unit type class | Annex G and Annex H NAL unit type class | Annex Error: Reference source not found NAL unit type class |
| --- | --- | --- | --- | --- | --- |
| 0 | Unspecified |  | non-VCL | non-VCL | non-VCL |
| 1 | Coded slice of a non-IDR picture slice_layer_without_partitioning_rbsp( ) | 2, 3, 4 | VCL | VCL | VCL |
| 2 | Coded slice data partition A slice_data_partition_a_layer_rbsp( ) | 2 | VCL | not applicable | not applicable |
| 3 | Coded slice data partition B slice_data_partition_b_layer_rbsp( ) | 3 | VCL | not applicable | not applicable |
| 4 | Coded slice data partition C slice_data_partition_c_layer_rbsp( ) | 4 | VCL | not applicable | not applicable |
| 5 | Coded slice of an IDR picture slice_layer_without_partitioning_rbsp( ) | 2, 3 | VCL | VCL | VCL |
| 6 | Supplemental enhancement information (SEI) sei_rbsp( ) | 5 | non-VCL | non-VCL | non-VCL |
| 7 | Sequence parameter set seq_parameter_set_rbsp( ) | 0 | non-VCL | non-VCL | non-VCL |
| 8 | Picture parameter set pic_parameter_set_rbsp( ) | 1 | non-VCL | non-VCL | non-VCL |
| 9 | Access unit delimiter access_unit_delimiter_rbsp( ) | 6 | non-VCL | non-VCL | non-VCL |
| 10 | End of sequence end_of_seq_rbsp( ) | 7 | non-VCL | non-VCL | non-VCL |
| 11 | End of stream end_of_stream_rbsp( ) | 8 | non-VCL | non-VCL | non-VCL |
| 12 | Filler data filler_data_rbsp( ) | 9 | non-VCL | non-VCL | non-VCL |
| 13 | Sequence parameter set extension seq_parameter_set_extension_rbsp( ) | 10 | non-VCL | non-VCL | non-VCL |
| 14 | Prefix NAL unit prefix_nal_unit_rbsp( ) /* specified in Annex G and Annex H */ | 2 | non-VCL | suffix dependent | suffix dependent |
| 15 | Subset sequence parameter set subset_seq_parameter_set_rbsp( ) /* specified in Annex G, Annex H and Annex Error: Reference source not found */ | 0 | non-VCL | non-VCL | non-VCL |
| 16 | Depth parameter set | 11 | non-VCL | non-VCL | non-VCL |
| 17..18 | Reserved |  | non-VCL | non-VCL | non-VCL |
| 19 | Coded slice of an auxiliary coded picture without partitioning slice_layer_without_partitioning_rbsp( ) | 2, 3, 4 | non-VCL | non-VCL | non-VCL |
| 20 | Coded slice extension slice_layer extension_rbsp( ) /* specified in Annex Error: Reference source not found and Annex H and Annex Error: Reference source not found */ | 2, 3, 4 | non-VCL | VCL | VCL |
| 21 | Coded slice extension for depth view components or 3DVC texture view component /*specified in Annex I */ slice_layer_extension_rbsp( ) /* specified in Annex I */ | 2, 3, 4 | non-VCL | non-VCL | VCL |
| 22..23 | Reserved |  | non-VCL | non-VCL | non-VCL |

In 7.4.1, add the following paragraph in the semantics of svc_extension_flag just before the semantics of rbsp_byte[ i ].

For nal_unit_type equal to 21, svc_extension_flag equal to 0 specifies that the coded slice NAL unit is a coded slice extension for a depth view component having the NAL unit header MVC extension, and svc_extension_flag equal to 1 specifies that the coded slice NAL unit is a coded slice extension for a texture or depth view component with the NAL unit header 3DVC extension. The value of DepthFlag is specified as follows.

DepthFlag = ( nal_unit_type  ! =  21 ) ? 0 : ( svc_extension_flag ? depth_flag : 1 )	(7-xx)

The value of svc_extension_flag shall be equal to 0 for coded video sequences conforming to one or more profiles specified in Annex I. Decoders conforming to one or more profiles specified in Annex I shall ignore (remove from the bitstream and discard) NAL units for which nal_unit_type is equal to 14, 20, or 21 and for which svc_extension_flag is equal to 1.

In 7.4. 3, Slice header semantics, add the following text after the sematics of “pic_parameter_set_id”:

pre_slice_header_src, pre_ref_lists_src, pre_pred_weight_table_src and pre_dec_ref_pic_marking_src specify if the respective syntax elements are present in the slice header, and, if not, the slice header from which the values of the respective syntax elements are taken as specified in Table J-XX and Table J-YY.

When a syntax element has an inferred value in the slice header from which its value is taken according to Table J-XX and Table J-YY, the syntax element value in the current slice header is equal to this inferred value. When a syntax element is not present and has no inferred value in the slice header from which its value is taken according to Table J-XX and Table J-YY, the syntax element is inferred to be absent in the current slice header.

pre_slice_header_src shall not be equal to 0.

When ViewCompOrder( DepthFlag, view_idx ) is smaller than ViewCompOrder( !DepthFlag, view_idx ), pre_slice_header_src, pre_ref_lists_src, pre_pred_weight_table_src and pre_dec_ref_pic_marking_src shall not be equal to 2

- Table J-XX – Respective syntax elements for pre_slice_header_src, pre_ref_lists_src, pre_pred_weight_table_src and pre_dec_ref_pic_marking_src

| Prediction indication syntax element | Respective syntax elements |
| --- | --- |
| pre_slice_header_src | colour_plane_id, frame_num, field_pic_flag, bottom_field_flag, idr_pic_id, pic_order_cnt_lsb, delta_pic_order_cnt_bottom, delta_pic_order_cnt[ 0 ], delta_pic_order_cnt[ 1 ], redundant_pic_cnt, direct_spatial_mv_pred_flag, cabac_init_idc, sp_for_switch_flag, slice_qs_delta, disable_deblocking_filter_idc, slice_alpha_c0_offset_div2, slice_beta_offset_div2, slice_group_change_cycle, depth_weighted_pred_flag, dmvp_flag, dps_id |
| pre_ref_lists_src | num_ref_idx_active_override_flag, num_ref_idx_l0_active_minus1, num_ref_idx_l1_active_minus1 and reference picture list modification syntax table |
| pre_pred_weight_table_src | pred_weight_table( ) syntax structure |
| pre_dec_ref_pic_marking_src | dec_ref_pic_marking( ) syntax structure |

- Table J-YY – Semantics of the values of pre_slice_header_src, pre_ref_lists_src, pre_pred_weight_table_src and pre_dec_ref_pic_marking_src

| Value of pre_slice_header_src, pre_ref_lists_src, pre_pred_weight_table_src or pre_dec_ref_pic_marking_src | Semantics |
| --- | --- |
| 0 | The respective syntax elements are not predicted but included in the slice header. |
| 1 | The values of the respective syntax elements are taken from the slice header of the first slice of the previous view component in decoding order having the same value of DepthFlag as the current slice, belonging to a dependent view of the current view, and residing in the same access unit. |
| 2 | The values of the respective syntax elements are taken from the first slice header of the first slice of the view component having the same view order index as the current slice and a different value of DepthFlag. |
| 3 | The values of the respective syntax elements are taken from the first slice header of the first slice of the view component in the same access unit having view order index equal to 0 and the same value of DepthFlag as the current slice. |

depth_weighted_pred_flag equal to 0 specifies that no depth-range-based weighted prediction is used for corresponding slice RBSP. depth_weighted_pred_flag equal to 1 specifies that depth-range-based weighted prediction is used for corresponding slice RBSP. When not present, depth_weighted_pred_flag is inferred to be equal to 0. When depth_weighted_pred_flag is equal to 1, process of derivation of prediction weights is specified in J.8.3.4 applies.

dmvp_flag equal to 0 specifies that the inter and inter-view prediction process specified in I.8.3 is applied. dmvp_flag equal to 1 specifies that the inter, inter-view, and view synthesis prediction process specified in J.8.3 is applied.

dps_id specifies the depth parameter set in use. The value of dps_id shall be in the range of 0 to TBD, inclusive. When dps_id is equal to 0, depth parameters are set according to syntax elements in seq_parameter_set_3dvc_extension( ) of the active sequence parameter set. When present, the value of dps_id shall be the same in all slice headers within an access unit. [Ed. Note (CY): we may need to add a paragraph on generating the depth parameter set with id equal to 0 based on the SPS.]

In 7.4.4, Slice data semantics, add the following text

mb_skip_type_flag specifies the reference picture type if a macroblock is skipped. mb_skip_type_flag equal to 0 specifies that the current macroblock uses a reference picture as derived in subclause 8.4.1.1 or 8.4.1.2.2. mb_skip_type_flag equal to 1 specifies that the current macroblock uses a synthetic reference component as reference. When mb_skip_type_flag is not present, it is inferred to be equal to 0. When mb_skip_type_flag is present, the variable MbVSSkipFlag is set equal to mb_skip_type_flag.

mb_vsskip_flag equal to 1 specifies that for the current macroblock, when decoding a P slice, mb_type shall be inferred to be P_Skip using a synthetic reference component as a reference and the macroblock type is collectively referred to as P macroblock type, or for which, when decoding a B slice, mb_type shall be inferred to be B_Skip using a synthetic reference component as a reference and the macroblock type is collectively referred to as B macroblock type. mb_vsskip_flag equal to 0 specifies that the current macroblock is neither P_Skip using a synthetic reference component as a reference nor B_Skip using using a synthetic reference component as a reference. When mb_vsskip_flag is not present, it is inferred to be equal to 0. When mb_vsskip_flag is present, the variable MbVSSkipFlag is set equal to mb_vsskip_flag.

leftMbVSSkipped is derived to be 1 if the left macroblock adjacent to the current macroblock is available and the MbVSSkipFlag of the left macroblock is equal to 1, leftMbVSSkipped is derived to be 0 otherwise.

upMbVSSkipped is derived to be 1 if the upper macroblock adjacent to the current macroblock is available and the MbVSSkipFlag of the upper macrolbock is equal to 1, upMbVSSkipped is derived to be 0 otherwise.

In 7.4.5, Macroblock layer semantics, add the following text

mb_direct_type_flag specifies the reference picture type if a macroblock is coded in direct mode. mb_direct_type_flag equal to 0 specifies that the current macroblock uses a reference picture as derived in subclause 8.4.1.1 or 8.4.1.2.2. mb_direct_type_flag equal to 1 specifies that the current macroblock uses a synthetic reference component as reference. When mb_direct_type_flag is not present, it is inferred to be equal to 0.

IvmpEnabledFlag is set to 1 when all of the following conditions are satisfied:

–	inside_view_mvp_flag is equal to 1 (inside view motion prediction is enabled);

–	the current view component is a depth view component;

–	ViewCompOrder( 0, view_idx ) is smaller than ViewCompOrder( 1, view_idx );

–	svc_extension_flag is equal to 1;

–	the current picture is a non-anchor picture and the slice type is not I or SI slice.

Otherwise, IvmpEnabledFlag is set to 0.

CheckIVMPAvailable is set to 0 if one of the following conditions is satisfied:

–	if reduced_resolution_flag is equal to 0, the co-located macroblock in the texture view component is intra coded, or view synthesis prediction is included for any partition in current macroblock. The co-located macroblock in the texture view component is specified in subclause 8.3.3.1.

–	if reduced_resolution_flag is equal to 1 and if any of the four co-located macroblocks in the texture view component is intra coded, or its mb_type equal to equal to P_8x8, P_8x8ref0, or B_8x8, or view synthesis prediction is included. The relationsip between current macroblock in the depth view component and its four co-located macroblocks in the texture view component is specified in subclause 8.3.3.1.

Otherwise, CheckIVMPAvaiable is set to 1.

mb_ivmp_flag equal to 1 indicates that the mb_type and motion information of the current macroblock are not present and are derived from the one or four co-located macroblocks of the texture view component (depending on reduced_resolution_flag is 0 or 1, respectively) in the same view component pair. mb_ivmp_flag equal to 0 indicates that the mb_type and motion information are present. The motion information includes reference indices and motion vectors. When not present, this flag is inferred to be equal to 0.

mb_disp_flag equal to 1 specifies that the depth intra skip prediction specified in J.8.7 is applied. mb_disp_flag equal to 0 specifies that the current macroblock is not intra skiped. When not present, this flag is inferred to be equal to 0.

Annex J

Multiview and depth video coding

(This annex forms an integral part of this Recommendation | International Standard)

This annex specifies multiview and depth video coding, referred to as 3DVC.

  - Scope

Bitstreams and decoders conforming to the profile specified in this annex are completely specified in this annex with reference made to clauses Error: Reference source not found-Error: Reference source not found and Annexes Error: Reference source not found-H.

  - Normative references

The specifications in clause Error: Reference source not found apply.

  - Definitions

For the purpose of this annex, the following definitions apply in addition to the definitions in clause I.Error: Reference source not found. These definitions are either not present in clause I.Error: Reference source not found or replace definitions in clause I.Error: Reference source not found.

- MVC texture view component: A texture view component composed of coded slice NAL units of nal_unit_type not equal to 21.

- texture view component: A coded representation of the texture of a view in a single access unit. A texture view component may be a 3DVC texture view component or an MVC texture view component.

- view component pair: A texture view component and a depth view component of the same view within the same access unit.

- view synthesis: A process to generate a virtual picture using coded texture view and depth view from the same access unit. The process is used to generate a synthetic reference component.

- view synthesis prediction: A prediction derived from samples of synthetic reference components for decoding a view component.

- VSP reference index: An identifier of the output of the 3DVC decoding process for view synthesis reference component generation when the inputs are the texture and depth view components with view order index equal to ( vOIdx – 1 – i ), where vOIdx is the view order index of the present texture view component and i is equal to the VSP reference index.

- synthetic reference component: A picture synthesized from coded texture views and depth views. A view synthesis reference component contains samples that may be used for view synthesis prediction. A view synthesis reference components are reference pictures, and never being outputted.

- 3DVC texture view component: A texture view component composed of coded slice NAL units of nal_unit_type equal to 21.

  - Abbreviations

The specifications in clause Error: Reference source not found apply with the following additions:

VSP	View Synthesis Prediction

  - Conventions

The specifications in clause Error: Reference source not found apply.

  - Source, coded, decoded and output data formats, scanning processes, and neighbouring relationships

The specifications in clause Error: Reference source not found apply with substitution of 3DVC sequence parameter set for sequence parameter set.

  - Syntax and semantics

This clause specifies syntax and semantics for coded video sequences that conform to one or more of the profiles specified in this annex.

    - Method of specifying syntax in tabular form

The specifications in subclause I.Error: Reference source not found apply.

    - Specification of syntax functions, categories, and descriptors

The specifications in subclause I.Error: Reference source not found apply.

    - Syntax in tabular form

      - NAL unit syntax

The syntax table is specified in subclause I.Error: Reference source not found.

        - NAL unit header 3DVC extension syntax

| nal_unit_header_3dvc_extension( ) { | C | Descriptor |
| --- | --- | --- |
| view_idx | All | u(8) |
| depth_flag | All | u(1) |
| non_idr_flag | All | u(1) |
| temporal_id | All | u(3) |
| anchor_pic_flag | All | u(1) |
| inter_view_flag | All | u(1) |
| } |  |  |

      - Raw byte sequence payloads and RBSP trailing bits syntax

        - Sequence parameter set RBSP syntax

The syntax table is specified in subclause I.Error: Reference source not found.

          - Sequence parameter set data syntax

The syntax table is specified in subclause I.Error: Reference source not found.

            - Scaling list syntax

The syntax table is specified in subclause I.Error: Reference source not found.

          - Sequence parameter set extension RBSP syntax

The syntax table is specified in subclause I.Error: Reference source not found.

          - Subset sequence parameter set RBSP syntax

The syntax table is specified in subclause I.Error: Reference source not found.

          - Sequence parameter set MVC extension syntax

The syntax table is specified in subclause I.7.3.2.1.4.

          - Sequence parameter set 3DVC extension syntax

The syntax table is specified in subclause I.7.3.2.1.5 is replaced by the following.

| seq_parameter_set_3dvc_extension( ) { | C | Descriptor |
| --- | --- | --- |
| for( i = 0; i <= num_level_values_signalled_minus1; i++ ) |  |  |
| for( j = 0; j <= num_applicable_ops_minus1[ i ]; j++ ) |  |  |
| applicable_op_num_depth_views_minus1[ i ][ j ] |  | ue(v) |
| depth_info_present_flag | 0 | u(1) |
| if( depth_info_present_flag ) { |  |  |
| 3dv_acquisition_idc | 0 | ue(v) |
| for( i = 0; i < num_views_minus1; i++ ) |  |  |
| view_id_3dv[ i ] | 0 | ue(v) |
| if( 3dv_acquisition_idc ) { |  |  |
| depth_ranges( 2, 0 ) |  |  |
| if( profile_idc = = 139 &&num_views_minus1 ) |  |  |
| vsp_param( 2, 0 ) |  |  |
| } |  |  |
| if ( profile_idc = = 139 ) |  |  |
| for( i = 0; i<= num_views_minus1; i++ ) |  |  |
| depth_preceding_texture_flag[ i ] | 0 | u(1) |
| } |  |  |
| if( profile_idc = = 139 && depth_info_present_flag ) { |  |  |
| reduced_resolution_flag | 0 | u(1) |
| slice_header_prediction_flag | 0 | u(1) |
| inside_view_mvp_flag | 0 | u(1) |
| disp_flag | 0 | u(1) |
| psip_flag | 0 | u(1) |
| nonlinear_depth_representation_num | 0 | ue(v) |
| for( i = 1; i <= nonlinear_depth_representation_num; i++ ) |  |  |
| nonlinear_depth_representation_model[ i ] | 0 | ue(v) |
| seq_view_synthesis_flag | 0 | u(1) |
| } |  |  |
| } |  |  |

        - Picture parameter set RBSP syntax

The syntax table is specified in subclause I.Error: Reference source not found.

        - Supplemental enhancement information RBSP syntax

The syntax table is specified in subclause I.Error: Reference source not found.

          - Supplemental enhancement information message syntax

The syntax table is specified in subclause I.Error: Reference source not found.

        - Access unit delimiter RBSP syntax

The syntax table is specified in subclause I.Error: Reference source not found.

        - End of sequence RBSP syntax

The syntax table is specified in subclause I.Error: Reference source not found.

        - End of stream RBSP syntax

The syntax table is specified in subclause I.Error: Reference source not found.

        - Filler data RBSP syntax

The syntax table is specified in subclause I.Error: Reference source not found.

        - Slice layer without partitioning RBSP syntax

The syntax table is specified in subclause I.Error: Reference source not found.

        - Slice data partition RBSP syntax

Slice data partition syntax is not present in coded video sequences conforming to one or more of the profiles specified in this annex.

        - RBSP slice trailing bits syntax

The syntax table is specified in subclause I.Error: Reference source not found.

        - RBSP trailing bits syntax

The syntax table is specified in subclause I.Error: Reference source not found.

        - Prefix NAL unit RBSP syntax

The syntax table is specified in subclause I.Error: Reference source not found.

        - Depth parameter set RBSP syntax

The syntax table is specified in subclause I.7.3.2.13 is replaced by the following.

| depth_parameter_set_rbsp( ) { | C | Descriptor |
| --- | --- | --- |
| depth_parameter_set_id | 11 | ue(v) |
| pred_direction | 11 | ue(v) |
| if( pred_direction = = 0 ¦ ¦ pred_direction = = 1 ) { |  |  |
| ref_dps_id0 | 11 | ue(v) |
| predWeight0 = 64 |  |  |
| } |  |  |
| if( pred_direction = = 0 ) { |  |  |
| ref_dps_id1 | 11 | ue(v) |
| pred_weight0 | 11 | u(6) |
| predWeight0 = pred_weight0 |  |  |
| } |  |  |
| num_views_minus1 | 11 | ue(v) |
| depth_ranges( pred_direction, depth_parameter_set_id ) |  |  |
| vsp_param_flag | 11 | u(1) |
| if( vsp_param_flag ) |  |  |
| vsp_param( pred_direction, depth_parameter_set_id ) |  |  |
| depth_param_additional_extension_flag | 11 | u(1) |
| if(depth_param_additional_extension_flag = = 1 ) |  |  |
| while( more_rbsp_data( ) ) |  |  |
| depth_param_additional_extension_data_flag | 11 | u(1) |
| rbsp_trailing_bits( ) |  |  |
| } |  |  |

          - Depth ranges syntax

| depth_ranges( predDirection, index ) { | C | Descriptor |
| --- | --- | --- |
| z_near_flag | 11 | u(1) |
| z_far_flag | 11 | u(1) |
| if( z_near_flag ) |  |  |
| 3dv_acquisition_element( 0, predDirection, 7, 0, ZNearSign, ZNearExp, ZNearMantissa, ZNearManLen ) |  |  |
| if( z_far_flag ) |  |  |
| 3dv_acquisition_element( 0, predDirection, 7, 0, ZFarSign, ZFarExp, ZFarMantissa, ZFarManLen ) |  |  |
| } |  |  |

          - 3DV acquisition element syntax

| 3dv_acquisition_element( deltaFlag, predDirection, precMode, expLen, OutSign, OutExp, OutMantissa, OutManLen ) { | C | Descriptor |
| --- | --- | --- |
| if( num_views_minus1 + 1 – deltaFlag > 1 ) |  |  |
| element_equal_flag | 11 | u(1) |
| if( element_equal_flag = = 0 ) |  |  |
| numValues = num_views_minus1 + 1 – deltaFlag |  |  |
| else |  |  |
| numValues = 1 |  |  |
| for( i = 0; i < numValues; i++ ) { |  |  |
| if( predDirection = = 2 && i = = 0 ) { |  |  |
| if( precMode = = 0 ) { |  |  |
| matissa_len_minus1 | 11 | u(5) |
| OutManLen[ index, i ] = manLen = mantissa_len_minus + 1 |  |  |
| } else |  |  |
| prec | 11 | u(5) |
| } |  |  |
| if( predDirection = = 2 ) { |  |  |
| sign0 | 11 | u(1) |
| OutSign[ index, i ] = sign0 |  |  |
| exponent0 | 11 | u(expLen) |
| OutExp[ index, i ] = exponent0 |  |  |
| if( precMode = = 1 ) { |  |  |
| if( exponent0 = = 0 ) |  |  |
| OutManLen[ index, i ] = manLen = Max( 0, prec – 30 ) |  |  |
| else |  |  |
| OutManLen[ index, i ] = manLen = Max( 0, exponent0 + prec – 31 ) |  |  |
| } |  |  |
| mantissa0 | 11 | u(manLen) |
| OutMantissa[ index, i ] = mantissa0 |  |  |
| } else { |  |  |
| skip_flag | 11 | u(1) |
| if( skip_flag = = 0 ) { |  |  |
| sign1 | 11 | u(1) |
| OutSign[ index, i ] = sign1 |  |  |
| exponent_skip_flag | 11 | u(1) |
| if( exponent_skip_flag = = 0 ) { |  |  |
| exponent1 | 11 | u(expLen) |
| OutExp[ index, i ] = exponent1 |  |  |
| } else |  |  |
| OutExp[ index, i ] = OutExp[ ref_dps_id0, i ] |  |  |
| mantissa_diff | 11 | se(v) |
| mantissaPred = (( OutMantissa[ ref_dps_id0, i ] * predWeight0 + OutMantissa[ ref_dps_id1, i ] * ( 64-predWeight0 ) + 32 ) >> 6 ) |  |  |
| OutMantissa[ index, i ] = mantissaPred + mantissa_diff |  |  |
| OutManLen[ index, i ] = OutManLen[ ref_dps_id0, i ] |  |  |
| } else { |  |  |
| OutSign[ index, i ] = OutSign[ ref_dps_id0, i ] |  |  |
| OutExp[ index, i ] = OutExp[ ref_dps_id0, i ] |  |  |
| OutMantissa[ index, i ] = OutMantissa[ ref_dps_id0, i ] |  |  |
| OutManLen[ index, i ] = OutManLen[ ref_dps_id0, i ] |  |  |
| } |  |  |
| } |  |  |
| } |  |  |
| if( element_equal_flag = = 1 ) { |  |  |
| for( i = 1; i < num_views_minus1 + 1 – deltaFlag; i++ ) { |  |  |
| OutSign[ index, i ] = OutSign[ index, 0 ] |  |  |
| OutExp[ index, i ] = OutExp[ index, 0 ] |  |  |
| OutMantissa[ index, i ] = OutMantissa[ index, 0 ] |  |  |
| OutManLen[ index, i ] = OutManLen[ index, 0 ] |  |  |
| } |  |  |
| } |  |  |
| } |  |  |

          - View synthesis parameters syntax

| vsp_param( predDirection, index ) { | C | Descriptor |
| --- | --- | --- |
| focal_length_x_flag | 11 | u(1) |
| focal_length_y_flag | 11 | u(1) |
| principal_point_x_flag | 11 | u(1) |
| principal_point_y_flag | 11 | u(1) |
| translation_flag | 11 | u(1) |
| if( focal_length_x_flag ) |  |  |
| 3dv_acquisition_element( 0, predDirection, 6, 1, FocalLengthXSign, FocalLengthXExp, FocalLengthXMantissa, FocalLengthXManLen ) |  |  |
| if( focal_length_y_flag ) |  |  |
| 3dv_acquisition_element( 0, predDirection, 6, 1, FocalLengthYSign, FocalLengthYExp, FocalLengthYMantissa, FocalLengthYManLen ) |  |  |
| if( principal_point_x_flag ) |  |  |
| 3dv_acquisition_element( 0, predDirection, 6, 1, PrincipalPointXSign, PrincipalPointXExp, PrincipalPointXMantissa, PrincipalPointXManLen ) |  |  |
| if( principal_point_y_flag ) |  |  |
| 3dv_acquisition_element( 0, predDirection, 6, 1, PrincipalPointYSign, PrincipalPointYExp, PrincipalPointYMantissa, PrincipalPointYManLen ) |  |  |
| if( translation_flag ) |  |  |
| 3dv_acquisition_element( 1, predDirection, 6, 1, TXSign, TXExp, TXMantissa, TXManLen ) |  |  |
| } |  |  |

        - Slice layer extension RBSP syntax

The syntax table is specified in subclause I.Error: Reference source not found.

      - Slice header syntax

The syntax table is specified in subclause I.Error: Reference source not found.

        - Reference picture list modification syntax

The syntax table is specified in subclause I.Error: Reference source not found.

          - Reference picture list MVC modification syntax

The syntax table is specified in subclause I.7.3.3.1.1

          - Reference picture list 3DVC modification syntax

| ref_pic_list_3dvc_modification( ) { | C | Descriptor |
| --- | --- | --- |
| if( slice_type % 5 != 2 && slice_type % 5 != 4 ) { |  |  |
| ref_pic_list_modification_flag_l0 | 2 | u(1) |
| if( ref_pic_list_modification_flag_l0 ) |  |  |
| do { |  |  |
| modification_of_pic_nums_idc | 2 | ue(v) |
| if( modification_of_pic_nums_idc = = 0 ¦ ¦ modification_of_pic_nums_idc = = 1 ) |  |  |
| abs_diff_pic_num_minus1 | 2 | ue(v) |
| else if( modification_of_pic_nums_idc = = 2 ) |  |  |
| long_term_pic_num | 2 | ue(v) |
| else if ( modification_of_pic_nums_idc = = 4 ¦ ¦ modification_of_pic_nums_idc = = 5 ) |  |  |
| abs_diff_view_idx_minus1 | 2 | ue(v) |
| else if( modification_of_pic_nums_idc = = 6 ) |  |  |
| vsp_ref_idx | 2 | ue(v) |
| } while( modification_of_pic_nums_idc != 3 ) |  |  |
| } |  |  |
| if( slice_type % 5 = = 1 ) { |  |  |
| ref_pic_list_modification_flag_l1 | 2 | u(1) |
| if( ref_pic_list_modification_flag_l1 ) |  |  |
| do { |  |  |
| modification_of_pic_nums_idc | 2 | ue(v) |
| if( modification_of_pic_nums_idc = = 0 ¦ ¦ modification_of_pic_nums_idc = = 1 ) |  |  |
| abs_diff_pic_num_minus1 | 2 | ue(v) |
| else if( modification_of_pic_nums_idc = = 2 ) |  |  |
| long_term_pic_num | 2 | ue(v) |
| else if ( modification_of_pic_nums_idc = = 4 ¦ ¦ modification_of_pic_nums_idc = = 5 ) |  |  |
| abs_diff_view_idx_minus1 | 2 | ue(v) |
| else if( modification_of_pic_nums_idc = = 6 ) |  |  |
| vsp_ref_idx | 2 | ue(v) |
| } while( modification_of_pic_nums_idc != 3 ) |  |  |
| } |  |  |
| } |  |  |

        - Prediction weight table syntax

The syntax table is specified in subclause I.Error: Reference source not found.

        - Decoded reference picture marking syntax

The syntax table is specified in subclause I.Error: Reference source not found.

      - Slice data syntax

The syntax table is specified in subclause I.Error: Reference source not found.

      - Macroblock layer syntax

The syntax table is specified in subclause I.Error: Reference source not found.

        - Macroblock prediction syntax

| mb_pred( mb_type ) { | C | Descriptor |
| --- | --- | --- |
| if( MbPartPredMode( mb_type, 0 ) = = Intra_4x4 ¦ ¦ MbPartPredMode( mb_type, 0 ) = = Intra_8x8 ¦ ¦ MbPartPredMode( mb_type, 0 ) = = Intra_16x16 ) { |  |  |
| if( MbPartPredMode( mb_type, 0 ) = = Intra_4x4 ){ |  |  |
| for( luma4x4BlkIdx=0; luma4x4BlkIdx<16; luma4x4BlkIdx++ ) { |  |  |
| prev_intra4x4_pred_mode_flag[ luma4x4BlkIdx ] | 2 | u(1) ¦ ae(v) |
| if( !prev_intra4x4_pred_mode_flag[ luma4x4BlkIdx ] ) |  |  |
| rem_intra4x4_pred_mode[ luma4x4BlkIdx ] | 2 | u(3) ¦ ae(v) |
| } |  |  |
| for( luma4x4BlkIdx=0; luma4x4BlkIdx<16; luma4x4BlkIdx++ ) { |  |  |
| if( psip_flag && Psip4x4Available && Intra4x4PredMode[luma4x4BlkIdx] = = 2 ){ |  |  |
| psip4x4_dir[ luma4x4BlkIdx ] | 2 | u(1) |
| for( idxRowCol=0; idxRowCol<4; idxRowCol++ ) |  |  |
| psip4x4_code_diff[ luma4x4BlkIdx ][ idxRowCol ] | 2 | se(v) ¦ ae(v) |
| [Ed. CAVLC syntax missing] |  |  |
| } |  |  |
| } |  |  |
| } |  |  |
| if( MbPartPredMode( mb_type, 0 ) = = Intra_8x8 ){ |  |  |
| for( luma8x8BlkIdx=0; luma8x8BlkIdx<4; luma8x8BlkIdx++ ) { |  |  |
| prev_intra8x8_pred_mode_flag[ luma8x8BlkIdx ] | 2 | u(1) ¦ ae(v) |
| if( !prev_intra8x8_pred_mode_flag[ luma8x8BlkIdx ] ) |  |  |
| rem_intra8x8_pred_mode[ luma8x8BlkIdx ] | 2 | u(3) ¦ ae(v) |
| } |  |  |
| for( luma8x8BlkIdx =0; luma8x8BlkIdx <4; luma8x8BlkIdx ++ ) { |  |  |
| if( psip_flag && Psip8x8Available && Intra8x8PredMode[luma4x4BlkIdx] = = 2 ){ |  |  |
| psip8x8_dir[ luma8x8BlkIdx ] | 2 | u(1) |
| for( idxRowCol=0; idxRowCol<8; idxRowCol++ ) |  |  |
| psip8x8_code_diff[ luma8x8BlkIdx ][idxRowCol] | 2 | se(v) ¦ ae(v) |
| } |  |  |
| } |  |  |
| } |  |  |
| if( MbPartPredMode( mb_type, 0 ) = = Intra_16x16 ) { |  |  |
| if( psip_flag && Psip16x16Available && Intra16x16PredMode = = 2 ){ |  |  |
| psip16x16_dir | 2 | u(1) |
| for( idxRowCol=0; idxRowCol<16; idxRowCol++ ) |  |  |
| psip16x16_code_diff[ idxRowCol ] | 2 | se(v) ¦ ae(v) |
| } |  |  |
| } |  |  |
| if( ChromaArrayType = = 1 ¦ ¦ ChromaArrayType = = 2 ) |  |  |
| intra_chroma_pred_mode | 2 | ue(v) ¦ ae(v) |
| } else if( MbPartPredMode( mb_type, 0 ) != Direct ) { |  |  |
| for( mbPartIdx = 0; mbPartIdx < NumMbPart( mb_type ); mbPartIdx++) |  |  |
| if( ( num_ref_idx_l0_active_minus1 > 0 ¦ ¦ mb_field_decoding_flag != field_pic_flag ) && MbPartPredMode( mb_type, mbPartIdx ) != Pred_L1 ) |  |  |
| ref_idx_l0[ mbPartIdx ] | 2 | te(v) ¦ ae(v) |
| for( mbPartIdx = 0; mbPartIdx < NumMbPart( mb_type ); mbPartIdx++) |  |  |
| if( ( num_ref_idx_l1_active_minus1 > 0 ¦ ¦ mb_field_decoding_flag != field_pic_flag ) && MbPartPredMode( mb_type, mbPartIdx ) != Pred_L0 ) |  |  |
| ref_idx_l1[ mbPartIdx ] | 2 | te(v) ¦ ae(v) |
| for( mbPartIdx = 0; mbPartIdx < NumMbPart( mb_type ); mbPartIdx++) |  |  |
| if( MbPartPredMode ( mb_type, mbPartIdx ) != Pred_L1 ) |  |  |
| for( compIdx = 0; compIdx < 2; compIdx++ ) |  |  |
| mvd_l0[ mbPartIdx ][ 0 ][ compIdx ] | 2 | se(v) ¦ ae(v) |
| for( mbPartIdx = 0; mbPartIdx < NumMbPart( mb_type ); mbPartIdx++) |  |  |
| if( MbPartPredMode( mb_type, mbPartIdx ) != Pred_L0 ) |  |  |
| for( compIdx = 0; compIdx < 2; compIdx++ ) |  |  |
| mvd_l1[ mbPartIdx ][ 0 ][ compIdx ] | 2 | se(v) ¦ ae(v) |
| } |  |  |
| } |  |  |

        - Sub-macroblock prediction syntax

The syntax table is specified in subclause I.Error: Reference source not found.

        - Residual data syntax

The syntax table is specified in subclause I.Error: Reference source not found.

          - Residual luma syntax

The syntax table is specified in subclause I.Error: Reference source not found.

          - Residual block CAVLC syntax

The syntax table is specified in subclause I.Error: Reference source not found.

          - Residual block CABAC syntax

The syntax table is specified in subclause I.Error: Reference source not found.

    - Semantics

Semantics associated with the syntax structures and syntax elements within these structures (in subclause J.7.3 and in subclause I.7.3 by reference in subclause J.7.3) are specified in this subclause and by reference to subclause I.7.4. When the semantics of a syntax element are specified using a table or a set of tables, any values that are not specified in the table(s) shall not be present in the bitstream unless otherwise specified in this Recommendation | International Standard.

      - NAL unit semantics

The semantics for the syntax elements in subclause 7.3.1 are specified in subclause I.7.3.1.

        - NAL unit header 3DVC extension semantics

- view_idx specifies the view oder index for the NAL unit.

- view_id is inferred to be equal to view_id[ view_idx ], where view_id[ ] is present in the active sequence parameter set.

- The variable VOIdx, representing the view order index of the view identified by view_id[ i ], is set equal to view_idx.

- depth_flag equal to 1 indicates that the current NAL unit belongs to a depth view component, depth_flag equal to 0 indicates that the current NAL unit belongs to a texture view component.

- non_idr_flag, temporal_id, anchor_pic_flag, and inter_view_flag have the same semantics as those syntax elements with the same names in Annex H.

        - Order of NAL units and association to coded pictures, access units, and video sequences

The specification of subclause I.7.4.1.2 applies.

          - Order of 3DVC sequence parameter set RBSPs and picture parameter set RBSPs and their activation

- The specification of subclause I.7.4.1.2.1 applies.

- In addition, the following applies for the activiation of depth parameter set.

- A depth parameter set RBSP includes parameters that can be referred to by the coded slice NAL units of one or more texture view or depth view components of one or more coded pictures.

- Each depth parameter set RBSP is initially considered not active at the start of the operation of the decoding process. At most one depth parameter set RBSP is considered as the active depth parameter set RBSP at any given moment during the operation of the decoding process, and when any particular depth parameter set RBSP becomes the active depth parameter set RBSP, the previously-active depth parameter set RBSP (if any) is deactivated.

- When a depth parameter set RBSP (with a particular value of depth_parameter_set_id) is not the active depth parameter set RBSP and it is referred to by a coded slice NAL unit (when dps_id is present in a slice header), it is activated. This depth parameter set RBSP is called the active depth parameter set RBSP until it is deactivated when another depth parameter set RBSP becomes the active depth parameter set RBSP. A depth parameter set RBSP, with that particular value of depth_parameter_set_id, shall be available to the decoding process prior to its activation. When a depth parameter set RBSP is activated, the same depth parameter set RBSP shall remain active for subsequent coded slice NAL units of the same access unit.

- If any depth parameter set RBSP is present that is never activated in the bitstream (i.e., it never becomes the active depth parameter set), its syntax elements shall have values that would conform to the specified constraints if it were activated by reference in an otherwise-conforming bitstream.

          - Order of access units and association to coded video sequences

The specification of subclause I.7.4.1.2.2 applies.

          - Order of NAL units and coded pictures and association to access units

The specification of subclause I.Error: Reference source not found applies with the following modifications.

The association of VCL NAL units to primary or redundant coded pictures is specified in subclause I.7.4.1.2.5.

          - Detection of the first VCL NAL unit of a primary coded picture

The specification of subclause I.7.4.1.2.4 applies.

          - Order of VCL NAL units and association to coded pictures

The specification of subclause I.7.4.1.2.5 applies with following modifications.

Each VCL NAL unit is part of a coded picture.

Let voIdx be the value of VOIdx of any particular VCL NAL unit. The order of the VCL NAL units within a coded picture is constrained as follows:

- For all VCL NAL units following this particular VCL NAL unit, the value of VOIdx shall be greater than or equal to voIdx.

- All VCL NAL units for a depth view component, if present, shall follow any VCL NAL unit of an MVC texture view component with a same value of VOIdx.

      - Raw byte sequence payloads and RBSP trailing bits semantics

        - Sequence parameter set RBSP semantics

The semantics specified in subclause I.Error: Reference source not found apply.

          - Sequence parameter set data semantics

The semantics specified in subclause I.Error: Reference source not found apply.

            - Scaling list semantics

The semantics specified in subclause I.Error: Reference source not found apply.

          - Sequence parameter set extension RBSP semantics

The semantics specified in subclause I.Error: Reference source not found apply.

          - Subset sequence parameter set RBSP semantics

The semantics specified in subclause I.Error: Reference source not found apply.

          - Sequence parameter set MVC extension semantics

The semantics specified in subclause I.7.4.2.1.4 apply with the substitution of texture view component or depth view component for view component

          - Sequence parameter set 3DVC extension semantics

The semantics specified in subclause I.7.4.2.1.4 apply with the following changes and additions.

depth_info_present_flag equal to 0 specifies that no depth view components are present in a coded video sequence in which this subset sequence parameter set RBSP is active. depth_info_present_flag equal to 1 specifies that depth view components may be present in a coded video sequence in which this subset sequence parameter set RBSP is active.

3dv_acquisition_idc equal to 0 indicates that no 3DV acquisition parameters (the closest and farthest depth values) are present in the sequence parameter set. 3dv_acquisition_idc equal to 1 indicates that 3DV acquisition parameters are present in the sequence parameter set and valid for the entire coded video sequence. 3dv_acquisition_idc equal to 2 indicates that 3DV acquisition parameters are present in the sequence parameter set and depth range parameter update NAL units may be present. 3dv_acquisition_idc values greater than 2 are reserved.

The function ViewIdTo3DVAcquisitionParamIndex( viewId ) is specified to return the value of i for which view_id_3dv[ i ] is equal to viewId in the active sequence parameter set.

depth_preceding_texture_flag[ i ] specifies the decoding order of depth view components in relation to texture view components. depth_preceding_texture_flag[ i ] equal to 1 indicates that the depth view component of the view with view_idx equal to i precedes the texture view component of the same view in decoding order in each access unit that contains both the texture and depth view components. depth_preceding_texture_flag[ i ] equal to 0 indicates that the texture view component of the view with view_idx equal to i precedes the depth view component of the same view in decoding order in each access unit that contains both the texture and depth view components.

The function ViewCompOrder( depthFlag, vOIdx ) is specified to return the following value:

vOIdx*2+ ( (depth_preceding_texture_flag[vOIdx] = = depthFlag) ? 0: 1).

reduced_resolution_flag equal to 1 specifies that the depth view components of a view component pair have a lower spatial resolution than the luma component of the texture view component of the same view component pair, and the width and height (as represented by pic_width_in_mbs_minus1+1 and pic_height_in_map_units_minus1+1 in the referred subset sequence parameter set) of the depth view components are both half of the width and height of all the texture view components. reduced_resolution_flag equal to 0 specifies that when both depth view components and texture view components are present, they have the same spatial resolution.

slice_header_prediction_flag equal to 0 indicates that slice header prediction from texture view component to depth view component or vice versa is disallowed. slice_header_prediction_flag equal to 1 indicates that the prediction is used.

inside_view_mvp_flag equal to 1 indicates inside view motion prediction is enabled for depth view components having view order index vOIdx when svc_extension_flag is equal to 1 and ViewCompOrder( 0, vOIdx) is smaller than ViewCompOrder( 1, vOIdx). inside_view_mvp_flag equal to 0 inidicates that inside view motion prediction is disabled for all view components referring the current sequence parameter set.

seq_view_synthesis_flag equal to 1 indicates view synthesis prediction is enabled. seq_view_synthesis_flag equal to 0 indicates that view synthesis prediction is disbable for all view components referring the current sequence parameter set.

disp_flag equal to 1 indicates depth intra skip prediction is enabled for view components refereeing the current sequence parameter set. disp_flag equal to 0 indicates that depth intra skip prediction is disbable for all view components referring the current sequence parameter set. When not present, disp_flag is inferred to be equal to 0.

psip_flag equal to 1 indicates plane segmentation based intra prediction is enabled. psip_flag equal to 0 indicates that plane segmentation based intra prediction is disbable for all view components referring the current sequence parameter set. When not present, psip_flag is inferred to be equal to 0.

nonlinear_depth_representation_num + 1 specifies the number of piecewise linear segments for mapping of depth values to a scale that is uniformly quantized in terms of disparity.

nonlinear_depth_representation_model[ i ] specifies the piecewise linear segments for mapping of depth values to a scale that is uniformly quantized in terms of disparity.

NOTE – When nonlinear_depth_representation_num is equal to 0, a depth sample represents a disparity normalized to the range of 0 to 255, inclusive, so that 0 corresponds to ZFar value and 255 corresponds to ZNear value. Depending on the value of nonlinear_depth_representation_num, a depth view component is composed of either depth samples that can be converted to disparity using a linear equation or nonlinearly transformed depth samples. If nonlinear_depth_representation_num is equal to 0, depth view component contains directly depth samples that are uniformly quantized in terms of disparity, i.e. depth samples that can be transformed to disparity using a linear equation. If nonlinear_depth_representation_num is greater than 0, depth view component contains nonlinearly transformed depth samples. 

Variable NdrInverse[ i ], as specified below, is used to transform depth sample values from nonlinear representation to the linear representation. The shape of this transform is defined by means of line-segment-approximation in two-dimensional linear-disparity-to-nonlinear-disparity space. The first (0, 0) and the last (255, 255) nodes of the curve are predefined. Positions of additional nodes are transmitted in form of deviations (nonlinear_depth_representation_model[ i ]) from the straight-line curve. These deviations are uniformly distributed along the whole range of 0 to 255, inclusive, with spacing depending on the value of nonlinear_depth_representation_num.

Variable NdrInverse[ i ] for i in the range of 0 to 255, inclusive, is specified as follows.

nonlinear_depth_representation_model[ 0 ] = 0
nonlinear_depth_representation_model[ nonlinear_depth_representation_num + 1 ] = 0
for( k=0; k<= nonlinear_depth_representation_num; ++k )
{
	pos1 = ( 255 * k ) / ( nonlinear_depth_representation_num + 1 )
	dev1 = nonlinear_depth_representation_model[ k ]
	pos2 = ( 255 * ( k+1 ) ) / ( nonlinear_depth_representation_num + 1 ) )
	dev2 = nonlinear_depth_representation_model[ k+1 ]

	x1 = pos1 - dev1
	y1 = pos1 + dev1
	x2 = pos2 - dev2
	y2 = pos2 + dev2

	for ( x = max( x1, 0 ); x <= min( x2, 255 ); ++x )
		NdrInverse[ x ] = Clip3( 0, 255, Round( ( ( x - x1 ) * ( y2 - y1 ) ) ÷ ( x2 - x1 ) + y1 ) )
}

        - Picture parameter set RBSP semantics

The semantics specified in subclause I.Error: Reference source not found apply.

        - Supplemental enhancement information RBSP semantics

The semantics specified in subclause I.Error: Reference source not found apply.

          - Supplemental enhancement information message semantics

The semantics specified in subclause I.Error: Reference source not found apply.

        - Access unit delimiter RBSP semantics

The semantics specified in subclause I.Error: Reference source not found apply.

        - End of sequence RBSP semantics

The semantics specified in subclause I.Error: Reference source not found apply.

        - End of stream RBSP semantics

The semantics specified in subclause I.Error: Reference source not found apply.

        - Filler data RBSP semantics

The semantics specified in subclause I.Error: Reference source not found apply.

        - Slice layer without partitioning RBSP semantics

The semantics specified in subclause I.Error: Reference source not found apply.

        - Slice data partition RBSP semantics

Slice data partition syntax is not present in bitstreams conforming to one or more of the profiles specified in Annex J.

        - RBSP slice trailing bits semantics

The semantics specified in subclause I.Error: Reference source not found apply.

        - RBSP trailing bits semantics

The semantics specified in subclause I.Error: Reference source not found apply.

        - Prefix NAL unit RBSP semantics

The semantics specified in subclause I.Error: Reference source not found apply.

        - Depth parameter set RBSP semantics

depth_parameter_set_id identifies the depth parameter set that is referred to in the slice header. The value of depth_parameter_set_id shall be in the range of 1 to TBD, inclusive.

pred_direction equal to 0 specifies that the 3DV acquisition variables (the closest and farthest depth value) for the base view may be predicted from the respective variables of two pictures. pred_direction equal to 1 specifies that the 3DV acquisition variables for the base view may be predicted from the respective variables of one picture. pred_direction equal to 2 specifies that the 3DV acquisition variables for the base view are not predicted. The value of pred_direction shall be in the range of 0 to 2, inclusive.

ref_dps_id0 specifies a first reference depth parameter set to be used in prediction of 3DV acquisition variables. The value of ref_dps_id0 shall be in the range of 0 to TBD, inclusive.

ref_dps_id1 specifies a second reference depth parameter set to be used in prediction of 3DV acquisition variables. The value of ref_dps_id1 shall be in the range of 0 to TBD, inclusive.

pred_weight0 specifies a weight associated with the a first reference depth parameter set derived from ref_dps_id0.

num_views_minus1 plus1 1 specifies the number of views for which depth parameters are specified in the included 3dv_acquisition_element ( ) structure.

vsp_param_flag equal to 1 indicates the presence of the vsp_param( ) syntax structure. vsp_param_flag equal to 0 indicates the absence of the vsp_param( ) syntax structure.

depth_param_additional_extension_flag equal to 0 indicates that no additional data follows within the depth range update RBSP prior to the RBSP trailing bits. The value of depth_param_additional_extension_flag shall be equal to 0. The value of 1 for depth_param_additional_extension_flag is reserved for future use by ITU-T | ISO/IEC. Decoders shall ignore all data that follows the value of 1 for depth_param_additional_extension_flag in a depth range update RBSP.

          - Depth ranges semantics

The contents of the syntax structure are controlled through input variables predDirection and index the semantics of which are as follows.

–	predDirection equal to 2 specifies that the first loop entry of the element is not predicted and coded in the sign, exponent, and mantissa syntax elements. predDirection equal to 0 or 1 specifies that the first loop entry of the element is predicted and a difference relative to a prediction value is coded in the difference syntax element.

–	index may be equal to the depth_parameter_set_id of the depth parameter set wherein the parameters are present.

z_near_flag equal to 0 specifies that the syntax elements specifying the closest depth value are not present in the syntax structure. z_near_flag equal to 1 specifies that the syntax elements specifying the closest depth value are present in the syntax structure.

z_far_flag equal to 0 specifies that the syntax elements specifying the farthest depth value are not present in the syntax structure. z_near_flag equal to 1 specifies that the syntax elements specifying the farthest depth value are present in the syntax structure.

          - 3DV acquisition element semantics

The syntax structure specifies the value of an element in 3DV acquisition information. The element may contain one or more loop entries i of the order specified by view_id_3dv syntax elements.

The contents of the syntax structure are controlled through input variables predDirection, precMode, and expLen the semantics of which are as follows.

–	deltaFlag equal to 0 specifies that the each loop entry corresponds to a value specific to the view indicated by i. deltaFlag equal to 1 specifies that each loop entry corresponds to a difference of values between the views indicated by i and i + 1.

–	predDirection equal to 2 specifies that the first loop entry of the element is not predicted and coded in the sign, exponent, and mantissa syntax elements. predDirection equal to 0 or 1 specifies that the first loop entry of the element is predicted and a difference relative to a prediction value is coded in the difference syntax element.

–	precMode equal to 0 specifies that the number of bits in the mantissa syntax element.

–	expLen specifies the number of bits in the exponent syntax element.

The syntax structure uses OutSign, OutExp, and OutMantissa variables for both input and output, where each variable is indexed by [ index, viewIdc ], index being an identifier (equal to either 0 when decoding depth ranges in sequence parameter set or depth_parameter_set_id value when decoding depth range parameter set) to a depth parameter set and viewIdc being a view indicator (in the order of views for 3DV acquisition parameters).

element_equal_flag equal to 0 specifies that the sign, exponent, and mantissa may not be identical to respective values for any two loop entries i and j. element_equal_flag equal to 1 specifies that the sign, exponent, and mantissa are identical to respective values for any two loop entries i and j.

mantissa_len_minus1 + 1 specifies the number of bits in the mantissa syntax element. The value of mantissa_len_minus1 shall be in the range of 0 to 31, inclusive.

prec specifies the exponent of the maximum allowable truncation error for the value represented by the sign, exponent, and mantissa given by 2−prec. The value of prec shall be in the range of 0 to 31, inclusive.

sign0 equal to 0 indicates that the sign of the value provided in the loop entry is positive. sign0 equal to 1 indicates that the sign is negative.

exponent0 specifies the exponent of the value provided by the loop entry. The value of exponent0 shall be in the range of 0 to 2expLen – 2, inclusive. The value 2expLen – 1 is reserved for future use by ITU-T | ISO/IEC. Decoders shall treat the value 2expLen – 1 as indicating an unspecified value.

mantissa0 specifies the mantissa of the value provided by the loop entry.

skip_flag equal to 0 specifies that syntax elements sign1, exponent_skip_flag and mantissa_diff are present for the loop entry. skip_flag equal to 1 specifies that elements sign1, exponent_skip_flag and mantissa_diff are not present for the loop entry.

sign1 equal to 0 indicates that the sign of the value provided in the loop entry is positive. sign1 equal to 1 indicates that the sign is negative.

exponent1, if present, specifies the exponent of the value provided by the loop entry. The value of exponent1 shall be in the range of 0 to 2expLen – 2, inclusive. The value 2expLen – 1 is reserved for future use by ITU-T | ISO/IEC. Decoders shall treat the value 2expLen – 1 as indicating an unspecified value.

mantissa_diff specifies the difference of the mantissa of the value provided by the loop entry relative to its prediction value.

          - View synthesis parameters semantics

The contents of the syntax structure are controlled through input variables predDirection and index the semantics of which are as follows.

–	predDirection equal to 2 specifies that the first loop entry of the element is not predicted and coded in the sign, exponent, and mantissa syntax elements. predDirection equal to 0 or 1 specifies that the first loop entry of the element is predicted and a difference relative to a prediction value is coded in the difference syntax element.

–	index identifies a depth parameter set.

focal_length_x_flag equal to 0 specifies that the syntax elements specifying the horizontal focal length are not present in the syntax structure. focal_length_x_flag equal to 1 specifies that the syntax elements specifying the horizontal focal length are present in the syntax structure.

focal_length_y_flag equal to 0 specifies that the syntax elements specifying the vertical focal length are not present in the syntax structure. focal_length_y_flag equal to 1 specifies that the syntax elements specifying the vertical focal length are present in the syntax structure.

principal_point_x_flag equal to 0 specifies that the syntax elements specifying the horizontal principal point are not present in the syntax structure. principal_point_x_flag equal to 1 specifies that the syntax elements specifying the horizontal principal point are present in the syntax structure.

principal_point_y_flag equal to 0 specifies that the syntax elements specifying the vertical principal point are not present in the syntax structure. principal_point_y_flag equal to 1 specifies that the syntax elements specifying the vertical principal point are present in the syntax structure.

translation_flag equal to 0 specifies that the syntax elements specifying the horizontal translation between views are not present in the syntax structure. translation_flag equal to 1 specifies that the syntax elements specifying the horizontal translation between views are present in the syntax structure.

        - Slice layer extension RBSP semantics

The semantics specified in subclause I.7.4.2.14 apply.

      - Slice header semantics

The semantics specified in subclause H.Error: Reference source not found apply with the substitution of texture view component or depth view component for view component and with the following modifications.

When nal_unit_type is equal to 1, 5, 20 or 21 with DepthFlag equal to 0, all constraints specified in subclause H.Error: Reference source not found apply only to the texture view components with the same value of VOIdx. When nal_unit_type is equal to 21 and DepthFlag is equal to 1, all constraints specified in subclause H.Error: Reference source not found apply only to the depth view components with the same value of VOIdx.

The value of the following 3DVC sequence parameter set syntax elements shall be the same across all coded slice NAL units of nal_unit_type 1, 5, 20 and 21 with DepthFlag equal to 0 of an access unit: chroma_format_idc.

The value of the following slice header syntax elements shall be the same across all coded slice NAL units of nal_unit_type 1, 5, 20 and 21 with DepthFlag equal to 0 of an access unit: field_pic_flag and bottom_field_flag.

        - Reference picture list modification semantics

The semantics specified in subclause I.Error: Reference source not found apply.

          - Reference picture list MVC modification semantics

The semantics specified in subclause I.7.4.3.1.1 apply.

          - Reference picture list 3DVC modification semantics

The semantics specified in subclause I.7.4.3.1.1 apply with the following additions.

The following additional table entry is inserted in Table H-1.

| modification_of_pic_nums_idc | Modification specified |
| --- | --- |
| 6 | vsp_ref_idx is present and corresponds to a VSP reference index |

modification_of_pic_nums_idc shall not be equal to 6, if seq_view_synthesis_flag is equal to 0.

        - Prediction weight table semantics

The semantics specified in subclause I.Error: Reference source not found apply.

        - Decoded reference picture marking semantics

The semantics specified in subclause I.Error: Reference source not found apply to each view independently, with "sequence parameter set" being replaced by "3DVC sequence parameter set", with "primary coded picture" being replaced by "texture view component” for nal_unit_type 1, 5, and 20 as well as nal_unit_type 21 when DepthFlag is equal to 1, and by “depth view component” for nal_unit_type 21 when DepthFlag is equal to 0.

      - Slice data semantics

The semantics specified in subclause I.Error: Reference source not found apply.

      - Macroblock layer semantics

The semantics specified in subclause I.Error: Reference source not found apply.

        - Macroblock prediction semantics

The semantics specified in subclause I.Error: Reference source not found apply with the following additions.

psip_4x4_dir specifies the pattern code direction for Intra_4x4_DC_PSIP mode luma block. psip_4x4_dir equal to 0 indicates that the PSIP pattern code is generated with row-by-row, and psip_4x4_dir equal to 1 indicates that the PSIP pattern code is generated with column-by-column.

psip_4x4_code_diff specifies the difference of pattern code between two adjacent rows or columns in the range of -4 to 3.

Psip4x4Available is set to 1 if eight neighbouring samples p[ x, y ] that are constructed luma samples prior to the deblocking filter process, with x = −1, y = 0..3 and with x = 0..3, y = −1 are marked as “available for Intra_4x4_prediction”.

psip_8x8_dir specifies the pattern code direction for Intra_8x8_DC_PSIP mode luma block. psip_8x8_dir equal to 0 indicates that the PSIP pattern code is generated with row-by-row, and psip_8x8_dir equal to 1 indicates that the PSIP pattern code is generated with column-by-column.

psip_8x8_code_diff specifies the difference of pattern code between two adjacent rows or columns in the range of -8 to 7.

Psip8x8Available is set to 1 if 16 neighbouring samples p[ x, y ] that are constructed luma samples prior to the deblocking filter process, with x = −1, y = 0..7 and with x = 0..7, y = −1 are marked as “available for Intra_8x8_prediction”.

psip_16x16_dir specifies the pattern code direction for Intra_16x16_DC_PSIP mode luma block. psip_16x16_dir equal to 0 indicates that the PSIP pattern code is generated with row-by-row, and psip_16x16_dir equal to 1 indicates that the PSIP pattern code is generated with column-by-column.

psip_16x16_code_diff specifies the difference of pattern code between two adjacent rows or columns in the range of -16 to 15.

Psip16x16Available is set to 1 if both the macroblocks with address mbAddrA and mbAddrB are all available.

        - Sub-macroblock prediction semantics

The semantics specified in subclause I.Error: Reference source not found apply.

        - Residual data semantics

The semantics specified in subclause I.Error: Reference source not found apply.

          - Residual luma semantics

The semantics specified in subclause I.Error: Reference source not found apply.

          - Residual block CAVLC semantics

The semantics specified in subclause I.Error: Reference source not found apply.

          - Residual block CABAC semantics

The semantics specified in subclause I.Error: Reference source not found apply.

  - 3DVC decoding process

This subclause specifies the decoding process for an access unit of a coded video sequence conforming to one or more of the profiles specified in Annex J. Specifically, this subclause specifies how the decoded picture with multiple texture view components and multiple depth view components is derived from syntax elements and global variables that are derived from NAL units in an access unit when the decoder is decoding the operation point identified by the target temporal level and the target output views.

The decoding process is specified such that all decoders shall produce numerically identical results for the target output views. Any decoding process that produces identical results for the target output views to the process described here conforms to the decoding process requirements of this Recommendation | International Standard.

Unless stated otherwise, the syntax elements and derived upper-case variables that are referred to by the decoding process specified in this subclause and all child processes invoked from the process specified in this subclause are the syntax elements and derived upper-case variables for the current access unit.

The target output views are either specified by external means not specified in this Specification, or, when not specified by external means, there shall be one target output view which is the base view. Let OutputVOIdxList be the list of VOIdx values, in increasing order of VOIdx, of all the target output views in one access unit. The list OutputVOIdxList shall not change within a coded video sequence.

For profiles specified in Annex J, a target output view includes both the texture view and the depth view, which have the same VOIdx.

All sub-bitstreams that can be derived using the sub-bitstream extraction process with pIdTarget equal to any value in the range of 0 to 63, inclusive, tIdTarget equal to any value in the range of 0 to 7, inclusive, viewIdTargetList consisting of any one or more viewIdTarget's identifying the views in the bitstream as inputs as specified in subclause J.8.5 shall result in a set of coded video sequences, with each coded video sequence conforming to one or more of the profiles specified in Annex Error: Reference source not found, Annex Error: Reference source not found, Annex I and Annex J.

Let vOIdxList be a list of integer values specifying the VOIdx values of the view components of the access unit. The variable VOIdxMax is set equal to the maximum value of the entries in the list vOIdxList, and the variable vOIdxMin is set to the minimum value of the entries in the list vOIdxList. VOIdxMax shall be the same for all access units within a coded video sequence. vOIdxMin shall be the same for all anchor access units within a coded video sequence. When the current access unit is an anchor access unit, the variable VOIdxMin is set to vOIdxMin.

The multiview video decoding process specified in this subclause is repeatedly invoked for each texture and depth view component with VOIdx from vOIdxMin to VOIdxMax, inclusive, which is present in the list vOIdxList, in increasing order of VOIdx and in decoding order of texture or depth view components as specified in subclause 7.4.1.2.5.

Outputs of the multiview video decoding process are decoded samples of the current primary coded picture including all decoded texture and depth view components.

For each texture view component and each depth view component, the specifications in clause J.Error: Reference source not found apply, with the decoding processes for reference picture lists construction and decoded reference picture marking being modified in subclauses J.8.1 and J.8.2, respectively. The 3DVC inter prediction, inter-view prediction and view synthesis prediction processes are specified in subclause J.8.3. The decoding process for depth range parameters is specified in subclause J.8.5. The decoding process for depth intra prediction is specified in subclause J.8.7. Additionally, the specification of bitstream subsets is specified in subclause .

    - 3DVC decoding process for reference picture lists construction

The specifications of subclause I.8.1 apply with the following changes and additions:

–	Modification process for reference picture lists is specified in subclause 8.1.1 instead of subclause I.8.1.2.

–	When DepthFlag is equal to 0, the variable VspRefExist is specified after applying subclause H.8.2 as follows.

–	If seq_view_synthesis_flag is equal to 0 (view synthesis prediction is disabled), VspRefExist is set to 0.

–	Otherwise, if the current slice is a P or SP slice and there exists at least one synthetic reference component in RefPicList0, or if  the current slice is a B slice and there exists at least one synthetic reference component in either RefPicList0 or RefPicList1, VspRefExist is set to 1;

–	Otherwise, VspRefExist is set to 0.

      - Modification process for reference picture lists

Input to this process is reference picture list RefPicList0 and, when decoding a B slice, also reference picture list RefPicList1.

Outputs of this process are a possibly modified reference picture list RefPicList0 and, when decoding a B slice, also a possibly modified reference picture list RefPicList1.

When ref_pic_list_modification_flag_l0 is equal to 1, the following ordered steps are specified:

- Let refIdxL0 be an index into the reference picture list RefPicList0. It is initially set equal to 0.

- The corresponding syntax elements modification_of_pic_nums_idc are processed in the order they occur in the bitstream. For each of these syntax elements, the following applies:

–	If modification_of_pic_nums_idc is equal to 0 or equal to 1, the process specified in subclause Error: Reference source not found is invoked with RefPicList0 and refIdxL0 given as input, and the output is assigned to RefPicList0 and refIdxL0.

–	Otherwise, if modification_of_pic_nums_idc is equal to 2, the process specified in subclause Error: Reference source not found is invoked with RefPicList0 and refIdxL0 given as input, and the output is assigned to RefPicList0 and refIdxL0.

–	Otherwise, if modification_of_pic_nums_idc is equal to 4 or equal to 5, the process specified in subclause 8.1.1.1 is invoked with RefPicList0 and refIdxL0 given as input, and the output is assigned to RefPicList0 and refIdxL0.

–	Otherwise, if modification_of_pic_nums_idc is equal to 6, the process specified in subclause 8.1.1.1 is invoked with RefPicList0 and refIdxL0 given as input, and the output is assigned to RefPicList0 and refIdxL0.

–	Otherwise (modification_of_pic_nums_idc is equal to 3), the modification process for reference picture list RefPicList0 is finished.

When ref_pic_list_modification_flag_l1 is equal to 1, the following ordered steps are specified:

- Let refIdxL1 be an index into the reference picture list RefPicList1. It is initially set equal to 0.

- The corresponding syntax elements modification_of_pic_nums_idc are processed in the order they occur in the bitstream. For each of these syntax elements, the following applies:

–	If modification_of_pic_nums_idc is equal to 0 or equal to 1, the process specified in subclause Error: Reference source not found is invoked with RefPicList1 and refIdxL1 given as input, and the output is assigned to RefPicList1 and refIdxL1.

–	Otherwise, if modification_of_pic_nums_idc is equal to 2, the process specified in subclause Error: Reference source not found is invoked with RefPicList1 and refIdxL1 given as input, and the output is assigned to RefPicList1 and refIdxL1.

–	Otherwise, if modification_of_pic_nums_idc is equal to 4 or equal to 5, the process specified in subclause 8.1.1.1 is invoked with RefPicList1 and refIdxL1 given as input, and the output is assigned to RefPicList1 and refIdxL1.

–	Otherwise, if modification_of_pic_nums_idc is equal to 6, the process specified in subclause 8.1.1.1 is invoked with RefPicList1 and refIdxL1 given as input, and the output is assigned to RefPicList1 and refIdxL1.

–	Otherwise (modification_of_pic_nums_idc is equal to 3), the modification process for reference picture list RefPicList1 is finished.

        - Modification process for reference picture lists for synthetic reference components

Inputs to this process are an index refIdxLX and a reference picture list RefPicListX (with X being 0 or 1).

Outputs of this process are an incremented index refIdxLX and a modified reference picture list RefPicListX.

The following procedure is conducted to place a synthetic reference component with VSP reference index equal to vsp_ref_idx into the index position refIdxLX, shift the position of any other remaining pictures to later in the list, and increment the value of refIdxLX:

for( cIdx = num_ref_idx_lX_active_minus1 + 1; cIdx > refIdxLX; cIdx− − )
	RefPicListX[ cIdx ] = RefPicListX[ cIdx − 1]
RefPicListX[ refIdxLX++ ] = synthetic reference component with VSP reference index equal to vsp_ref_idx

    - 3DVC decoded reference picture marking process

The specifications of subclause I.8.2 apply.

    - 3DVC inter prediction, inter-view prediction, and view synthesis prediction process

Processes described in this subclause are invoked when decoding P and B macroblock types, and when nal_unit_type is equal to 21.

The function UniformDepth( depthSample, zN, zF ) is specified to return the zValue specified as follows.

zValue = 1 ÷ ( depthSample ÷ 255 * ( 1 ÷ zN – 1 ÷ zF ) + 1 ÷ zF )	(I.8-XX)

The function dispQPel( depthSample, zN, zF, f, dTX, dPX) is specified to return dispQPel specified as follows.

dispQPel = Floor( ( f * dTX ÷ UniformDepth( depthSample, zN, zF ) + dPX ) * 4 )	(I.8-XX)

The function Disparity( depthSample, zN, zF, f, dTX ) is specified to return disparityValue specified as follows.

disparityValue = dispQPel( depthSample, zN, zF, f, dTX, 0)	(I.8-XX)

[Ed.note: depthSample, camera parameters, like dTX, and zN, zF need satisfy certain constraints. Check the following among others and modify the equations above, if needed. dTX is always non-negative? The depth to be encoded, and zN, zF are provided in a right-hand coordinate system? WarpDir should be drived from camer parameters than index?]

When DepthFlag is equal to 0 and dmvp_flag is equal to 1, the variables DepthRefPicList0, DepthRefPicList1 for B slices, and DepthCurrPic are specified as follows. The variable DepthRefPicList0 is specified to consist of the depth view components of the view component pairs for which the texture view components are in RefPicList0 in the order that RefPicList0[ i ] and DepthRefPicList0[ i ] form a view component pair for any value of i = 0.. num_ref_idx_l0_active_minus1. The variable DepthRefPicList1 is specified for B slices to consist of the depth view components of the view component pairs for which the texture view components are in RefPicList1 in the order that RefPicList1[ i ] and DepthRefPicList1[ i ] form a view component pair for any value of i = 0.. num_ref_idx_l1_active_minus1. The variable DepthCurrPic is specified to be the upsampled decoded sample array of the depth view component of the view component pair for which the texture view component is the current texture view component.

Subclause H.8.4 is applied with the following the following changes.

–	The following paragraph is added after the paragraph starting with “Outputs of this process are …”:

When nal_unit_type is equal to 21 and DepthFlag is equal to 0, subclause 8.3.2 is invoked to decode the sample arrays for each synthetic reference picture included in RefPicListX for X equal to 0 and, if the current slice is a B slice, 1.

–	When nal_unit_type is equal to 21, DepthFlag is equal to 0, and dmvp_flag is equal to 1, subclause 8.3.1 is invoked instead of subclause 8.4.1.

–	When nal_unit_type is equal to 21, DepthFlag is equal to 1, and mb_ivmp_flag is equal to 1, subclause 8.3.3 is invoked instead of subclause 8.4.1.

–	The following additional step applied between steps 3 and 4 in subclause 8.4.

–	When (nal_unit_type is equal to 21 and depth_weighted_pred_flag is equal to 1 and (slice_type % 5) is equal to 0 or 3) or (depth_weighted_bipred_flag is equal to 1 and (slice_type % 5) is equal to 1), the derivation process for prediction weights in depth-range-based weighted prediction in subclause 8.3.4is invoked.

      - Derivation process for motion vector components and reference indices

Inputs to this process are:

–	a macroblock partition mbPartIdx,

–	a sub-macroblock partition subMbPartIdx.

Outputs of this process are:

–	luma motion vectors mvL0 and mvL1 and when ChromaArrayType is not equal to 0, the chroma motion vectors mvCL0 and mvCL1,

–	reference indices refIdxL0 and refIdxL1,

–	prediction list utilization flags predFlagL0 and predFlagL1,

–	a motion vector count variable subMvCnt.

The specifications of subclause 8.4.1 apply with the following changes.

–	The following paragraphs in subclause 8.4.1:

For the derivation of the variables mvL0 and mvL1 as well as refIdxL0 and refIdxL1, the following applies.

–	If mb_type is equal to P_Skip, the derivation process for luma motion vectors for skipped macroblocks in P and SP slices in subclause 8.4.1.1 is invoked with the output being the luma motion vectors mvL0 and reference indices refIdxL0, and predFlagL0 is set equal to 1. mvL1 and refIdxL1 are marked as not available and predFlagL1 is set equal to 0. The motion vector count variable subMvCnt is set equal to 1.

–	Otherwise, if mb_type is equal to B_Skip or B_Direct_16x16 or sub_mb_type[ mbPartIdx ] is equal to B_Direct_8x8, the derivation process for luma motion vectors for B_Skip, B_Direct_16x16, and B_Direct_8x8 in B slices in subclause 8.4.1.2 is invoked with mbPartIdx and subMbPartIdx as the input and the output being the luma motion vectors mvL0, mvL1, the reference indices refIdxL0, refIdxL1, the motion vector count variable subMvCnt, and the prediction utilization flags predFlagL0 and predFlagL1.

are replaced by:

For the derivation of the variables mvL0 and mvL1 as well as refIdxL0 and refIdxL1, the following applies:

–	If mb_type is equal to P_Skip, the following applies:

–	If MbVSSkipFlag is equal to 0, the following applies:

–	If nal_unit_type is equal to 21 and DepthFlag is equal to 0 and dmvp_flag is equal to 1, the depth-based derivation process for luma motion vectors for skipped macroblock in P and SP slices in subclause 8.3.1.1 is invoked with the output being the luma motion vectors mvL0 and refernece indices refIdxL0, and predFlagL0 is set equal to 1.

–	Otherwise (nal_unit_type is not equal to 21 or DepthFlag is equal to 1 or dmvp_flag is equal to 0), the derivation process for luma motion vectors for skipped macroblock in P and SP slices in subclause 8.4.1.1 is invoked with the output being the luma motion vectors mvL0 and refernece indices refIdxL0, and predFlagL0 is set equal to 1.

–	Otherwise (MbVSSkipFlag is equal to 1), the derivation process for luma motion vectors for VSP skipped macroblock in P and SP slices in subclause 8.3.1.2 is invoked with the output being the luma motion vectors mvL0 and reference indices refIdxL0, and predFlagL0 is set equal to 1.

–	mvL1 and refIdxL1 are marked as not available and predFlagL1 is set equal to 0. The motion vector count variable subMvCnt is set equal to 1.

–	Otherwise, if mb_type is equal to B_Skip or B_Direct_16x16 or sub_mb_type[ mbPartIdx ] is equal to B_Direct_8x8, the following applies.

–	The variable vspFlag is specified as follows:

vspFlag = ! ( ( mb_type  = =  B_Skip  &&  MbVSSkipFlag  = =  0 )  | |  ( ( mb_type  = =  B_Direct_16x16  | |  sub_type[ mbPartIdx ]  = =  B_Direct_8x8 )  &&  !mb_direct_type_flag ) )

–	If vspFlag is equal to 0 and nal_unit_type is equal to 21 and DepthFlag is equal to 0 and dmvp_flag is equal to 1, the depth-based derivation process for luma motion vectors for B_Skip, B_Direct_16x16, and B_Direct_8x8 in B slices in subclause 8.3.1.3 is invoked with mbPartIdx and subMbPartIdx as the input and the output being the luma motion vectors mvL0, mvL1, the reference indices refIdxL0, refIdxL1, the motion vector count variable subMvCnt, and the prediction utilization flags predFlagL0 and predFlagL1.

–	Otherwise, if vspFlag is equal to 0 either nal_unit_type is not equal to 21 or DepthFlag is equal to 1 or dmvp_flag is equal to 0, the derivation process for luma motion vectors for B_Skip, B_Direct_16x16, and B_Direct_8x8 in B slices in subclause 8.4.1.2 is invoked with mbPartIdx and subMbPartIdx as the input and the output being the luma motion vectors mvL0, mvL1, the reference indices refIdxL0, refIdxL1, the motion vector count variable subMvCnt, and the prediction utilization flags predFlagL0 and predFlagL1.

–	Otherwise (vspFlag is equal to 1), the derivation process for luma motion vectors for B_Skip, B_Direct_16x16, and B_Direct_8x8 in B slices in subclause 8.3.1.6 is invoked.

–	The following sentence in subclause 8.4.1:

4.	When predFlagLX is equal to 1, the derivation process for luma motion vector prediction in subclause 8.4.1.3 is invoked with mbPartIdx subMbPartIdx, refIdxLX, and currSubMbType as the inputs and the output being mvpLX.

is replaced by

4.	When predFlagLX is equal to 1 and DepthFlag is equal to 0 and dmvp_flag is equal to 1, the derivation process for luma motion vector prediction in subclause 8.3.1.7 is invoked with mbPartIdx subMbPartIdx, refIdxLX, and currSubMbType as the inputs and the output being mvpLX. When predFlagLX is equal to 1 and either DepthFlag is equal to 1 or dmvp_flag is equal to 0, the derivation process for luma motion vector prediction in subclause 8.4.1.3 is invoked with mbPartIdx subMbPartIdx, refIdxLX, and currSubMbType as the inputs and the output being mvpLX.

        - Depth-based derivation process for luma motion vectors for skipped macroblocks in P and SP slices

This process is invoked when mb_type is equal to P_Skip, nal_unit_type is equal to 21, DepthFlag is equal to 0, dmvp_flag is equal to 1 and MbVSSkipFlag is equal to 0.

Outputs of this process are:

–	the motion vector mvL0,

–	the reference index refIdxL0.

For the derivation of the motion vector mvL0 and refIdxL0 of a P_Skip macroblock type, the following ordered steps are specified:

- The process specified in subclause 8.3.1.4 is invoked with mbPartIdx set equal to 0, subMbPartIdx set equal to 0, currSubMbType set equal to "na", and listSuffixFlag equal to 0 as input and the output is assigned to the motion vectors mvCand [ 0 ] and the reference indices refIdxCand[ 0 ].

- The process specified in subclause Error: Reference source not found is invoked with mbPartIdx set equal to 0, subMbPartIdx set equal to 0, currSubMbType set equal to "na", and listSuffixFlag set equal to 0 as input and with mbAddrN\mbPartIdxN\subMbPartIdxN, reference indices refIdxCand[ i ] and the motion vectors mvCand[ i ] as outputs with i equal to 1, 2 and 3 corresponding to neighbouring partition A, B, and C, respectively.

- When refIdxCand[ i ] is equal to -1 for all values of i = 0 .. 3, the variables mvL0 and refIdxL0 are specified as follows

refIdxCand[ 0 ] = 0, mvCand[ 0 ][ 0 ] 	= 0, mvCand[ 0 ][ 1 ] = 0

- The following variables are initialised:

refIdxL0=-1, MvL0[ 0 ]=0, MvL0[ 1 ] =0, cIdx=0

- While refIdxL0 is smaller than 0 and cIdx is smaller than 4, the following apply

refIdxL0= refIdxCand[ cIdx ], mvL0 = mvCand[ cIdx ]

        - Derivation process for luma motion vectors for VSP skipped macroblocks in P and SP slices

This process is invoked when mb_type is equal to P_Skip, nal_unit_type is equal to 21, DepthFlag is equal to 0, and MbVSSkipFlag is equal to 1.

Outputs of this process are the motion vector mvL0 and the reference index refIdxL0.

The reference index refIdxL0 for a VSP skipped macroblock is derived as the synthetic picture that appears first in RefPicList0.

The motion vector mvL0 is set equal to zero motion vector.

        - Derivation process for luma motion vectors for B_Skip, B_Direct_16x16, and B_Direct_8x8

Inputs to this process are current macroblock partition index mbPartIdx and subMbPartIdx.

Outputs of this process are the reference indices refIdxL0, refIdxL1, the motion vectors mvL0 and mvL1, the motion vector count variable subMvCnt, and the prediction list utilization flags, predFlagL0 and predFlagL1.

For the derivation of output, the following ordered steps are specified:

- Let the variable currSubMbType be set equal to sub_mb_type[ mbPartIdx ].

- The process specified in subclause 8.3.1.4 is invoked with mbPartIdx set equal to 0, subMbPartIdx set equal to 0, currSubMbType and listSuffixFlag set equal to 0 as input and the output is assigned to the motion vectors mvCandL0[ 0 ] and the reference indices refIdxCandL0[ 0 ].

- The process specified in subclause 8.3.1.4 is invoked with mbPartIdx set equal to 0, subMbPartIdx set equal to 0, currSubMbType and listSuffixFlag set equal to 1 as input and the output is assigned to the motion vectors mvCandL1[ 0 ] and the reference indices refIdxCandL1[ 0 ].

- The process specified in subclause Error: Reference source not found is invoked with mbPartIdx set equal to 0, subMbPartIdx set equal to 0, currSubMbType, and listSuffixFlag set equal to 0 as inputs and the output is assigned to the motion vectors mvCandL0[ i ] and the reference indices refIdxCandL0[ i ] with i equal to 0, 1, and 2 corresponding to neighbouring partition A, B, and C, respectively.

- The process specified in subclause Error: Reference source not found is invoked with mbPartIdx set equal to 0, subMbPartIdx set equal to 0, currSubMbType, and listSuffixFlag set equal to 1 as inputs and the output is assigned to the motion vectors mvCandL1[ i ] and the reference indices refIdxCandL1[ i ] with i equal to 0, 1, and 2 corresponding to neighbouring partition A, B, and C, respectively.

NOTE 1 – The motion vectors mvL0N, mvL1N and the reference indices refIdxL0N, refIdxL1N are identical for all 4x4 sub-macroblock partitions of a macroblock.

- When refIdxCandL0[ i ] and refIdxCandL1[ i ] are both equal to -1 for all values of i = 0 .. 3, the following applies

refIdxCandL0[ 0 ] = 0, mvCandL0[ 0 ][ 0 ] = 0, mvCandL0[ 0 ][ 1 ] = 0
refIdxCandL1[ 0 ] = 0, mvCandL1[ 0 ][ 0 ] = 0, mvCandL1[ 0 ][ 1 ] = 0

- The following variables are initialised:

refIdxL0 = -1, MvL0[ 0 ] = 0, MvL0[ 1 ] = 0
refIdxL1 = -1, MvL1[ 0 ] = 0, MvL1[ 1 ] = 0

- refIdxL0, refIdxL1, mvL0 and mvL1 are derived as follows:

for( cIdx = 0;cIdx<4 && refIdxL0 < 0 && refIdxL1 < 0 ; cIdx ++) {
	mvL0 = mvCandL0[ cIdx ]
	refIdxL0 = refIdxCandL0[ cIdx ]
	mvL1 = mvCandL1[cIdx ]
	refIdxL1 = refIdxCandL1[cIdx ]
}

        - Derivation process for the disparity vector and the inter-view reference

Inputs to this process are depth reference view component depthPic, the location of a top-left sample ( dbx1, dby1 ) of a partition and the listSuffixFlag.

Outputs of this process are a picture InterViewPic, an offset vector dv and a variable InterViewAvailable

Set InterViewAvailable equal to 0.

The following applies to derive an inter-view reference picture or inter-view only reference picture, InterViewPic, with X set to 1 when listFuffixFlag is 1 or 0 otherwise:

for( cIdx = 0;cIdx<num_ref_idx_l0_active_minus1 + 1 && !InterViewAvailable; cIdx ++)
	if ( view order index of RefPicList0[ cIdx ] is equal to 0) {
		InterViewPic = RefPicList0[ cIdx ]
		InterViewAvailable = 1
	}

When InterViewAvailable is equal to 1, the following steps apply in order.

–	The process specified in subclause Error: Reference source not found is invoked with mbPartIdx set equal to 0, subMbPartIdx set equal to 0, currSubMbType set equal to "na", and listSuffixFlag set equal to 0 as input and with reference indices refIdxCandL0[ i ] and the motion vectors mvCandL0[ i ] as outputs with i equal to 0, 1, and 2 corresponding to neighbouring partition A, B, and C, respectively.

–	The process specified in subclause Error: Reference source not found is invoked with mbPartIdx set equal to 0, subMbPartIdx set equal to 0, currSubMbType set equal to "na", and listSuffixFlag set equal to 1 as input and with reference indices refIdxCandL1[ i ] and the motion vectors mvCandL1[ i ] as outputs with i equal to 0, 1, and 2 corresponding to neighbouring partition A, B, and C, respectively.

–	Set the dv and DvAvailable as follows

for( cIdx = 0;cIdx<3 && !DvAvailable; cIdx ++) {
	if (view order index of RefPicList0[ refIdxCandLX[ cIdx ] ] is equal to 0 ) {
		dv[ 0 ] = mvCandLX[ cIdx ][ 0 ]
		dv[ 1 ] = mvCandLX[ cIdx ][ 1 ]
		DvAvailable = 1
	}

–	When DvAvailable is equal to 0, the following apples.

depth = depthPic[dbx1 +(partWidth << 1) -1, dby1+ (partHeight << 1) -1 ]
index = ViewIdTo3DVAcquisitionParamIndex( view_id of the current view )
refIndex = ViewIdTo3DVAcquisitionParamIndex( view_id of the InterViewPic )

dv[ 0 ] = 0
dv[ 1 ] = Disparity( NdrInverse[depth], ZNear[ dps_id, index ], ZFar[dps_id, index ],
  		FocalLengthX[dps_id, index ], AbsTX[ index ] – AbsTX[ refIndex ] )

        - Derivation process for the inter-view motion vector in inter-view reference

Inputs to this process are mbPartIdx, subMbPartIdx, and listSuffixFlag.

Outputs of this process are the motion vector mvCorrespond and the reference index refIdxCorrespond.

Inter-view reference picture InterViewPic and an offset vector dv are derived as specified by the following ordered steps:

–	The inverse macroblock scanning process as specified in subclause 6.4.1is invoked with CurrMbAddr as the input and the output is assigned to ( x1, y1 ).

–	The inverse macroblock partition scanning process specified in subclause 6.4.2.1 is invoked with mbPartIdx as the input and the output assigned to ( dx1, dy1 ).

–	The inverse sub-macroblock partition scanning process specified in subclause 6.4.2.2 is invoked with mbPartIdx and subMbPartIdx as the input and the output assigned to ( dx2, dy2 ).

–	The process specified in subclause 8.3.1.4 is invoked with depthPic set to DepthCurrPic, dbx1 set to x1 + dx1 + dx2, dby1 set to y1 + dy1 + dy2 and listSuffixFlag as input and InterViewPic, an offset vector dv and an variable InterViewAvailable as outputs.

The refIdxCorrespond and mvCorrespond are set as follows.

–	If InterViewAvailable is equal to 0, refIdxCorrespond is set to -1, and mvCorrespond [ 0 ] and mvCorrespond [ 1 ] are both set to 0.

–	Otherwise, the following step applies in order.

–	The variable luma4x4BlkIdx is derived as  (4 * mbPartIdx + subMbPartIdx).

–	The inverse 4x4 luma block scanning process as specified in subclause Error: Reference source not found is invoked with luma4x4BlkIdx as the input and ( x, y ) as the output. In addition, ( xCorrespond, yCorrespond ) is set equal to ( x + ( dv[ 0 ]>>4 ), y + ( dv [ 1 ]>>4 ) ) and mbAddrCorrespond is set equal to ( ( CurrMbAddr / PicWidthInMbs ) + ( dv[1] >>6 ) ) * PicWidthInMbs + ( CurrMbAddr % PicWidthInMbs ) + ( dv[0] >>6 ).

–	Set mbTypeCorrespond to the syntax element mb_type of the macroblock with address mbAddrCorrespond inside the picture InterViewPic. When mbTypeCorrespond is equal to P_8x8, P_8x8ref0, or B_8x8, subMbTypeCorrespond is set to be the syntax element sub_mb_type of the macroblock with address mbAddrCorrespond inside the picture InterViewPic.

–	Set mbPartIdxCorrespond to the macroblock partition index of the corresponding partition and subMbPartIdxCorrespond to the sub-macroblock partition index of the corresponding sub-macroblock partition. The derivation process for macroblock and sub-macroblock partition indices as specified in subclause Error: Reference source not found is invoked with the luma location equal to ( xCorrespond, yCorrespond ), the macroblock type equal to mbTypeCorrespond, and when mbTypeCorrespond is equal to P_8x8, P_8x8ref0, or B_8x8, the list of sub-macroblock types subMbTypeCorrespond as the inputs and the outputs are the macroblock partition index mbPartIdxCorrespond and the sub-macroblock partition index subMbPartIdxCorrespond.

–	The motion vector mvCorrespond and the reference index refIdxCorrespond are derived as follows.

–	If the macroblock mbAddrCorrespond is coded in as Intra prediction mode, both components of mvCorrespond are set equal to 0 and refIdxCorrespond is set equal to –1.

–	Otherwise (the macroblock mbAddrCorrespond is not coded as Intra prediction mode), the prediction utilization flags predFlagLXCorrespond is set equal to PredFlagLX[ mbPartIdxCorrespond ], the prediction utilization flag of the macroblock partition mbAddrCorrespond\mbPartIdxCorrespond of the picture InterViewPic. In addition, the following applies.

–	When predFlagLXCorrespond is equal to 1, the mvCorrespond and the reference index refIdxCorrespond are set equal to MvLX[ mbPartIdxCorrespond ][ subMbPartIdxCorrespond ] and RefIdxLX[ mbPartIdxCorrespond ], respectively, which are the motion vector mvLX and the reference index refIdxLX that have been assigned to the (sub-)macroblock partition mbAddrCorrespond\mbPartIdxCorrespond\subMbPartIdxCorrespond inside the picture InterViewPic.

        - Derivation process for luma motion vectors for VSP skipped/direct macroblocks in B slices

Outputs of this process are the motion vector mvL0, mvL1 and the reference index refIdxL0, refIdxL1.

The reference index refIdxLX for a VSP skipped/direct macroblock is derived as the synthetic reference component that appears first in the reference picture list X, with X being replaced by 0 or 1. When there is no synthetic picture in the reference picture list X, refIdxLX is set equal to 0.

The motion vector mvLX is set equal to zero motion vector, with X being replaced by 0 or 1.

        - Derivation process for luma motion vector prediction

Inputs to this process are:

–	the macroblock partition index mbPartIdx,

–	the sub-macroblock partition index subMbPartIdx,

–	the reference index of the current partition refIdxLX (with X being 0 or 1),

–	the variable currSubMbType.

Output of this process is the prediction mvpLX of the motion vector mvLX (with X being 0 or 1).

The specifications of subclause 8.4.1.3 apply with the following changes.

–	The following additional sentence is applied

If refIdxLX is not equal to refIdxLXN for any N = A, B, or C and X equal to 0 or 1, the following applies:

mbAddrN\mbPartIdxN\subMbPartIdxN is marked as not available
refIdxLXN = -1
mvLXN[ 0 ] = 0
mvLXN[ 1 ] = 0

after the following paragraph in subclause 8.4.1.3:

The derivation process for the neighbouring blocks for motion data in subclause Error: Reference source not found is invoked with mbPartIdx, subMbPartIdx, currSubMbType, and listSuffixFlag = X (with X being 0 or 1 for refIdxLX being refIdxL0 or refIdxL1, respectively) as the input and with mbAddrN\mbPartIdxN\subMbPartIdxN, reference indices refIdxLXN and the motion vectors mvLXN with N being replaced by A, B, or C as the output.

–	The following additional sentence is applied

–	Otherwise, If refIdxLX is a reference index to an inter-view reference component or an inter-view only reference component, the depth-based derivation process for median luma motion vector prediction in subclause 8.3.1.8 is invoked with mbAddrN\mbPartIdxN\subMbPartIdxN, mvLXN, refIdxLXN with N being replaced by A, B, or C, and refIdxLX as the inputs and the output is assigned to the motion vector predictor mvpLX.

after the following paragraph in subclause 8.4.1.3

–	Otherwise, if MbPartWidth( mb_type ) is equal to 8, MbPartHeight( mb_type ) is equal to 16, mbPartIdx is equal  mvpLX = mvLXCto 1, and refIdxLXC is equal to refIdxLX, the motion vector predictor mvpLX is derived by:

mvpLX = mvLXC

        - Depth-based derivation process for median luma motion vector prediction

Inputs to this process are:

–	the neighbouring partitions mbAddrN\mbPartIdxN\subMbPartIdxN (with N being replaced by A, B, or C),

–	the motion vectors mvLXN (with N being replaced by A, B, or C) of the neighbouring partitions,

–	the reference indices refIdxLXN (with N being replaced by A, B, or C) of the neighbouring partitions,,

–	the reference index refIdxLX of the current partition.

Output of this process is the motion vector prediction mvpLX.

When either partition mbAddrN\mbPartIdxN\subMbPartIdxN is not available or refIdxLXN is not equal to refIdxLX, mvLXN is derived as specified by the following ordered steps:

1.	The inverse macroblock scanning process as specified in subclause 6.4.1is invoked with CurrMbAddr as the input and the output is assigned to ( x1, y1 ).

2.	The inverse macroblock partition scanning process specified in subclause 6.4.2.1 is invoked with mbPartIdx as the input and the output assigned to ( dx1, dy1 ).

3.	The inverse sub-macroblock partition scanning process specified in subclause 6.4.2.2 is invoked with mbPartIdx and subMbPartIdx as the input and the output assigned to ( dx2, dy2 ).

4.	The modification process of inter-view motion vector in median luma motion vector prediction as specified in subclause 8.3.1.8.1 is invoked with depthPic being equal to DepthRefPicList0[ refIdxL0 ], dbx1 being equal to x1 + dx1 + dx2, dby1 being equal to y1 + dy1 + dy2, and mv being equal to mvL0 as inputs and the output is assigned to the motion vector mvLXN.

Each component of the motion vector prediction mvpLX is given by the median of the corresponding vector components of the motion vector mvLXA, mvLXB, and mvLXC:

mvpLX[ 0 ] = Median( mvLXA[ 0 ], mvLXB[ 0 ], mvLXC[ 0 ] )                 (J8-XX)

mvpLX[ 1 ] = Median( mvLXA[ 1 ], mvLXB[ 1 ], mvLXC[ 1 ] )                 (J8-XX)

          - Modification process for inter view motion vector in median luma motion vector prediction

Inputs to this process are

–	depth reference view component depthPic,

–	the location of a top-left sample ( dbx1, dby1 ) of a partition,

–	a motion vector mv,

Outputs of this process are:

–	the motion vector mv.

Let refViewId be the view_id value of depthPic.

The following ordered steps apply:

- Let numSamples be partWidth * partHeight.

- The variable maxDepth is specified as follows:

maxDepth = INT_MIN
for( j = 0; j < partHeight; j++ )
	for( i = 0; i < partWidth; i++ )		(J-8-XX)
		if( depthPic[ dbx1 + i, dby1 + j ] > maxDepth ) maxDepth = depthPic[ dbx1 + i, dby1 + j ]

- The variable mv is specified as follows:

index = ViewIdTo3DVAcquisitionParamIndex( view_id )
refIndex = ViewIdTo3DVAcquisitionParamIndex( refViewId )

mv[ 0 ] = Disparity( NdrInverse[ maxDepth ], ZNear[ dps_id, index ], ZFar[dps_id, index ],
	FocalLengthX[dps_id, index ], AbsTX[ index ] – AbsTX[ refIndex ] )
mv[ 1 ] = 0

      - 3DVC decoding process for view synthesis reference component generation

This process is invoked when decoding a texture view component which refers to a synthetic reference component.

Inputs of this process are a decoded texture view component srcTexturePicY and, if chroma_format_idc is equal to 1 srcTexturePicCb and srcTexturePicCr, and a decoded depth view component srcDepthPic of the same view component pair.

Output of this process is a sample array of a synthetic reference component vspPic consisting of 1 sample array vspPicY when chroma_format_idc is equal to 0, or 3 sample arrays vspPicY, vspPicCb, and vspPicCr when chroma_format_idc is equal to 1.

For the derivation of output, the following ordered steps are specified.

- When reduced_resolution_flag is equal to 0 and chroma_format_idx is equal to 1, the chroma upsampling process specified in subclause 8.3.2.1 is invoked twice, the first time with srcTexturePicCb as input and normTexturePicCb as output, and the second time with srcTexturePicCr as input and normTexturePicCr as output.

- The picture warping and hole filling process specified in subclause 8.3.2.2 is invoked with srcPicY set to srcTexturePictureY, srcPicCb set to normTexturePicCb (when chroma_format_idc is equal to 1), srcPicCr set to normTexturePicCr (when chroma_format_idc is equal to 1), and depPic set to normDepthPic as inputs, and the output assigned to vspPicY, and if chroma_format_idc is equal to 1, vspPicCb and vspPicCr.

        - Chroma upsamping from 4:2:0 to 4:4:4 process

This process is invoked only when chroma_format_idc is equal to 1 and reduced_resolution_flag is equal to 0.

Input is a sample array of a view component inputPic.

Output is a sample array of a view component outputPic.

For each y from 0 to PicHeightInSamplesL – 1, inclusive, the following applies:

–	For each x from 0 to PicWidthInSamplesL – 1, includsive, the following applies:

–	outputPic[y][x] = inputPic[y/2][x/2]

        - Picture warping and hole-filling process

[Ed.note: splatting needs to be added.] [Ed. Note (CY): JCT2-A0023 needs to be integrated.]

Inputs of this process are decoded a luma component of the texture view component srcPicY and, if chroma_format_idc is equal to 1 two chroma components srcPicCb and srcPicCr, and a depth picure depPic.

Outputs of this process is a sample array of a synthetic reference component vspPic consisting of 1 sample array vspPicY when chroma_format_idc is equal to 0, or 3 sample arrays vspPicY, vspPicCb, and vspPicCr when chroma_format_idc is equal to 1.

If ViewIdTo3DVAcquisitionParamIndex( view_id of the current view ) is smaller than ViewIdTo3DVAcquisitionParamIndex( view_id of the input texture view component ), the warping direction WarpDir is set to 0, otherwise, WarpDir is set to 1. [Ed.note. This is not a safe way to derive WarpDir. It should be derived from camera parameters.]

Invoke 8.3.2.2.1 to generate the look up table dispTable.

A resolution factor resFac is set to 1 when reduced_resolution_flag is equal 0 and 2 when reduced_resolution_flag is equal to 1.

For each row i, i from 0 to height-1, (wherein height is the height of the depth array), inclusive, 8.3.2.2.2 and 8.3.2.2.3 are invoked sequentially with the resFac*i-th row of srcPicY srcPicYRow0, resFac*i+1-th row of srcPicY srcPicYRow1 when refFac is 2, the i-th row of scrPicCb scrPicCbRow, the i-th row of scrPicCr scrPicCrRow, the i-th row of depth picture depPicRow and WarpDir as inputs and the i-th row of vspPicY vspPicYRow0, resFac*i+1-th row of vspPicY vspPicYRow1 when refFac is 2, the i-th row of vspPicCb vspPicCbRow, and the i-th row of vspPicCr vspPicCrRow as outputs.

          - Look up table from disparity to depth generation process

[Ed.note:Inputs and outputs TBD].

For each d from 0 to 255, dispTable[d] is set as follows:

dispTable[d] = dispQPel ( NdrInverse[ d ], ZNear[dps_id, index ], ZFar[dps_id, index ], FocalLengthX[dps_id, index ], AbsTX[ index ] – AbsTX[ refIndex ] , PrincipalPointX[ index] – PrincipalPointX[ refIndex]), wherein index and refIndex are derived by the following formulas:

index = ViewIdTo3DVAcquisitionParamIndex(view_id of the current view)

refIndex = ViewIdTo3DVAcquisitionParamIndex(ViewId of the input texture view component)

Note that each entry of dispTable is represented as integer value in the unit of 1/4 –pel position of luma component of the input texture view componet.

          - Fractional pixel to integer pixel conversion

Inputs to this process is one row of reference luma samples srcPicYRow0, another row of reference luma samples when resFac is equal 2 srcPicYRow1, a row of reference cb samples scrPicCbRow, a row of reference cr samples scrPicCrRow, and a row of depth samples depPicRow.

Outputs to this process is one row of reference luma samples srcPicYRowT0, another row of reference luma sample srcPicYRowT1 when resFac is equal to 2, a row of reference cb samples scrPicCbRowT, a row of reference cr samples scrPicCrRowT, and a row of disparity samples disRow.

For each j from 0 to RowWidthY-1 (RowWidthY is the width of the luma array), the following steps apply in order, by setting scrPicYRow to srcPicYRow0, and srcPicYRowT to srcPicYRowT0. When reduced_resolution_flag is equal to 0, in addition, the following steps apply in order again by setting scrPicYRow to srcPicYRow1, and srcPicYRowT to srcPicYRowT1.

- DisQ = dispTable[depPicRow[j>>reduced_resolution_flag]]

- DisI = Sign(DisQ) * (Abs(DisQ) >> 2), Fracpos = DisQ-( DisI <<2).

- The following steps apply in order

–	If Fracpos is equal to 0, the following applies

–	srcPicYRowT[j] = 	srcPicYRow[r]

–	If Fracpos is equal to 2 or -2, k= (Fracpos>0 ? j: j +1), the following applies

–	srcPicYRowT[j]= (srcPicYRow[k-1]*20+ srcPicYRow[k]*20- srcPicYRow[k-2]*5- srcPicYRow[k+1]*5+srcPicYRow[k-3] + srcPicYRow[k+2]+16)>>5

–	If Fracpos is equal to 1, 3, -1 or -3, k= (Fracpos>0 ? j: j +1), the following applies

–	ty= (srcPicYRow[k-1]*20+ srcPicYRow[k]*20- srcPicYRow[k-2]*5- srcPicYRow[k+1]*5+srcPicYRow[k-3] + srcPicYRow[k+2]+16)>>5.

–	s = (Fracpos = = 3 ? -1 : (Fracpos = = -3 ? 1 : 0))

–	srcPicYRowT[j]= (ty+ srcPicYRow[j+s]+1)>>1.

–	If Fracpos is not equal to 0, srcPicYRowT[j] = clip (srcPicYRowT[j], 0, 255)

- disRow[j>>reduced_resolution_flag] = DisI

For each j from 0 to RowWidthC-1 (RowWidthC is the width of the chroma array), the following steps apply in order

- DisQ = dispTable[depPicRow[j]]

- When reduced_resolution_flag is equal to 1, DisQ = Sign(DisQ)*(Abs(DisQ) >> 1).

- DisI = Sign(DisQ)*(Abs(DisQ) >> 2), Fracpos = DisQ-( DisI <<2).

- The following steps apply in order for chroma pixels

–	If Fracpos is equal to 0, the following applies

–	srcCrRowT[j] =			srcCrRow[j]

–	srcCbRowT[j] =			srcCbRow[j]

–	If Fracpos is equal to 2 or -2, k= (Fracpos>0 ? j: j+1), the following applies

–	srcCrRowT[j]= (srcPicCrRow[k-1] + srcPicCrRow[k]+1)>>1.

–	srcCbRowT[j]= (srcPicCbRow[k-1] + srcPicCbRow[k]+1)>>1.

–	If Fracpos is equal to 1, 3, -1 or -3, k= (Fracpos>0 ? j-1: j+1), the following applies

–	srcCrRowT[j]= (srcPicCrRow[j]*u + srcPicCrRow[k]*v+2)>>2, wherein v = Abs(Fracpos), u= 4-v.

–	srcCbRowT[j]= (srcPicCbRow[j]*u + srcPicCbRow’[k]*v+2)>>2.

          - Row warping and hole-filling process

Inputs to this process is a row of reference luma samples srcPicYRowT0, another row of reference luma samples srcPicYRowT1 when reduced_resolution_flag is equal to 1, a row of reference cb samples srcPicCbRow and a row of reference cr samples srcPicCrRow, a row of depth samples depPicRow, and a warping direction WarpDir.

Outputs of this process is a row of target luma samples, vspPicYRow0, another row of target luma samples vspPicYRow1 when reduced_resolution_flag is equal to1, a row of target cb samples vspPicCbRow, and a row of target cr samples vspPicCrRow.

Set PixelStep as follows: PixelStep = WarpDir ? -1 : 1.

Set RowWidthT to be the width of the luma sample row.

Set RowWidthC to be the width of the chroma sample row.

The following steps are carried out in order.

- Set j = 0, prevL = 0, prevC =0, jDirL = (RowWidthT-1)*WarpDir, jDirC = (RowWidthC-1)*WarpDir

- Set curL = jDirL + disRow[jDirC] and curC = jDirC +(disRow[jDirC] >> reduced_resolution_flag).

- If curL is smaller than RowWidthT and curL is equal or larger than 0, do the following; otherwise, go to step 4.

–	Invoke pixel warping process 8.3.2.2.3.1 with srcPicRow equal to srcPicYRowT0, vspPicRow equal to vspPicYRow0, posSrc equal to jDirL and the posDst equal to curL as inputs.

–	If reduced_resolution_flag is equal to 1, invoke pixel warping process 8.3.2.2.3.1 with srcPicRow equal to srcPicYRowT1, vspPicRow equal to vspPicYRow1, posSrc equal to jDirL and the posDst equal to curL as inputs.

–	If (curL -prevL) is unequal to PixelStep, the following apply

–	Invoke 8.3.2.2.3.2 to fill holes with srcPicRow equal to srcPicYRowT0, vspPicRow equal to vspPicYRow0 and a position pair of (prevL+ PixelStep, curL - PixelStep) as inputs;

–	When reduced_resolution_flag is equal to 1, invoke 8.3.2.2.3.2 to fill holes with srcPicRow equal to srcPicYRowT1, vspPicRow equal to vspPicYRow1, and a position pair of (prevL+ PixelStep, curL - PixelStep) as inputs;

–	prevL = curL, jDirL = jDirL + (PixelStep<< reduced_resolution_flag).

- When curC is smaller than RowWidthC and curC is equal to or larger than 0, the following steps apply in order:

–	Invoke pixel warping process 8.3.2.2.3.1 with srcPicRow equal to srcPicCbRow, vspPicRow equal to vspPicCbRow, posSrc equal to jDirC, and posDst is equal to curC as inputs;

–	Invoke pixel warping process 8.3.2.2.3.1 with srcPicRow equal to srcPicCrRow, vspPicRow equal to vspPicCrRow, posSrc equal to jDirC, and posDst is equal to curC as inputs;

–	If (curC -preC) is unequal to PixelStep, the following apply

–	Invoke 8.3.2.2.3.2 to fill holes with vspPicRow equal to vspPicCbRow, and a position pair of (prevC + PixelStep, curC – PixelStep) as inputs;

–	Invoke 8.3.2.2.3.2 to fill holes with vspPicRow equal to vspPicCrRow, and a position pair of (prevC + PixelStep, curC – PixelStep) as inputs;

–	prevC = curC, jDirC = jDirC + PixelStep.

- The following steps apply in order:

–	j++.

–	If j is equal to RowWidth, go to step 6; otherwise, go to step 2.

- The following steps apply in order:

–	If prevL is unequal to WarpDir*(RowWidthT-1)), the following apply

–	Invoke 8.3.2.2.3.2 with vspPicRow equal to vspPicYRow0, and a position pair of (prevL+PixelStep, WarpDir*(RowWidth-1)) as inputs.

–	If reduced_resolution_flag is equal to 1, invoke 8.3.2.2.3.2 with vspPicRow equal to vspPicYRow1, and a position pair of (prevL+PixelStep, WarpDir*(RowWidth-1)) as inputs.

–	If prevC is unequal to WarpDir*(RowWidthC – 1), the following steps apply in order:

–	Invoke 8.3.2.2.3.2 to fill holes with vspPicRow equal to vspPicCbRow, and position pair of (prevC + PixelStep, WarpDir*(RowWidthC – 1)) as inputs.

–	Invoke 8.3.2.2.3.2 to fill holes with vspPicRow equal to vspPicCrRow, and position pair of (prevC + PixelStep, WarpDir*(RowWidthC – 1)) as inputs.

            - Pixel warping process

Inputs to this process include all the inputs

–	row of source samples, srcPicRow,

–	row of target samples, vspPicRow,

–	source pixel position, posSrc,

–	target pixel position, posDst.

Outputs of this process is modified sample rows of vspPicRow.

vspPicRow[posDst] is set equal to srcPicRow[posSrc].

            - Hole pixel filling process

Inputs to this process are

–	row of target samples, vspPicRow,

–	pixel position pair (p1, p2),

–	reference pixel position, posRef.

Outuput of the process is modified sample row of vspPicRow.

Set posLeft and posRight as follows:

–	posLeft = (p1 < p2 ? p1, p2);

–	posRight = (p1 < p2 ? p2, p1).

The posRef is derived as follows: [ed. Note (CY): check whether the handling of the corner case is still needed.]

–	If posLeft is equal to 0, posRef is set to posRight+1;

–	Otherwise, if posRight is equal to RowWidth-1, posRef is set to posLeft-1;

–	Otherwise, if tempDepRow[posLeft -1] is smaller than tempDepRow[posRight +1], posRef is set to posLeft -1;

–	Otherwise, posRef is set to posRight +1.

For each pos from posLeft to posRight, inclusive, set vspPicRow[pos] to be equal to vspPicRow[posRef].

      - Derivation process for inside view motion prediction

Inputs to this process are:

–	macroblock types, luma motion vectors and reference indices of one/four co-located macroblocks in the texture view component of the current macroblock with address CurrMbAddr in the depth view component.

Outputs of this process are variables for the current macroblock in the depth view component:

–	macroblock type mb_type and sub-macroblock type sub_mb_type[ mbPartIdx ] for each sub-macroblock (with mbPartIdx being the macroblock partition index for the corresponding sub-macroblock)

–	motion vectors, reference indices refIdxLX (X being 0 and 1) and prediction modes

The co-located macroblocks are identified by 8.3.3.1.

When reduced_resolution_flag is equal to 0, the macroblock type, reference indices, prediction modes and luma motion vectors are derived by 8.3.3.3; when reduced_resolution_flag is equal to 1, the macroblock type, reference indices, prediction modes and luma motion vectors are derived by 8.3.3.4.

        - Deriviation process for inside view co-located macroblocks

Input to this process is the address of one macroblock in the depth view component.

Output to this process is the address of one/four co-located macroblocks of the texture view component (depending on reduced_resolution_flag is 0/1) in the same access unit.

When reduced_resolution_flag is equal to 0, the address of the co-located macroblock in the texture view componenent is set to CurrMbAddr.

When reduced_resolution_flag is equal to 1, the following applies.

–	For a macoblock with address CurrMbAddr in the depth view component, the addresses ColMbAddr[ mbPartIdx ] (mbPartIdx  = 0..3) of its four co-located macroblocks in the texture view component are defined as follows:

mb_x = ( CurrMbAddr % PicWidthInMbs ) << 1

mb_y = ( CurrMbAddr / PicWidthInMbs ) <<1		(I-8-XX)

ColMbAddr[ mbPartIdx ] = ( mb_y + mbPartIdx / 2 ) * ( PicWidthInMbs << 1 ) + ( mb_x + mbPartIdx % 2 )

        - Motion vectors scaling process

Input to this process is a motion vector.

Output to this process is the modified motion vector after scaling.

If reduced_resolution_flag is equal to 1, this motion vector scaling process is invoked.

Denote a motion vector by MV[2] with MV[0] and MV[1] represent the horizontal value and the vertical value, respectively. The scaling process is specified by the following formulas:

MV[0] = (MV[0] + 1) >> 1;

MV[1] = (MV[1] + 1) >> 1.		(I-8-XX)

        - Derivation process for motion information from one inside view co-located macroblock

This process is invoked when reduced_resolution_flag is equal to 0.

The reference indices, luma motion vectors of the co-located macroblock denoted by ColMb are assigned to the current macroblock.

Denote the reference indices of each reference picture list X of one 8x8 partition mbPartIdx by refIdxLXCol[ mbPartIdx ] (X being 0..1).

–	If both of the following conditions are false, macroblock type and sub-macroblock types of ColMb are inherited by current macroblock.

–	Macroblock type of ColMb is B_8x8 and at least one of the sub-macroblock types is B_Direct_8x8,

–	Macroblock type of ColMb is P_Skip, B_skip or B_Direct_16x16.

–	Otherwise (at least one of the above conditions is true)

–	If the current slice is a P slice, mb_type is set to P_8x8 and sub_mb_type[ mbPartIdx ] is set to P_L0_8x8

–	Otherwise (the current slice is a B slice), mb_type is set to B_8x8, and

–	If both refIdxLXCol[ mbPartIdx ] (X being 0..1) are not equal to -1, sub_mb_type[ mbPartIdx ] is set to B_Bi_8x8;

–	Otherwise, sub_mb_type[ mbPartIdx ] is set to B_LX_8x8, when refIdxLXCol[ mbPartIdx ] is not equal to -1.

        - Derivation process for motion information from inside view co-located macroblocks

This process is invoked when reduced_resolution_flag is equal to 1.

mb_type is set to P_8x8 if the current slice is a P slice or B_8x8 if the current slice is a B slice.

Denote the macroblock with address ColMbAddr[ mbPartIdx ] by ColMb and its macroblock type by mbtypeCol.

–	If mbtypeCol is equal to P_Skip, P_L0_16x16, B_LX_16x16, B_Bi_16x16 or B_Skip or B_Direct_16x16 for B slices (X being 0 or 1), the following ordered steps are specified:

- Reference indices are inherited by the 8x8 partition mbPartIdx. Luma motion vectors of ColMb are inherited by the 8x8 partition mbPartIdx and scaled by the subclause 8.3.3.2.

- SubPredMode[ mbPartIdx ] is derived as follows:

–	If both of list 0 and list 1 reference indices of ColMb are not equal to -1, BiPred is assigned to SubPredMode[ mbPartIdx ].

–	Otherwise (list X reference index of ColMb is not equal to -1), Pred_LX is assigned to SubPredMode[ mbPartIdx ].

- sub_mb_type[ mbPartIdx ] is derived as follows:

–	If the prediction mode is Pred_LX, sub_mb_type[mbPartIdx] is set to P_L0_8x8 for P slices and B_LX_8x8 for B slices, respectively.

–	Otherwise, sub_mb_type[ mbPartIdx ] is set to B_Bi_8x8 for B slices.

–	Otherwise, let refIdxLXCol[ i ] (i being 0..3) denotes the reference index of four 8x8 partitions of ColMb. The index i of the top-left and bottom-right 8x8 partition is 0 and 3, respectively, as depicted in Figure 6-11. The following ordered steps are specified:

- Reference indices, luma motion vectors of the 8x8 partition with index mbPartIdx are derived as follows:

–	If both of refIdxLXCol[0] and refIdxLXCol[3] are -1, refIdxLX[mbPartIdx] is set to -1.

–	Otherwise, if refIdxLXCol[0] is equal to refIdxLXCol[3], refIdxLXCol[0] is assigned to refIdxLX[mbPartIdx]. The sub-macroblock partition size for 8x8 partition mbPartIdx is set to 8x4, or 4x8 correspoding to the partition size of ColMb equal to 16x8 or 8x16, respectively. The luma motion vectors of the two 8x8 partitions are inherited by the two 8x4 or 4x8 partitions of the 8x8 partiotion mbPartIdx and scaled by the subclause 8.3.3.2.

–	Otherwise, if refIdxLXCol[i] (i being 0 or 3) is unequal to -1 and refIdxLXCol[j] (j being 3 or 0) is equal to -1, refIdxLXCol[i] is assigned to refIdxLX[mbPartIdx] and the luma motion vectors of 8x8 partition with index i in ColMb are inherited by the 8x8 partiotion mbPartIdx and scaled by the subclause 8.3.3.2.

–	Otherwise, refIdxLXCol[i] is assigned to refIdxLX[mbPartIdx] with i equal to mbPartIdx and the luma motion vectors of the 8x8 partition with index i in ColMb are inherited by the 8x8 partition mbPartIdx and scaled by the subclause 8.3.3.2.

- Sub-macroblock partition size MxN is derived as follows:

–	If the sub-macroblock partition size is set to 8x4 or 4x8 in the above step, MxN is set to 8x4 or 4x8.

–	Otherwise, MxN is set to 8x8.

- SubPredMode[ mbPartIdx ] and sub_mb_type[ mbPartIdx ] of the 8x8 partition mbPartIdx are derived as follows:

–	If current slice is a P slice, Pred_L0 and P_L0_MxN are assigned to SubPredMode[ mbPartIdx ] and sub_mb_type[ mbPartIdx ], respectively.

–	Otherwise, if refIdxLX[mbPartIdx] (X being 0 or 1) is equal to -1 and refIdxLY[mbPartIdx] (Y being 1 or 0) is unequal to -1, Pred_LY and B_LY_MxN are assigned to SubPredMode[ mbPartIdx ] and sub_mb_type[ mbPartIdx ], respectively.

–	Otherwise, BiPred and B_Bi_MxN are assigned to SubPredMode[ mbPartIdx ] and sub_mb_type[ mbPartIdx ], respectively.

      - Derivation of prediction weights in depth-range-base weighted prediction

For (nal_unit_type is equal to 21 and depth_weighted_pred_flag is equal to 1 and (slice_type % 5) is equal to 0 or 3) or (depth_weighted_bipred_flag is equal to 1 and (slice_type % 5) is equal to 1), the derivation process for prediction weights in depth-range-based weighted prediction described in this subclause is invoked.

Inputs to this process are

– the reference indices refIdxL0 and refIdxL1,

– the prediction utilization flags predFlagL0 and predFlagL1.

Outputs of this process are variables for weighted prediction logWDC, w0C, o0C, w1C, o1C, where C being replaced by L (luma).

When predFlagL0 is equal to 1, the following is applied.

–	The derivation process for a single prediction weight in depth-range-based weighted prediction specified in subclause J.8.3.4.1 is invoked with zN1 equal to ZNear[ dps_id1, index ], zF1 equal to ZFar[ dps_id1, index ], zN2 equal to ZNear[ dps_id2, refIndex ], zF2 equal to ZFar[ dps_id2, refIndex ], and X equal to 0.

When predFlagL1 is equal to 1 and depth_weighted_bipred_flag is equal to 1, the following is applied.

–	The derivation process for a single prediction weight in depth-range-based weighted prediction specified in sub-clause J.8.3.4.1 is invoked with zN1 equal to ZNear[ dps_id1, index], zF1 equal to ZFar[ dps_id1, index  ], zN2 equal to ZNear[ dps_id2, refIndex ], zF2 equal to ZFar[ dps_id2, refIndex ], and X equal to 1.

The variables index, refIndex, dps_id1 and dps_id2 are derived as following:

index = ViewIdTo3DVAcquisitionParamIndex( view_id of the current view );
refIndex = ViewIdTo3DVAcquisitionParamIndex( view_id of the view of the reference picture);
dps_id1 is set equal to the dps_id of the current picture;

dps_id2 is set equal to the dps_id of the reference picture;

        - Derivation of weight and offset parameteres

–	The variable wXC is calculated through the following steps.

- scale_w is set to 8;

- Calculate variable W_factora as follows:

temp1 = ZF2 - ZN2; 
temp2 = ZF2;
x = ( temp1 + ( temp2 >> 1 ) ) / temp2;
sign = ( (temp1 - x * temp2) < 0 ) ? -1 : 1;
W_factora = (x<<scale_w);
W_factora += ( ((temp1 - x * temp2) << scale_w) + sign * (temp2 >> 1) ) / temp2;

- Calculate variable W_factorb as follows:

temp1 = ZF1;
temp2 = ZF1 – ZN1;
x = ( temp1 + ( temp2 >> 1 ) ) / temp2;
sign = ( (temp1 - x * temp2) < 0 ) ? -1 : 1;
W_factorb = (x<<scale_w);
W_factorb += ( ((temp1 - x * temp2) << scale_w) + sign * (temp2 >> 1) ) / temp2;

- Calculate variable W_factorc as follows:

temp1 = ZN1;
temp2 = ZN2;
x = ( temp1 + ( temp2 >> 1 ) ) / temp2;
sign = ( (temp1 - x * temp2) < 0 ) ? -1 : 1;
W_factorc = (x<<scale_w);
W_factorc += ( ((temp1 - x * temp2) << scale_w) + sign * (temp2 >> 1) ) / temp2;

- Calculate variable wX:

wX= (W_factora*W_factorb*W_factorc + (1<<(scale_w*3- logWD-1)) ) >> (scale_w*3- logWD);
wX=Clip(-127,128,wX);

–	The variable oX is calculated through the following derivation process.

- scale_o is set to 8

- Calculate variable O_factora as follows:

O_factora=( (ZN1<<(scale_o))+( ZF2>>1) ) / ZF2;

- Calculate variable O_factorb:

temp1 = ZF1- ZF2;
temp2 = ZF1- ZN1;
sign= (temp1 < 0) ? -1 : 1;
x = ( temp1 + sign * ( temp2 >> 1 ) ) / temp2;
sign = ( (temp1 - x * temp2) < 0 ) ? -1 : 1;
O_factorb = (x<<scale_o);
O_factorb += ( ((temp1 - x * temp2) << scale_o) + sign * (temp2 >> 1) ) / temp2;

- Calculate variable o0:

oX=( O_factora*O_factorb + (1<<(scale_o*2-8-1)) ) >> (scale_o*2-8);
oX =Clip(-127,128, oX);

    - Specification of bitstream subsets

The specifications of subclause I.8.5 apply.

    - Decoding process for depth range parameters

This process is invoked for decoding of the depth_ranges( ) syntax structure and, if present, the vsp_param( ) syntax structure in sequence parameter set 3DVC extension and for decoding of depth parameter set RBSP.

Inputs of this process are the variables obtained from parsing the active depth range parameter set RBSP:

- the variables relative to the closest depth: ZNearSign[dps_id, i ], ZNearExp[dps_id, i ], ZNearMantissa[dps_id, i ], ZNearManLen[dps_id, i ];

- the variables relative to the farthest depth: ZFarSign[dps_id, i ], ZFarExp[dps_id, i ], ZNearMantissa[dps_id, i ], ZNearManLen[dps_id, i ].

- the variables relative to the horizontal focal length: FocalLengthXSign[ dps_id, i ], FocalLengthXExponent[ dps_id, i ], FocalLengthXMantissa[ dps_id, i ], FocalLengthXManLen[ dps_id, i ];

- the variables relative to the vertical focal length: FocalLengthYSign[ dps_id, i ], FocalLengthYExponent[ dps_id, i ], FocalLengthYMantissa[ dps_id, i ], FocalLengthYManLen[ dps_id, i ];

- the variables relative to the horizontal principal point: PrincipalPointXSign[ dps_id, i ], PrincipalPointXExponent[ dps_id, i ], PrincipalPointXMantissa[ dps_id, i ], PrincipalPointXManLen[ dps_id, i ];

- the variables relative to the vertical principal point: PrincipalPointYSign[ dps_id, i ], PrincipalPointYExponent[ dps_id, i ], PrincipalPointYMantissa[ dps_id, i ], PrincipalPointYManLen[ dps_id, i ];

- the variables relative to the horizontal translation delta: TXSign[ dps_id, i ], TXExponent[ dps_id, i ], TXMantissa[ dps_id, i ], TXManLen[ dps_id, i ];

- the variables relative to the horizontal translation parameter: AbsTX[ dps_id, 0 ].

where i is the index to the order of views for 3DV acquisition parameters.

Outputs of this process are

–	the closest depth values ZNear[ dps_id, i ] and the respective variables respective variables ZNearSign[ dps_id, i ], ZNearExponent[ dps_id, i ], ZNearMantissa[ dps_id, i ], ZNearManLen[ dps_id, i ];

–	the farthest depth values ZFar[ dps_id, i ] and the respective variables respective variables ZFarSign[ dps_id, i ], ZFarExponent[ dps_id, i ], ZFarMantissa[ dps_id, i ], ZFarManLen[ dps_id, i ].

–	the horizontal focal length values FocalLengthX[ dps_id, i ] and the respective variables respective variables FocalLengthXSign[ dps_id, i ], FocalLengthXExponent[ dps_id, i ], FocalLengthXMantissa[ dps_id, i ], FocalLengthXManLen[ dps_id, i ];

–	the vertical focal length values FocalLengthY[ dps_id, i ] and the respective variables respective variables FocalLengthYSign[ dps_id, i ], FocalLengthYExponent[ dps_id, i ], FocalLengthYMantissa[ dps_id, i ], FocalLengthYManLen[ dps_id, i ];

–	the horizontal principal point values PrincipalPointX[ dps_id, i ] and the respective variables respective variables PrincipalPointXSign[ dps_id, i ], PrincipalPointXExponent[ dps_id, i ], PrincipalPointXMantissa[ dps_id, i ], PrincipalPointXManLen[ dps_id, i ];

–	the vertical principal point values PrincipalPointY[ dps_id, i ] and the respective variables respective variables PrincipalPointYSign[ dps_id, i ], PrincipalPointYExponent[ dps_id, i ], PrincipalPointYMantissa[ dps_id, i ], PrincipalPointYManLen[ dps_id, i ];

–	the horizontal translation delta values TX[ dps_id, i ] and the respective variables respective variables TXSign[ dps_id, i ], TXExponent[ dps_id, i ], TXMantissa[ dps_id, i ], TXManLen[ dps_id, i ];

–	the horizontal translation values AbsTX[ dps_id, i ] relative to AbsTX[ dps_id, 0 ]

Some of the views for which the 3DV acquisition information is specified may not be present in the coded video sequence.

The output variables x in Table I-X are derived as follows from the respective variables f, s, e, n, and v indicated in Table I-X.

–	If f is equal to 0 in depth range update RBSP, f in the active sequence parameter set 3DVC extension shall be equal to 1 and x[ dps_id, i ] = x[ SPS, i ], s[ dps_id, i ] = s[ 0, i ], e[ dps_id, i ] = e[ 0, i ], and n[ dps_id, i ] = n[ 0, i ] for all values of i and for “SPS” pointing to the active sequence parameter set. [Ed. Note (CY): if we have created a 0 depth parameter set, I believe this paragraph is not needed. In other words, shall we use this paragraph to create the depth parameter set with id equal to 0?]

–	Else if f is equal to 1, the variable x computed as follows for [ dps_id, i ] where i is index to the order of views for 3DV acquisition parameters:

–	If 0 < e < 127, x = (−1)s * 2e−31 * (1 + n ÷ 2v).

–	Otherwise (e is equal to 0), x = (−1)s * 2−(30+v) * n.

NOTE – The above specification is similar to that found in IEC 60559:1989, Binary floating-point arithmetic for microprocessor systems.

- Table I-X – Association between depth parameter variables and syntax elements.

| x | f | s | E | n | v |
| --- | --- | --- | --- | --- | --- |
| ZNear | z_near_flag | ZNearSign | ZNearExp | ZNearMantissa | ZNearManLen |
| ZFar | z_far_flag | ZFarSign | ZFarExp | ZFarMantissa | ZFarManLen |
| FocalLengthX | focal_length_flag | FocalLengthXSign | FocalLengthXExp | FocalLengthXMantissa | FocalLengthXManLen |
| FocalLengthY | focal_length_flag | FocalLengthYSign | FocalLengthYExp | FocalLengthYMantissa | FocalLengthYManLen |
| PrincipalPointX | principal_point_flag | PrincipalPointXSign | PrincipalPointXExp | PrincipalPointXMantissa | PrincipalPointXManLen |
| PrincipalPointY | principal_point_flag | PrincipalPointYSign | PrincipalPointYExp | PrincipalPointYMantissa | PrincipalPointYManLen |
| TX | translation_flag | TXSign | TXExp | TXMantissa | TXManLen |

The variable AbsTX is specified as follows:

AbsTX[ dps_id, 0 ] = 0
for( i = 1; i < num_views_minus1; i ++ )
	AbsTX[dps_id, i ] = AbsTX[dps_id,  i – 1 ] + TX[dps_id,  i – 1 ]

    - In-loop depth filtering process

The in-loop depth filtering process is invoked for every decoded depth view component prior to decoding the next depth view component in decoding order, when jvf_idc in the active sequence parameter set is greater than 0.

Input to this process is:

–	the decoded sample array of the current depth view component DP0.

Output of this process is:

–	the filtered sample array of the current depth view component DP0.

If no depth view component precedes, in decoding order, DP0 in the current access unit, the in-loop depth filtering process has no effect on the decoding process. Otherwise, the following applies.

Variable ndrForward[ i ] for i in the range of 0 to 255, inclusive, is specified as follows.

nonlinear_depth_representation_model[ 0 ] = 0
nonlinear_depth_representation_model[ nonlinear_depth_representation_num + 1 ] = 0
for( k=0; k<= nonlinear_depth_representation_num; ++k )
{
	pos1 = ( 255 * k ) / ( nonlinear_depth_representation_num + 1 )
	dev1 = nonlinear_depth_representation_model[ k ]
	pos2 = ( 255 * ( k+1 ) ) / ( nonlinear_depth_representation_num + 1 ) )
	dev2 = nonlinear_depth_representation_model[ k+1 ]

	x1 = pos1 + dev1
	y1 = pos1 - dev1
	x2 = pos2 + dev2
	y2 = pos2 - dev2

	for( x = max( x1, 0 ); x <= min( x2, 255 ); ++x )
		NdrForward[ x ] = Clip3( 0, 255, Round( ( ( x - x1 ) * ( y2 - y1 ) ) ÷ ( x2 - x1 ) + y1 ) )
}

The function ndrForwardLERP( value ) is specified to return variable retValue as specified below.

valueInt = Floor( value )
if( valueInt < 0 ) retValue = 0
else if( valueInt >= 255) retValue = 255
else {
	frc = value – valueInt
	retValue = ndrForward[ value_int ] * ( 1 – frc ) + ndrForward[ value_int + 1 ] * frc;  
}

Let numDepthPics be the number of depth view components preceding DP0 in decoding order in the current access unit. Let dPn, be a sample array of a decoded depth view component preceding DP0 in decoding order in the current access unit where n is in the range of 1 to numDepthPics, inclusive.

The function QuantizedDepth( uD, zN, zF ) is specified to return qD specified as follows:

qD = ( 255 * ( 1 ÷ uD – 1 ÷ zF ) ÷ ( 1 ÷ zN – 1 ÷ zF ) )

The following procedure is conducted for each value of n in the range of 1 to numDepthPics, inclusive, where srcViewId is set to the view_id of dPn:

idx = ViewIdTo3DVAcquisitionParamIndex( view_id )
srcIdx = ViewIdTo3DVAcquisitionParamIndex( srcViewId )
for( j = 0; j < PicHeightInSamplesL ; j++ ) {
	for( i = 0; i < PicWidthInSamplesL ; i++ ) {
		uD0[ i ] = UniformDepth( NdrInverse[ DP0[ i, j ] ], ZNear[ dps_id, idx ], ZFar[ dps_id, idx ]
		uDn[ i ] = UniformDepth( NdrInverse[ dPn[ i, j ] ], ZNear[ dps_id, srcIdx ], ZFar[ dps_id, srcIdx ]
	}
	for( i = 0; i < PicWidthInSamplesL ; i++ ) {
		x = Round( ( Disparity( NdrInverse[ dPn[ i, j ] ], ZNear[ dps_id, idx ], ZFar[ dps_id, idx ], 
			FocalLengthX[ dps_id, idx ], AbsTX[ idx ] – AbsTX[ srcIdx ] ) + 2 ) / 4 )
		if( x >= 0  &&  x < PicWidthInSamplesL  &&  Abs( uD0[ x ] – uDn[ i ] ) < jvf_threshold ) )
			uD0[ x ] = ( uD0[ x ] + uDn[ i ] ) ÷ 2
	}
	for( i = 0; i < PicWidthInSamplesL ; i++ )
		DP0[ i, j ] = Round( ndrForwardLERP ( QuantizedDepth( uD0[ i ], ZNear[ dps_id, idx ], ZFar[ dps_id, idx ] ) ) )
}

    - Depth intra prediction process

This subclause is invoked for a coded slice with DepthFlag equal to 1, when disp_flag is equal to 1 or psip_flag is equal to 1.

Subclause 8.3 is applied with the following changes.

–	If mb_disp_flag is equal to 1, subclause 8.7.1 is invoked

–	Modification process for intra 4x4 precition is specified in subclause 8.7.1 instead of subclause 8.3.1.

–	Modification process for intra 8x8 precition is specified in subclause 8.7.1 instead of subclause 8.3.2.

–	Modification process for intra 16x16 precition is specified in subclause 8.7.14 instead of subclause 8.3.3.

      - Depth intra skip prediction process

Input to this process is a (PicWidthInSamplesL)x(PicHeightInSamplesL) array cSL containing constructed luma samples prior to the deblocking filter process of neighbouring macroblocks

Outputs of this process are Intra prediction luma samples for the current macroblock predL[ x, y ].

The 32 neighbouring samples p[ x, y ] that are constructed luma samples prior to the deblocking filter process, with x = −1, y = 0..15 and with x = 0..15, y = −1, are derived as specified by the following ordered steps:

- The derivation process for neighbouring locations in subclause Error: Reference source not found is invoked for luma locations with ( x, y ) assigned to ( xN, yN ) as input and mbAddrN and ( xW, yW ) as output.

- Each sample p[ x, y ] with x = −1, y = 0..15 and with x = 0..15, y = −1 is derived as follows.

–	If mbAddrA or mbAddrB are not available, the value of the sample p[ x, y ] is 128:

–	Otherwise, the value of the sample p[ x, y ] is derived as specified by the following ordered steps:

- The location of the upper-left luma sample of the macroblock mbAddrN is derived by invoking the inverse macroblock scanning process in subclause Error: Reference source not found with mbAddrN as the input and the output is assigned to ( xM, yM ).

- Depending on the variable MbaffFrameFlag and the macroblock mbAddrN, the sample value p[ x, y ] is derived as follows.

–	If MbaffFrameFlag is equal to 1 and the macroblock mbAddrN is a field macroblock,

p[ x, y ] = cSL[ xM + xW, yM + 2 * yW ]	(8-1)

–	Otherwise (MbaffFrameFlag is equal to 0 or the macroblock mbAddrN is a frame macroblock),

p[ x, y ] = cSL[ xM + xW, yM + yW ]	(8-2)

The depth intra skip prediction estimates the Intra 16x16PredMode from the upper-left neighboring luma samples.

Set left_line[k], up_line[k], left_i16mode, up_i16mode, left_change, and up_change as follows:

–	left_line[k] is equal to p[-1,y], with y=0…15, and k equal to y.

–	up_line[k] is equal to p[x,-1], with x=0…15, and k equal to x.

–	left_i16mode is equal to Intra 16x16PredMode of the left macroblock if the left macroblock is coded as I16x16 mode or DISP_mode. Otherwise, it is equal to 4.

–	up_i16mode is equal to Intra 16x16PredMode of the upper macroblock if the upper macroblock is coded as I16x16 mode or DISP_mode. Otherwise, it is equal to 4.

–	left_change is set to 0, for each k from 0 to 15 inclusive, if left_line[k] is not equal to left_line[k+1], left_change is increased by 1.

–	up_change is set to 0, for each k from 0 to 15, inclusive, if up_line[k] is not equal to up_line[k+1], up_change is increased by 1.

The Intra 16x16PredMode is derived as follows: [Ed. Note (CY): editorial improvements/corrections related to indentations are needed.]

–	If mbAddrA and mbAddrB are available, the following applies.

–	If left_change is equal to 0 and up_change is equal to 0, the following applies.

–	If up_i16mode is equal to 0 and left_i16mode is equal to 0, the variable Intra 16x16PredMode is set equal to 0.

–	Otherwise, if up_i16mode is equal to 1 and left_i16modeis  equal to 1, the variable Intra 16x16PredMode is set equal to 1.

–	Otherwise, if left_line[0] is less than up_line[0], the variable Intra 16x16PredMode is set equal to 0.

–	Otherwise, if up_line[0] is less than left_line[0], the variable Intra 16x16PredMode is set equal to 1.

–	Otherwise, the variable Intra 16x16PredMode is set equal to 2.

–	Otherwise, if left_change is equal to 0 and up_change is larger than 0, the variable Intra 16x16PredMode is set equal to 0.

–	Otherwise, if left_change is larger than 0 and up_change is equal to 0, the variable Intra 16x16PredMode is set equal to 1.

–	Otherwise, the following applies

–	If up_i16mode is equal to 0 and left_i16mode is equal to 0, the variable Intra 16x16PredMode is set equal to 0.

–	Otherwise, if up_i16mode is equal to 1 and left_i16mode is equal to 1, the variable Intra 16x16PredMode is set equal to 1.

–	Otherwise, if up_i16mode is equal to 0 and left_i16mode is not equal to 1, the variable Intra 16x16PredMode is set equal to 0.

–	Otherwise, if left_i16mode is equal to 1 and up_i16mode is not equal to 0, the variable Intra 16x16PredMode is set equal to 1.

–	Otherwise, if left_change is less than up_change, the variable Intra 16x16PredMode is set equal to 0.

–	Otherwise, if up_change is less than left_change, the variable Intra 16x16PredMode is set equal to 1.

–	Otherwise, if abs(left_line[0] – left_line[15]) is less than abs(up_line[0] – up_line[15]), the variable Intra 16x16PredMode is set equal to 0.

–	Otherwise, if abs(up_line[0] –up_line[15]) is less than abs(left_line[0] –left_line[15]), the variable Intra 16x16PredMode is set equal to 1.

–	Otherwise, the variable Intra 16x16PredMode is set equal to 2.

–	Otherwise, if mbAddrB is availabe, the variable Intra 16x16PredMode is set equal to 0.

–	Otherwise, if mbAddrA is availabe, the variable Intra 16x16PredMode is set equal to 1.

–	Otherwise, the variable Intra 16x16PredMode is set equal to 2.

Let predL[ x, y ] with x, y = 0..15 denote the prediction samples for the 16x16 luma block samples. Depending on Intra16x16PredMode, one of the Intra_16x16 prediction modes specified in subclauses Error: Reference source not found to Error: Reference source not found is invoked.

When mb_disp_flag is equal to 1, CodedBlockPatternLuma and all values of LumaLevel, ChromaDCLevel, ChromaACLevel are set equal to 0 for the current macroblock.

Subclause 8.7.2.1 is applied with the following changes. [Ed. Consider the best location of this section that modifies 8.7.2.1.]

      - Intra_4x4 prediction process for luma samples

The specification of subclause 8.3.1 apply

        - Derivation process for Intra4x4PredMode

The specification of subclause 8.3.1 apply with the following changes:

–	The Table 8-2 in subclause 8.3.1 is replaced by

- Table ‎8-1 – Specification of Intra4x4PredMode[ luma4x4BlkIdx ] and associated names

| Intra4x4PredMode[ luma4x4BlkIdx ] | Name of Intra4x4PredMode[ luma4x4BlkIdx ] |
| --- | --- |
| 0 | Intra_4x4_Vertical (prediction mode) |
| 1 | Intra_4x4_Horizontal (prediction mode) |
| 2 | Intra_4x4_DC_PSIP (prediction mode) |
| 3 | Intra_4x4_Diagonal_Down_Left (prediction mode) |
| 4 | Intra_4x4_Diagonal_Down_Right (prediction mode) |
| 5 | Intra_4x4_Vertical_Right (prediction mode) |
| 6 | Intra_4x4_Horizontal_Down (prediction mode) |
| 7 | Intra_4x4_Vertical_Left (prediction mode) |
| 8 | Intra_4x4_Horizontal_Up (prediction mode) |

–	The following term ‘Intra_4x4_DC’ is replaced by ‘Intra_4x4_DC_PSIP’.

The specification of subclause 8.3.1.1 is applied with the following changes.

–	The following paragraphs in 4th procedure of derivation of Intra4x4PredMode[luma4x4BliIdx] in subclause 8.3.1.1:

–	predIntra4x4PredMode = Min( intraMxMPredModeA, intraMxMPredModeB )

is replaced by

if(psip_flag)

predIntra4x4PredMode = Intra_4x4_DC_PSIP_MODE;

else

predIntra4x4PredMode = Min( intraMxMPredModeA, intraMxMPredModeB )

        - Derivation process for RepVal4x4 and PredictionMap4x4

Processes described in this subclause are invoked when Intra4x4PredMode is 2, Psip4x4Available is equal to 1, and nal_unit_type is equal to 21.

Inputs to this process are

–	the index of a 4x4 luma block luma4x4BlkIdx,

–	an (PicWidthInSamplesL)x(PicHeightInSamplesL) array cSL containing constructed luma samples prior to the deblocking filter process of neighbouring macroblocks.

Output of this process are the variable RepVal4x4[luma4x4BlkIdx][i], with i=0,1, and PredictionMap4x4[luma4x4BlkIdx][x,y], with x, y=0..3.

For the derivation of the RepVal4x4[luma4x4BlkIdx], the following ordered steps are specified:

- Let p0[x] = p[x,-1], with x=0..3, and p0[y+4] = p[-1,y], with y=0..3.

- Let Th = ( Min(p0[0], .. , p0[7] ) + Max(p0[0], .., p0[7]) ) >> 1.

- Set sum1 = sum2 = n1 = n2 = 0.

- For each n from 0 through 7 inclusive, sum1 = sum1 + p0[n] and n1 = n1 + 1, if p0[n] <= Th.

- For each n from 0 through 7 inclusive, sum2 = sum2 + p0[n] and n2 = n2 + 1, if p0[n] > Th.

- Set avg1 = sum1 ÷ n1, avg2 = sum2 ÷ n2.

- Set RepVal4x4[luma4x4BlkIdx][0] = avg1, RepVal4x4[luma4x4BlkIdx][1] = avg2.

- Reset Th = ( avg1 + avg2 ) >> 1.

For the derivation of the PredictionMap4x4[luma4x4BlkIdx], the following ordered steps are specified.

–	If psip4x4_dir[luma4x4BlkIdx] is equal to 0,

- If p[x,−1] <= Th, set PredictionMap4x4[luma4x4BlkIdx][x,-1] = 0, with x=0..3. Otherwise, set PredictionMap4x4[luma4x4BlkIdx][x,-1] = 1.

- Convert PredictionMap4x4[luma4x4BlkIdx][x,-1], with x=0…3. to PredictionCode[-1] by Table 8-3.

- For each n from 0 through 3 inclusive, PredictionCode[n] = (PredictionCode[n-1] + psip4x4_code_diff[luma4x4BlkIdx][n]+8)%8

- For each n from 0 through 3 inclusive, convert PredictionCode[n] to PredictionMap4x4[luma4x4BlkIdx][x,n] with x=0..3 by Table 8-3.

–	If psip4x4_dir [luma4x4BlkIdx] is equal to 1,

- If p[-1,y] <= Th, set PredictionMap4x4[luma4x4BlkIdx][-1,y] = 0, with y=0..3. Otherwise, set PredictionMap4x4[luma4x4BlkIdx][-1,y] = 1.

- Convert PredictionMap4x4[luma4x4BlkIdx][-1,y], with y=0…3. to PredictionCode[-1] by Table 8-3.

- For each n from 0 to 3 inclusive, PredictionCode[n] = ( PredictionCode[n-1] + psip4x4_code_diff[luma4x4BlkIdx][n]+8) % 8

- For each n from 0 to 3 inclusive, , convert PredictionCode[n] to PredictionMap4x4[luma4x4BlkIdx][n,y] with y=0..3 by Table 8-3.

- Table ‎8-3 – Specification of PredictionCode and PredictionMap

| PredictionCode | PredictionMap4x4[luma4x4BlkIdx][x,A] with x=0…3 PredictionMap4x4[luma4x4BlkIdx][B,y] with y=0…3 |
| --- | --- |
| 0 | 0 0 0 0 |
| 1 | 0 0 0 1 |
| 2 | 0 0 1 1 |
| 3 | 0 1 1 1 |
| 4 | 1 1 1 1 |
| 5 | 1 1 1 0 |
| 6 | 1 1 0 0 |
| 7 | 1 0 0 0 |

        - Intra_4x4 sample prediction

The specification of subclause 8.3.1.2 apply.

          - Intra_4x4 sample prediction

The specification of subclause 8.3.1.2.1 apply.

          - Specification of Intra_4x4_Horizontal prediction mode

The specification of subclause 8.3.1.2.2 apply.

          - Specification of Intra_4x4_DC_PSIP prediction mode

The specifications of subclause 8.3.1.2.3 apply with the following changes.

–	The following paragraphs in subclause 8.3.1.2.3:

–	If all samples p[ x, −1 ], with x = 0..3, and p[ −1, y ], with y = 0..3, are marked as "available for Intra_4x4 prediction", the values of the prediction samples pred4x4L[ x, y ], with x, y = 0..3, are derived by

pred4x4L[ x, y ] = ( p[ 0, −1 ] + p[ 1, −1 ] + p[ 2, −1 ] + p[ 3, −1 ] +

p[ −1, 0 ] + p[ −1, 1 ] + p[ −1, 2 ] + p[ −1, 3 ] + 4 ) >> 3  					(8-48)

is replaced by

–	If all samples p[ x, −1 ], with x = 0..3, and p[ −1, y ], with y = 0..3, are marked as "available for Intra_4x4 prediction", inputs of this process is RepVal[luma4x4BlkIdx] [n], with n=0,1, and PredictionMap4x4[luma4x4BlkIdx][x,y] with x, y=0..3, and the values of the prediction samples pred4x4L[ x, y ], with x, y = 0..3, are derived by

pred4x4L[ x, y ] = RepVal4x4[luma4x4BlkIdx][ PredictionMap4x4[luma4x4BlkIdx][ x, y] ] 		(8-48)

          - Specification of Intra_4x4_Diagonal_Down_Left prediction mode

The specification of subclause 8.3.1.2.4 apply.

          - Specification of Intra_4x4_Diagonal_Down_Right prediction mode

The specification of subclause 8.3.1.2.5 apply.

          - Specification of Intra_4x4_Vertical_Right prediction mode

The specification of subclause 8.3.1.2.6 apply.

          - Specification of Intra_4x4_Horizontal_Down prediction mode

The specification of subclause 8.3.1.2.7 apply.

          - Specification of Intra_4x4_Vertical_Left prediction mode

The specification of subclause 8.3.1.2.8 apply.

          - Specification of Intra_4x4_Horizontal_Up prediction mode

The specification of subclause 8.3.1.2.9 apply.

      - Intra_8x8 prediction process for luma samples

The specification of subclause 8.3.2 apply

        - Derivation process for Intra8x8PredMode

The specification of subclause 8.3.1 apply with the following changes :

–	The Table 8-3 in subclause 8.3.1 is replaced by

- Table ‎8-4 – Specification of Intra8x8PredMode[ luma8x8BlkIdx ] and associated names

| Intra8x8PredMode[ luma8x8BlkIdx ] | Name of Intra8x8PredMode[ luma8x8BlkIdx ] |
| --- | --- |
| 0 | Intra_8x8_Vertical (prediction mode) |
| 1 | Intra_8x8_Horizontal (prediction mode) |
| 2 | Intra_8x8_DC_PSIP (prediction mode) |
| 3 | Intra_8x8_Diagonal_Down_Left (prediction mode) |
| 4 | Intra_8x8_Diagonal_Down_Right (prediction mode) |
| 5 | Intra_8x8_Vertical_Right (prediction mode) |
| 6 | Intra_8x8_Horizontal_Down (prediction mode) |
| 7 | Intra_8x8_Vertical_Left (prediction mode) |
| 8 | Intra_8x8_Horizontal_Up (prediction mode) |

–	The following term ‘Intra_8x8_DC’ is replaced by ‘Intra_8x8_DC_PSIP’.

The specification of subclause 8.3.2.1 is applied with the following changes.

–	The following paragraphs in 4th procedure of derivation of Intra8x8PredMode[luma8x8BliIdx] in subclause 8.3.2.1:

–	predIntra8x8PredMode = Min( intraMxMPredModeA, intraMxMPredModeB )

is replaced by

if(psip_flag)

predIntra8x8PredMode = Intra_8x8_DC_PSIP_MODE;

else

predIntra8x8PredMode = Min( intraMxMPredModeA, intraMxMPredModeB )

        - Derivation process for RepVal8x8 and PredictionMap8x8

Processes described in this subclause are invoked when Intra8x8PredMode is 2, Psip8x8Available is equal to 1, and nal_unit_type is equal to 21.

Inputs to this process are

–	the index of a 8x8 luma block luma8x8BlkIdx,

–	an (PicWidthInSamplesL)x(PicHeightInSamplesL) array cSL containing constructed luma samples prior to the deblocking filter process of neighbouring macroblocks.

Output of this process are the variable RepVal8x8[luma8x8BlkIdx][i], with i=0,1, and PredictionMap8x8[luma8x8BlkIdx][x,y], with x, y=0..7.

For the derivation of the RepVal8x8[luma8x8BlkIdx], the following ordered steps are specified:

- Let p0[x] = p[x,-1], with x=0..7, and p0[y+8] = p[-1,y], with y=0..7.

- Let Th = ( Min(p0[0], .. , p0[15] ) + Max(p0[0], .., p0[15]) ) >> 1.

- Set sum1 = sum2 = n1 = n2 = 0.

- For each n from 0 through 15 inclusive, sum1 = sum1 + p0[n] and n1 = n1 + 1, if p0[n] <= Th.

- For each n from 0 through 15 inclusive, sum2 = sum2 + p0[n] and n2 = n2 + 1, if p0[n] > Th.

- Set avg1 = sum1 ÷ n1, avg2 = sum2 ÷ n2.

- Set RepVal8x8 [luma8x8BlkIdx][0] = avg1, RepVal8x8[luma8x8BlkIdx][1] = avg2.

- Reset Th = ( avg1 + avg2 ) >> 1.

For the derivation of the PredictionMap8x8[luma8x8BlkIdx], the following ordered steps are specified.

–	If psip8x8_dir[luma8x8BlkIdx] is equal to 0,

- If p[x,−1] <= Th, set PredictionMap8x8[luma8x8BlkIdx][x,-1] = 0, with x=0..7. Otherwise, set PredictionMap8x8[luma8x8BlkIdx][x,-1] = 1.

- Convert PredictionMap8x8[luma8x8BlkIdx][x,-1], with x=0..7. to PredictionCode[-1] by Table 8-5.

- For each n from 0 through 7 inclusive, PredictionCode[n] = ( PredictionCode[n-1] + psip8x8_code_diff[luma8x8BlkIdx][n]+16)%16

- For each n from 0 through 7 inclusive, convert PredictionCode[n] to PredictionMap8x8[luma8x8BlkIdx][x,n] with x=0..7 by Table 8-5.

–	If psip8x8_dir [luma8x8BlkIdx] is equal to 1,

- If p[-1,y] <= Th, set PredictionMap8x8[luma8x8BlkIdx][-1,y] = 0, with y=0..7. Otherwise, set PredictionMap8x8[luma8x8BlkIdx][-1,y] = 1.

- Convert PredictionMap8x8[luma8x8BlkIdx][-1,y], with y=0…7. to PredictionCode[-1] by Table 8-5.

- For each n from 0 through 7 inclusive , PredictionCode[n] = ( PredictionCode[n-1] + psip8x8_code_diff[luma8x8BlkIdx][n]+16)%16

- For each n from 0 through 7 inclusive, convert PredictionCode[n] to PredictionMap8x8[luma8x8BlkIdx][n,y] with y=0..7 by Table 8-5.

- Table ‎8-5 – Specification of PredictionCode and PredictionMap

| PredictionCode | PredictionMap8x8[luma8x8BlkIdx][x,A] with x=0..7 PredictionMap8x8[luma8x8BlkIdx][B,y] with y=0..7 |
| --- | --- |
| 0 | 0 0 0 0 0 0 0 0 |
| 1 | 0 0 0 0 0 0 0 1 |
| 2 | 0 0 0 0 0 0 1 1 |
| 3 | 0 0 0 0 0 1 1 1 |
| 4 | 0 0 0 0 1 1 1 1 |
| 5 | 0 0 0 1 1 1 1 1 |
| 6 | 0 0 1 1 1 1 1 1 |
| 7 | 0 1 1 1 1 1 1 1 |
| 8 | 1 1 1 1 1 1 1 1 |
| 9 | 1 1 1 1 1 1 1 0 |
| 10 | 1 1 1 1 1 1 0 0 |
| 11 | 1 1 1 1 1 0 0 0 |
| 12 | 1 1 1 1 0 0 0 0 |
| 13 | 1 1 1 0 0 0 0 0 |
| 14 | 1 1 0 0 0 0 0 0 |
| 15 | 1 0 0 0 0 0 0 0 |

        - Intra_8x8 sample prediction

The specification of subclause 8.3.2.2 apply.

          - Reference sample filtering process for Intra_8x8 sample prediction

The specification of subclause 8.3.2.2.1 apply.

          - Specification of Intra_8x8_Vertical prediction mode

The specification of subclause 8.3.2.2.2 apply.

          - Specification of Intra_8x8_ Horizontal prediction mode

The specification of subclause 8.3.2.2.3 apply.

          - Specification of Intra_8x8_ DC_PSIP prediction mode

The specifications of subclause 8.3.2.2.3 apply with the following changes.

–	The following paragraphs in subclause 8.3.2.2.3:

–	If all samples p[ x, −1 ], with x = 0..7, and p[ −1, y ], with y = 0..7, are marked as "available for Intra_8x8

prediction", the values of the prediction samples pred8x8L[ x, y ], with x, y = 0..7, are derived by

pred8x8L[ x, y ]=(x'=07p'[x',−1]+y'=07p'[−1,y']+8)>>4	  					(‎8-91)

is replaced by

–	If all samples p[ x, −1 ], with x = 0..7, and p[ −1, y ], with y = 0..7, are marked as "available for Intra_8x8 prediction", inputs of this process is RepVal8x8[luma8x8BlkIdx] [n], with n=0,1, and predictionMap8x8[luma8x8BlkIdx][x,y] with x, y=0..7, and the values of the prediction samples pred8x8L[ x, y ], with x, y = 0..7, are derived by

pred8x8L[ x, y ] = RepVal8x8[ luma8x8BlkIdx ][ PredictionMap8x8[luma8x8BlkIdx][ x, y] ] 		(8-91)

          - Specification of Intra_8x8_ Diagonal_Down_Left prediction mode

The specification of subclause 8.3.2.2.5 apply.

          - Specification of Intra_8x8_ Diagonal_Down_Right prediction mode

The specification of subclause 8.3.2.2.6 apply.

          - Specification of Intra_8x8_ Vertical_Right prediction mode

The specification of subclause 8.3.2.2.7 apply.

          - Specification of Intra_8x8_ Horizontal_Down prediction mode

The specification of subclause 8.3.2.2.8 apply.

          - Specification of Intra_8x8_ Vertical_Left prediction mode

The specification of subclause 8.3.2.2.9 apply.

          - Specification of Intra_8x8_ Horizontal_Up prediction mode

The specification of subclause 8.3.2.2.10 apply.

      - Intra_16x16 prediction process for luma samples

The specification of subclause 8.3.3 apply with the following changes:

–	The Table 8-4 in subclause 8.3.3 is replaced by

- Table ‎8-6 – Specification of Intra16x16PredMode and associated names

| Intra16x16PredMode | Name of Intra16x16PredMode |
| --- | --- |
| 0 | Intra_16x16_Vertical (prediction mode) |
| 1 | Intra_16x16_Horizontal (prediction mode) |
| 2 | Intra_16x16_DC_PSIP (prediction mode) |
| 3 | Intra_16x16_Plane (prediction mode) |

        - Derivation process for RepVal16x16 and PredictionMap16x16

Processes described in this subclause are invoked when Intra16x16PredMode is 2, Psip16x16Available is equal to 1, and nal_unit_type is equal to 21.

Input to this process is a (PicWidthInSamplesL)x(PicHeightInSamplesL) array cSL containing constructed luma samples prior to the deblocking filter process of neighbouring macroblocks.

Output of this process are the variable RepVal16x16[i], with i=0,1, and PredictionMap16x16[x,y], with x, y=0..15.

For the derivation of the RepVal16x16, the following ordered steps are specified:

- Let p0[x] = p[x,-1], with x=0..15, and p0[y+16] = p[-1,y], with y=0..15.

- Let Th = ( Min(p0[0], .. , p0[31] ) + Max(p0[0], .., p0[31]) ) >> 1.

- Set sum1 = sum2 = n1 = n2 = 0.

- For each n from 0 through 31 inclusive, sum1 = sum1 + p0[n] and n1 = n1 + 1, if p0[n] <= Th.

- For each n from 0 through 31 inclusive, sum2 = sum2 + p0[n] and n2 = n2 + 1, if p0[n] > Th.

- Set avg1 = sum1 ÷ n1, avg2 = sum2 ÷ n2.

- Set RepVal16x16 [0] = avg1, RepVal16x16 [1] = avg2.

- Reset Th = ( avg1 + avg2 ) >> 1.

For the derivation of the PredictionMap16x16, the following ordered steps are specified.

–	If psip16x16_dir is equal to 0,

- If p[x,−1] <= Th, set PredictionMap16x16[x,-1] = 0, with x=0..15. Otherwise, set PredictionMap16x16[x,-1] = 1.

- Convert PredictionMap16x16[x,-1], with x=0…15, to PredictionCode[-1] by Table 8-7.

- For each n from 0 to 15 inclusive, PredictionCode[n] = ( PredictionCode[n-1] + psip16x16_code_diff[n]+32)%32

- For each n from 0 to 15 inclusive, convert PredictionCode[n] to PredictionMap16x16 [x,n] with x=0..15 by Table 8-7.

–	If psip16x16_dir is equal to 1,

- If p[-1,y] <= Th, set PredictionMap16x16[-1,y] = 0, with y=0..15. Otherwise, set PredictionMap16x16[-1,y] = 1.

- Convert PredictionMap16x16[-1,y], with y=0..15. to PredictionCode[-1] by Table 8-7.

- For each n from 0 to 15 inclusive, PredictionCode[n] = ( PredictionCode[n-1] + psip16x16_code_diff[n]+32) %32

- For each n from 0 to 15 inclusive, convert PredictionCode[n] to PredictionMap16x16n,y] with y=0..15 by Table 8-7.

- Table ‎8-7 – Specification of PredictionCode and PredictionMap

| PredictionCode | PredictionMap16x16[x,A] with x=0..15 PredictionMap16x16[B,y] with y=0..15 | PredictionCode | PredictionMap16x16[x,A] with x=0..15 PredictionMap16x16[B,y] with y=0..15 |
| --- | --- | --- | --- |
| 0 | 0 0 0 0 0 0 0 0 0 0 0 0 0 0 0 0 | 16 | 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 |
| 1 | 0 0 0 0 0 0 0 0 0 0 0 0 0 0 0 1 | 17 | 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 0 |
| 2 | 0 0 0 0 0 0 0 0 0 0 0 0 0 0 1 1 | 18 | 1 1 1 1 1 1 1 1 1 1 1 1 1 1 0 0 |
| 3 | 0 0 0 0 0 0 0 0 0 0 0 0 0 1 1 1 | 19 | 1 1 1 1 1 1 1 1 1 1 1 1 1 0 0 0 |
| 4 | 0 0 0 0 0 0 0 0 0 0 0 0 1 1 1 1 | 20 | 1 1 1 1 1 1 1 1 1 1 1 1 0 0 0 0 |
| 5 | 0 0 0 0 0 0 0 0 0 0 0 1 1 1 1 1 | 21 | 1 1 1 1 1 1 1 1 1 1 1 0 0 0 0 0 |
| 6 | 0 0 0 0 0 0 0 0 0 0 1 1 1 1 1 1 | 22 | 1 1 1 1 1 1 1 1 1 1 0 0 0 0 0 0 |
| 7 | 0 0 0 0 0 0 0 0 0 1 1 1 1 1 1 1 | 23 | 1 1 1 1 1 1 1 1 1 0 0 0 0 0 0 0 |
| 8 | 0 0 0 0 0 0 0 0 1 1 1 1 1 1 1 1 | 24 | 1 1 1 1 1 1 1 1 0 0 0 0 0 0 0 0 |
| 9 | 0 0 0 0 0 0 0 1 1 1 1 1 1 1 1 1 | 25 | 1 1 1 1 1 1 1 0 0 0 0 0 0 0 0 0 |
| 10 | 0 0 0 0 0 0 1 1 1 1 1 1 1 1 1 1 | 26 | 1 1 1 1 1 1 0 0 0 0 0 0 0 0 0 0 |
| 11 | 0 0 0 0 0 1 1 1 1 1 1 1 1 1 1 1 | 27 | 1 1 1 1 1 0 0 0 0 0 0 0 0 0 0 0 |
| 12 | 0 0 0 0 1 1 1 1 1 1 1 1 1 1 1 1 | 28 | 1 1 1 1 0 0 0 0 0 0 0 0 0 0 0 0 |
| 13 | 0 0 0 1 1 1 1 1 1 1 1 1 1 1 1 1 | 29 | 1 1 1 0 0 0 0 0 0 0 0 0 0 0 0 0 |
| 14 | 0 0 1 1 1 1 1 1 1 1 1 1 1 1 1 1 | 30 | 1 1 0 0 0 0 0 0 0 0 0 0 0 0 0 0 |
| 15 | 0 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 | 31 | 1 0 0 0 0 0 0 0 0 0 0 0 0 0 0 0 |

        - Specification of Intra_16x16_Vertical prediction mode

The specification of subclause 8.3.3.1 apply.

        - Specification of Intra_16x16_Horizontal prediction mode

The specification of subclause 8.3.3.2 apply.

        - Specification of Intra_16x16_DC_PSIP prediction mode

The specifications of subclause 8.3.3.3 apply with the following changes.

–	The following paragraphs in subclause 8.3.3.3:

–	If all samples p[ x, −1 ], with x = 0..15, and p[ −1, y ], with y = 0..15, are marked as "available for Intra_16x16

prediction", the prediction for all luma samples in the macroblock is given by

predL[ x, y ] = (x'=015px',−1+y'=015p−1,y'+16)>>5, with x, y = 0..15	(‎8-118)

is replaced by

–	If all samples p[ x, −1 ], with x = 0..15, and p[ −1, y ], with y = 0..15, are marked as "available for Intra_16x16

prediction",, inputs of this process is RepVal16x16 [n], with n=0,1, and PredictionMap16x16[x,y] with x, y=0..15, and the prediction for all luma samples in the macroblock is given by

predL[ x, y ] = RepVal16x16[ PredictionMap16x16[ x, y] ] 								(8-118)

        - Specification of Intra_16x16_Plane prediction mode

The specification of subclause 8.3.3.4 apply.

      - Intra prediction process for chroma samples

The specification of subclause 8.3.4 apply.

      - Sample construction process for I_PCM macroblocks

    - The specification of subclause 8.3.5 apply.Deblocking filter process

The specifications in 8.7 apply.

      - Filtering process for block edges

The specifications in 8.7.1 apply.

      - Filtering process for a set of samples across a horizontal or vertical block edge

The specifications in 8.7.2 apply.

        - Derivation process for the luma content dependent boundary filtering strength

The specifications in 8.7.2.1 apply with the following changes:

–	The following paragraph is added after the paragraph starting with “Output of this process is the variable bS”:

The variable bS is derived as follows if psip_flag is equal to 1.

–	If psip_flag is equal to 1, both of the samples p0 and q0 is in a macroblock coded using a I16x16 Mode and both of the samples p0 and q0 is in a macroblock coded using an Intra_16x16_DC_PSIP_MODE, a value of bS equal to 0 is the output:

–	Otherwise, if psip_flag is equal to 1, both of the samples p0 and q0 is in a macroblock coded as I8x8 Mode and both of the samples p0 and q0 is in a macroblock coded using an Intra_8x8_DC_PSIP_MODE, a value of bS equal to 0 is the output:

–	Otherwise, if psip_flag is equal to 1, both of the samples p0 and q0 is in a macroblock coded as I4x4 Mode and both of the samples p0 and q0 is in a macroblock coded using an Intra_4x4_DC_PSIP_MODE, a value of bS equal to 0 is the output:

–	Otherwise, if psip_flag is equal to 1, either the samples p0 or q0 is in a macroblock coded as I16x16 Mode with Intra_16x16_DC_PSIP_MODE, the following applies

–	If the block edge is also a macroblock edge, a value of bS equal to 1 is the output:

–	Otherwise, a value of bS equal to 0 is the output:

–	Otherwise, if psip_flag is equal to 1, either the samples p0 or q0 is in a macroblock coded as I8x8 Mode with Intra_8x8_DC_PSIP_MODE, intra prediction mode, the following applies

–	If the block edge is also a macroblock edge, a value of bS equal to 1 is the output:

–	Otherwise, a value of bS equal to 0 is the output:

–	Otherwise, if psip_flag is equal to 1, either the samples p0 or q0 is in a macroblock coded as I4x4 Mode with Intra_4x4_DC_PSIP_MODE intra prediction mode, the following applies

–	If the block edge is also a macroblock edge, a value of bS equal to 1 is the output:

–	Otherwise, a value of bS equal to 0 is the output:

The variable bS is derived as follows if disp_flag is equal to 1. mb_type is DISPMODE if mb_disp_flag is equal to 1.

–	If verticalEdgeFlag is euqal to 1, both of the samples p0 and q0 is in a macroblock coded using a DISPMODE and both of the samples p0 and q0 is in a macroblock coded using an Intra_16x16_Horizontal intra prediction mode, a value of bS equal to 0 is the output:

–	Otherwise, if verticalEdgeFlag is euqal to 0, both of the samples p0 and q0 is in a macroblock coded using a DISPMODE and both of the samples p0 and q0 is in a macroblock coded using an Intra_16x16_Vertical intra prediction mode, a value of bS equal to 0 is the output:

–	Otherwise, if both of the samples p0 and q0 is in a macroblock coded using a DISPMODE, the following applies

–	If the block edge is also a macroblock edge, a value of bS equal to 2 is the output:

–	Otherwise, a value of bS equal to 0 is the output:

–	Otherwise, if either the samples p0 or q0 is in a macroblock coded using a DISPMODE, the following applies

–	If the block edge is also a macroblock edge, a value of bS equal to 4 is the output:

–	Otherwise, a value of bS equal to 3 is the output:

        - Derivation process for the thresholds for each block edges

The specifications in 8.7.2.2 apply.

        - Filtering process for edges with bS less than 4

The specifications in 8.7.2.3 apply.

        - Filtering process for edges for bS equal to 4

The specifications in 8.7.2.4 apply.

  - Parsing process

The specifications in clause Error: Reference source not found apply. Additionally, the following modifications related to the syntax element mb_ivmp_flag are specified and added.

    - Alternative CABAC parsing process for slice data and macroblock layer in depth extension

Subclause 9.1.1specifies the initialisation process for the alternative CABAC parsing process for slice data and macroblock layer in depth extension.

Subclause  specifies the binarization process for the alternative CABAC parsing process for slice data and macroblock layer in depth extension.

Subclause 9.1.3 specifies the decoding process flow for the alternative CABAC parsing process for slice data and macroblock layer in depth extension.

      - Initialisation process

Outputs of this process are the initialised CABAC context variables indexed by ctxIdx.

Tables I-3 and I-4 contain the values of the variables n and m used in the initialisation of context variables that are assigned to syntax elements mb_ivmp_flag, mb_vsskip_flag, and mb_direct_type_flag. Table I-5 contains the values of the variables n and m used in the initialisation of context variables that are assigned to syntax element mb_disp_flag, psip4x4_dir, psip4x4_code_diff, psip8x8_dir, psip8x8_code_diff, psip16x16_dir, and psip16x16_code_diff. The initialization proces for two variables pStateIdx and valMPS is the same as other syntax elements, as defined in Equation 9-3. For all other syntax elements in subclause 7.3.5 the initialisation process of context variables as specified in subclause Error: Reference source not found applies.

In Table I-2, the ctxIdx for which initialisation is needed for I, P and B slices are listed. Also listed is the table number that includes the values of m and n needed for the initialisation.

- Table I-2 – Association of ctxIdx and syntax elements in the initialisation process

| Syntax element |  | Table |  | Slice type |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | I |  | P |  | B |
| mb_ivmp_flag | Error: Reference source not found3 |  |  |  | 1031..1033 |  | 1031..1033 |  |
| mb_vsskip_flag | Table I-4 |  |  |  | 1034..1036 |  | 1037..1039 |  |
| mb_direct_type_flag | Table I-4 |  |  |  |  |  | 1040..1042 |  |
| mb_disp_flag | Table I-5 |  | 1043..1045 |  | 1043..1045 |  | 1043..1045 |  |
| psip4x4_dir | Table I-5 |  | 1046 |  | 1046 |  | 1046 |  |
| psip4x4_code_diff | Table I-5 |  | 1047..1048 |  | 1047..1048 |  | 1047..1048 |  |
| psip8x8_dir | Table I-5 |  | 1046 |  | 1046 |  | 1046 |  |
| psip8x8_code_diff | Table I-5 |  | 1047..1048 |  | 1047..1048 |  | 1047..1048 |  |
| psip16x16_dir | Table I-5 |  | 1046 |  | 1046 |  | 1046 |  |
| psip16x16_code_diff | Table I-5 |  | 1047..1048 |  | 1047..1048 |  | 1047..1048 |  |

- Table I-3 – Values of variables m and n for ctxIdx from 1031 to 1033

| Initialisation variables | ctxIdx |  |  |
| --- | --- | --- | --- |
|  | 1031 | 1032 | 1033 |
| m | 0 | 0 | 0 |
| n | 64 | 64 | 64 |

- Table I-4 – Values of variables m and n for ctxIdx from 1034 to 1042

| Value of cabac_init_idc | Initialisation variables | ctxIdx |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 1034 | 1035 | 1036 | 1037 | 1038 | 1039 | 1040 | 1041 | 1042 |
| 0 | m | 23 | 23 | 21 | 18 | 9 | 29 | -46 | -20 | 1 |
|  | n | 33 | 2 | 0 | 64 | 43 | 0 | 127 | 104 | 67 |
| 1 | m | 22 | 34 | 16 | 26 | 19 | 40 | -45 | -15 | -4 |
|  | n | 25 | 0 | 0 | 34 | 22 | 0 | 127 | 101 | 76 |
| 2 | m | 29 | 25 | 14 | 20 | 20 | 29 | -32 | -22 | -2 |
|  | n | 16 | 0 | 0 | 40 | 10 | 0 | 127 | -117 | 74 |

- Table I-5 – Values of variables m and n for ctxIdx from 1043 to 1048

| Initialisation variables | ctxIdx |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 1043 | 1044 | 1045 | 1046 | 1047 | 1048 |
| m | 0 | 0 | 0 | 0 | 0 | 0 |
| n | 64 | 64 | 64 | 64 | 64 | 64 |

      - Binarization process

Input to this process is a request for a syntax element.

Output of this process is the binarization of the syntax element, maxBinIdxCtx, ctxIdxOffset, and bypassFlag.

Associated with each binarization or binarization part of a syntax element is a specific value of the context index offset (ctxIdxOffset) variable and a specific value of the maxBinIdxCtx variable as given in Table I-5.

The variable bypassFlag is set equal to 0.

The possible values of the context index ctxIdx are in the range 1031 to 1048, inclusive. The value assigned to ctxIdxOffset specifies the lower value of the range of ctxIdx assigned to the corresponding binarization or binarization part of a syntax element.

- Table I-5 – Syntax elements and associated types of binarization, maxBinIdxCtx, and ctxIdxOffset

| Syntax element | Type of binarization | maxBinIdxCtx | ctxIdxOffset |
| --- | --- | --- | --- |
| mb_ivmp_flag (P, B slices) | FL, cMax=1 | 0 | 1031 |
| mb_vsskip_flag (P slices only) | FL, cMax=1 | 0 | 1034 |
| mb_vsskip_flag (B slices only) | FL, cMax=1 | 0 | 1037 |
| mb_direct_type_flag (B slices only) | FL, cMax=1 | 0 | 1040 |
| mb_disp_flag (I, P, B slices) | FL, cMax=1 | 0 | 1043 |
| psip4x4_dir psip8x8_dir psip16x16_dir | FL, cMax=1 | 0 | 1046 |
| psip4x4_code_diff psip8x8_code_diff psip16x16_code_diff | as specified in subclause 9.1.2.1 | 0 | 1047 |

        - Binarization process for psip4x4_code_diff, psip8x8_code_diff, and psip16x16_code_diff

Input to this process is a request for a binarization for the syntax element psip4x4_code_diff, psip8x8_cod_diff, and psip16x16_code_diff.

Output of this process is the binarization of the syntax element.

The bin string of psip4x4_code_diff, psip8x8_cod_diff, and psip16x16_code_diff is derived by the U binarization of the mapped value of the syntax element, where the assignment rule between the signed value of psip4x4_code_diff, psip8x8_cod_diff, psip16x16_code_diff and its mapped value is given as specified in Table 9-3.

      - Decoding process flow

Input to this process is a binarization of the requested syntax element, maxBinIdxCtx, bypassFlag and ctxIdxOffset as specified in subclause Error: Reference source not found.

Output of this process is the value of the syntax element.

This process specifies how each bit of a bit string is parsed for each syntax element.

After parsing each bit, the resulting bit string is compared to all bin strings of the binarization of the syntax element and the following applies.

–	If the bit string is equal to one of the bin strings, the corresponding value of the syntax element is the output.

–	Otherwise (the bit string is not equal to one of the bin strings), the next bit is parsed.

While parsing each bin, the variable binIdx is incremented by 1 starting with binIdx being set equal to 0 for the first bin.

The parsing of each bin is specified by the following two ordered steps:

1.	Given binIdx, maxBinIdxCtx and ctxIdxOffset, ctxIdx is derived as specified in subclause  9.1.3.1.

2.	Given ctxIdx, the value of the bin from the bitstream as specified in subclause Error: Reference source not found is decoded.

        - Derivation process for ctxIdx

Inputs to this process are binIdx, maxBinIdxCtx and ctxIdxOffset.

Output of this process is ctxIdx.

Error: Reference source not found6 shows the assignment of ctxIdx increments (ctxIdxInc) to binIdx for all ctxIdxOffset values for the syntax mb_ivmp_flag.

The ctxIdx to be used with a specific binIdx is the sum of ctxIdxOffset and ctxIdxInc, which is found in Error: Reference source not found6. When more than one value is listed in Table I-6 or Error: Reference source not found for a binIdx, the assignment process for ctxIdxInc for that binIdx is further specified in the subclauses given in parenthesis of the corresponding table entry.

All entries in Table I-6 labelled with "na" correspond to values of binIdx that do not occur for the corresponding ctxIdxOffset.

- Table I-6 – Assignment of ctxIdxInc to binIdx for the ctxIdxOffset values related to the syntax elements mb_ivmp_flag, mb_vsskip_flag, mb_direct_type_flag, mb_disp_flag, psip4x4_dir, psip4x4_code_diff, psip8x8_dir, psip8x8_code_diff, psip16x16_dir, and psip16x16_code_diff

| ctxIdxOffset | binIdx |  |
| --- | --- | --- |
|  | 0 | >= 1 |
| 1031 | 0,1,2 (subclause 9.1.3.2 ) | na |
| 1034 | 0,1,2 (subclause 9.1.3.23 ) | na |
| 1037 | 0,1,2 (subclause 9.1.3.23 ) | na |
| 1040 | 0,1,2 (subclause 9.1.3.24 ) | na |
| 1043 | 0,1,2 (subclause 9.1.3.5) | na |
| 1046 | 0 | na |
| 1047 | 0 | 1 |

        - Derivation process of ctxIdxInc for the syntax element mb_ivmp_flag

Output of this process is ctxIdxInc.

The derivation process for neighbouring macroblocks specified in subclause Error: Reference source not found is invoked and the output is assigned to mbAddrA and mbAddrB.

Let the variable condTermFlagN (with N being either A or B) be derived as follows.

–	If mbAddrN is not available or mb_ivmp_flag for the macroblock mbAddrN is equal to 0, condTermFlagN is set equal to 1.

–	Otherwise (mbAddrN is available and mb_ivmp_flag for the macroblock mbAddrN is equal to 1), condTermFlagN is set equal to 0.

The variable ctxIdxInc is derived by:

ctxIdxInc = condTermFlagA + condTermFlagB		(I-9-XX)

        - Derivation process of ctxIdxInc for the syntax element mb_vsskip_flag

Output of this process is ctxIdxInc.

The derivation process for neighbouring macroblocks specified in subclause Error: Reference source not found is invoked and the output is assigned to mbAddrA and mbAddrB.

Let the variable condTermFlagN (with N being either A or B) be derived as follows.

–	If mbAddrN is not available or mb_vsskip_flag for the macroblock mbAddrN is equal to 1, condTermFlagN is set equal to 0.

–	Otherwise (mbAddrN is available and mb_vsskip_flag for the macroblock mbAddrN is equal to 0), condTermFlagN is set equal to 1.

The variable ctxIdxInc is derived by:

ctxIdxInc = condTermFlagA + condTermFlagB		(I-9-XX)

        - Derivation process of ctxIdxInc for the syntax element mb_direct_type_flag

Output of this process is ctxIdxInc.

The derivation process for neighbouring macroblocks specified in subclause Error: Reference source not found is invoked and the output is assigned to mbAddrA and mbAddrB.

Let the variable condTermFlagN (with N being either A or B) be derived as follows.

–	If any of the following conditions is true, condTermFlagN is set to 0:

–	If mbAddrN is not available,

–	Both mb_vsskip_flag and mb_direct_type_flag for the macroblock mbAddrN is equal to 0

–	Otherwise, condTermFlagN is set equal to 1.

The variable ctxIdxInc is derived by:

ctxIdxInc = condTermFlagA + condTermFlagB		(I-9-XX)

        - Derivation process of ctxIdxInc for the syntax element mb_disp_flag

Output of this process is ctxIdxInc.

The derivation process for neighbouring macroblocks specified in subclause Error: Reference source not found is invoked and the output is assigned to mbAddrA and mbAddrB.

Let the variable condTermFlagN (with N being either A or B) be derived as follows.

–	If mbAddrN is not available or mb_disp_flag for the macroblock mbAddrN is equal to 1, condTermFlagN is set equal to 0.

–	Otherwise (mbAddrN is available and mb_disp_flag for the macroblock mbAddrN is equal to 0), condTermFlagN is set equal to 1.

The variable ctxIdxInc is derived by:

ctxIdxInc = condTermFlagA + condTermFlagB		(I-9-XX)

  - Profiles and levels

The specifications in Annex I apply. Additional profiles and specific values of profile_idc are specified in the following.

The profiles that are specified in subclause J.10.1 are also referred to as the profiles specified in Annex J.

    - Profiles

All constraints for picture parameter sets that are specified in the following are constraints for picture parameter sets that become the active picture parameter set or an active view picture parameter set inside the bitstream. All constraints for 3DVC sequence parameter sets that are specified in the following are constraints for 3DVC sequence parameter sets that become the active 3DVC sequence parameter set or an active view 3DVC sequence parameter set inside the bitstream.

      - 3D Enhanced High profile

Bitstreams conforming to the 3D Enhanced High profile shall obey the following constraints:

–	The base view bitstream as specified in subclause Error: Reference source not found shall obey all constraints of the High profile specified in subclause Error: Reference source not found and all active sequence parameter sets shall fulfill one of the following conditions:

–	profile_idc is equal to 77 or constraint_set1_flag is equal to 1,

–	profile_idc is equal to 100.

–	Only I, P, and B slice types may be present.

–	NAL unit streams shall not contain nal_unit_type values in the range of 2 to 4, inclusive.

–	Arbitrary slice order is not allowed.

–	Picture parameter sets shall have num_slice_groups_minus1 equal to 0 only.

–	Picture parameter sets shall have redundant_pic_cnt_present_flag equal to 0 only.

–	When the profile_idc is equal to 139 in a sequence parameter set, the frame_mbs_only_flag in the sequence parameter set shall be equal to 1.

–	3DVC sequence parameter sets shall have chroma_format_idc equal to 0 only.

–	3DVC sequence parameter sets shall have bit_depth_luma_minus8 equal to 0 only.

–	3DVC sequence parameter sets shall have bit_depth_chroma_minus8 equal to 0 only.

–	3DVC sequence parameter sets shall have qpprime_y_zero_transform_bypass_flag equal to 0 only.

–	For each access unit, the value of level_idc for all active view 3DVC sequence parameter set RBSPs shall be the same as the value of level_idc for the active 3DVC sequence parameter set RBSP.

–	Reference picture list 0 for any coded slice shall contain at most one synthetic reference component. Reference picture list 1 for any coded slice shall contain at most one synthetic reference component that shall be the same as the synthetic reference picture included in reference picture list 0 for the same coded slice, if any. The synthetic reference component in reference picture list 0 or 1 of a coded slice of a view component shall be the same as the synthetic reference component in reference picture list 0 or 1, respectively, of any other coded slice of the same view component.

–	The level constraints specified for the 3D Enhanced High profile in subclause I.10.2 shall be fulfilled.

Conformance of a bitstream to the 3D Enhanced High profile is indicated by profile_idc being equal to 139.

Decoders conforming to the 3D Enhanced High profile at a specific level shall be capable of decoding all bitstreams in which both of the following conditions are true:

- All active 3DVC sequence parameter sets have one or more of the following conditions fulfilled:

–	profile_idc is equal to 139,

–	profile_idc is equal to 138,

–	profile_idc is equal to 128,

–	profile_idc is equal to 118 and constraint_set5_flag is equal to 1,

–	profile_idc is equal to 100,

–	profile_idc is equal to 77 or constraint_set1_flag is equal to 1.

- All active 3DVC sequence parameter sets have one or more of the following conditions fulfilled:

–	level_idc or (level_idc and constraint_set3_flag) represent a level less than or equal to the specific level,

–	level_idc[ i ] or (level_idc[ i ] and constraint_set3_flag) represent a level less than or equal to the specific level.

    - Levels

The specification of subclause I.10.2 apply.

      - Level limits common to 3D High profiles

The specification of subclause I.10.2.1 apply.

      - Profile specific level limits

- In bitstreams conforming to the 3D Enhanced High profile, 3DVC sequence parameter sets shall have frame_mbs_only_flag equal to 1 for the levels specified in Error: Reference source not found.

  - Byte stream format

The specifications in Annex Error: Reference source not found apply.

  - 3DVC hypothetical reference decoder

The specifications in Annex Error: Reference source not found apply with substituting 3DVC sequence parameter set for MVC sequence parameter set.

  - Video usability information

The specifications in Annex I.14 apply with substituting 3DVC sequence parameter set for MVC sequence parameter set. The mvc_vui_parameters_extension() contained in 3DVC sequence parameter set apply for the 3DV operation points.

--End

[Ed.Notes: Open issues]